3GPP TSG RAN WG4 Meeting #103-e	R4-2210122

E-meeting, 09 May 2022 – 20 May 2022

Agenda Item:	5.3.3

Source: 	Ericsson

Title:	Discussion on number of carriers for NR SA in rel-17

Document for:	Discussion

# Introduction

In this contribution, we provide our views on number of carriers to be supported for NR SA in rel-17.

# Discussion

In last meeting several CRs [1][2][3] are agreed for Rel-15/16 and Rel-17 for correcting the number of carriers supported by UE in DL and UL based on the CA configuration supported in 38.101-1/2/3. Principle followed for agreeing those CRs was, RRM should support at least the CA configurations supported by RF and RRM should not down scope the capability supported in RF.

Following the same principle, our understanding from the CA combinations defined in TS 38.101-2 and TS 38.101-3, RAN4 needs to update the number of serving carriers supported- for DL and UL. Existing specification is copied below for reference.

- 3.6.2.1 Number of serving carriers for SA Requirements for standalone NR with NR PCell are applicable for the UE configured with the following number of serving NR CCs: - up to 10 NR DL CCs in total, with 1 UL (or 2 UL if SUL is configured) in PCell and up to 8 UL (or 9 UL if SUL is configured) in total for SCells. - SUL may be configured together with one of the UL

As per TS 38.101-2, for DL, 16 carriers should be supported (e.g., for CA configuration of CA_n260M-n261M, etc.) and for DL 9 carriers should be supported (e.g., CA_n2A-n260M, etc. [5]).

Proposal 1: Number of serving carriers capabilities of clause 3.6.2.1 shall be revised to following

- up to 16 NR DL CCs in total, with 1 UL (or 2 UL if SUL is configured) in PCell and up to 8 UL (or 9 UL if SUL is configured) in total for SCells.

- SUL may be configured together with one of the UL

# Summary and Conclusion

In this contribution we have analysed RRM requirements of FR2 inter-band UL CA for IBM UE and made following proposals and observations.

Proposal 1: Number of serving carriers capabilities of clause 3.6.2.1 shall be revised to following

- up to 16 NR DL CCs in total, with 1 UL (or 2 UL if SUL is configured) in PCell and up to 8 UL (or 9 UL if SUL is configured) in total for SCells.

- SUL may be configured together with one of the UL.

# References

- R4-2206802

- R4-2207096

- R4-2207112

- TS 38.101-2

- TS 38.101-3