3GPP TSG-RAN WG4 Meeting#103-e	R4-22101446

Online, 9th May – 20th May, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on RedCap PDSCH/SDR Requirements

Agenda item:	9.19.5.2.1

Document for:	Discussion

- Introduction

In RAN4#102-e, simulation assumptions were agreed to define demodulation and CQI requirements for RedCap [1]. In this paper, we share our simulation results for PDSCH/SDR tests based on these assumptions.

- Simulation Results for PDSCH (FDD 15KHz FR1) Tests

Based on the agreed simulation assumptions [1], we provide the PDSCH simulation results (i.e., required SNR) below for FDD 15KHz FR1 test cases at 70% of peak rate.

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Reference from TS38.101-4 Table | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 5.2.2.1.1-3 Test 1-1 | 1 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 5.2.2.1.1-3 Test 1-4 | 9.6 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 5.2.2.1.1-4 Test 2-1 | 13.8 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 5.2.2.1.1-3 Test 1-1 | -2.5 |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 5.2.2.1.1-3 Test 1-4 | 5.6 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 5.2.2.1.1-4 Test 2-1 | 17 |

- Simulation Results for PDSCH (TDD 30KHz FR1) Tests

Based on the agreed simulation assumptions [1], we provide the PDSCH simulation results (i.e., required SNR) below for TDD 30KHz FR1 test cases at 70% of peak rate.

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Reference from TS38.101-4 Table | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 5.2.2.1.1-3 Test 1-1 | 1.2 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 5.2.2.1.1-3 Test 1-4 | 9.9 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 5.2.2.1.1-4 Test 2-1 | 14.1 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 5.2.2.1.1-3 Test 1-1 | -2.3 |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 5.2.2.1.1-3 Test 1-4 | 5.8 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 5.2.2.1.1-4 Test 2-1 | 17.4 |

- Simulation Results for Sustained Data Rate (SDR) Tests

Based on the agreed simulation assumptions [1], we provide the SDR simulation results (i.e., the maximum supported MCS from Table 1) below for both FDD 15KHz and TDD 15KHz FR1 test cases.

| 1Rx UE or 2Rx UE | Duplex/SCS | Propagation condition | MCS |
| --- | --- | --- | --- |
| 1Rx UE | FDD 15 KHz | AWGN |  |
| 1Rx UE | TDD 30 KHz | AWGN |  |

- Conclusions

In this paper, we share our simulation results for the PDSCH and SDR tests.

6. References

[1] R4-2207206, “WF on RedCap demodulation and CQI reporting requirements,” 3GPP TSG RAN WG4 Meeting #102-e, Online, Feb 21 – Mar 3, 2022.