3GPP TSG-RAN WG4 Meeting#103-e	R4-2210147

Online, 9th May – 20th May, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on RedCap PDCCH/PBCH requirements

Agenda item:	9.19.5.2.2

Document for:	Discussion

- Introduction

In RAN4#102-e, simulation assumptions were agreed to define demodulation and CQI requirements for RedCap [1]. In this paper, we share our simulation results for PDCCH/PBCH based on these assumptions.

- Simulation Results for PDCCH (FDD 15KHz FR1) Tests

Based on the agreed simulation assumptions [1], we provide the PDCCH simulation results (i.e., required SNR) below for FDD 15KHz FR1 test cases at 1% pm-dsg.

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Reference from TS38.101-4 | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 5.3.2.1.1-1 Test 3 | 12.28 |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 5.3.2.1.2-1 Test 3 |  |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 5.3.2.1.1-1 Test 3 | 4.2 |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 5.3.2.1.2-1 Test 3 | -3.08 |

- Simulation Results for PDCCH (TDD 30KHz FR1) Tests

Based on the agreed simulation assumptions [1], we provide the PDCCH simulation results (i.e., required SNR) below for TDD 15KHz FR1 test cases at 1% pm-dsg.

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Reference from TS38.101-4 | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x1 | 5.3.2.2.1-1 Test 2 | 6.57 |
| 1Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 5.3.2.2.2-1 Test 1 | 0.69 |
| 2Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x2 low | 5.3.2.2.1-1 Test 2 | 0.97 |
| 2Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 5.3.2.2.2-1 Test 1 | -3.06 |

- Simulation Results for PBCH Tests

Based on the agreed simulation assumptions [1], we provide the PBCH simulation results (i.e., the required SNR) below for both FDD 15KHz and TDD 15KHz FR1 test cases.

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 |  |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 |  |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 |  |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 |  |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 |  |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 |  |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 |  |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 |  |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 |  |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 |  |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |

- Conclusions

In this paper, we share our simulation results for the PDCCH and PBCH tests.

6. References

[1] R4-2207206, “WF on RedCap demodulation and CQI reporting requirements,” 3GPP TSG RAN WG4 Meeting #102-e, Online, Feb 21 – Mar 3, 2022.