3GPP TSG-RAN WG4 Meeting # 103-e              									     R4-2210163

Electronic Meeting, 9st of May – 20th of May, 2022

Agenda item:			9.12.3.2

Source:	THALES

Title:	Draft text proposal for Clause 7.3.3.3.2 OTA reference sensitivity - TR 38.863

Document for:	Approval

# Introduction

During RAN WG4 Meeting RAN4#102-e, a work split has been proposed to complete Section 6 and Section 7 of TR 38.863. In this document, a TP is proposed to update some of the subsections related to RF coexistence.

# Discussion

In this contribution is proposed to add information with respect to Section 7.3.3.3.2 OTA reference Sensitivity of TR 38.863 [1] by including some information related to agreements for TS 38.108.

Proposal 1: To update the TR with the attached TP

# Conclusions

In this contribution, the following proposal is made:

Proposal 1: To update the TR with the attached TP

# Reference

[1]	TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

# Text proposals for TR 38.863

### ------------------------------------------------<Start of TP>---------------------------------------------------

## 7.3.3.3.4              OTA in-band selectivity and blocking

### 7.3.3.3.4.1 				OTA adjacent channel selectivity

OTA Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1, the OTA wanted signal and the interfering signal are specified in table 7.3.3.3.4.1-1 and table 7.3.3.3.4.1-2 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1 of TS 38.108 [x]. The characteristic of the interfering signal is further specified in [annex D] of TS 38.108 [x].

The OTA ACS requirement is applicable outside the SAN RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the SAN RF Bandwidth edges or Radio Bandwidth edges.

Table 7.3.3.3.4.1-1: OTA ACS requirement for SAN type 1-O

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20 (Note 1) | EISminSENS + 6 dB | LEO: -60 – ΔminSENS GEO: -57 – ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the SAN for that bandwidth NOTE 2: EISminSENS depends on the SAN channel bandwidth |  |  |

Table 7.3.3.3.4.1-2: OTA ACS interferer frequency offset for SAN type 1-O

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz OFDM NR signal, 15 kHz SCS, 25 RBs |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |

### ------------------------------------------------<End of TP>---------------------------------------------------