3GPP TSG-RAN4 Meeting #103-e	R4-2210193

# Online, 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0464 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS38.101-2 TIB for inter-band UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add FR2 inter-band UL CA as Rel-17 feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add PC3 framework and PC1/3/5 TIB values for FR2 UL CA to agreed draft CR R4-2206600 at the RAN4#102-e. - Add PC1/3/5 MPRPA-PA values with square bracket Summary of Change in draft CR R4-2206600 is follow: - Requirement framework applied to classes 1 and 5. - The requirement framework only applies for FR2 inter-band UL CA between different frequency groups based on IBM. - PC1 framework limited to n260+n261 - PC5 framework limited to n257+n259 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 inter-band UL CA will not be standardized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.5A.3, 6.2A, 6.3A, 6.4A, 6.5A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE is configured to transmit and receive multiple contiguously aggregated carriers.

Bidirectional spectrum: UL/DL common spectrum in which the UE supports the configuration of uplink or downlink CCs.

Beam correspondence: the ability of the UE to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Cumulative aggregated channel bandwidth: The cumulative aggregated channel bandwidth is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs inside the bidirectional spectrum of the UE.

EIRP(Link=Link angle, Meas=Link angle): measurement of the UE such that the link angle is aligned with the measurement angle. EIRP (indicator to be measured) can be replaced by EIS, Frequency, EVM, carrier Leakage, In-band eission and OBW.

EIRP(Link=TX beam peak direction, Meas=Link angle): measurement of the EIRP of the UE such that the measurement angle is aligned with the beam peak direction within an acceptable measurement error uncertainty. EIRP (indicator to be measured) can be replaced by Frequency, EVM, carrier Leakage, In-band emission and OBW

EIRP(Link=Spherical coverage grid, Meas=Link  angle): measurement of the EIRP spherical coverage of the UE such that the EIRP link and measurement angles are aligned with the directions along the spherical coverage grid within an acceptable measurement error uncertainty. Alternatively, the spherical coverage grid can be replaced by the beam peak search grid as the results from the beam peak search can be re-used for spherical coverage.

EIS (effective isotropic sensitivity): sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

EIS(Link=RX beam peak direction, Meas=Link angle): measurement of the EIS of the UE such that the measurement angle is aligned with the RX beam peak direction within an acceptable measurement error uncertainty.

NOTE 1:	The sensitivity is the minimum received power level at which specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

Fallback group: Group of carrier aggregation bandwidth classes for which it is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group

IBM(Independent Beam Management): A UE that supports inter-band CA with IBM selects its DL and UL beam(s) for all CCs in each configured band based on DL reference signals measurements made in that band.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Link angle: a DL-signal AoA from the view point of the UE, as described in Annex J. If the beam lock function is used to lock the UE beam(s), the link angle can become any arbitrary AoA once the beam lock has been activated.

Measurement angle: the angle of measurement of the desired metric from the view point of the UE, as described in Annex J

radiated interface boundary: operating band specific radiated requirements reference point where the radiated requirements apply

radiated requirements reference point: for the RF measurement setup, the radiated requirements reference point is located at the centre of the quiet zone. From the UE perspective the reference point is the input of the UE antenna array

RX beam peak direction: direction where the maximum total component of RSRP and thus best total component of EIS is found

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

TX beam peak direction: direction where the maximum total component of  EIRP is found

TRP(Link=TX beam peak direction, Meas=TRP grid): measurement of the TRP of the UE such that the measurement angles are aligned with the directions of the TRP grid points within an acceptable measurement uncertainty while the link angle is aligned with the TX beam peak direction

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle

< end of changes >

< start of changes >

3.2	Symbols

For the purposes of the present document, the following symbols apply:

∆EIRPBC	The beam correspondence tolerance, where ∆EIRPBC = EIRP2 – EIRP1

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔRB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB	Allowed reference sensitivity relaxation due to support for inter-band CA operation

ΔRIB,P,n	Allowed relaxation to reference sensitivity due to support for inter-band CA operation, per band in a combination of supported bands

ΔRIB,S,n	Allowed relaxation to EIS spherical coverage due to support for inter-band CA operation, per band in a combination of supported bands

ΔTIB	Allowed relaxation to EIRP requirements due to support for inter-band CA operation

ΔTIB,P,n	Allowed relaxation to peak EIRP requirements due to support for inter-band CA operation, per band in a combination of supported bands

ΔTIB,S,n	Allowed relaxation to EIRP spherical coverage due to support for inter-band CA operation, per band in a combination of supported bands

ΔMBP,n	Allowed relaxation to each, minimum peak EIRP and reference sensitivity due to support for multi-band operation, per band in a combination of supported bands

ΔMBS,n	Allowed relaxation to each, EIRP spherical coverage and EIS spherical coverage due to support for multi-band operation, per band in a combination of supported bands

BWChannel	Channel bandwidth

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWGB	max( BWGB,Channel(k) )

BWGB,Channel(k)	Minimum guard band defined in sub-clause 5.3A.2 of carrier k

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EIRP1	The measured total EIRP based on the beam the UE chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping

EIRP2	The measured total EIRP based on the beam yielding highest EIRP in a given direction, which is based on beam correspondence with relying on UL beam sweeping

EIRPmax	The applicable maximum EIRP as specified in sub-clause 6.2.1

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

F_center	The center frequency of an allocated block of PRBs

FC	RF reference frequency for the carrier center on the channel raster, given in table 5.4.2.2-1

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block.

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block.

FC, low	The Fc of the lowest carrier, expressed in MHz.

FC, high	The Fc of the highest carrier, expressed in MHz.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge, low	The lower edge of Aggregated Channel Bandwidth, expressed in MHz. Fedge, low = FC, low - Foffset, low.

Fedge, high	The upper edge of Aggregated Channel Bandwidth, expressed in MHz. Fedge, high = FC, high + Foffset, high.

FInterferer	Frequency of the interferer

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FUL_Meas	The sub-carrier frequency for which the equalizer coefficient is evaluated

GBChannel	Minimum guard band defined in sub-clause 5.3.3

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

LCRB,Max	Maximum number of RB for a given Channel bandwidth and sub-carrier spacing

Max()	The largest of given numbers

Min()	The smallest of given numbers

MPRf,c	Maximum output power reduction for carrier f of serving cell c

MPRnarrow	Maximum output power reduction due to narrow PRB allocation

MPRWT	Maximum power reduction due to modulation orders, transmit bandwidth configurations, waveform types

nPRB	Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB,low	Transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	Transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c

Pint	The intermediate power point as defined in table 6.3.4.2-2

PInterferer	Modulated mean power of the interferer

Pmax	The maximum UE output power as specified in sub-clause 6.2.1

Pmin	The minimum UE output power as specified in sub-clause 6.3.1

P-MPRf,c	The Power Management UE Maximum Power Reduction for carrier f of serving cell c

PPowerClass	Nominal UE power class (i.e., no tolerance) as specified in sub-clause 6.2.1

PRB	The transmitted power per allocated RB, measured in dBm

PTMAX,f,c	The measured total radiated power for carrier f of serving cell c

PUMAX	The measured configured maximum UE output power

Pw	Power of a wanted DL signal

RBstart	Indicates the lowest RB index of transmitted resource blocks

SCSlow	SCS for the lowest assigned component carrier in clause 5.3A.1

SCShigh	SCS for the highest assigned component carrier in clause 5.3A.1

SSREF	SS block reference frequency position

T(∆P)	The tolerance T(∆P) for applicable values of ∆P (values in dB)

TRPmax	The maximum TRP for the UE power class as specified in sub-clause 6.2.1

< end of changes >

< start of changes >

5.5A.3	Configurations for inter-band CA

Table 5.5A.3-1: NR CA configurations for inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 | 100 | 200 | 400 |  |
| CA_n257A-n259A | CA_n257A-n259A | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | 50 | 100 | 200 | 400 |  |
| CA_n257A-n259G | CA_n257A-n259A | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259G |  |  |  |  |
| CA_n257A-n259H | CA_n257A-n259A | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259H |  |  |  |  |
| CA_n257A-n259I | CA_n257A-n259A | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259I |  |  |  |  |
| CA_n257A-n259J | CA_n257A-n259A | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259J |  |  |  |  |
| CA_n257A-n259K | CA_n257A-n259A | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259K |  |  |  |  |
| CA_n257A-n259L | CA_n257A-n259A | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259L |  |  |  |  |
| CA_n257A-n259M | CA_n257A-n259A | n257 | 50 | 100 | 200 | 400 | 0 |
|  |  | n259 | CA_n259M |  |  |  |  |
| CA_n257G-n259A | CA_n257A-n259A | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | 50 | 100 | 200 | 400 |  |
| CA_n257G-n259G | CA_n257A-n259A | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259G |  |  |  |  |
| CA_n257G-n259H | CA_n257A-n259A | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259H |  |  |  |  |
| CA_n257G-n259I | CA_n257A-n259A | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259I |  |  |  |  |
| CA_n257G-n259J | CA_n257A-n259A | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259J |  |  |  |  |
| CA_n257G-n259K | CA_n257A-n259A | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259K |  |  |  |  |
| CA_n257G-n259L | CA_n257A-n259A | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259L |  |  |  |  |
| CA_n257G-n259M | CA_n257A-n259A | n257 | CA_n257G |  |  |  | 0 |
|  |  | n259 | CA_n259M |  |  |  |  |
| CA_n257H-n259A | CA_n257A-n259A | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | 50 | 100 | 200 | 400 |  |
| CA_n257H-n259G | CA_n257A-n259A | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259G |  |  |  |  |
| CA_n257H-n259H | CA_n257A-n259A | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259H |  |  |  |  |
| CA_n257H-n259I | CA_n257A-n259A | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259I |  |  |  |  |
| CA_n257H-n259J | CA_n257A-n259A | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259J |  |  |  |  |
| CA_n257H-n259K | CA_n257A-n259A | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259K |  |  |  |  |
| CA_n257H-n259L | CA_n257A-n259A | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259L |  |  |  |  |
| CA_n257H-n259M | CA_n257A-n259A | n257 | CA_n257H |  |  |  | 0 |
|  |  | n259 | CA_n259M |  |  |  |  |
| CA_n257I-n259A | CA_n257A-n259A | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | 50 | 100 | 200 | 400 |  |
| CA_n257I-n259G | CA_n257A-n259A | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259G |  |  |  |  |
| CA_n257I-n259H | CA_n257A-n259A | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259H |  |  |  |  |
| CA_n257I-n259I | CA_n257A-n259A | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259I |  |  |  |  |
| CA_n257I-n259J | CA_n257A-n259A | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259J |  |  |  |  |
| CA_n257I-n259K | CA_n257A-n259A | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259K |  |  |  |  |
| CA_n257I-n259L | CA_n257A-n259A | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259L |  |  |  |  |
| CA_n257I-n259M | CA_n257A-n259A | n257 | CA_n257I |  |  |  | 0 |
|  |  | n259 | CA_n259M |  |  |  |  |
| CA_n258A-n260A | - | n258 | 50 | 100 | 200 | 400 | 0 |
|  |  | n260 | 50 | 100 | 200 | 400 |  |
| CA_n260A-n261A | CA_n260A-n261A CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | 50 | 100 | 200 | 400 | 0 |
|  |  | n261 | 50 | 100 | 200 | 400 |  |
| CA_n260A-n261G |  | n260 | 50 | 100 | 200 | 400 | 0 |
|  |  | n261 | CA_n261G |  |  |  |  |
| CA_n260A-n261H |  | n260 | 50 | 100 | 200 | 400 | 0 |
|  |  | n261 | CA_n261H |  |  |  |  |
| CA_n260A-n261I |  | n260 | 50 | 100 | 200 | 400 | 0 |
|  |  | n261 | CA_n261I |  |  |  |  |
| CA_n260A-n261J |  | n260 | 50 | 100 | 200 | 400 | 0 |
|  |  | n261 | CA_n261J |  |  |  |  |
| CA_n260A-n261K |  | n260 | 50 | 100 | 200 | 400 | 0 |
|  |  | n261 | CA_n261K |  |  |  |  |
| CA_n260A-n261L |  | n260 | 50 | 100 | 200 | 400 | 0 |
|  |  | n261 | CA_n261L |  |  |  |  |
| CA_n260A-n261M |  | n260 | 50 | 100 | 200 | 400 | 0 |
|  |  | n261 | CA_n261M |  |  |  |  |
| CA_n260G-n261A | CA_n260A-n261A CA_n260G CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | CA_n260G |  |  |  | 0 |
|  |  | n261 | 50 | 100 | 200 | 400 |  |
| CA_n260G-n261G |  | n260 | CA_n260G |  |  |  | 0 |
|  |  | n261 | CA_n261G |  |  |  |  |
| CA_n260G-n261H |  | n260 | CA_n260G |  |  |  | 0 |
|  |  | n261 | CA_n261H |  |  |  |  |
| CA_n260G-n261I |  | n260 | CA_n260G |  |  |  | 0 |
|  |  | n261 | CA_n261I |  |  |  |  |
| CA_n260G-n261J |  | n260 | CA_n260G |  |  |  | 0 |
|  |  | n261 | CA_n261J |  |  |  |  |
| CA_n260G-n261K |  | n260 | CA_n260G |  |  |  | 0 |
|  |  | n261 | CA_n261K |  |  |  |  |
| CA_n260G-n261L |  | n260 | CA_n260G |  |  |  | 0 |
|  |  | n261 | CA_n261L |  |  |  |  |
| CA_n260G-n261M |  | n260 | CA_n260G |  |  |  | 0 |
|  |  | n261 | CA_n261M |  |  |  |  |
| CA_n260H-n261A | CA_n260A-n261A CA_n260G CA_n260H CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | CA_n260H |  |  |  | 0 |
|  |  | n261 | 50 | 100 | 200 | 400 |  |
| CA_n260H-n261G |  | n260 | CA_n260H |  |  |  | 0 |
|  |  | n261 | CA_n261G |  |  |  |  |
| CA_n260H-n261H |  | n260 | CA_n260H |  |  |  | 0 |
|  |  | n261 | CA_n261H |  |  |  |  |
| CA_n260H-n261I |  | n260 | CA_n260H |  |  |  | 0 |
|  |  | n261 | CA_n261I |  |  |  |  |
| CA_n260H-n261J |  | n260 | CA_n260H |  |  |  | 0 |
|  |  | n261 | CA_n261J |  |  |  |  |
| CA_n260H-n261K |  | n260 | CA_n260H |  |  |  | 0 |
|  |  | n261 | CA_n261K |  |  |  |  |
| CA_n260H-n261L |  | n260 | CA_n260H |  |  |  | 0 |
|  |  | n261 | CA_n261L |  |  |  |  |
| CA_n260H-n261M |  | n260 | CA_n260H |  |  |  | 0 |
|  |  | n261 | CA_n261M |  |  |  |  |
| CA_n260I-n261A | CA_n260A-n261A CA_n260G CA_n260H CA_n260I CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | CA_n260I |  |  |  | 0 |
|  |  | n261 | 50 | 100 | 200 | 400 |  |
| CA_n260I-n261G |  | n260 | CA_n260I |  |  |  | 0 |
|  |  | n261 | CA_n261G |  |  |  |  |
| CA_n260I-n261H |  | n260 | CA_n260I |  |  |  | 0 |
|  |  | n261 | CA_n261H |  |  |  |  |
| CA_n260I-n261I |  | n260 | CA_n260I |  |  |  | 0 |
|  |  | n261 | CA_n261I |  |  |  |  |
| CA_n260I-n261J |  | n260 | CA_n260I |  |  |  | 0 |
|  |  | n261 | CA_n261J |  |  |  |  |
| CA_n260I-n261K |  | n260 | CA_n260I |  |  |  | 0 |
|  |  | n261 | CA_n261K |  |  |  |  |
| CA_n260I-n261L |  | n260 | CA_n260I |  |  |  | 0 |
|  |  | n261 | CA_n261L |  |  |  |  |
| CA_n260I-n261M |  | n260 | CA_n260I |  |  |  | 0 |
|  |  | n261 | CA_n261M |  |  |  |  |
| CA_n260J-n261A | CA_n260A-n261A CA_n260G CA_n260H CA_n260I CA_n260J CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | CA_n260J |  |  |  | 0 |
|  |  | n261 | 50 | 100 | 200 | 400 |  |
| CA_n260J-n261G |  | n260 | CA_n260J |  |  |  | 0 |
|  |  | n261 | CA_n261G |  |  |  |  |
| CA_n260J-n261H |  | n260 | CA_n260J |  |  |  | 0 |
|  |  | n261 | CA_n261H |  |  |  |  |
| CA_n260J-n261I |  | n260 | CA_n260J |  |  |  | 0 |
|  |  | n261 | CA_n261I |  |  |  |  |
| CA_n260J-n261J |  | n260 | CA_n260J |  |  |  | 0 |
|  |  | n261 | CA_n261J |  |  |  |  |
| CA_n260J-n261K |  | n260 | CA_n260J |  |  |  | 0 |
|  |  | n261 | CA_n261K |  |  |  |  |
| CA_n260J-n261L |  | n260 | CA_n260J |  |  |  | 0 |
|  |  | n261 | CA_n261L |  |  |  |  |
| CA_n260J-n261M |  | n260 | CA_n260J |  |  |  | 0 |
|  |  | n261 | CA_n261M |  |  |  |  |
| CA_n260K-n261A | CA_n260A-n261A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | CA_n260K |  |  |  | 0 |
|  |  | n261 | 50 | 100 | 200 | 400 |  |
| CA_n260K-n261G |  | n260 | CA_n260K |  |  |  | 0 |
|  |  | n261 | CA_n261G |  |  |  |  |
| CA_n260K-n261H |  | n260 | CA_n260K |  |  |  | 0 |
|  |  | n261 | CA_n261H |  |  |  |  |
| CA_n260K-n261I |  | n260 | CA_n260K |  |  |  | 0 |
|  |  | n261 | CA_n261I |  |  |  |  |
| CA_n260K-n261J |  | n260 | CA_n260K |  |  |  | 0 |
|  |  | n261 | CA_n261J |  |  |  |  |
| CA_n260K-n261K |  | n260 | CA_n260K |  |  |  | 0 |
|  |  | n261 | CA_n261K |  |  |  |  |
| CA_n260K-n261L |  | n260 | CA_n260K |  |  |  | 0 |
|  |  | n261 | CA_n261L |  |  |  |  |
| CA_n260K-n261M |  | n260 | CA_n260K |  |  |  | 0 |
|  |  | n261 | CA_n261M |  |  |  |  |
| CA_n260L-n261A | CA_n260A-n261A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | CA_n260L |  |  |  | 0 |
|  |  | n261 | 50 | 100 | 200 | 400 |  |
| CA_n260L-n261G |  | n260 | CA_n260L |  |  |  | 0 |
|  |  | n261 | CA_n261G |  |  |  |  |
| CA_n260L-n261H |  | n260 | CA_n260L |  |  |  | 0 |
|  |  | n261 | CA_n261H |  |  |  |  |
| CA_n260L-n261I |  | n260 | CA_n260L |  |  |  | 0 |
|  |  | n261 | CA_n261I |  |  |  |  |
| CA_n260L-n261J |  | n260 | CA_n260L |  |  |  | 0 |
|  |  | n261 | CA_n261J |  |  |  |  |
| CA_n260L-n261K |  | n260 | CA_n260L |  |  |  | 0 |
|  |  | n261 | CA_n261K |  |  |  |  |
| CA_n260L-n261L |  | n260 | CA_n260L |  |  |  | 0 |
|  |  | n261 | CA_n261L |  |  |  |  |
| CA_n260L-n261M |  | n260 | CA_n260L |  |  |  | 0 |
|  |  | n261 | CA_n261M |  |  |  |  |
| CA_n260M-n261A | CA_n260A-n261A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M | n260 | CA_n260M |  |  |  | 0 |
|  |  | n261 | 50 | 100 | 200 | 400 |  |
| CA_n260M-n261G |  | n260 | CA_n260M |  |  |  | 0 |
|  |  | n261 | CA_n261G |  |  |  |  |
| CA_n260M-n261H |  | n260 | CA_n260M |  |  |  | 0 |
|  |  | n261 | CA_n261H |  |  |  |  |
| CA_n260M-n261I |  | n260 | CA_n260M |  |  |  | 0 |
|  |  | n261 | CA_n261I |  |  |  |  |
| CA_n260M-n261J |  | n260 | CA_n260M |  |  |  | 0 |
|  |  | n261 | CA_n261J |  |  |  |  |
| CA_n260M-n261K |  | n260 | CA_n260M |  |  |  | 0 |
|  |  | n261 | CA_n261K |  |  |  |  |
| CA_n260M-n261L |  | n260 | CA_n260M |  |  |  | 0 |
|  |  | n261 | CA_n261L |  |  |  |  |
| CA_n260M-n261M |  | n260 | CA_n260M |  |  |  | 0 |
|  |  | n261 | CA_n261M |  |  |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |  |  |  |

< end of changes >

< start of changes >

6.2A.1	UE maximum output power for CA

For downlink intra-band contiguous and non-contiguous carrier aggregation with a single uplink component carrier configured in the NR band, the maximum output power is specified in clause 6.2.1.

For uplink intra-band contiguous and non-contiguous carrier aggregation for any CA bandwidth class, the maximum output power is specified in clause 6.2.1.

For inter-band uplink CA with two NR bands with each UL band configured with a single CC, the maximum power requirements are applicable per band, with both carriers active with non-zero UL RB allocation. The maximum output power requirements for TRP and EIRP are applicable per carrier and are specified in tables 6.2.1.x-2. The minimum peak values for EIRP are defined in Tables 6.2.1.x-1 and further relaxed by ΔTIB,P,n specified in Table 6.2A.1-3. The peak EIRP requirements are verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle). The EIRP spherical coverage requirements are determined from the percentile rank in Tables 6.2.1.x-3 and the EIRP level specified in Tables 6.2.1.x-3 is reduced by the parameter ΔTIB,S,n specified in Table 6.2A.1-3. The spherical coverage EIRP requirements are verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

The inter-band CA spherical coverage requirement for each power class is met if the intersection set of spherical coverage areas exceeds the common coverage requirement. Intersection set of spherical coverage areas is defined as a fraction of area of full sphere measured around the UE where both bands meet their defined individual EIRP spherical coverage requirements for inter-band CA operation. The common coverage requirement is determined as <100-percentile rank> %, where ‘percentile rank’ is the percentile value in the specification of spherical coverage for that power class from sub-clause 6.2.1.

The ΔTIB parameters are specified in tables 6.2A.1-x.

Table 6.2A.1-1:  ΔTIB EIRP relaxations for inter-band UL CA for power class 1

| NR CA configuration | NR band | ΔTIB,P,n (dB) | ΔTIB,S,n (dB) |
| --- | --- | --- | --- |
| CA_n260A-n261A | n260 | 0.0 | 0.5 |
|  | n261 | 0.0 | 0.5 |

Table 6.2A.1-2:  reserved for future use

Table 6.2A.1-3:  ΔTIB EIRP relaxations for inter-band UL CA for power class 3

| NR CA configuration | NR band | ΔTIB,P,n (dB) | ΔTIB,S,n (dB) |
| --- | --- | --- | --- |
| CA_n257A-n259A | n257 | 5.01 | 5.51 |
|  | n259 | 5.02 | 5.52 |
| CA_n260A-n261A | n260 | 5.03 | 5.53 |
|  | n261 | 5.04 | 5.54 |
| NOTE 1: n257 peak relaxation is 0.7 dB and n257 spherical relaxations is 1.2 dB when MPRC_CA in Table 6.2A.2.4-1 or MPRNC_CA in Table 6.2A.2.4.2-1 is applied for n257. NOTE 2: n259 peak relaxation is 0.5 dB and n259 spherical relaxations is 1.0 dB when MPRC_CA in Table 6.2A.2.4-1 or MPRNC_CA in Table 6.2A.2.4.2-1 is applied for n259. NOTE 3: n260 peak relaxation is 0.0 dB and n260 spherical relaxations is 0.5 dB when MPRC_CA in Table 6.2A.2.4-1 or MPRNC_CA in Table 6.2A.2.4.2-1 is applied for n260. NOTE 4: n261 peak relaxation is 0.0 dB and n261 spherical relaxations is 0.5 dB when MPRC_CA in Table 6.2A.2.4-1 or MPRNC_CA in Table 6.2A.2.4.2-1 is applied for n261. |  |  |  |

Table 6.2A.1-4:  reserved for future use

Table 6.2A.1-5:  ΔTIB EIRP relaxations for inter-band UL CA for power class 5

| NR CA configuration | NR band | ΔTIB,P,n (dB) | ΔTIB,S,n (dB) |
| --- | --- | --- | --- |
| CA_n257A-n259A | n257 | 0.7 | 1.2 |
|  | n259 | 0.5 | 1.0 |

Power class 3 is default power class

< end of changes >

< start of changes >

6.2A.2.1	General

The UE is defined to be configured for CA operation when it has at least one of UL or DL configured for CA. In CA operation, the UE may reduce its maximum output power due to higher order modulations and transmit bandwidth configurations. This Maximum Power Reduction (MPR) is defined in clauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2, shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

When the maximum output power of a UE is modified by MPR, the power limits specified in clause 6.2A.4 apply. The requirements in the following clauses are applicable to the following CA configurations:

- intra-band contiguous uplink CA, with the aggregated channel bandwidth no greater than 800 MHz.

- intra-band non-contiguous uplink CA with UL frequency separation no greater than 1400 MHz, and no more than 3 sub-blocks. A sub-block may consist of single CC or multiple contiguous CCs.

- inter-band uplink CA with two NR bands, and each UL band is configured with a single CC.

- In case the CA configuration consists of a single UL CC, MPR for contiguous UL CA applies and where necessary, BWchannel shall be used as BWchannel_CA.

< end of changes >

< start of changes >

6.2A.2.2.3	Maximum output power reduction for power class 1 inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, MPR for each configured UL band is:

MPRinter-band_CA = max(MPRsingleBand, MPRPA-PA)

Where:

MPRsingleBand is the MPR specified in clause 6.2.2.1

MPRPA-PA is specified in Table 6.2A.2.1-1 and applies only when both bands have non-zero UL RB allocations, 0 dB otherwise.

Table 6.2A.2.1-1: MPRPA-PA for Inter-band CA in FR2 for PC1

| NR CA Band | MPR PA-PA (dB) |
| --- | --- |
| CA_n260A-n261A | [max (0, 6 - 10*log10(PRB))] |

[Where:

PRB is the number non-zero power UL RBs in the band with the narrower allocation.]

< end of changes >

< start of changes >

6.2A.2.4.3	Maximum output power reduction for power class 3 inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, MPR for each configured UL band is:

MPRinter-band_CA = max(MPRsingleBand, MPRPA-PA)

Where:

MPRsingleBand is the MPR specified in clause 6.2.2.3, clause 6.2A.2.4.1, and clause 6.2A.2.4.2.

MPRPA-PA is specified in Table 6.2A.2.4-1 and applies only when both bands have non-zero UL RB allocations, 0 dB otherwise.

Table 6.2A.2.4-1: MPRPA-PA for Inter-band CA in FR2 for PC3

| NR CA Band | MPR PA-PA (dB) |
| --- | --- |
| CA_n257A-n259A | [max (0, 6 - 10*log10(PRB))] |
| CA_n260A-n261A |  |

[Where:

PRB is the number non-zero power UL RBs in the band with the narrower allocation.]

< end of changes >

< start of changes >

6.2A.2.6	Maximum output power reduction for power class 5

For power class 5, MPR specified in sub-clause 6.2A.2.4.1 applies for intra-band contiguous UL CA and sub-clause 6.2A.2.4.2 applies for intra-band non-contiguous UL CA.

For inter-band carrier aggregation with uplink assigned to two NR bands, MPR for each configured UL band is:

MPRinter-band_CA = max(MPRsingleBand, MPRPA-PA)

Where:

MPRsingleBand is the MPR specified in clause 6.2.2.5

MPRPA-PA is specified in Table 6.2A.2.5-1 and applies only when both bands have non-zero UL RB allocations, 0 dB otherwise.

Table 6.2A.2.5-1: MPRPA-PA for Inter-band CA in FR2 for PC5

| NR CA Band | MPR PA-PA (dB) |
| --- | --- |
| CA_n257A-n259A | [max (0, 6 - 10*log10(PRB))] |

[Where:

PRB is the number non-zero power UL RBs in the band with the narrower allocation.]

< end of changes >

< start of changes >

6.3A.1.1	Minimum output power for power class 1

For intra-band contiguous and non-contiguous carrier aggregation, the minimum output power shall not exceed the values specified in Table 6.3A.1.1-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3A.1.1-1: Minimum output power for power class 1

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261, n262 | 50 | 4 | 47.58 |
|  | 100 | 4 | 95.16 |
|  | 200 | 4 | 190.20 |
|  | 400 | 4 | 380.28 |

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the minimum output power is defined per carrier and is specified in clause 6.3.1.1.

6.3A.1.2	Minimum output power for power class 2, 3, and 4

For intra-band contiguous and non-contiguous carrier aggregation, the minimum output power shall not exceed the values specified in Table 6.3A.1.2-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3A.1.2-1: Minimum output power for CA for power class 2, 3, and 4

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n259, n260, n261, n262 | 50 | -13 | 47.58 |
|  | 100 | -13 | 95.16 |
|  | 200 | -13 | 190.20 |
|  | 400 | -13 | 380.28 |
| NOTE 1: n260 is not applied for power class 2. NOTE 2: n259 is not applied for power class 2 and 4. |  |  |  |

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the minimum output power is defined per carrier and is specified in clause 6.3.1.2.

6.3A.1.3	Minimum output power for power class 5

For intra-band contiguous and non-contiguous carrier aggregation, the minimum output power shall not exceed the values specified in Table 6.3A.1.3-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3A.1.2-1: Minimum output power for CA for power class 5

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n259 | 50 | -6 | 47.52 |
|  | 100 | -6 | 95.04 |
|  | 200 | -6 | 190.08 |
|  | 400 | -6 | 380.16 |

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the minimum output power is defined per carrier and is specified in clause 6.3.1.3.

< end of changes >

< start of changes >

6.2A.4	Configured transmitted power for CA

Table 6.2A.4-1: Void

6.2A.4.1	Configured transmitted power for intra-band UL CA

A UE configured with carrier aggregation can configure its maximum output power for each uplink  activated serving cell c and its total configured maximum output power PCMAX. The definition of the configured UE maximum output power PCMAX,f,c for each carrier f of a serving cell c is used for power headroom reporting for carrier f of serving cell c only and is in accordance with that specified in clause 6.2.4 with parameters MPR, A-MPR and P-MPR replaced with those specified in subclause 6.2A.2, 6.2A.3 and 6.2.4, respectively. The UE maximum configured power PCMAX in a transmission occasion is determined by the UL grants for carriers f of all serving cells c with non-zero granted power in the respective reference point.

For uplink intra-band contiguous carrier aggregation, MPR is specified in clause 6.2A.2. PCMAX is calculated under the assumption that power spectral density for each RB in each component carrier is same.

The configured UE maximum output power PCMAX shall be set such that the corresponding measured total peak EIRP PUMAX is within the following bounds

PPowerclass – MAX(MAX(MPR, A-MPR)  + ΔMBP,n, P-MPR) – MAX{T(MAX(MPR, A-MPR)),T(P-MPR)} ≤ PUMAX ≤ EIRPmax

with PPowerclass the peak EIRP  as specified in sub-clause 6.2A.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2A.1, MPR as specified in sub-clause 6.2A.2, A-MPR as specified in sub-clause 6.2A.3, ΔMBP,n the peak EIRP relaxation as specified in clause 6.2.1, P-MPR the power management term for the UE as described in 6.2.4.

The measured configured power PUMAX for carrier aggregation is defined as

PUMAX=10log10c,f(c)pUMAX,f,c

where  pUMAX,f,c is the linear value of the measured power PUMAX,f,c for carrier f=f(c) of serving cell c. The measured total radiated power PTMAX for carrier aggregation is defined as

PTMAX=10log10c,f(c)pTMAX,f,c

where pTMAX,f,c is the linear value of the measured total radiated power PTMAX,f,c for carrier f = f(c) of serving cell c. The total radiated power PTMAX is bounded by

PTMAX ≤ TRPmax

where TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2A.1.

The tolerance T(ΔP) for applicable values of ΔP (values in dB) is specified in Table 6.2A.4.1-1.

Table 6.2A.4.1-1: PUMAX tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n259, n260, n261, n262 | P = 0 | 0 |
|  | 0 < P ≤ 2 | 1.5 |
|  | 2 < P ≤ 3 | 2.0 |
|  | 3 < P ≤ 4 | 3.0 |
|  | 4 < P ≤ 5 | 4.0 |
|  | 5 < P ≤ 10 | 5.0 |
|  | 10 < P ≤ 15 | 7.0 |
|  | 15 < P ≤ X | 8.0 |
| NOTE: X is the value such that Pumax lower bound, PPowerclass - P – T(P) = minimum output power specified in clause 6.3A.1 |  |  |

6.2A.4.2	Configured transmitted power for inter-band UL CA

[A UE can configure its maximum output power for each uplink band when it is configured for inter-band UL carrier aggregation with two NR bands each with a single UL CC. For each uplink band n, the configured UE maximum output power PCMAX,f,c,n for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11].

The configured UE maximum output power PCMAX,f,c,n for carrier f of a serving cell c in band n shall be set such that the corresponding measured peak EIRP PUMAX,f,c,n is within the following bounds

PPowerclass + PIBE – MAX(MAX(MPRf,c,n, A- MPRf,c,n) + ΔTIBP,n, P-MPRf,c,n) – MAX{T(MAX(MPRf,c,n, A- MPRf,c,n,)), T(P-MPRf,c,n)} ≤ PUMAX,f,c,n ≤ EIRPmax,n

while the corresponding measured total radiated power in uplink band n, PTMAX,f,c,n , is bounded by

PTMAX,f,c,n ≤ TRPmax,n

with PPowerclass the UE power class as specified in sub-clause 6.2.1, EIRPmax,n the applicable maximum EIRP as specified in sub-clause 6.2A.1 for uplink band n and TRPmax,n the applicable maximum TRP as specified in sub-clause 6.2A.1 for uplink band n. MPRf,c,n as specified in sub-clause 6.2A.2 , A-MPRf,c,n as specified in sub-clause 6.2A.3, ΔTIBP,n the peak EIRP relaxation as specified in clause 6.2A.1 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1. The requirement is verified in beam peak direction.

PIBE, mpr-PowerBoost-FR2-r16 and maxUplinkDutyCycle-FR2 are described in clause 6.2.4.

P-MPRf,c,n is the power management maximum output power reduction P-MPRf,c in band n. P-MPRf,c is defined in clause 6.2.4.

The tolerance T(∆P) for applicable values of ∆P (values in dB) in each band is specified in Table 6.2.4-1.]

< end of changes >

< start of changes >

6.3A.2	Transmit OFF power for CA

For intra-band contiguous and non-contiguous carrier aggregation, the transmit OFF power is defined as the TRP in the channel bandwidth per component carrier when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of it sports.

The transmit OFF power shall not exceed the values specified in Table 6.3A.2-1 for each operating band supported.

Table 6.3A.2-1: Transmit OFF power for CA

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257, n258, n259, n260, n261, n262 | -35 | -35 | -35 | -35 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the transmit OFF power specified in clause 6.3.2.1 is applicable for each CC when the transmitter is OFF on all CCs. The transmitter is considered to be OFF when the UE is not allowed to transmit on any of its ports.

< end of changes >

< start of changes >

6.3A.3	Transmit ON/OFF time mask for CA

For intra-band contiguous and non-contiguous UL carrier aggregation, the general output power ON/OFF time mask specified in clause 6.3.3.2 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.2 shall only be applicable for each component carrier when all the component carriers are OFF.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single  CC, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each CC during the ON power period and the transient periods. The OFF period is specified in clause 6.3.3.1 for each CC separately when all the CCs are OFF.

< end of changes >

< start of changes >

6.4A	Transmit signal quality for CA

6.4A.0	General

For intra-band contiguous and non-contiguous carrier aggregation, the requirements in this clause apply if the UE has at least one of UL or DL configured for CA.

6.4A.1	Frequency error

The requirements in this clause apply to UEs of all power classes.

For intra-band contiguous and non-contiguous carrier aggregation, the UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequencies per band shall be accurate to within ± 0.1 PPM observed over a period of 1ms of cumulated measurement intervals compared to the carrier frequency of primary component carrier received from the gNB.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the frequency error requirement is specified in clause 6.4.1 and is applicable for each CC with all CCs active with non-zero UL RB allocation.

The frequency error is defined as a directional requirement. The requirement is verified in beam locked mode on beam peak direction.

6.4A.2	Transmit modulation quality

6.4A.2.0	General

For intra-band contiguous and non-contiguous carrier aggregation, the requirements in clauses 6.4A.2.1, 6.4A.2.2, and 6.4A.2.3.

All the parameters defined in clause 6.4A.2 are defined using the measurement methodology specified in Annex F.

All the requirements in 6.4A.2 are defined as directional requirement. The requirements are verified in beam locked mode on beam peak direction.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the transmit modulation quality requirements are specified in clause 6.4.2 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

6.4A.2.1	Error Vector magnitude

The requirements in this clause apply to UEs of all power classes. For intra-band contiguous and non-contiguous carrier aggregation, the Error Vector Magnitude requirement of clause 6.4.2.2 is defined for each component carrier. Requirements only apply with PRB allocation in one of the component carriers. Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform.

In case the parameter 3300 or 3301 is reported from UE via the parameter txDirectCurrentLocation in UplinkTxDirectCurrenList IE (as defined in TS 38.331 [13]),  carrier leakage measurement requirement in clause 6.4A.2.2 and 6.4A.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

For intra-band contiguous and non-contiguous carrier aggregation, the UE is defined to be configured for CA operation when it has at least one of UL or DL configured for CA.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the Error Vector Magnitude requirements are specified in clause 6.4.2.1 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

6.4A.2.2	Carrier leakage

6.4A.2.2.1	General

Carrier leakage is an additive sinusoid waveform. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

Note:	When UE has DL configured for intra-band non-contiguous CA, carrier leakage may land outside the spectrum occupied by all configured UL and DL CC.

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The requirement is verified with the test metric of Carrier Leakage (Link=TX beam peak direction, Meas=Link angle).

6.4A.2.2.2	Carrier leakage for power class 1

For intra-band contiguous and non-contiguous carrier aggregation, when carrier leakage is contained inside the spectrum occupied by all configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4A.2.2.2-1 for power class 1 UEs.

Table 6.4A.2.2.2-1: Minimum requirements for relative carrier leakage for power class 1

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 17 dBm | -25 |
| 4 dBm ≤ EIRP ≤ 17 dBm | -20 |

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the carrier leakage requirements are specified in clause 6.4.2.2.2 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

6.4A.2.2.3	Carrier leakage for power class 2

For intra-band contiguous and non-contiguous carrier aggregation, when carrier leakage is contained inside the spectrum occupied by all configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4A.2.2.3-1 for power class 2.

Table 6.4A.2.2.3-1: Minimum requirements for relative carrier leakage power class 2

| Parameters | Relative limit (dBc) |
| --- | --- |
| EIRP > 6 dBm | -25 |
| -13 dBm ≤ EIRP ≤ 6 dBm | -20 |

6.4A.2.2.4	Carrier leakage for power class 3

For intra-band contiguous and non-contiguous carrier aggregation, when carrier leakage is contained inside the spectrum occupied by all configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4A.2.2.4-1 for power class 3 UEs.

Table 6.4A.2.2.4-1: Minimum requirements for relative carrier leakage power class 3

| Parameters | Relative limit (dBc) |
| --- | --- |
| Output power > 0 dBm | -25 |
| -13 dBm ≤ Output power EIRP ≤ 0 dBm | -20 |

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the carrier leakage requirements are specified in clause 6.4.2.2.4 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

6.4A.2.2.5	Carrier leakage for power class 4

For intra-band contiguous and non-contiguous carrier aggregation, when carrier leakage is contained inside the spectrum occupied by all configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4A.2.2.5-1 for power class 4 UEs.

Table 6.4A.2.2.5-1: Minimum requirements for relative carrier leakage power class 4

| Parameters | Relative limit (dBc) |
| --- | --- |
| Output power > 11 dBm | -25 |
| -13 dBm ≤ Output power EIRP ≤ 11 dBm | -20 |

6.4A.2.2.6	Carrier leakage for power class 5

For intra-band contiguous and non-contiguous carrier aggregation, when carrier leakage is contained inside the spectrum occupied by all configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4A.2.2.6-1 for power class 5 UEs.

Table 6.4A.2.2.6-1: Minimum requirements for relative carrier leakage power class 5

| Parameters | Relative limit (dBc) |
| --- | --- |
| Output power > 7 dBm | -25 |
| -6 dBm ≤ Output power EIRP ≤ 7 dBm | -20 |

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the carrier leakage requirements are specified in clause 6.4.2.2.6 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

6.4A.2.3	Inband emissions

6.4A.2.3.1	General

For intra-band contiguous and non-contiguous carrier aggregation, the Inband emission requirement is defined over the spectrum occupied by all configured UL and DL CCs. The measurement interval is as defined in clause 6.4.2.4. The requirement is verified with the test metric of In-band emission (Link=TX beam peak direction, Meas=Link angle).

For intra-band contiguous and non-contiguous carrier aggregation, the requirements in this clause apply with all component carriers active and with one single contiguous PRB allocation in one of uplink component carriers. The inband emission is defined as the interference falling into the non-allocated resource blocks for all component carriers.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the in-band emissions requirements are applicable for each CC with all CCs active with non-zero UL RB allocation.

6.4A.2.3.2	Inband emissions for power class 1

For intra-band contiguous and non-contiguous carrier aggregation, the average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.2-1 for power class 1 UEs.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the in-band emissions requirements are specified in clause 6.4.2.3.2 and are applicable for each CC with all CCs active with non-zero UL RB allocation, for power class 1 UEs.

Table 6.4A.2.3.2-1: Requirements for in-band emissionsfor power class 1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 | Output power > 27 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 27 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 17 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | 4 dBm ≤ Output power ≤ 17 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

6.4A.2.3.3	Inband emissions for power class 2

For intra-band contiguous and non-contiguous carrier aggregation, the average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.3-1 for power class 2.

Table 6.4A.2.3.3-1: Requirements for in-band emissions for power class 2

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 | Output power > 16 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 16 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 6 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 6 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

6.4A.2.3.4	Inband emissions for power class 3

For intra-band contiguous and non-contiguous carrier aggregation, the average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.4-1 for power class 3 UEs.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the in-band emissions requirements are specified in clause 6.4.2.3.4 and are applicable for each CC with all CCs active with non-zero UL RB allocation, for power class 1 UEs.

Table 6.4A.2.3.4-1: Requirements for in-band emissions for power class 3

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 | Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

6.4A.2.3.5	Inband emissions for power class 4

For intra-band contiguous and non-contiguous carrier aggregation, the average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.5-1 for power class 4 UEs.

Table 6.4A.2.3.5-1: Requirements for in-band emissions for power class 4

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 | Output power > 21 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 21 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 11 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 11 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

6.4A.2.3.6	Inband emissions for power class 5

For intra-band contiguous and non-contiguous carrier aggregation, the average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4A.2.3.6-1 for power class 6 UEs.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the in-band emissions requirements are specified in clause 6.4.2.3.6 and are applicable for each CC with all CCs active with non-zero UL RB allocation, for power class 5 UEs.

Table 6.4A.2.3.6-1: Requirements for in-band emissions for power class 5

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated RB in allocated component carrier and not allocated component carriers (NOTE 2) |
| IQ Image | dB | -25 | Output power > 17 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 17 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 7 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -6 dBm ≤ Output power ≤ 7 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (- 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: Image frequencies for UL CA are specified in relation to either UL or DL carrier frequency. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency, or in the two RBs immediately adjacent to the DC frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth for kth allocated component carrier (see Figure 5.3.3-1). NOTE 7: EVM s the limit for the modulation format used in the allocated RBs. NOTE 8: ∆RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth), and may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 9: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 10: All powers are EIRP in beam peak direction. |  |  |  |  |

6.4A.2.4	EVM equalizer spectrum flatness

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the EVM equalizer spectrum flatness requirements are specified in clause 6.4.2.4 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

< end of changes >

< start of changes >

6.5A.1.0	General

The occupied bandwidth for UL CA is defined as a directional requirement. The requirement is verified in beam locked mode on beam peak direction. In case the intra-band CA configuration consists of a single UL CC, the occupied bandwidth requirement defined in subclause 6.5.1 applies.

< end of changes >

< start of changes >

6.5A.1.3	Occupied bandwidth for inter-band UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the occupied bandwidth requirements is specified in clause 6.5.1 and is applicable for each CC with all CCs active with non-zero UL RB allocation.

< end of changes >

< start of changes >

6.5A.2.1.0	General

For intra-band CA, the requirements specified in this clause shall apply if the UE has at least one of UL or DL configured for CA or if the UE is configured for single CC operation with different channel bandwidths in UL and DL carriers. In case the CA configuration consists of a single UL CC, spectrum emission mask defined in subclause 6.5.2.1 applies. Spectral emission mask requirements do not apply at any frequency where IBE requirements of clause 6.4A.2.3 apply.

The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction).

< end of changes >

< start of changes >

6.5A.2.1.3	Spectrum emission mask for inter-band UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the spectrum emission mask is specified in Table 6.5.2.1-1 for each CC separately while both CCs are active with non-zero UL RB allocation. If for some frequency spectrum emission masks of CCs overlap, then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency a CC spectrum emission mask overlaps with the channel bandwidth of another CC, then the emission mask does not apply for that frequency.

< end of changes >

< start of changes >

6.5A.2.3.3	Adjacent channel leakage ratio for CA inter-band UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the NR Adjacent Channel Leakage power Ratio (NRACLR) is applicable for each CC while both CCs are active with non-zero UL RB allocation and the requirement is specified in clause 6.5.2.3.

< end of changes >

< start of changes >

6.5A.3	Spurious emissions for CA

6.5A.3.0	General spurious emissions for CA

6.5A.3.0.0	General

This clause specifies the spurious emission requirements for carrier aggregation. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction).

For intra-band CA, in case the CA configuration consists of a single UL CC, spurious emissions requirements defined in subclause 6.5.3 apply. Spurious emissions requirements do not apply at any frequency where IBE requirements of clause 6.4A.2.3 apply.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5A.3.0.1	Spurious emissions for intra-band contiguous UL CA

For intra-band contiguous UL carrier aggregation, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) from the edge of the UL aggregated channel bandwidth, where FOOB is defined as the twice the UL aggregated channel bandwidth. For frequencies ΔfOOB greater than FOOB, the spurious emission requirements in Table 6.5.3-2 are applicable.

6.5A.3.0.2	Spurious emissions for intra-band non-contiguous UL CA

For intra-band non-contiguous UL carrier aggregation, the spurious emission requirement is defined as a composite spurious emission requirement which is a combination of individual spurious emission requirements defined for each UL sub-block. The limits in Table 6.5.3-2 apply for the frequency ranges that are more than FOOB (MHz) from the edge of each UL sub-block but excludes frequency ranges that coincide with another UL sub-block. No spurious emission limit applies in the gap between neighbouring UL sub-blocks if the frequency span between the lowest edge of the upper sub-block and the highest edge of the lower sub-block is smaller than FOOB_L + FOOB_H.

6.5A.3.0.3	Spurious emissions for inter-band UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the spurious emission requirements are specified in clause 6.5.3 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

6.5A.3.1	Spurious emission band UE co-existence for UL CA

This clause specifies the requirements for the specified contiguous or non-contiguous ULcarrier aggregation configurations for coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

For intra-band contiguous, non-contiguous carrier aggregation and inter-band carrier aggregation, the requirements in Table 6.5A.3-1 apply.

Table 6.5A.3.1-1: Requirements for CA

| CA operating band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band / frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 | 2 |
| CA_n258 | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n262 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n257_n259 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 |  |
|  | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: The protection of frequency range 23600-24000 MHz is meant for protection of satellite passive services. |  |  |  |  |  |  |  |

< end of changes >

< start of changes >

6.6A	Beam correspondence for CA

For intra-band CA in FR2, the same beam correspondence relationship for beam management is supported across CCs in Rel-15 and no requirement is specified. Beam correspondence performance for intra-band CA is fulfilled if the beam correspondence requirements defined in clause 6.6 is met for non-CA case.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, with all CCs active with non-zero UL RB allocation, the following beam correspondence requirements apply for each CC:

- The minimum peak EIRP requirement specified for UL inter-band CA in 6.2A, based on test conditions in clause 6.6. Reference signal power adjustments by MBS,n are replaced by ΔRIB,S,n, where referenced.

- The common spherical coverage requirement specified for UL inter-band CA in 6.2A, based on test conditions in clause 6.6. Reference signal power adjustments by MBS,n are replaced by ΔRIB,S,n, where referenced.

< end of changes >