3GPP TSG-RAN WG4 Meeting #103-e	R4-2210220

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.19.3.1.2

Source: 	Qualcomm Incorporated

Title: 	Mobility requirements for RedCap UEs

Document for:	Discussion

# Introduction

In RAN#90e meeting, a Rel-17 work item for support of reduced capability NR devices was approved and the WID was further updated in RAN#92e [1]. Two key objectives of the work-item are to provide support for 1 Rx UEs and UE bandwidth reduction.

We provided the simulation results for cell detection and SSB index identification in [2] and presented our views on the impact of 1Rx on cell detection and PBCH detection periods. Based on the simulation results, we provide our views on the mobility requirements for 1Rx UEs in this contribution.

We will also discuss some issues related to Redcap specific initial BWPs which were introduced to support reduced BW operation.

# Handover delay

Based on our simulation results presented in [2], we need two additional samples for cell detection. We therefore propose to increase the handover delay by 2 samples.

Observation 1: For cell detection, 2 additional SSB samples are needed for 1 Rx UEs

Proposal 1: For Handover delay requirements, specify Tsearch as:

- If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 3*Trs ms.

- If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 5* Trs ms

# Handover with Redcap specific initial BWP

A RedCap UE may be configured with an additional Redcap specific initial BWPs for random access procedures. Such BWPs may not be configured with CD-SSBs. This means that the CD-SSB would be configured in the non-RedCap specific initial BWP, while the ROs will be configured in Redcap specific BWPs.

In this case, during handover procedure, the UE would first perform search and measurements on the non-Redcap specific initial BWP (which contains the CD-SSB) and then performs a BWP switch to Redcap specific BWP (which contains the ROs) to transmit RACH. So an additional BWP switching delay is needed as compared to R15/16 requirements to handle this scenario.

Observation 2: A Redcap UE needs to perform a BWP switch from non-Redcap specific initial BWP (which contains CD-SSBs) to Redcap-specific initial BWP (which contains ROs) to transmit RACH when the Redcap-specific initial BWP is configured for RA only.

Proposal 2: When the Redcap specific initial DL BWP is configured for RA only, and there is no SSB available in the redcap specific initial BWP, define the interruption time as

  - Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin + TBWP-switching-delay ms

  - Where, TBWP-switching-delay is the BWP switching delay

Note: The same extension (TBWP-switching-delay) will apply to RRC re-establishment delay and RRC connection release with re-direction delay as well

# HD-FDD impact

In the last RAN4 meeting it was agreed that [3]:

The UE shall meet HO requirements provided the following is met:

- SSB is available at the UE once every SMTC period during Tsearch.

- One SSB is available during T∆

- One SSB is available during Tiu

Here, the term “available at the UE” is not very clear and needs some clarification. The SSB may be available at the UE, but if it collides with the UL transmission can it still be considered as available? We think “available at the UE in the DL slot/symbol” makes more sense to avoid any ambiguity.

Observation 3: The term “available at the UE” is not very clear and may cause some ambiguity.

Proposal 3: The UE shall meet HO requirements provided the following is met:

- SSB is available in the downlink slot/symbols at the UE once every SMTC period during Tsearch.

- One SSB is available in the downlink slot/symbols during T∆

- One SSB is available in the downlink slot/symbols and does not overlap with corresponding RACH occasion during Tiu

# Conclusion

Observation 1: For cell detection, 2 additional SSB samples are needed for 1 Rx UEs

Proposal 1: For Handover delay requirements, specify Tsearch as:

- If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 3*Trs ms.

- If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 5* Trs ms

Observation 2: A Redcap UE needs to perform a BWP switch from non-Redcap specific initial BWP (which contains CD-SSBs) to Redcap-specific initial BWP (which contains ROs) to transmit RACH when the Redcap-specific initial BWP is configured for RA only.

Proposal 2: When the Redcap specific initial DL BWP is configured for RA only, and there is no SSB available in the redcap specific initial BWP, define the interruption time as

  - Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin + TBWP-switching-delay ms

  - Where, TBWP-switching-delay is the BWP switching delay

Note: The same extension (TBWP-switching-delay) will apply to RRC re-establishment delay and RRC connection release with re-direction delay as well

Observation 3: The term “available at the UE” is not very clear and may cause some ambiguity.

Proposal 3: The UE shall meet HO requirements provided the following is met:

- SSB is available in the downlink slot/symbols at the UE once every SMTC period during Tsearch.

- One SSB is available in the downlink slot/symbols during T∆

- One SSB is available in the downlink slot/symbols and does not overlap with corresponding RACH occasion during Tiu

# References

[1] RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson, RAN#92e

[2] R4-2119561, “UE complexity reduction impact on measurement procedures”, Qualcomm, RAN4#101e

[3] R4-2206950, “WF on RedCap RRM requirements”, Ericsson, RAN4#102e