3GPP TSG-RAN WG4 Meeting # 103-e												       R4-2210314

Electronic Meeting, 09 – 20 May 2022

Agenda item:			9.12.3.

Source:	Moderator (CATT)

Title:	Email discussion summary for [103-e][308] NTN_Solutions_SANRF

Document for:	Information

Introduction

This E-mail discussion will focus on the remaining open issue for SAN type 1-H and 1-O. the corresponding TPs for SAN RF will also be reviewed under this E-mail thread.

The following agenda items are covered.

    - Satellite Access Node RF requirements

      - TX requirements for radiated characteristics

      - RX requirements for radiated characteristics

      - Tx requirements for conducted characteristics

      - Rx requirements for conducted characteristics

The aim to complete all the remaining issue and have all pending TP for 38.108 approved in this meeting.

Topic #1: Remaining issue for SAN

Companies’ contributions summary

| T-doc |  | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2208247 | Open issue on conducted requirement for SAN | CATT | withdrawn |
| R4-2208248 | Open issue on radiated requirement for SAN | CATT | withdrawn |
| R4-2208883 | NTN - SAN TX radiated requirements: remaining issues | Ericsson | Proposal1: The SAN spurious requirement should be specified according to the following Table 1: Spurious frequency range; Prated,c,TRP(W); Basic limit (dBm); Measurement bandwidth. 30 MHz – 12.75 GHz; ≤ 50 W; -13; 4 kHz. > 50 W; 10 Log(Prated,c,TRP(W)) – 30. Table 1: SAN spurious requirement Proposal2: The following OBUE masks should be specified for SAN GEO and LEO classes according to the following Table 3 and Table 4: Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; 7dBm-75foffsetMHz-0.002 dBm; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); 0 dBm; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Aligned with SAN spurious limit; 4kHz. Table 3: SAN GEO Class OBUE basic limits Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; -7dBm-75foffsetMHz-0.002dBm+ ΔLEOType; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); -14 dBm + ΔLEOType; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Aligned with SAN spurious limit; 4kHz. NOTE: ΔLEOType = 0dBm for LEO 600 km satellite and 6dBm for LEO 1200 km satellite. Table 4: SAN LEO/LEO600 OBUE basic limits |
| R4-2209923 | On SAN Spurious Emission requirements for Radiated Characteristics | Nokia, Nokia Shanghai Bell | Proposal: RAN 4 to adopt option 1 as the reference table for Spurious Emission. Option 1 Spurious frequency range; Basic limit GEO class (dBm); Basic limit LEO class (dBm); Measurement bandwidth. 9kHz – 12.75 GHz; [-13]; [-13]; 4 kHz. |
| R4-2208882 | NTN - SAN TX conducted requirements: remaining issues | Ericsson | Observation1: SAN output power level could exceed 50 W depending on SAN class and considered channel bandwidth. Proposal1: The SAN spurious requirement should be specified according to the following table: Spurious frequency range; Prated,c,sys(W); Basic limit (dBm); Measurement bandwidth. 30 MHz – 12.75 GHz; ≤ 50 W; -13; 4 kHz. > 50 W; 10 Log(Prated,c,sys(W)) – 30. Proposal2: The SAN GEO and LEO OBUE masks should be specified according to the following tables: SAN GEO Class - OBUE basic limits Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; 7dBm-75foffsetMHz-0.002 dBm; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); 0 dBm; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Aligned with SAN spurious limit; 4kHz. SAN LEO Class - OBUE basic limits Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; -7dBm-75foffsetMHz-0.002dBm+ ΔLEOType; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); -14 dBm + ΔLEOType; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Aligned with SAN spurious limit; 4kHz. NOTE: ΔLEOType = 0dBm for LEO 600 km satellite and 6dBm for LEO 1200 km satellite. Proposal3: The absolute ACLR requirement for SAN might not be needed but, if RAN4 decides to specify SAN absolute ACLR, it should be equal to -13 dBm / 4 kHz. |
| R4-2209361 | Discussion on SAN OoB mask and spurious emission requirements | Huawei, HiSilicon, Thales | Observation 1: For SAN, the spurious emission should be same, no matter the frequency range is located in or out of operating band. Thus, from spurious emission regulation perspective, there is no need to assume the operation band filter for SAN. Proposal 1: It is proposed to use for SAN requirements the ITU-R recommendations with the Out-of-band mask & spurious emission framework from ITU-R SM.1541-6 and ITU-R SM.329. Observation 2: Based on the ITU regulation, the boundary between the out-of-band and spurious domain is the frequency point at 250% of the necessary bandwidth from the carrier centre (2*CBW from the channel edge) Observation 3: If the boundary between the out-of-band and spurious domain for SAN follow NR FR1 base station rule based on a 5 MHz channel bandwidth, placing it at 12.5 MHz from the carrier centre (10 MHz from the channel edge), a stricter requirement can be observed for SAN RF bandwidth larger than 5MHz, e.g. 10, 15 or 20 MHz RF bandwidth. Proposal 2: The boundary between the out-of-band mask and spurious domain for SAN should be specified as 2*BWChannel from the channel edge based on ITU regulation. Observation 4: The spurious domain emission limits are different between output power less than 50W (47dBm) and larger than 50W (47dBm) based on the ITU regulation. Observation 5: Due to the difference of mean output power and Tx chain linearity performance between SAN and BS, the implementation are different between SAN and BS. Thus, we can’t simply follow the requirements or principles of base station when we specify the OoB mask and spurious emission requirements for SAN. Proposal 3: Considering the ITU regulation, the following spurious emissions limits are proposed for SAN. Spurious frequency range; Basic limit GEO/LEO class (dBm); Measurement bandwidth. 30 MHz –5th harmonic of the upper frequency edge of the DL operating band (NOTE 3, 4, 5, 6); When Prated,c,sys ≤ 47dBm : -13 When Prated,c,sys > 47dBm : Prated,c,sys-60 (NOTE 1); 4 kHz (NOTE 2). NOTE 1: Prated,c,sys (dBm) is declared by the manufacturer. NOTE 2: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 3: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5, Table 1. Values as in ITU-R SM.329 [x], Table 10. NOTE 4: Lower frequency limit is replaced by 800 MHz, according to ITU-R SM.329-12, when using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. [NOTE 5: This spurious frequency range applies only to SAN type 1-H.] [NOTE 6: Applies only for band n255 and n256.]. Proposal 4: Considering the ITU regulation ITU-R SM.1541-6, the following OoB mask limits are proposed for SAN. Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits dBm; Measurement bandwidth. 0 MHz f < 2* BWChannel; 0.002 MHz f_offset < 2* BWChannel + 0.002 MHz; maxSE limit, PSDchannel – 40*log10 f_offset-0.002BWChannel*2+1dBm; 4kHz. NOTE 1: PSDchannel = Prated,c – 10log10(BWChannel) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [3], Annex 5 OoB domain emission limits for space services.. |
| R4-2209592 | Further discussion on conducted Tx requirements of satellite access node | ZTE Corporation | Proposal 1: to define the NTN SAN transmitter UEM for LEO600 and LEO1200 as shown in Table 1-1 for GEO and Table 1-2 for LEO. Proposal 2: to define the NTN SAN transmitter spurious emission requirement as shown in Table 6.6.5.2.1-1 Proposal 3: to use ΔfOBUE =10dB and ΔfOOB=20MHz as baseline for band n256 and n255 unless there is any other filtering implementation concerns. |
| R4-2209924 | On SAN Spurious Emission requirements for Conducted Characteristics | Nokia, Nokia Shanghai Bell | Observation 1: In order to define modifications of the Radiated requirements compared to conducted requirements, it is better to use the relative frequency range to the downlink operating band, as defined in TS 38.104. Proposal 1: RAN 4 to adopt option 1 in [6] as the reference table for Spurious Emission. |
| R4-2210082 | Discussion on SAN OBUE | THALES | Proposal 1: Agree with the following OBUE SAN requirements (updated from Option 2 (see R4-2207456) based on EVM requirements): Table x.x-1. SAN GEO Class OBUE limit values Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits dBm; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; 1dBm-45foffsetMHz-0.002dB; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); -3 dBm; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Not applicable for B<100 MHz; 4kHz. Proposal 2: Agree with the following OBUE SAN requirements (updated from Option 2 (see R4-2207456) based on EVM requirements): Table x.x-2. SAN LEO Class OBUE basic limits Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits dBm; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; -3.0dBm-45foffsetMHz-0.002dB+20×log10A600dB; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); -7.0dBm+20×log10A600dB (Note 1); 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Not applicable for B<100 MHz; 4kHz. Note 1: A is the distance from the SAN to the Earth at SAN nadir direction. For example, A=600 for LEO@600km and A=1200 for LEO@1200km.. |
| R4-2210090 | Discussion on SAN Spurious Emissions | THALES | Proposal 1: Agree with the following SAN spurious emission limit requirements (updated from Option 2 (see R4-2207456)): Table 1. Basic spurious emission limit for GEO and LEO SAN classes Spurious frequency range; Basic limit GEO class (dBm); Basic limit LEO class (dBm); Measurement bandwidth; Notes. 30 MHz – 5th harmonic (NOTE 2, 4, 5); When Prated,c,sys ≤ 47dBm : -13 When Prated,c,sys > 47dBm : Prated,c,sys-60 ( NOTE 3); 4 kHz (NOTE 1); See NOTES 1, 2, 3, 4 ,5. NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5, Table 1. Applies only for the fundamental frequency range 600 MHz - 5.2 GHz (e.g. for n255 and n256 frequency bands). NOTE 3: Values as in ITU-R SM.329 [x], Table 10. NOTE 4: Lower frequency limit is replaced by 800 MHz, according to ITU-R SM.329-12, when using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. NOTE 5: This spurious frequency range applies only to SAN type 1-H.. |
| R4-2210110 | On SAN OBUE definition using ITU-R recommendation | THALES | Proposal 1: Consider Annex 5 of ITU-R recommendation SM.1541-6 (Unwanted emissions in the out-of-band domain) for the definition of the OBUE. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: ΔfOBUE

- Proposals

  - Option 1: use ΔfOBUE=10MHz as baseline for n256 and n255

  - Option 2: The boundary between the out-of-band mask and spurious domain for SAN should be specified as 2*BWChannel from the channel edge based on ITU regulation.

- Recommended WF

  - TBA

Sub-topic 1-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2: ΔfOOB

- Proposals

  - Option 1: use ΔfOOB=20MHz as baseline for n256 and n255

  - Option 2: other, please specify

- Recommended WF

  - TBA

Sub-topic 1-3 spurious emission

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3: Spurious emission

- Proposals

  - Option 1: spurious emission should be specified as in the following table (Ericsson)

| Spurious frequency range | Prated,c,TRP(W) | Basic limit (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 30 MHz – 12.75 GHz | ≤ 50 W | -13 | 4 kHz |
|  | > 50 W | 10 Log(Prated,c,sys(W)) – 30 |  |

  - Option 2: specify the spurious emissions as in the following table (Huawei, Thales)

| Spurious frequency range | Basic limit GEO/LEO class (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz –5th harmonic of the upper frequency edge of the DL operating band (NOTE 3, 4, 5, 6) | When Prated,c,sys ≤ 47dBm : -13 When Prated,c,sys > 47dBm : Prated,c,sys-60 (NOTE 1) | 4 kHz (NOTE 2) |
| NOTE 1: Prated,c,sys (dBm) is declared by the manufacturer. NOTE 2: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 3: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5, Table 1. Values as in ITU-R SM.329 [x], Table 10. NOTE 4: Lower frequency limit is replaced by 800 MHz, according to ITU-R SM.329-12, when using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. [NOTE 5: This spurious frequency range applies only to SAN type 1-H.] [NOTE 6: Applies only for band n255 and n256.] |  |  |

  - Option 3: specify the spurious emissions as in the following table (ZTE)

| Spurious frequency range | Basic limit (Note 5) | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz | When Prated,c,sys ≤ 50W : -13 When Prated,c,sys > 50W : 10*log(Prated,c,sys)-30 | 4 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz |  | 4 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 4 kHz | Note 1 |
| 1 GHz– 12.75 GHz |  | 4 kHz | Note 1, Note 2 |
| NOTE 1: Prated,c,sys (dBm) is declared by the manufacturer. NOTE 2: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 3: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5, Table 1. Values as in ITU-R SM.329 [x], Table 10. NOTE 4: Lower frequency limit is replaced by 800 MHz, according to ITU-R SM.329-12, when using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. [NOTE 5: This spurious frequency range applies only to SAN type 1-H.] [NOTE 6: Applies only for band n255 and n256.] |  |  |  |

- Recommended WF

  - TBA

Sub-topic 1-4

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-4-1: Principles to define OBUE requirements for SAN

- Proposals

  - Option 1: Specify satellite access node OBUE based on TN BS OBUE/TN MSR BC1 OBUE and scaling according to ACLR

  - Option 2: Follow Annex 5 of ITU recommendation SM.1541-6 to define SAN OBUE requirements.

- Recommended WF

  - TBA

Issue 1-4-2: OBUE requirements

- Proposals

  - Option 1: OBUE according to TN BS OBUE/TN MSR BC1 OBUE.

Option 1a:

- SAN GEO Class - OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 7dBm-75foffsetMHz-0.002 dBm | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 0 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Aligned with SAN spurious limit | 4kHz |

- SAN LEO Class - OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | -7dBm-75foffsetMHz-0.002dBm+ ΔLEOType | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -14 dBm + ΔLEOType | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Aligned with SAN spurious limit | 4kHz |
| NOTE: ΔLEOType = 0dBm for LEO 600 km satellite and 6dBm for LEO 1200 km satellite |  |  |  |  |

Option 1b:

Table 1-1. SAN GEO Class OBUE limit values

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz |  | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -3 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | -9dBm | 4kHz |
| NOTE: Assuming with lowest achievable EVM for GEO without power backoff is similar as QPSK EVM requirements to derive the starting point of first slope of UEM with 2dB implementation margin. |  |  |  |

Table 1-2. SAN LEO Class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | +X | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -13+X | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | -13+X | 4kHz |
| NOTE 1: For LEO600, X is equal to 0dB and LEO1200, X is equal to min(10*log10( Prated,c,sys)+30-41 ,6); NOTE 2: Assuming with lowest achievable EVM for LEO without power backoff is similar as 16QAM EVM requirements to derive the starting point of first slope of UEM with 2dB implementation margin. |  |  |  |

  - Option 2: OBUE according to Annex 5 of ITU recommendation SM.1541-6

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits dBm | Measurement bandwidth |
| --- | --- | --- | --- |

| 0 MHz f < 2* BWChannel | 0.002 MHz f_offset < 2* BWChannel + 0.002 MHz | maxSE limit, PSDchannel – 40*log10 f_offset-0.002BWChannel*2+1dBm | 4kHz |
| --- | --- | --- | --- |
| NOTE 1: PSDchannel = Prated,c – 10log10(BWChannel) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [3], Annex 5 OoB domain emission limits for space services. |  |  |  |

- Recommended WF

  - TBA

Sub-topic 1-5

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-5: Absolute ACLR

- Proposals

  - Option 1: Specify absolute ACLR requirement as -13dBm/4kHz

  - Option 2: Do not specify absolute ACLR requirement

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Ericsson | Issue 1-1: This would depend on the OBUE agreement. Issue 1-2: option 1 Issue 1-3: Ok with those limits for 1-H (we also proposed to use Prated,c,sys for 1-H), option 1 or option 2 would be acceptable. But the notes in option 2 are not ok: - Note 3: If we add this note, we shall be more precise, copying all ITU conditions: Lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency, according to ITU-R SM.329-12, for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelength Also, if we have this note, we would need a manufacturer declared parameter for this. - Note 5 is not needed as we only have conducted requirements for 1-H SAN - Note 6 might not be needed as well as we only have those 2 bands. Issue 1-4-1: see issue 1-4-2. Issue 1-4-2: We prefer option 1a, option 2 could be further discussed: To Huawei/Thales: with option 2, how this limit would fit with ACLR requirement? What attenuation level we would end up with this formula comparing to ACLR? THALES to Ericsson: ACLR should be seen as independent requirement from OBUE. The 2 requirements should be considered separately. Please also see our comment below. |
| ZTE | Issue 1-1: We support the option 1 and this is depend on front-end filter design in 3GPP instead of ITU recommendation; Issue 1-2: We support the option 1 otherwise the requirement is not complete. Issue 1-3: We don’t understand that why <30MHz is precluded in the spurious emission, there are lots of serive in that frequency range, why we don’t need any protection. For TN system, we also have lots of low bands, however within that frequency ranges, we still define the requirements; Issue 1-4-2: Option 2 is not related with ACLR requirement proposed in the spec, it also violate the basic principle how we define the OBUE mask in the past. In addition, we don;t understand that some compaines could have quite different views on the same requirement, i just don’t understand it . We don’t have strong opinions whether we need to define absolute ACLR requirement, for different SAN class, there would be different absolute ACLR requirements; |
| Huawei | Issue 1-1: I doubt whether we still need to use the concept “OBUE” for SAN. Based on the current proposals, the spurious emission requirements within operating band and out of operating band are same. Thus, even if we don’t specify ΔfOBUE, SAN has to meet the spurious emission requirements within operating band, which are same with the spurious emission requirements out of operating band. It means transmitter filter is helpless from spurious emission requirements perspective. Issue 1-2: option 1 Issue 1-3: Generally, we are OK with both option 1 and option 2. For the basic limit of option 1, the unit of Prated,c,sys is watt, but the unit of basic limit is dBm. That may cause some confusion. Thus, it’s better to align all the units (dBm). For option 2, brackets are added on note 5 and 6 for further discussion. In my understanding, Thales want to further discuss Spurious frequency range for SAN 1-O. I’m open for it. For note 6, I agree with Ericsson that Note 6 might not be needed as well as we only have those 2 bands. Issue 1-4-1: Option 2. Based on the WF R4-2207456, we have agreed the general principle during GTW session: follow ITU recommendation 1541 (SM.1541) at least for the frequency offset range within first two break points. We should honor the previous agreement. Issue 1-4-2: Option 2, option 2 is aligned with ITU recommendation 1541. To Ericsson / ZTE, when the frequency offset is equal to channel bandwidth, the PSD attenuation is 19 dB which is similar to 14dB ACLR GEO and 24dB ACLR LEO. Comments on Option 1a, when ΔLEOType = 0dBm, the basic limit in frequency offset 5~10MHz is -14 dBm/4kHz, but spurious emission is only -13 dBm/4kHz. That means the basic limit of OoB mask is stricter than the spurious emission. It isn’t reasonable from requirements perspective. Comments on Option 1b, when X is equal to 0dB for LEO600, the basic limit for frequency offset which is larger than 5MHz is -13dBm/4kHz. That means the basic limit of OoB mask is equal to or stricter than the spurious emission. In other words, the boundary between OoB mask and spurious emission is 5MHz for each channel bandwidth. It isn’t reasonable from requirements perspective. |
| THALES | Issue 1-1: Thales agrees with Option 2, nevertheless, by using the ITU-R SM.1541-6 OBUE mask, the limit between OBUE domain and spurious domain should be the frequency where the OBUE mask intersects the spurious domain limit. For 30 MHz Bandwidth, the intersection frequency is about 42 MHz from the edges of the channel. Issue 1-2: Option 1 Issue 1-3: Option 2 Issue 1-4-1: Option 2 Issue 1-4-2: Thales supports Option 2 if agreeable by RAN4 WG. Note that the OBUE mask requirement (and the mathematical operation of integration of the OBUE mask) does not reflect the ACLR performance, and should not consider ACLR requirement in the definition or computation of OBUE. For this reason, it is important to emphasize that respecting the OBUE mask will not guarantee/imply the ACLR requirements. ACLR is an independent requirement and shall be considered separate from OBUE. If not, we support Option 3 below, (justification can be found in TDoc R4-2210082): Option 3: Table 6.6.4.2-1: SAN GEO Class OBUE basic limits Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits dBm; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; 1dBm-45foffsetMHz-0.002dB; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); -3 dBm; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Not applicable for B<100 MHz; 4kHz. Table 6.6.4.2-2: SAN LEO Class OBUE basic limits Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits dBm; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; -3.0dBm-45foffsetMHz-0.002dB+20×log10A600dB; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); -7.0dBm+20×log10A600dB (NOTE 1); 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Not applicable for B<100 MHz; 4kHz. NOTE 1: A is the distance from the SAN to the Earth at SAN nadir direction. For example, A=600 for LEO@600km and A=1200 for LEO@1200km.. |
| CATT | Issue 1-1: Depends on the OBUE discussion. If the conclusion for OBUE is to follow TN BS OBUE/TN MSR BC1 OBUE, Option 1 should be used forΔfOBUE. Issue 1-2: Option 1. Issue 1-3: It seems there is not too much difference between these options. Would like to have clarification why protection for 30MHz is not needed? The needed notes need to be clean up in the final TP. Issue 1-4-1: Open for further discussion. Issue 1-4-2: Open for further discussion between Option 1a and Option 2. Issue 1-5: We are ok to omit absolute ACLR requirement if no one thinks it necessary. |
| Ligado Networks | Issue 1-1: Option 2 if Option 2 is agreed to for Issue 1-4-2; will have a consistent definition with ITU-R recommendation Issue 1-2: Option 1 Issue 1-3: Option 2; consistent with ITU-R Recommendations Issue 1-4-1: Option 2 Issue 1-4-2: Option 2 as it is consistent with the ITU-R recommendations Issue 1-5: Option 2 |

Example 2

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Issue 1-1: ΔfOBUE - Proposals - Option 1: use ΔfOBUE=10MHz as baseline for n256 and n255 - Option 2: The boundary between the out-of-band mask and spurious domain for SAN should be specified as 2*BWChannel from the channel edge based on ITU regulation. Agreement in Tuesday GTW Option 2: The boundary between the out-of-band mask and spurious domain for SAN should be specified as 2*BWChannel from the channel edge based on ITU regulation. Issue 1-2: ΔfOOB - Proposals - Option 1: use ΔfOOB=20MHz as baseline for n256 and n255 - Option 2: other, please specify Agreement in the Tuesday GTW Option 1: use ΔfOOB=20MHz as baseline for n256 and n255 Issue 1-3: Spurious emission - Proposals - Option 1: spurious emission should be specified as in the following table (Ericsson) Spurious frequency range; Prated,c,TRP(W); Basic limit (dBm); Measurement bandwidth. 30 MHz – 12.75 GHz; ≤ 50 W; -13; 4 kHz. > 50 W; 10 Log(Prated,c,sys(W)) – 30. - Option 2: specify the spurious emissions as in the following table (Huawei, Thales) Spurious frequency range; Basic limit GEO/LEO class (dBm); Measurement bandwidth. 30 MHz –5th harmonic of the upper frequency edge of the DL operating band (NOTE 3, 4, 5, 6); When Prated,c,sys ≤ 47dBm : -13 When Prated,c,sys > 47dBm : Prated,c,sys-60 (NOTE 1); 4 kHz (NOTE 2). NOTE 1: Prated,c,sys (dBm) is declared by the manufacturer. NOTE 2: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 3: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5, Table 1. Values as in ITU-R SM.329 [x], Table 10. NOTE 4: Lower frequency limit is replaced by 800 MHz, according to ITU-R SM.329-12, when using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. [NOTE 5: This spurious frequency range applies only to SAN type 1-H.] [NOTE 6: Applies only for band n255 and n256.]. - Option 3: specify the spurious emissions as in the following table (ZTE) Spurious frequency range; Basic limit (Note 5); Measurement bandwidth; Notes. 9 kHz – 150 kHz; When Prated,c,sys ≤ 50W : -13 When Prated,c,sys > 50W : 10*log(Prated,c,sys)-30; 4 kHz; Note 1, Note 4. 150 kHz – 30 MHz; 4 kHz; Note 1, Note 4. 30 MHz – 1 GHz; 4 kHz; Note 1. 1 GHz– 12.75 GHz; 4 kHz; Note 1, Note 2. NOTE 1: Prated,c,sys (dBm) is declared by the manufacturer. NOTE 2: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 3: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5, Table 1. Values as in ITU-R SM.329 [x], Table 10. NOTE 4: Lower frequency limit is replaced by 800 MHz, according to ITU-R SM.329-12, when using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. [NOTE 5: This spurious frequency range applies only to SAN type 1-H.] [NOTE 6: Applies only for band n255 and n256.]. Agreement in Tuesday GTW Option 2 is agreed Spurious frequency range; Basic limit GEO/LEO class (dBm); Measurement bandwidth. 30 MHz –5th harmonic of the upper frequency edge of the DL operating band (NOTE 3, 4, 5, 6); When Prated,c,sys ≤ 47dBm : -13 When Prated,c,sys > 47dBm : Prated,c,sys-60 (NOTE 1); 4 kHz (NOTE 2). RAN4 can further revisit the low frequency boundary (30MHz) in maintenance phase if technical justification identified. Further discuss the “Notes” in the table over email Issue 1-4-1: Principles to define OBUE requirements for SAN - Proposals - Option 1: Specify satellite access node OBUE based on TN BS OBUE/TN MSR BC1 OBUE and scaling according to ACLR - Option 2: Follow Annex 5 of ITU recommendation SM.1541-6 to define SAN OBUE requirements. - Recommended WF - TBA Agreement in Tuesday GTW Option 2: Follow Annex 5 of ITU recommendation SM.1541-6 to define SAN OBUE requirements. Issue 1-4-2: OBUE requirements - Proposals - Option 1: OBUE according to TN BS OBUE/TN MSR BC1 OBUE. Option 1a: - SAN GEO Class - OBUE basic limits Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; 7dBm-75foffsetMHz-0.002 dBm; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); 0 dBm; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Aligned with SAN spurious limit; 4kHz. - SAN LEO Class - OBUE basic limits Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; -7dBm-75foffsetMHz-0.002dBm+ ΔLEOType; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); -14 dBm + ΔLEOType; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; Aligned with SAN spurious limit; 4kHz. NOTE: ΔLEOType = 0dBm for LEO 600 km satellite and 6dBm for LEO 1200 km satellite. Option 1b: Table 1-1. SAN GEO Class OBUE limit values Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits (Note 1, 2) dBm; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); -3 dBm; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; -9dBm; 4kHz. NOTE: Assuming with lowest achievable EVM for GEO without power backoff is similar as QPSK EVM requirements to derive the starting point of first slope of UEM with 2dB implementation margin.. Table 1-2. SAN LEO Class OBUE basic limits Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits (Note 1, 2) dBm; Measurement bandwidth. 0 MHz f < 5 MHz; 0.002 MHz f_offset < 5.002 MHz; +X; 4kHz. 5 MHz f < min(10 MHz, fmax); 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax); -13+X; 4kHz. 10 MHz f fmax; 10.002 MHz f_offset < f_offsetmax; -13+X; 4kHz. NOTE 1: For LEO600, X is equal to 0dB and LEO1200, X is equal to min(10*log10( Prated,c,sys)+30-41 ,6); NOTE 2: Assuming with lowest achievable EVM for LEO without power backoff is similar as 16QAM EVM requirements to derive the starting point of first slope of UEM with 2dB implementation margin.. - Option 2: OBUE according to Annex 5 of ITU recommendation SM.1541-6 Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits dBm; Measurement bandwidth. 0 MHz f < 2* BWChannel; 0.002 MHz f_offset < 2* BWChannel + 0.002 MHz; maxSE limit, PSDchannel – 40*log10 f_offset-0.002BWChannel*2+1dBm; 4kHz. NOTE 1: PSDchannel = Prated,c – 10log10(BWChannel) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [3], Annex 5 OoB domain emission limits for space services.. Agreement in Tuesday GTW - Option 2 0 MHz f < 2* BWChannel; 0.002 MHz f_offset < 2* BWChannel + 0.002 MHz; maxSE limit, PSDchannel-[X] – 40*log10 f_offset-0.002BWChannel*2+1dBm; 4kHz. - X = 3dB for LEO - X= 0dB for GEO RAN4 can further revisit the requirements including the value(X) during maintenance phase with technical input from companies. Issue 1-5: Absolute ACLR - Proposals - Option 1: Specify absolute ACLR requirement as -13dBm/4kHz - Option 2: Do not specify absolute ACLR requirement Agreement in Tuesday GTW Option 2: Do not specify absolute ACLR requirement |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #2: TP review for 38.108

TP review will be based on each contribution. A sub-folder is set for each TP in the Round1 folder. If there is no comments for a TP after 1st round, it will be considered agreeable and no revisit in the 2nd round.

Companies’ contributions summary

| T-doc | Title | Company |
| --- | --- | --- |
| R4-2208888 | pCR to TS 38.108 - Alignement | Ericsson |
| R4-2208887 | pCR to TS 38.108 - Transmitter spurious requirement | Ericsson |
| R4-2210059 | pCR for Clause 10.6.2 Minimum requirement for SAN type 1-O - TS 38.108 | THALES |
| R4-2210159 | Draft text proposal for Clause 7.3.3.3.1 OTA sensitivity - TR 38.863 | THALES |
| R4-2210161 | Draft text proposal for Clause 7.3.3.3.2 OTA reference sensitivity - TR 38.863 | THALES |
| R4-2210162 | Draft text proposal for Clause 7.3.3.3.3 OTA dynamic range - TR 38.863 | THALES |
| R4-2210163 | Draft text proposal for Clause 7.3.3.3.2 OTA reference sensitivity - TR 38.863 | THALES |
| R4-2210164 | Draft text proposal for Clause 7.3.3.3.7 OTA receiver intermodulation - TR 38.863 | THALES |
| R4-2210165 | Draft text proposal for Clause 7.3.3.3.8 OTA in-channel selectivity - TR 38.863 | THALES |
| R4-2210213 | Draft text proposal for Clause 7.3.3.3.4 OTA in-band selectivity and blocking - TR 38.863 | THALES |
| R4-2208249 | TP for 38.863: clause 7.3.2 Conducted transmission characteristics | CATT |
| R4-2208250 | TP for 38.108: clause 6 on unwanted emissions | CATT |
| R4-2209528 | TP to TS 38.108 on 6.0 Conducted transmitter characteristics | HUGHES Network Systems Ltd; Hughes/EchoStar |
| R4-2209677 | TP to TS 38.108: TS corrections; RF requirements | Huawei, HiSilicon |
| R4-2209999 | pCR for Clause 6.6.3 Adjacent Channel Leakage Power Ratio - TS 38.108 | THALES |
| R4-2210000 | Tentative pCR for Clause 6.6 Unwanted emissions and Clause 6.6.4 OBUE - TS 38.108 | THALES |
| R4-2210001 | pCR for Clause 6.6.5 Transmitter spurious emissions - TS 38.108 | THALES |
| R4-2210004 | Draft text proposal for Clause 7.3.2.2.5 Transmitter spurious emissions - TR38.863 | THALES |
| R4-2210116 | Tentative draft pCR for Clause 7.3.2.2.4.2 Operating band unwanted emissions - TR 38.863 | THALES |
| R4-2210154 | Draft text proposal for Clause 7.3.2.2.1 SAN output power - TR 38.863 | THALES |
| R4-2210216 | Draft TP for TS 38.108 Section 6.6.4 Operating band unwanted emissions | Inmarsat |
| R4-2208663 | TP to TS 38.108 on Conducted receiver characteristics | ZTE Corporation |
| R4-2209678 | TP to TR 38.863: Conducted reference sensitivity | Huawei, HiSilicon |
| R4-2209679 | TP to TR 38.863: Conducted Rx dynamic range | Huawei, HiSilicon |
| R4-2210030 | pCR for Clause 7.4 In-band selectivity and blocking - TS 38.108 | THALES |
| R4-2210042 | pCR for Clause 7.5 Out-of-band blocking - TS 38.108 | THALES |
| R4-2210049 | pCR for Clause 7.6 Receiver spurious emissions - TS 38.108 | THALES |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: TBA

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

Sub-topic 2-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2: TBA

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

Example 2

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208888 | Thales: Comments have been provided in the draft folder R4-2208888 |
|  | CATT: comments have been uploaded to the TP in the folder. |
| R4-2208887 | Thales: Comments have been provided in the draft folder R4-2208887 |
|  | CATT: comments have been uploaded to the TP in the folder. |
| R4-2210059 | Ericsson: Table 10.6.2.1-2 title should be updated: ΔfOOB offset for NR satellite operating bands in FR1 ZTE: this is under the discussion in issue 1-2.CATT: comments have been uploaded to the TP in the folder. THALES: Comments have been taken into account Huawei: please note that almost the same correction is captured in R4-2209677. I the meantime, we have also identified that there might have been an error in the field strength value calculation – this is something that we would like to double-check during the second round. |
| R4-2210159 | Ericsson: We should not copy TS text in the TR, that doesn't bring anything, we succeeded not doing this before.. Better to just briefly describe requirement and explain how sensitivity will apply for SAN. To be revised. ZTE: similar comments as Ericsson. CATT: comments have been uploaded to the TP in the folder. Thales: Since these contributions have been already accepted for the TS, it is normal to have something similar in the TR. The purpose is not to copy&paste but to have some text proposals for the TR. We should still have some text/contribution that we will adapt in later stages. Other companies are encouraged to contribute. If you have better idea please provide comments in the pCRs, we would like to finish the TR and not leave any blank spaces. THALES2: We have simplified the pCR accordingly. We deleted the unnecessary text. The idea was to have something in the respective sections, since for the time being the sections were empty and no other contribution. |
| R4-2210161 | Ericsson: Same comment as R4-2210159, there should not be any “shall” in a TR. To be simplified. ZTE: please don’t copy&paste the TS into TR similar as comment from Ericsson CATT: comments have been uploaded to the TP in the folder. Thales: Since these contributions have been already accepted for the TS, it is normal to have something similar in the TR. The purpose is not to copy&paste but to have some text proposals for the TR. We should still have some text/contribution that we will adapt in later stages. Other companies are encouraged to contribute. If you have better idea please provide comments in the pCRs, we would like to finish the TR and not leave any blank spaces. THALES2: We have simplified the pCR accordingly. We deleted the unnecessary text and kept the tables (we can remove them if no need to consider them here). The idea was to have something in the respective sections, since for the time being the sections were empty and no other contribution. |

| R4-2210162 | Ericsson: Same comment as R4-2210159, there should not be any “shall” in a TR. To be simplified. ZTE: please don’t copy&paste the TS into TR similar as comment from Ericsson CATT: comments have been uploaded to the TP in the folder. Thales: Since these contributions have been already accepted for the TS, it is normal to have something similar in the TR. The purpose is not to copy&paste but to have some text proposals for the TR. We should still have some text/contribution that we will adapt in later stages. Other companies are encouraged to contribute. If you have better idea please provide comments in the pCRs, we would like to finish the TR and not leave any blank spaces. THALES2: We have simplified the pCR accordingly. We deleted the unnecessary text and kept the tables (we can remove them if no need to consider them here). The idea was to have something in the respective sections, since for the time being the sections were empty and no other contribution. |
| --- | --- |
| R4-2210163 | Ericsson: Same comment as R4-2210159, there should not be any “shall” in a TR. To be simplified. ZTE: please don’t copy&paste the TS into TR similar as comment from Ericsson CATT: comments have been uploaded to the TP in the folder. Thales: Since these contributions have been already accepted for the TS, it is normal to have something similar in the TR. The purpose is not to copy&paste but to have some text proposals for the TR. We should still have some text/contribution that we will adapt in later stages. Other companies are encouraged to contribute. If you have better idea please provide comments in the pCRs, we would like to finish the TR and not leave any blank spaces. THALES2: We have simplified the pCR accordingly. We deleted the unnecessary text and kept the tables (we can remove them if no need to consider them here). The idea was to have something in the respective sections, since for the time being the sections were empty and no other contribution. |
| R4-2210164 | Ericsson: No need to add "as the conducted receiver intermodulation" THALES; updated accordingly. |
| R4-2210165 | Ericsson: Same comment as R4-2210159, there should not be any “shall” in a TR. To be simplified. ZTE: please don’t copy&paste the TS into TR similar as comment from Ericsson CATT: comments have been uploaded to the TP in the folder. THALES: We have simplified the pCR accordingly. We deleted the unnecessary text and kept the tables (we can remove them if no need to consider them here). The idea was to have something in the respective sections, since for the time being the sections were empty and no other contribution. |
| R4-2210213 | Ericsson: Same comment as R4-2210159, there should not be any “shall” in a TR. To be simplified. ZTE: please don’t copy&paste the TS into TR similar as comment from Ericsson CATT: comments have been uploaded to the TP in the folder. THALES; corrected accordingly |
| R4-2208249 | Thales: See draft folder R4-2208249 |
| R4-2208250 | Ericsson: ACLR: "The ACLR limits in table 6.6.3.2-1/2 or [TBD]dBm/MHz": the TBD depends if RAN4 agrees to specify ACLR absolute limit. ZTE: update the absolute ACLR limit if there is agreement reached in this meeting. Thales: Comments have been provided in the draft folder R4-2208249 Note that Thales had contributed on the same section 6.6.3, See Tdoc R4-2209999. Tdocs should be merged once RAN4 WG agrees on the ACLR specifications. CATT: comments have been uploaded to the TP in the folder. THALES2: The SAN ACLR absolute basic limit should not be defined since there is no SAN power class definition or requirement. |
| R4-2209528 | Ericsson: To be merged with R4-2209677 |
| R4-2209677 | Ericsson: Ok to add Prated,c,sys,GEO and Prated,c,sys,LEO but they have to be added to the symbols clause. Table 10.6.2.1-1 title: ΔfOOB offset for NR satellite operating bands in FR1 To be merged with R4-2210059 and R4-2208888CATT: comments have been uploaded to the TP in the folder. Thales: Comments have been provided in the draft folder R4-2209677 |
| R4-2209999 | Ericsson: This pCR should not have been submitted, it’s not according to the agreed work split, R4-2208250 shall be used instead. This pCR should be noted or even withdrawn. Also, this is a TS, not a TR so no explanation is expected here: The SAN ACLR absolute basic limit is not defined since there is no SAN power class definition or requirement Thales: The pCR should be merged if lead company submitted a contribution (which occurs to be the case for this section). There is no restriction not to submit a contribution to the meeting, otherwise is not fair nor reasonable. This was explained in RAN4#102-e meeting by RAN4 RF chair. Any comments directly on the pCR are welcome. We are fine with removing the explanation in the TS. CATT: comments have been uploaded to the TP in the folder. THALES: agree to merge with CATT contribution R4-2208250. However, The SAN ACLR absolute basic limit should not be defined since there is no SAN power class definition or requirement. |
| R4-2210000 | Ericsson: This pCR should not have been submitted, it’s not according to the agreed work split, R4-2210216 shall be used instead. This pCR should be noted or even withdrawn. Thales: The pCR should be merged if lead company submitted a contribution (which occurs to be the case for this section). There is no restriction not to submit a contribution to the meeting, otherwise is not fair nor reasonable. This was explained in RAN4#102-e meeting by RAN4 RF chair. Any comments directly on the pCR are welcome. THALES2: Updated with the meeting agreements. The document can be resubmitted as it is for approval, or material directly reused (merged) by lead company (Inmarsat) in R4-2210216. |
| R4-2210001 | Ericsson: This pCR should not have been submitted, it’s not according to the agreed work split, R4-2208887 shall be used instead. This pCR should be noted or even withdrawn. Also, we have already agreed some meetings ago the 10Log factor won’t apply here. Thales: The pCR should be merged if lead company submitted a contribution (which occurs to be the case for this section). There is no restriction not to submit a contribution to the meeting, otherwise is not fair nor reasonable. This was explained in RAN4#102-e meeting by RAN4 RF chair. Any comments directly on the pCR are welcome. CATT: comments have been uploaded to the TP in the folder. |
| R4-2210004 | Ericsson: Note 3 is not needed Note 4 needs more clarification, to be aligned with ITU: NOTE 4: Lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency 800 MHz, according to ITU-R SM.329-12, for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelengthwhen using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. Also, if we have note 4, this should be a manufacturer declaration then ZTE: to be revised and to capture the agreement. CATT: comments have been uploaded to the TP in the folder. Thales: We prefer to keep Note 3 as a reference. Note 4 has been aligned as requested. |
| R4-2210116 | Ericsson: To be aligned with the agreements on OBUE. ZTE: to be revised and to capture the agreement. THALES: Revised to capture the agreement. |
| R4-2210154 | Ericsson: Better to align with R4-2209677 which has a clearer wording. |
| R4-2210216 | Ericsson: Sub-clause 6.6.4.3 is not needed, we have already agreed the 10LogX won't apply here. To be updated anyway with RAN4 agreements. ZTE: to be revised with the agreement reached if possible |
| R4-2208663 | Ericsson: OK, but should probably be merged with R4-2209677? ZTE: draft CR could be merged into the big CR at the end, this draft CR could be endorsed. |
| R4-2209678 | Thales: Comments have been provided in the draft folder R4-2209678 |
| R4-2209679 | Ericsson: See commented file Thales: Comments have been provided in the draft folder R4-2209678 |
| R4-2210030 | Ericsson: There is no "OFDM" in UL… To be merged with R4-2209677 ZTE: please keep the DFT-s-OFDM, either CP-OFDM or DFT-s-OFDM, Thales: Comments have been provided in the draft folder R4-2210030 THALES2: CP-OFDM modification has been made. This corresponds to Scenario 6 from coexistence analysis where TN DL (TDD) CP-OFDM interfering with NTN Satellite UL. |
| R4-2210042 | Ericsson: To be merged with R4-2209677 ZTE: this is under the discussion. Thales: We agree to merge R4-2209677 with R4-2210042. (Lead of the section: Thales) |
| R4-2210049 | Ericsson: Why should we remove Rx spurious? That has never been agreed before… ZTE: we have the concerns to remove the Rx spuirou emission requirements. Thales: From satellite industry, definition of Rx Spurious emissions is not clear. Please clarify the rationale behind. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CR/TP statue update recommendation |
| --- | --- |
| R4-2208888 | To be revised |
| R4-2208887 | To be revised |
| R4-2210059 | To be revised |
| R4-2210159 | To be revised |
| R4-2210161 | To be revised |
| R4-2210162 | To be revised |
| R4-2210163 | To be revised |
| R4-2210164 | To be revised |
| R4-2210165 | To be revised |
| R4-2210213 | To be revised |
| R4-2208249 | To be revised |
| R4-2208250 | To be revised |
| R4-2209528 | To be revised |
| R4-2209677 | To be merged with R4-2210042 |
| R4-2209999 | To be merged with leading company CR |
| R4-2210000 | To be merged with leading company CR |
| R4-2210001 | To be merged with leading company CR |
| R4-2210004 | To be revised |
| R4-2210116 | To be revised |
| R4-2210154 | To be revised |
| R4-2210216 | To be revised |
| R4-2208663 | To be endorsed. |
| R4-2209678 | To be revised |
| R4-2209679 | To be revised |
| R4-2210030 | To be revised |
| R4-2210042 | To be revised |
| R4-2210049 | To be revised. Continue discussion in the 2nd round whether to remove this requirement or not. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-220xxxx | WF on remaining issue for SAN RF requirements | CATT |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2208248 |  | Open issue on radiated requirement for SAN | CATT | Withdrawn |  |
| R4-2208883 |  | NTN - SAN TX radiated requirements: remaining issues | Ericsson | To be noted |  |
| R4-2209923 |  | On SAN Spurious Emission requirements for Radiated Characteristics | Nokia, Nokia Shanghai Bell | To be noted |  |
| R4-2208882 |  | NTN - SAN TX conducted requirements: remaining issues | Ericsson | To be noted |  |
| R4-2209361 |  | Discussion on SAN OoB mask and spurious emission requirements | Huawei, HiSilicon, Thales | To be noted |  |
| R4-2209592 |  | Further discussion on conducted Tx requirements of satellite access node | ZTE Corporation | To be noted |  |
| R4-2209924 |  | On SAN Spurious Emission requirements for Conducted Characteristics | Nokia, Nokia Shanghai Bell | To be noted |  |
| R4-2210082 |  | Discussion on SAN OBUE | THALES | To be noted |  |
| R4-2210090 |  | Discussion on SAN Spurious Emissions | THALES | To be noted |  |
| R4-2210110 |  | On SAN OBUE definition using ITU-R recommendation | THALES | To be noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)