3GPP TSG-RAN WG4 Meeting # 103-e														R4-2210357

Electronic Meeting, May 09 – 20, 2022

Source:		Verizon

Title:						TP for TR 37.717-11-11 for DC_48_n2

Agenda item:		8.11.2

Document for:		Approval

# 1. Introduction

This contribution is a text proposal to include DC_48_n2 in TR 37.717-11-11.

# 2. Text Proposal

<Start of Text Proposal>

## 6.1.x	DC_48_n2

### 6.1.x.1	Operating bands for EN-DC

Table 6.1.X.1-1: Band combinations EN-DC (two bands)

| EN-DC band | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_48A_n2A | DC_48A_n2A | No |

#### 6.1.x.2		Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_48A_n2A | 23 | +2/-3 |

### 6.1.x.3	Spurious emission band UE co-existence for DC

Table 6.1.x.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48_n2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

### 6.1.x.4	MSD analysis for DC

Both CA_n2-n48 and DC_2_n48 combinations have been specified in 38.101-1. Also, the related co-existence study of CA_n2-n48 is included in Section 6.36.2 of TR 38.716-02-00. For this DC_48-n2 combo, the same co-existence issues are identified,

- The 2nd harmonic skirt interference for band n2 UL will impact the receive frequency of band 48

- The 4th order IMD may fall into Rx frequencies of band n2 and no IMD falls into its band 48 Rx frequencies since band 48 is TDD band.

Thus additional reference sensitivity exceptions for 2nd harmonic in DC_48_n2 is derived to below, based on 38.101-1.

Table 6.1.x.4-1: Reference sensitivity exceptions due to UL harmonic for EN-DC FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n2 | 482, 13 | 27.3 | 24.4 | 22.4 | 21.2 |  | 19.0 | 18.0 | 17.1 | 16.3 | 15.4 | 15.0 | 14.5 | 14 |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n2, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The uplink configuration for reference sensitivity exceptions from 38.101-1 is applied below for UL harmonic interference.

Table 6.1.x.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (KHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n2 | 48 | 15 | 25 | 50 | 75 | 100 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

The additional MSD required due to 4th IMD for DC_48_n2 is derived from CA_n2-n48 defined in 38.101-1 as below for the dual uplink configuration.

Table 6.1.x.4-3: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A_n2A | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
|  | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A |

#### 6.1.x.5	∆TIB and ∆RIB values

For DC_48_n2, the TIB,c and RIB,c values are given in the tables below.

Table 6.x.1.5-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_48_n2 | 48 | 0.8 |
|  | n2 | 0.6 |

Table 6.x.1.5-2: ΔRIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_48_n2 | 48 | 0.5 |
|  | n2 | 0.2 |

<End of Text Proposal>