3GPP TSG-RAN WG4 Meeting #103-e	R4-2210393

# Electronic Meeting, 9 - 20 May, 2022

Source:	Huawei, HiSilicon

Title:	TP for 38.717-02-01 CA_n3A-n78C with UL_n78C

Agenda item:	8.5.2

Document for:	Approval

1 Background

This contribution provides updated text proposal on the NR CA band combination CA_n3A-n78C with UL_n78C.

2 Text Proposal

---Start of changes---

## 6.11		CA_n3-n78

### 6.11.1		Common for 1 band UL and 2 bands UL CA

#### 6.11.1.1		Operating bands for CA

Table 6.11.1.1-1: CA band combination of band n3+n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 6.11.1.2		Channel bandwidths per operating band for CA

Table 6.11.1.2-1: Supported bandwidths per CA band combination of band n3+n78

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n78(2A) | CA_n78(2A) | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n78C | CA_n78C | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78C_BCS2 |  |

#### 6.11.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n3 + n78 have been already completed and captured into TR 37.865-01-01, there is no IMD issue for band n3 + n78.

Referring to clause 7.3.2.2 from TR 38.862, we can get the IMD order for contiguous UL CA as below.

WGap = lowest band edge of n78 - highest DL band edge of n1 = 3300 – 1880 = 1420 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(200, 500) = 200 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 17.

Since IMD order is > 11, MSD can be ignored.

Referring to clause 7.3.2.2 from TR 38.862, we can get the IMD order for non-contiguous UL CA as below.

WGap = lowest band edge of n78 - highest DL band edge of n1 = 3300 – 1880 = 1420 MHz.

Min(maxBW separation class, UL band bandwidth) = Min(600, 500) = 500 MHz.

IMD order = 2*ceil(WGap /Min(maxBW separation class, UL band bandwidth) + 1 = 7.

Since IMD order is < 9, MSD due to IMD7 can’t be ignored.

#### 6.11.1.4		∆TIB and ∆RIB values

For CA_n3-n78, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

#### 6.11.1.5		REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration..

Referring to MSD due to IMD7 for DL_n3A-n77(2A)_UL _n77(2A) specified in TS 38.101-1, the MSD due to IMD7 for DL_n3A-n78(2A)_UL _n78(2A) can be introduced as below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78 | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7 |
|  | n78 | 3310 | 10 | 1 (RBstart=0) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBstart=0) | 3790 |  |  |  |

#### 6.11.1.6	    OOB blocking exception requirements

Table 6.11.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

---End of changes---

Reference

[1]	RP-211058, “Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation