3GPP TSG-RAN WG4 Meeting # 103-e 													R4-2210524

Electronic Meeting, May 9-20, 2022

Agenda item:			9.11.1  9.11.2.3

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [103-e][321] NR_perf_enh2_Demod_Part1

Document for:	Information

Introduction

This email thread discusses the general and CRS-IM part in the Rel-17 further demodulation performance enhancement WI in agenda 9.11.1 and 9.11.2.3.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to provide comments in section 1.3, 2.3 and 3.3.

  - According to Session Chair’s announcement, the GTW session for CRS-IM would probably be switched to Thursday. It is appreciated if companies can add your comments & response before 20:00 UTC Wednesday, and so moderator can provide a brief summary before the GTW session.

- 2nd round: 4 sub-threads will be created in round 2

  - 1 sub-thread on ‘[103-e][321] NR_perf_enh2_Demod_Part1 - Updated work plan’ (led by China Telecom)

    - Revision of updated work plan to incorporate the round 1 comment.

  - 1 sub-thread on ‘[103-e][321] NR_perf_enh2_Demod_Part1 - WF on test for scenario 2’ (led by HW)

    - WF on the test with only inter-RAT MO configured for scenario 2, which cover the issues in sub-topic 2-1 for both 15kHz and 30kHz SCS

  - 1 sub-thread on ‘[103-e][321] NR_perf_enh2_Demod_Part1 - CRs and simulation assumptions for 15kHz CRS-IM’ (led by China Telecom)

    - Cover the revision of draft CRs and simulation assumptions discussed in section 2.3.2

  - 1 sub-thread on ‘[103-e][321] NR_perf_enh2_Demod_Part1 - CRs and simulation assumptions for 30kHz CRS-IM’ (led by CMCC)

    - Cover the revision of draft CRs discussed in section 3.3.2, and summary of agreed simulation assumptions for 30 kHz CRS-IM.

Topic #1: General

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2209417 | China Telecom | Updated work plan for Further enhancement on NR demodulation performance WI |

Open issues

No open issues.

Companies views’ collection for 1st round

Updated work plan comments collection

| tdoc number | Comments collection |
| --- | --- |
| R4-2209417 | Apple: We agreed in last meeting not to define requirements in Scenario 2 for MMSE-IRC in ICI. Also, the SR from RAN#95e indicates completion in June 2022, should the last row for RAN4 #104 (Aug 2022) be deleted? |
|  | Qualcomm: CR Work Split for 30kHz is pending on the outcome of whether to define tests for 30kHz SCS. CMCC should be added for simulation results collection for 30kHz, if it is agreed. |

Summary for 1st round

| tdoc number | Title/Content | CRs/TPs Status update recommendation |
| --- | --- | --- |
| R4-2209417, CTC | Updated work plan | Revised |

Discussion on 2nd round

| tdoc | Comments collection |
| --- | --- |
| R4-2210926, CTC, Updated work plan | China Telecom: Based on the first round comments as listed below (Thanks for all the comments!): Comments collection. Apple: We agreed in last meeting not to define requirements in Scenario 2 for MMSE-IRC in ICI. Also, the SR from RAN#95e indicates completion in June 2022, should the last row for RAN4 #104 (Aug 2022) be deleted?. Qualcomm: CR Work Split for 30kHz is pending on the outcome of whether to define tests for 30kHz SCS. CMCC should be added for simulation results collection for 30kHz, if it is agreed.. We made following updates to the WP: - Removed all action points related to Scenario 2 for IRC tests in the May and Aug meetings. - Update the CRS-IM related action points in the May and Aug meetings, according to the latest agreements and status. - Add CMCC as the responsible company for LTE CRS-IM for 30kHz SCS scenario result collection. |
|  | CMCC: We have one comment about LTE CRS-IM, We think the ‘Collection of ideal and impairment simulation results for 30kHz SCS’ is still needed in RAN4 #104, since the simulation assumption is revised in this meeting. Could you please help to add this sub-bullet? Thanks a lot! |
|  | China Telecom: Thanks Shiyuan for the comments. Based on the status for CRS-IM. I made following updates to the action points for the May and Aug meetings: |

Topic #2: CRS-IM for general and 15kHz SCS scenario

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2209527 | ZTE | CRS-IM simulation results (15 kHz FDD and TDD) |
| R4-2209408 | China Telecom | Simulation results for CRS-IM for 15kHz SCS scenario |
| R4-2209409 | China Telecom | Observation 1: UE can always ensure high reliable PBCH decoding for the first dominant interference cell. For the second interference cell, considering the real network deployment, UE can assume channel bandwidth can be the same for all LTE interference cells. Proposal 1: Define another set of test setup with only inter-RAT MO configured, for the UE supporting CRS-IM without the assistance of network signalling on LTE channel bandwidth, and no requirement difference with the test the new NWA signalling on LTE CBW configured. Proposal 2: For the test with only inter-RAT MO configured, supportive for TE to start calculating the PDSCH TP after N inter-RAT measurement periods. We support N=4 which is a common NW configuration. Observation 2: CRS-IM under 2Tx CRS interference can achieve similar performance gain compared with 4Tx interference under FDD condition, and can achieve even larger performance gain under TDD condition. Observation 3: 2Tx is very common network configuration in practical and it is the scenario in our deployment. Proposal 3: Support to use 2Tx for scenario 1. Observation 4: For 2Tx cases, there is only 12 REs per PRB which are occupied by the serving cell CRS, which means overhead of 12 can already ensure the effective code rate will not increase by serving cell CRS rate matching. Proposal 4: In case 2Tx CRS port is agreed for scenario 1, we propose to use overhead of 12. Observation 5: In Rel-16 LTE-NR coexistence tests, no user data is scheduled on slots with LTE PBCH/PSS/SSS. And LTE PBCH/PSS/SSS may also impact the PDSCH performance. Proposal 5: Not to schedule PDSCH data on slots with LTE PBCH/PSS/SSS for both scenario 1 and scenario 2. |
| R4-2207803 | Apple Inc. | Observation #1: Demodulation requirements typically do not have inter-RAT MO configured. Observation #2: Max TP would be different between test cases with and without inter-RAT MO Observation #3: Impact of errors in detection of parameters with inter-RAT measurement may not be accounted for in simulation results. Observation #4: Test requirements for schemes with NWA signaling and with detection of parameters by inter-RAT measurements cannot be the same. Proposal #1: Do not define requirement in scenario 2 with only inter-RAT MO configured. |
| R4-2208259 | Nokia, Nokia Shanghai Bell | Capability#1 (Scenario 1) Observation 1: It is unclear if Capability #1 should directly state that UE can perform CRS-IM both with and without Rel-17 new RRC network assistant signalling. Observation 2: The statement that “UE can perform CRS-IM without Rel-17 new RRC network assistant signalling in scenario 1” opens for NWA also in scenario 1, it is unclear if requirements shall be created for the case where NWA is provided in scenario 1. Proposal 1: RAN4 to discuss if NWA shall be part of optional capability #1 description. Proposal 2: RAN4 to discuss if requirements shall be introduced for scenario 1 both with and without NWA signalling. Capability#2+Capability#3 (Scenario 2) Observation 3: It is currently not decided to introduce test-setup and/or requirements for inter-RAT MO configured. Observation 4: In case test-setup and/or requirements for inter. RAT MO configured is agreed, it should be discussed if a UE tested for one of the capabilities (capability#2 / capability#3) would not require being tested for the other capability. Proposal 3: In case test-setup and/or requirements for inter-RAT MO configured are agreed, a UE tested for either capability#2 or capability#3 can claim support for the capability not tested. |
| R4-2208260 | Nokia, Nokia Shanghai Bell | Test setup for 15 kHz SCS scenario Test setup for scenario 2 Observation 1: In most cases, CBW of the first dominant interferer will be the same as for the 2nd interferer. Observation 2: It can be assumed, that with sufficiently high measurement accuracy that there is no critical performance loss with compared to NWA when the CBW is detected using InterRAT MO. Observation 3: We do not see it as relevant for the test, if the interferer CBW is detected by the UE before the scheduling of PDSCH. In addition, we assume there is no change in the interferer CBW during the test run. Observation 4: In case the CBW is detected with sufficiently high measurement accuracy by the UE, there is little to no difference between testing with InterRAT MO and NWA, hence the same requirements can be used for both. Proposal 1: Define only one set of requirements for scenario 2 for both InterRAT MO and NWA. The requirements shall be derived from simulations assuming NWA. Proposal 2: Define test cases for both InterRAT MO and NWA. Proposal 3: With one set of performance requirements and if a UE supports both InterRAT MO and NWA for CRS-IM, it is only required that the UE pass the requirements for one of the test cases. Tx antenna and LTE CRS port number Observation 5: 2 CRS ports are the most common setup used by networks. Proposal 4: Use 2 CRS ports in case simulations show no difference between 2 CRS and 4 CRS ports for scenario2. |
| R4-2208261 | Nokia, Nokia Shanghai Bell | Draft CR for 38_101-4 CRS-IM 15KHz SCS Scenario - General and applicability sections |
| R4-2208420 | CMCC | Draft CR on TDD PDSCH CRS-IM demod requirements for Scenario2 with overlapping spectrum for LTE and NR 15kHz SCS |
| R4-2208421 | CMCC | Proposal 1: Also define the set of test setup with only inter-RAT MO configured. Proposal 2: The same test requirements for CRS-IM can be applied in the two sets of test setup. Proposal 3: During the beginning of the test, N x inter-RAT measurement period is needed for UE to acquire LTE channel bandwidth before PDSCH scheduling of serving cell. Proposal 4: Inter-RAT MO is only configured during the period of N x inter-RAT measurement. |
| R4-2209147 | ZTE Corporation | Proposal 1: It is necessary to define the other set of test set up with only inter-RAT MO configured. Proposal 2: N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration and N is 4. Proposal 3: The inter-RAT MO is only configured during the beginning of the test. |
| R4-2209404 | China Telecom | Simulation assumptions for CRS-IM (for 15kHz FDD and TDD) |
| R4-2209410 | China Telecom | Draft CR on adding FRC for CRS-IM 15kHz SCS test requirements |
| R4-2209411 | China Telecom | Draft CR on FDD PDSCH CRS-IM demod requirements for DSS Scenario |
| R4-2209692 | Ericsson | Proposal 1: Define the other set of test setup with only inter-RAT MO configured. Proposal 2: Assume no error in LTE CBW detection. Proposal 3: Apply same test requirements for two sets of test setup. Observation 1: For the testing of the other test setup with inter-RAT MO configured, UE needs time to do the blind detection to acquiring LTE CBW. In this case, the Network needs to consider the measurement period before UE starting CRS-IM receiver. Observation 2: RRM session has already defined the measurement time as TMeasure, E-UTRAN FDD. Proposal 4: Follow the RRM definition and consider the whole timing for UE to acquire neighboring LTE cells CBW to be the TMeasure, E-UTRAN FDD = 2s Proposal 5: Consider such scheduling time to be as one of the applicability rules and captured in the specification. Proposal 6: Capture the time needed for UE to acquire LTE channel bandwidth as N x inter-RAT measurement period into the LS. Observation 3: there is reasonable gain (>1dB) for 2Tx, 2Rx and 2Tx, 4Rx. |
| R4-2209694 | Ericsson | Simulation results for CRS-IM |
| R4-2209695 | Ericsson | Draft CR on TDD PDSCH CRS-IM demod requirements for DSS Scenario |
| R4-2209794 | MediaTek Inc. | Observation 1: The performance gap between LLR weighting and Rel-15 CRS serving cell rate matching is quite small for both 2T2R and 2T4R cases. Proposal 1: We propose not to define CRS-IM receiver requirements for the case of 2 CRS ports. |
| R4-2209795 | MediaTek | Draft CR to TS38.101-4, interference model for CRS-IM receiver |
| R4-2209815 | Huawei, HiSilicon | Observation 1: If only test with NWA is introduced, the type of UEs not relaying on NWA will can’t be tested. Proposal 1: Introduce the test with inter-RAT MO configured. Proposal 2: Use Gap pattern 0 for CRS-IM scenario 2 requirements definition. Proposal 3: The test design should satisfy the RRM requirements that UE shall be able to identify a new detectable FDD cell within TIdentify, E-UTRAN FDD (6240ms for gap pattern 0) for FDD or T_(Identify,E-UTRAN TDD) (6240ms for gap pattern 0) for TDD Observation 2: It was agreed inter-RAT MO configuration is one of signalling to inform UE enable CRS-IM, there is the risk that UE will disable CRS-IM during the test that if inter-RAT MO is only configured during the beginning of the test. Proposal 4: Configure inter-RAT MO through the test. Observation 3: Considering the limitation of MGL and MGRP, UE can only perform PBCH decoding one time per 40ms. Observation 2: The PBCH BLER of stronger interference cell is about 0 for both 4T2R and 4T4R. The PBCH BLER of weaker interference cell is 0.1405 for 4T2R and 0.0149 for 4T4R. Proposal 5: Set time for decoding PBCH to 500ms to ensure the probability that UE decode weaker cell’s PBCH successfully is at least 99.99% Proposal 6: PDSCH is scheduled after TIdentify, E-UTRAN FDD+500ms for FDD and TIdentify, E-UTRAN TDD+ 500ms for FDD Proposal 7: Reuse the requirement of case with NWA for case with inter-RAT MO. |
| R4-2209816 | Huawei, HiSilicon | Simulation results for CRS-IM with 15kHz SCS Proposal 1: Only consider 2 CRS ports for scenario 1. |
| R4-2209817 | Huawei, HiSilicon | Draft CR: Introduction of CRS-IM receiver for 15kHz SCS FDD scenario 2 |
| R4-2210191 | Qualcomm Incorporated | Proposal 1: Do not define the test setup with only inter-RAT MO configured. |
| R4-2209738 | Nokia, Nokia Shanghai Bell | Draft CR for 38_101-4 Abbreviations section |

Open issues summary

Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2

Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured

- Status in RAN#102-e in the WF R4-2207239

  - Define one set of test setup with the new NWA signalling on LTE CBW configured.

  - FFS whether to define the other set of test setup with only inter-RAT MO configured:

    - FFS whether the same test requirements for CRS-IM can be applied in the two sets of test setup, considering that:

      - Whether or not to assume no error in LTE CBW detection: 1) based on PBCH decoding and/or power detection for the two interferers with different power level, or 2) based on PBCH decoding and/or power detection for the first dominant interferer.

      - TE does not start PDSCH scheduling of serving cell until UE acquires LTE channel bandwidth, further discuss the time needed for UE to acquire LTE channel bandwidth:

    - Option A: N x inter-RAT measurement period where N is the number of inter-RAT measurement configuration. One candidate value for N is 4, and other values are not precluded. FFS for the inter-RAT measurement period.

    - Other options are not precluded

      - Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test

- Proposals on whether to define the test with only inter-RAT MO configured:

  - Option 1: Yes (China Telecom, Nokia, CMCC, ZTE, E///, Huawei)

    - CTC, ZTE, HW: It is important to have a test setup to verify the UE performance of obtaining the CHBW information by power detection and/or PBCH decoding.

  - Option 2: No (Apple, QC)

    - QC: It mixes demod and RRM aspects significantly and requirements are expected to be similar to the test defined with NWA signaling.

- Recommended WF

  - Further discuss and aim to make decision in the first week of the meeting

Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced)

- Proposals

  - Option 1: Yes (China Telecom, Nokia, CMCC, ZTE, E///, HW)

    - CTC, Nokia, CMCC: UE can always ensure high reliable PBCH decoding for the first dominant interference cell. For the second interference cell, UE can assume channel bandwidth be the same for all LTE interference cells, which is aligned with the default assumptions of the same CHBW for LTE cells in scenario 1.

    - ZTE, E///: It is reasonable to assume no error for PBCH decoding

    - HW:

      - UE can assume reliable PBCH decoding. For the second interference cell, the PBCH can be decoded successfully with probability of 99.99% assuming 5 independent PBCH transmissions.

      - UE can also do blind detection of LTE CHBW without PBCH decoding, which can achieve the similar LLR weighting performance compared to with new NWA on LTE CHBW.

  - Option 2: No (Apple)

    - Apple: Max TP would be different between test cases with and without inter-RAT MO, as inter-RAT MO would result in some unavailable subframes for PDSCH transmission. Impact of errors in detection of parameters with inter-RAT measurement may not be accounted for in simulation results.

- Recommended WF

  - Two aspects are considered:

    - Regarding the impact of errors in LTE CHBW information acquisition by PBCH decoding and/or power detection, majority companies think it is reasonable to assume error-free LTE CHBW information acquisition. Can we go with majority companies’ view?

    - Regarding the unavailable slots for NR PDSCH transmission, it is discussed in Issue 2-1-3.

Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced)

- Proposals

  - Option 1: TE to wait N x inter-RAT measurement periods before starts PDSCH scheduling where N is the number of inter-RAT measurement configuration, and N=4 (China Telecom, CMCC, ZTE)

  - Option 2: PDSCH is scheduled after TIdentify, E-UTRAN FDD+500ms for FDD and TIdentify, E-UTRAN TDD+ 500ms for FDD, where 500ms is the time for UE to decode cell 2 PBCH within 5 coherent times (100ms for TDLA30-10) to reach 99.99% (1 - 0.145) PBCH accuracy (Huawei)

    - According to the RRM requirements, TIdentify, E-UTRAN FDD is 3840ms for gap pattern 0 for FDD and TDD.

    - With the above, PDSCH is scheduled after 3840 +500ms = 4340ms for FDD and TDD.

  - Option 3: Follow the RRM definition and consider the whole timing for UE to acquire neighboring LTE cells CBW to be the TMeasure, E-UTRAN FDD = 2s, and consider such scheduling time to be as one of the applicability rules and captured in the specification, and capture the time needed for UE to acquire LTE channel bandwidth as N x inter-RAT measurement period into the LS (E///)

- Recommended WF

  - There are 3 aspects to be discussed:

    - 1) In the test with only inter-RAT MO configured for scenario 2 (if introduced), is it agreeable to schedule NR PDSCH and measure the throughput after a certain time period?

    - 2) The length of the time period: 4.34 s (in option 2) or 2 s (in option 3)?

    - 3) Is it necessary to send LS to RAN2 and to define a timer based on the time period needed?

Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test

- Proposals

  - Option 1: Inter-RAT MO is only configured during period of N x inter-RAT measurement (CMCC, ZTE)

  - Option 2: Configure inter-RAT MO through the test (Huawei)

    - HW: Taking into account it was agreed inter-RAT MO configuration is one of signalling to inform UE enable CRS-IM, there is the risk that UE will disable CRS-IM during the test that if inter-RAT MO is only configured during the beginning of the test.

- Recommended WF

  - Encourage comments to each of the options

  - For proponent of option 2, it is encouraged to feedback whether there will be unavailable slots for NR PDSCH throughout the whole test.

Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2

- Proposals on the test applicability in case it is agreed define two sets of test setup for scenario 2

  - Option 1: If a UE supports both InterRAT MO and NWA for CRS-IM (Capability #2 and Capability #3), it is only required that the UE pass the requirements for one of the test cases (Nokia)

- Recommended WF

  - Encourage feedback

Sub-topic 2-2: Test parameters

Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1

- Status in RAN#102-e in the WF R4-2207239

  - For scenario 1, companies to bring simulation results for both 2 CRS and 4 CRS ports, and further decide whether to define requirements for 2 CRS or 4 CRS ports in the next meeting based on the performance gain.

  - For scenario 2, only cover 4 CRS ports

- Companies’ observations based on CRS-IM performance for 2Tx

  - CTC, Huawei: CRS-IM under 2Tx CRS interference can achieve similar performance gain compared with 4Tx CRS interference for FDD, and can achieve larger performance gain for TDD.

  - E///: There is reasonable gain (>1dB) for 2Tx, 2Rx and 2Tx, 4Rx

  - MTK: The performance gap between LLR weighting and Rel-15 CRS serving cell rate matching is quite small for both 2T2R and 2T4R cases

- Proposals

  - Option 1: 2 CRS ports (CTC, Nokia, Huawei)

  - Option 2: 4 CRS ports (MTK)

- Recommended WF

  - For scenario 1, can we go with 2 CRS ports based on the majorities’ view?

Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1

- Agreement in RAN#101-e in the WF R4-2120705

  - Overhead as 18 when Rel-15 CRS-RM is configured for the target cell (Scenario 1), and overhead as 0 for scenario 2.

- Proposals

  - Option 1: In case 2Tx CRS port is agreed for scenario 1, use overhead of 12. (CTC)

    - CTC: For 2Tx cases, there is only 12 REs per PRB which are occupied by the serving cell CRS, which means overhead of 12 can already ensure the effective code rate will not increase by serving cell CRS rate matching.

    - CTC, HW, E///, MTK: 12 is used in the simulation for 2 CRS ports.

- Recommended WF

  - Is option 1 agreeable?

Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS

- Proposals

  - Option 1: Not to schedule PDSCH data on slots with LTE PBCH/PSS/SSS for both scenario 1 and scenario 2. (CTC)

    - CTC: In Rel-16 LTE-NR coexistence tests, no user data is scheduled on slots with LTE PBCH/PSS/SSS. LTE PBCH/PSS/SSS may also impact the NR PDSCH performance.

- Recommended WF

  - Encourage feedback.

Sub-topic 2-3: UE capability description and testing for scenario 1

Issue 2-3-1: UE capability description and testing for scenario 1

- Related agreement in the approved LS in R4-2207238

  - With the above default network configuration assumptions, RAN4 has agreed that for UE supporting Capability #1, the UE can perform CRS-IM without Rel-17 new RRC network assistant signalling in scenario 1. For UE supporting Capability #2, the UE can perform CRS-IM without Rel-17 new RRC network assistant signalling in scenario 2 with 15 kHz SCS when MeasObjectEUTRA IE is configured and the configured measurement gaps overlap with neighbour LTE cell PBCH position.

  - Meanwhile, new RRC based network assistant signalling is agreed to be introduced to assist CRS-IM in Rel-17, and the Rel-17 new RRC signalling are optionally to be indicated to UE supporting Capability #1 and Capability #2.

- Proposals:

  - Proposal 1: RAN4 to discuss if NWA shall be part of optional capability #1 description (Nokia)

    - Nokia: It is not clear, if capability#1 should state that it can work also with NWA in the capability section.

| Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS ([Capability #1]), with and without the assistance of network signaling on LTE channel bandwidth | FR1 FDD | PDSCH | Clause 5.2.2.1.x Clause 5.2.3.1.x | UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. |
| --- | --- | --- | --- | --- |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.x Clause 5.2.3.2.x |  |

  - Proposal 2: RAN4 to discuss if requirements shall be introduced for scenario 1 both with and without NWA signalling. (Nokia)

- Recommended WF

  - Encourage feedback

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2 Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 Sub-topic 2-2: Test parameters Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS Sub-topic 2-3: UE capability description and testing for scenario 1 Issue 2-3-1: UE capability description and testing for scenario 1 |
|  | Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2 Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 Sub-topic 2-2: Test parameters Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS Sub-topic 2-3: UE capability description and testing for scenario 1 Issue 2-3-1: UE capability description and testing for scenario 1 |
| Huawei | Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configure We strongly support option 1. As proposed in our contribution, we should introduce one test case for UE supporting CRS-IM in scenario 2 without NWA. Such UE will not be verified if no test is introduced. As for some companies’ concern that it will mix the requirements with RRM and demod. We summary the test setup as follows to make more clear. The whole test can be divided to two period. - The measurements Gap is configured through the test. - Period 1 is used for identifying the LTE cells, measurement and performing PBCH decoding, without PDSCH transmission. UE shall get the cell ID and CBW information after period 1. - Period 2 is used for PDSCH decoding, PDSCH will be transmitted in all DL slots except for slots in measurements gap which means slots in measurement gap are not used for ACK/NACK statistics. Although absolute TP is reduced in such test, our requirements is targeted for relative throughput which means the target SNR will not be affected. If we set feasible duration for period 1 to ensure UE identify the LTE cells correctly and acquire correct CBW information, there is no difference from demodulation view compared to test with NWA except for some slots not used for PDSCH transmission which is not a big issue as analysis above. Also, measurement gap in Period 2 is used to indicate UE the presence of LTE cells and keep the CRS-IM open, we don’t define any measurement requirements in Period 2. Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) Based on our understanding, there are following difference for test with inter-RAT MO configured compared to test with NWA. - Errors of identifying LTE cells - Errors of decoding PBCHs - Unavailable slots for PDSCH transmission As analysed in Issue 2-1-1. 1) and 3) can be ignored by setting feasible duration for Period 1. For 2), as we discussed in issue 2-1-3, it can also be ignored if the decoding times is enough. Therefore, we think the requirements can be reused from NWA test. Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) As proposed in our contribution, Extra time for CBW information detection includes duration of identifying LTE cells and PBCH decoding. The duration of identifying LTE cells which is specified by RRM can be denoted as follows: TIdentify, E-UTRAN FDD=TBasicIdentify∙480TInter1∙CSSFinterRAT ms, TBasicIdentify=480ms CSSFinterRAT=1 TInter1=60 (For Gap Pattern ID0) (In Table 9.4.1-1 of TS 38.133) TIdentify, E-UTRAN FDD=3840ms For PBCH decoding, our understanding is it is more feasible to make UE acquire both CBW information from PBCHs. I.e. UE will successfully decodes the two PBCHs. It may can’t be guaranteed that all LTE cells have same CBWs, especially for the boundary between two regions. Some companies proposed to follow the scenario 1 that all neighbouring cells have same CBW. Our concern is that for DSS scenario, NWA information can be indicated to UE if default channel bandwidth assumption is not valid. However, for scenario 2, NWA information for CBW can’t be transmitted to UE not relying on NWA if neighbouring LTE cells don’t have same CBW. For times of decoding PBCH, we have provided our simulation results and analysis in our contribution that 500ms is enough to make UE decode PBCH of weaker cells successfully. Therefore, we propose to set the total time for cell identification and PBCH decoding to 4.5s. Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test We support option 2. Based on our understanding, inter-RAT MO configuration is used to signal UE to open CRS-IM in scenario 2 for UE not relying on NWA(As per agreement from last meeting). UE can decode PBCH and perform measurements in each gaps to track the updates of interferences and adjust algorithm. If inter-RAT MO is closed during the test, UE will lost all the information of neighbouring cells and probably close the CRS-IM. Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 Option 1 is OK for us. Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 Prefer option 1 since more gain can be observed for TDD Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 Option 1 is fine for us Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS Option 1 is fine for us. Issue 2-3-1: UE capability description and testing for scenario 1 For proposal 1, we don’t think red part is necessary. As per agreement in approved LS, support CRS-IM in DSS scenario can be equivalent to support CRS-IM in DSS scenario without NWA. For proposal 2, we don’t support introduce test with NWA in scenario 1. From the point of UE capability view, we have not introduced such a UE supporting NWA. |
| CMCC | Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2 Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured Support Option 1. The test setup with NWA can’t verify the UE behavior of obtaining the CBW configuration by power detection and/or PBCH decoding, it is necessary to define the test setup without NWA for UE with Capability #2 and Capability #4. Or else, in real network without new NWA configuration, the performance of this kind of UE can’t be guaranteed. Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) Support Option 1. when UE capable of Capability #2 and #4, CRS-IM performance can be guaranteed by following: 1. The first dominant PBCH can be decoded correctly with high SINR and PBCH combination. 2. For the second dominant PBCH, there is a high probability that PBCH can also be decoded correctly with PBCH combination. 3. If UE couldn’t obtain the CBW information of the second dominant cell, then a. It is typical case that the CBW configuration is aligned between LTE neighbouring cells. As long as UE know the bandwidth of one interference cell, then the CRS-IM performance can keep same, since the interference from second cell can be detected by power difference detection in LLR weighting processing. b. UE can also perform power detection to obtain the CBW information. With all above inurement, we think the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2. Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) We think Option 1 and Option 3 are similar options which UE should decode PBCH during N*TMeasure, E-UTRAN XDD. If UE has already detected these neighbouring cells, then we think this period is enough for UE to decode PBCHs, since multiple PBCHs can be detected and decoded during this period. While if UE hasn’t detected theses neighbouring cells, then we think Option 2 (TIdentify, E-UTRAN XDD +500ms) make more sense. Therefore, we prefer to use either ‘Option 1 + detected neighbouring cells’ or ‘Option 2 + undetected neighbouring cells’. Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test we prefer to configure inter-RAT MO during the beginning of the test. The inter-RAT MO is included in RRC-Reconfiguration message, we don’t expect frequent RRC-Reconfiguration message in real network. Another issue is whether MG is configured only during the beginning of the test or throughout the test. In order to avoid the CRS-IM suspend issue which raised by HW, we are fine with configure the MG throughout the test. PDSCH will not be transmitted in DL slots contains measurements gap. Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 We don’t prefer to introduce such applicability rule. If UE supports both Capability #2 and #3, then whether UE use capability#2 or capability #3 depends on network provide new NWA or not. Both scenarios are valid in real network, therefore, it is better to verify that UE can perform well under two scenarios. Sub-topic 2-2: Test parameters Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 We would like to try the compromise proposal: 2 Ports for FDD and 4 Ports for TDD. Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 Option 1. Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS We are ok with Option 1. Sub-topic 2-3: UE capability description and testing for scenario 1 Issue 2-3-1: UE capability description and testing for scenario 1 For UE capability description, we don’t support to add the clarification part, especially the “LTE channel bandwidth” part, which is not the main issue for Scenario 1. For testing, we think there is no need to define two test cases for scenario 1, since UE will follow the default assumption to obtain the CBW, and no additional error will be introduced of this UE behavior. |
| Apple | Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2 Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured We support Option 2 – not to introduce requirements with only inter-RAT MO configured. Our concern is that we are mixing RRM and demod requirements with such a test set up. If requirements are expected to be the same for the 2 UE capabilities, we don’t see why we need to define requirements with Capability #2. Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) We prefer to define requirements with NWA for CBW configured and the requirements will also be applicable to UE supporting capability #2. <Added 05/11 17:00 UTC> Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) - 1) In the test with only inter-RAT MO configured for scenario 2 (if introduced), is it agreeable to schedule NR PDSCH and measure the throughput after a certain time period? - Sounds reasonable - 2) The length of the time period: 4.34 s (in option 2) or 2 s (in option 3)? - We need to further check. The MG configuration in the start would only allow for a measurements during gaps. Need to account for measuring only during gaps – every 20ms or 40ms based on the MGRP. - 3) Is it necessary to send LS to RAN2 and to define a timer based on the time period needed? - We don’t think this is necessary. Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test Inter-RAT MO is configured by RRC, do we expect to have RRC re-configuration during the test to disable the inter-RAT MO? Also, in scenario 2 inter-RAT MO is the assumption if no other NWA is configured to indicate presence of interfering LTE cells. So the inter-RAT MO should be enabled throughout the test. <End Added 05/11 17:00 UTC> Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 We prefer to define requirements with NWA for CBW configured and the requirements will also be applicable to UE supporting capability #2. Do we expect UE to signal both capability #2 and #3? How does NW interpret such UE capability? Sub-topic 2-2: Test parameters Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 We are fine with the recommended WF. Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 We are fine with the recommended WF. Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS We support Option 1. Sub-topic 2-3: UE capability description and testing for scenario 1 Issue 2-3-1: UE capability description and testing for scenario 1 For DSS scenario we have agreement that the UE assumes interference CBW is the same as serving cell, if no additional NWA is configured, so we don’t see why this need to be added. Since for scenario 2 there is no serving cell config, UE needs to depend on either NWA or its own detection for LTE CBW. Is the NWA signalling in proposal #2 for LTE CBW or other parameters? |
| China Telecom | Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2 Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured We support option 1. Otherwise, the UE capability #2 and #3 cannot be differentiated by RAN4 requirements. Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) Yes. For the slots with NR PDSCH scheduling, the data throughput in the two test setups is the same. Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) Our views on the 3 aspects: - 1) In the test with only inter-RAT MO configured for scenario 2 (if introduced), is it agreeable to schedule NR PDSCH and measure the throughput after a certain time period? OK - 2) The length of the time period: 4.34 s (in option 2) or 2 s (in option 3)? The difference between option 2 and 3 is whether or not the cell identification time is accounted. In our understanding, time/frequency synchronization from cell identification procedure is needed to perform correct PBCH decoding. So, option 2 is preferred for the test setup. - 3) Is it necessary to send LS to RAN2 and to define a timer based on the time period needed? We don’t see a clear need for a new timer, and would like to also hear other companies’ view. Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test Our preference is option 1 (i.e., inter-RAT MO is only configured during PBCH decoding/detection). For option 2, with measurement gap always exists in the period for PDSCH throughput statistics, the test will become complicated. One middle way would be to not configure measurement gap (although inter-RAT MO is always configured) in the period for PDSCH throughput statistics. Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 Option 1 is ok, and our preference is to test capability #2 (CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth) if UE supports both. Sub-topic 2-2: Test parameters Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 Option 1. As far as we know, 2 CRS port is the most typical configuration for DSS scenario with legacy LTE deployment. We can also accept CMCC’s proposal: 2 Ports for FDD and 4 Ports for TDD. We understand their intention that 4 CRS ports can be used for LTE TDD scenarios due to higher frequency. Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 Option 1 Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS Option 1 Sub-topic 2-3: UE capability description and testing for scenario 1 Issue 2-3-1: UE capability description and testing for scenario 1 For scenario 1, we don’t support to introduce one more test with Rel-17 new NWA signalling. The new NWA signalling is needed only when the default network configurations are not valid. However, in the current test setup for scenario 1, all the default configurations are assumed. |
| Qualcomm | Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2 Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured Support Option 2. We do not want to mix demod and RRM aspects. Given that most companies agree that the requirements will be the same for both capabilities, we do not want to introduce a complex test setup. Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) Yes, if PBCH decoding is given enough time. However, for power detection method, it’s reliability will depend on UE implementation and may have some errors. Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) - Yes. We need to provide some time to UE to decode PBCH reliably. During that time, we cannot schedule PDSCH. - The length of time period should be decided based on feedback from RRM experts. - There is no need to define a timer in RAN2 because we are only focusing on demod requirements and there will be no RRM requirements for this feature. Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test In our opinion, it should be throughout the test so that UE doesn’t stop doing CRS-IM in the middle of the test because of no IRAT meas gap configured for a long time. Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 There should be an applicability rule. We can decide the details after the decision on second test setup. Sub-topic 2-2: Test parameters Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 We still prefer 4 Tx ports because that case is more severe for UE since it needs to estimate interference on more symbols and apply mitigation on more symbols Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 Ok with Option 1. Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS Ok with Option 1. Sub-topic 2-3: UE capability description and testing for scenario 1 Issue 2-3-1: UE capability description and testing for scenario 1 We do not agree with such a change. It was agreed that for DSS scenario, default assumption is to use same LTE bandwidth as in rate matching configuration. If that assumption is not true, either BS can indicate the correct BW or performance degradation is expected. There is no expectation from UE to try to detect LTE BW without NWA in this case. So, “without NWA” contradicts that understanding. |
| ZTE | Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2 Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured Option 1 is ok. Based on our understanding, in last meeting ,we have defined two UE capabilities for scenario 2, one is support NWA signalling, another is support inter-RAT MO. Because these two UE capabilities have different ways to acquire LTE channel bandwidth, it is reasonable to introduce the test with only inter-RAT MO configuration. Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) UE can make sure sufficiently high measurement accuracy by channel bandwidth detection. And the PDSCH transmission always after UE acquire the LTE channel bandwidth. TE start calculating the PDSCH TP after N inter-RAT measurement periods. So there is no requirement difference with the test the new NWA signalling on LTE CBW configured. Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) - In the test with only inter-RAT MO configured for scenario 2 (if introduced), is it agreeable to schedule NR PDSCH and measure the throughput after a certain time period? We agree with this. - The length of the time period: 4.34 s (in option 2) or 2 s (in option 3)? From TS 38.133, it indicates the UE shall be able to identify a new detectable FDD cell within TIdentify, E-UTRAN FDD. So if UE identify a cell, it shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN FDD. UE needs total time is TIdentify, E-UTRAN FDD to decode PBCH. On the other hand, if UE has identified FDD cell, the total time is TMeasure, E-UTRAN FDD. In order to make sure UE can decode PBCH accurately, We think option 2 is reasonable in test. - Is it necessary to send LS to RAN2 and to define a timer based on the time period needed? There is no necessary send LS to RAN2. Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test We are OK with option 1. In our understanding, inter-RAT MO configuration is used to signal UE to open CRS-IM in scenario 2. If inter-RAT MO is configured during the beginning of the test, UE will open CRS-IM in all test. Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 Option 1 is ok, and we prefer to test capability #2. Sub-topic 2-2: Test parameters Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 We are ok with Option 1. Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 We support option 1. Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS Option 1 is OK. Sub-topic 2-3: UE capability description and testing for scenario 1 Issue 2-3-1: UE capability description and testing for scenario 1 For proposal 1, we don’t support to add the red part. In our understanding, UE have clearly way to acquire LTE channel bandwidth in DSS scenario. For proposal 2, there is no necessary to introduce for scenario 1 with NWA. In DSS scenario, we have clarify that UE will follow the default assumption to obtain the CBW. |
| Nokia, Nokia Shanghai Bell | Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2 Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured As using inter-RAT MO is agreed in previous meetings as a valid way to determine CBW of the interference cell, it will make sense to create test configurations for this feature. The requirements themselves can be shared between NWA and MO setups. This will also ensure that devices not supporting NWA are tested even though requirements for inter-RAT MO and NWA are decided to be the same. We are also open to add applicability rules to only test one of the configurations. Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) Our assumption is, that a UE will be able to detect the CBW information with sufficiently high measurement accuracy to not have performance loss with respect to NWA. Hence it is enough to for RAN4 to create requirements based on the NWA configuration. There might be a slight difference in performance values, which should be within acceptable levels to re-use requirements. Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) We recognise the need to give the UE some time to detect the CBW. Option1 seems to be the most strait forward solution based on RRM requirements. Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test It is not expected, that the CBW will change during the test. This means, that if a UE detect the CBW in the beginning, it is not required from the perspective of continuous monitoring the CBW to have inter-RAT MO configured during the test. We do however understand the point from HW, that there might be a risk, that a UE will disable CRS-IM in case it does not continuously detect inter-RAT MO during the test. Also, from a practical perspective the MO would stay configured as long as the UE stays in the NR cell. In the end, both options are fine from our perspective, hence UE vendors can propose the option which makes most sense for their implementations. Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 The requirements are intended to test the performance of the CRS-IM part of the implementation, not so much the CBW detection, which already covered in RRM with inter-RAT measurement requirements. As such we see it sufficient for a UE to pass either one or the other requirement; certainly, this should hold true in the case where the same requirements are shared between both CRS-IM setups. Concerning the question raised (among others) by Apple: “Do we expect UE to signal both capability #2 and #3? How does NW interpret such UE capability?” We would expect that the UE signals all the capabilities of features it supports, with the NW then deciding which feature to use. I.e., the NW can then either use explicit NWA or not/MO, and in both cases the CRS-IM can be carried out at the UE side. Sub-topic 2-2: Test parameters Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 It remains our preference to reduce testing load by having requirements only for the most common deployment, i.e., 2 CRS ports. However, similar to prior agreements, we would like to hear if some companies have observed significant performance gains with 4 port CRS (e.g., larger than 1dB). Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 Agree to option 1, using Xoverhead of 12. Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS Option 1 seems to be inline with prior way of working. Sub-topic 2-3: UE capability description and testing for scenario 1 Issue 2-3-1: UE capability description and testing for scenario 1 Please let us quickly explain our thinking behind this proposal: We are in scenario 1 (i.e., CRS-RM is configured and DSS between NR serving cell and LTE interference cell), i.e., the UE is always able to employ CRS-IM based on the CRS-RM configuration alone. While writing the CRs it came to our mind that CRS-IM performance might differ, depending on if we additionally have CRS-IM Rel-17 NWA or not, since this new signaling potentially provides more information than CRS-RM alone. We wanted to bring this observation to the attention of the group, to decide if the same performance requirements should be applicable to both. In our opinion, yes, we should have the same requirement(s). And then make it clear in the applicability rules, that both cases are covered by the same requirement(s). However, we don’t have strong opinion here. As long as every company agrees that the performance does not differ, we are fine with not having the extension. |
| MediaTek | Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured We prefer Option 2, not to introduce requirements with only inter-RAT MO configured. Option 1 will increase the test complexity by considering inter-RAT MO and PBCH decoding. Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) If is agreed to define two sets of test setup and there will be no PBCH detection error, we are fine to define same CRS-IM test requirements. Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 We are OK to Option 1 if two test setups are introduced for scenario 2 Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 There is more gain for 4 ports case from our simulation results. However, we are OK to 2 ports. Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 We are OK to the recommended WF. Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS We are OK to Option 1. Issue 2-3-1: UE capability description and testing for scenario 1 We think it is not necessary to make such a clarification. There is only one capability for scenario 1 and NW will provide NWA only if it does not follow the default assumptions. |
| Ericsson | Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2 Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured We prefer option 1. We share similar view as Huawei that such kind of UE without NWA signalling should have chance to be tested. It gets RRM requirements involved, but we think it is clearer after the clarification on which RRM requirement will be referred and for what reason. RAN4 has already defined UE demodulation requirements considering RRM requirements; for example, HTS-DPS requirements is specified based on the assumption UE tracks the TCI which requires L1-RSRP measurements. Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced) We prefer option 1. Except the reliability of PBCH as mentioned before, we also leave some processing time for UE as discussed in issue 2-1-3. With the additional processing time for UE to do the blind detection and PBCH decoding, we believe that we can apply the same test requirement for two sets of test setup in scenario 2. Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced) As mentioned in CTC’s comments, we discuss over these three aspects: - 1) In the test with only inter-RAT MO configured for scenario 2 (if introduced), is it agreeable to schedule NR PDSCH and measure the throughput after a certain time period? We think it is necessary to schedule PDSCH data and collect the throughput (in the test) after a certain time period. It is very important to let the BS (and TE in the testing) knows when to start calculating the PDSCH throughput, otherwise the BS (TE in the testing) may start to schedule PDSCH before UE can acquire the LTE channel bandwidth information and in the worst-case UE cannot pass the CRS-IM tests. - 2) The length of the time period: 4.34 s (in option 2) or 2 s (in option 3)? We prefer to have a clear value for such period. We are fine with either option 2 or 3. Option 2 with longer time period seems a safer choice. - 3) Is it necessary to send LS to RAN2 and to define a timer based on the time period needed? Yes. We think it is necessary to add such information and description into the LS and send it to RAN2. Because we need to clarify it in the specification so that the network can have the same understanding as UE to schedule the PDSCH data after a certain period in the real case. Otherwise, the network does not have such assumption and no related content in the specification that can indicate the network when to start to schedule the PDSCH data if we only have such description in the RAN4 simulation assumption and test case description. Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test We support option 2, to configure inter-RAT MO through the test. Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2 We are fine with option 1. Sub-topic 2-2: Test parameters Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1 We are fine with option 1. Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1 We are fine with option 1. Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS We are fine with option 1. Sub-topic 2-3: UE capability description and testing for scenario 1 Issue 2-3-1: UE capability description and testing for scenario 1 For scenario 1, we are not in favor of introducing one more test with NWA signaling, as UE can refer to the default assumption. |
| Ericsson 2 | E///: Updated on Thursday For issue C, as agreed in Today’s GTW session, we will continue discuss it in the first round. ‘ We think there is a need to have such description into the LS and send it to RAN2 for information. We would like to first clarify that we don’t propose to define a new timer in demodulation discussion. We don’t propose to define new RRM requirements either. Our intention is to set a certain time value, which will be used for UE processing the CBW of LTE neighboring cells. And, such time value can be obtained by reusing the existing RRM requirements, like we mentioned in our previous comments and proposals. So, no new defined timer, no new define RRM requirement mixed with demodulation discussion. Just a reusing from the existing requirement to let the UE and the BS have the same understanding. The reason why we propose to capture this timer into the spec. is that the BS behaviour in the real network needs some guidances on when to start schdueling the PDSCH. If we only consider to capture this into the simulation assumption or test cases description, then what will the BS do in the real network? Even though in the testing, the TE knows it and will wait some periods like we discussed, the real BS does not have such guidance. That is the main reason we would like to have it in the spec. for indicating the BS behaviour.’ Our intention is to ask RAN2 to capture the conclusion of Issue 2-1-3 for the description of capability #2. Example (It is up to RAN2 which spec (TS38.331 or TS38.306) capture it) Capability #2: NR UE capable of performing CRS-IM in scenario 2 with 15 kHz SCS without Rel-17 new network assistant signalling on LTE channel bandwidth. UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbor LTE cell PBCH position. UE is expected to acquire neighbor LTE channel bandwidth information within the inter-RAT E-UTRA cell measurement period T (T is specified in TS38.133).. Capability #4: NR UE capable of performing CRS-IM in scenario 2 with 30kHz SCS without Rel-17 new network assistant signalling on LTE channel bandwidth. UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbor LTE cell PBCH position. UE is expected to acquire neighbor LTE channel bandwidth information within the inter-RAT E-UTRA cell measurement period T (T is specified in TS38.133).. |

Simuation assumption and CR comments collection

| tdoc number | Title/Content | Comment collection |
| --- | --- | --- |
| R4-2209404, CTC | Simulation assumptions for CRS-IM (for 15kHz FDD and TDD) | Moderator’s note: To be revised based on the discussion in section 2.2. |
| R4-2209738, Nokia | Draft CR 38_101-4 Abbreviations section |  |
| R4-2208261, Nokia | Draft CR General and applicability sections | CMCC: The Applicability notes part may need to be revised based on the discussion of Issue 2-1-5. |
|  |  | CTC: In our understanding, for UE only supporting Capability #2 or #3, we do not need extra applicability notes since each test case will simply apply to each capability. However, the last meeting’s LS did not exclude the possibility for UE to report supporting both Capability#2 and #3. In such cases, only test Capability #2 can verify the test purpose but not only testing Capability #3. Considering the above, we think the applicability note for Capbility #2 ‘[The requirements apply only if capability #3 is not tested]’ should be removed. |
|  |  | Qualcomm: We should have a separate table for Applicability rules if those are defined between Capability#2 and #3. Nokia: Thank you for checking our CR and the comment. However, we have some difficulty understanding what is meant by “a separate table”. Does this mean a separate row? Or could you give some more details of what you have in mind? Though, the question might not be relevant any longer after issue 2-1-5 has been decided. |
|  |  | Apple2: We don’t think both Capability #2 and Capability #3 can be reported by UE and the notes can be removed. |
|  |  | Nokia2: We have the following observation from the LS: The above capabilities are applicable for FR1 only without FDD/TDD difference. The above information from LS states no difference between FDD/TDD, however since there are different clauses for FDD and TDD, we have kept the FDD and TDD separate. |
| R4-2209411, CTC | Draft CR Requirements for FDD Scenario 1 | Qualcomm: We also need a note saying that MBSFN and CRS Muting are not configured on serving and Nbr LTE cells. |
|  |  | Apple2: Section numbers need to be aligned with other requirements. Suggest using X as we have many requirements introduced in 38.101-4 for PDSCH in this meeting from Rel-17, or we try to align between different email threads. |
| R4-2209695, Ericsson | Draft CR Requirements for TDD Scenario 1 | CTC: Suggest to align the legacy 101-4 test requirement format similar (duplex mode, TDD UL-DL pattern should be captured in the table, rank/MCS/overhead can only captured in the FRC, propagation channel/antenna config only captured in the test requirement table, etc) |
|  |  | Qualcomm: Need to align the test parameter tables with other sections. Some of the parameters need to be removed. Also, we need to mention that CRS Muting is not configured on serving or Nbr LTE cell. One of 2Tx or 4Tx will have to be removed based on the discussion. |
|  |  | Apple2: Align test parameter tables with other sections. Section numbers need to be aligned with other requirements. Suggest using X as we have many requirements introduced in 38.101-4 for PDSCH in this meeting from Rel-17, or we try to align between different email threads. Need to add note that MBSFN, CRS muting are not configured on serving or inf cells. |
| R4-2209817, Huawei, HiSilicon | Draft CR Requirements for FDD Scenario 2 | CMCC: We think the Test num in Table 5.2.2.1.17-5 and Table 5.2.3.1.17-5 should be 2-1, and they should also be included in Tests purpose. |
|  |  | CTC: 1) Duplex mode is missing in the test parameter table. 2) Add note to the INR definition reference 3) Interference modelling, rank probability, MBSFN config, TM, CRS muting config is missing in the LTE parameter table 4) The existing FRC cannot be reused if we agree not to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS |
|  |  | Qualcomm: Need to mention that LTE Nbr cells will not have MBSFN and CRS Muting configured. |
|  |  | Apple2: Align test parameter tables with other sections (TDD). Section numbers need to be aligned with other requirements. Suggest using X as we have many requirements introduced in 38.101-4 for PDSCH in this meeting from Rel-17, or we try to align between different email threads. Need to add note that MBSFN, CRS muting are not configured on inf cells. |
| R4-2208420, CMCC | Draft CR Requirements for TDD Scenario 2 | CTC: 1) The number of slots between PDSCH and corresponding HARQ-ACK information should be ‘Specific to each TDD UL-DL pattern and as defined in Annex A.1.2’ 2) MBSFN config, CRS muting config is missing in the LTE parameter table. 3) Suggest to use INR instead of ‘Interference power level’ since it will be defined in the annex. |
|  |  | Qualcomm: We also need a note saying that MBSFN and CRS Muting are not configured on Nbr LTE cells. |
|  |  | Apple2: Align the sub-section name, test purpose with FDD requirements. Align test parameter tables with other sections. Section numbers need to be aligned with other requirements. Suggest using X as we have many requirements introduced in 38.101-4 for PDSCH in this meeting from Rel-17, or we try to align between different email threads. |
| R4-2209410, CTC | Draft CR FRC for15kHz SCS | Qualcomm: For TDD RMCs, Slot indices for number of DMRS REs need to be same as the slots with grants. |
| R4-2209795, MediaTek | Draft CR interference model | CTC: The INR definition is duplicated with R4-2208258 for IRC testing modeling. |
|  |  | Qualcomm: “where is Ior(i)(j) is” should be “where Ior(i)(j) is”. “Noc” should be N_{oc}^{(j)}. Please replace all the TS references with reference number in Section 2 of 38.101-4. |
|  |  | MediaTek: Thanks for comments from CTC and QC. @CTC: We can refer to that sub-section. @Qualcomm: We will modify the draft CR according to the suggestion. |
|  |  | Apple2: We would like to check if the interference model assumes random selection of rank on a per sub-band basis or the rank is the same for the full band. We don’t recall agreement on per sub-band random rank from simulation assumption discussion. We think this should be in a separate section as for IRC we have interference from NR cells and here we have interference from LTE cells, its okay to reference the INR definition from that introduced for IRC. |

Summary for 1st round

Open issues

Sub-topic 2-1: Test with only inter-RAT MO configured for scenario 2

Issue 2-1-1: Whether to introduce the test with only inter-RAT MO configured

- Status in RAN#102-e in the WF R4-2207239

  - Define one set of test setup with the new NWA signalling on LTE CBW configured.

  - FFS whether to define the other set of test setup with only inter-RAT MO configured:

    - ……

GTW Agreement:

The baseline assumption: RAN4 will introduce the test case with only inter-RAT MO configured if the test feasibility with proper test set-up can be confirmed.

Issue 2-1-2: Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 (if introduced)

- GTW Agreement: RAN4 target to specify single set of requirements if two sets of test set-up introduced which is pending on further checking on the test set-up and the performance with power detection method.

Issue 2-1-3: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced)

Summary of round 1 discussion

- Issue A): In the test with only inter-RAT MO configured for scenario 2 (if introduced), is it agreeable to schedule NR PDSCH and measure the throughput after a certain time period?

  - Yes (CMCC, HW, China Telecom, QC, ZTE, E///, Apple)

- Issue B): The length of the time period

  - Option 1 & 3: 2s (E///, Nokia, CMCC& ZTE - for detected neighbouring cells)

    - Follow the RRM definition and consider the whole timing for UE to acquire neighboring LTE cells CBW to be the TMeasure, E-UTRAN FDD = 2s

  - Option 2: 4.34 s (Huawei, China Telecom, E///, CMCC & ZTE - for undetected neighbouring cells)

    - PDSCH is scheduled after TIdentify, E-UTRAN FDD+500ms for FDD and TIdentify, E-UTRAN TDD+ 500ms for FDD, where 500ms is the time for UE to decode cell 2 PBCH within 5 coherent times (100ms for TDLA30-10) to reach 99.99% (1 - 0.145) PBCH accuracy

    - According to the RRM requirements, TIdentify, E-UTRAN FDD is 3840ms for gap pattern 0 for FDD and TDD.

    - With the above, PDSCH is scheduled after 3840 +500ms = 4340ms for FDD and TDD.

  - Option 4: decided based on feedback from RRM experts (QC)

- Issue C): Is it necessary to send LS to RAN2 and to define a timer based on the time period needed?

  - Yes (E///)

    - E///: We need to clarify the time in the specification, so that the network can have the same understanding as UE to schedule the PDSCH data after a certain period in the real case.

  - No (China Telecom, QC, ZTE, Apple, Intel)

GTW Agreement on Thursday:

- Issue A: In the test with only inter-RAT MO configured for scenario 2 (if introduced), it’s agreeable to schedule NR PDSCH and measure the throughput after a certain time period

- Issue B: Further discuss the candidate options

- Further discuss in 1st round for Issue C.

Summary of round 1 discussion on Issue C:

- E/// elaborated further on the motivation and the proposal, but with no further feedback from companies showing concern in the email discussion and in GTW. Moderator would think the proposal is not agreeable.

Recommendation for 2nd round discussion:

- Following the GTW agreement, further discuss the candidate options for Issue B.

Issue 2-1-4: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test

Summary of round 1 discussion

  - Option 1: Inter-RAT MO is configured during the beginning of the test, and applied throughout the test. The measurement gap exists throughout the test, and PDSCH is not transmitted in DL slots in the measurements gap. (CMCC, HW, QC, [ZTE], Nokia, E///, Apple)

    - QC, HW: With option 1, UE doesn’t stop doing CRS-IM in the middle of the test because of no IRAT meas gap configured for a long time.

  - Option 2: Inter-RAT MO is configured during the beginning of the test. Meanwhile, not configure measurement gap (although inter-RAT MO is always configured) for the period for PDSCH throughput statistics (China Telecom)

Recommendation for 2nd round discussion:

- The difference between the two options is whether measurement gap exists in the second period. Is option 1 acceptable to all companies?

- If option 1 is agreeable, further discuss the measurement gap configuration in the test.

Issue 2-1-5: Test applicability if two test setups are introduced for scenario 2

- Summary of round 1 discussion

  - Option 1: If a UE supports both InterRAT MO and NWA based CRS-IM (i.e., Capability #2 and #3 for 15kHz SCS, Capability #4 and #5 for 30kHz SCS), it is only required that the UE pass the requirements for one of the test cases (Nokia, HW, China Telecom, ZTE, MTK, E///)

    - Option 1A: test capability #2 for 15kHz and capability #4 for 30kHz  (without the assistance of network signaling on LTE channel bandwidth) if UE supports both Inter-RAT MO and NWA based CRS-IM. (China Telecom, ZTE)

    - Question:

      - Apple question: If UE signal both capability #2 and #3, how does NW interpret such UE capability?

      - Nokia answer: UE signals all the capabilities of features it supports, NW then deciding which feature to use. i.e., the NW can then either use explicit NWA or MO

  - Option 2: Better to verify that UE can perform well under two scenarios. If UE supports both Capability #2 and #3, then whether UE use capability#2 or capability #3 depends on network provide new NWA or not. (CMCC)

Recommendation for 2nd round discussion:

  - Check if option 1 is agreeable. If so, is option 1A to test capability #2 or #4 agreeable?

Sub-topic 2-2: Test parameters

Issue 2-2-1: Tx antenna and LTE CRS port number for scenario 1

- Proposals on LTE CRS port number for scenario 1

  - Option 1: 2 CRS ports (CTC, [Nokia], Huawei, Apple, ZTE, MTK, E///)

  - Option 2: 4 CRS ports (MTK, QC)

  - Option 3: 2 Ports for FDD and 4 Ports for TDD (CMCC, CTC)

- GTW Agreement: Option 3 agreed

Issue 2-2-2: Overhead for TBS determination if 2Tx is used for scenario 1

- Tentative agreement:

  - Use overhead of 12 for 2Tx cases for scenario 1. (CTC, HW, CMCC, Apple, QC, ZTE, Nokia, MTK, E///)

Issue 2-2-3: Whether to schedule NR PDSCH data on slots with LTE PBCH/PSS/SSS

- Tentative agreement:

  - Not to schedule PDSCH data on slots with LTE PBCH/PSS/SSS for both scenario 1 and scenario 2. (CTC, HW, CMCC, Apple, QC, ZTE, Nokia, MTK, E///)

Sub-topic 2-3: UE capability description and testing for scenario 1

Issue 2-3-1: UE capability description and testing for scenario 1

- Tentative agreement:

  - Do not add the red part in proposal 1 (Huawei, CMCC, Apple, QC, ZTE, MTK)

| Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS ([Capability #1]), with and without the assistance of network signaling on LTE channel bandwidth | FR1 FDD | PDSCH | Clause 5.2.2.1.x Clause 5.2.3.1.x | UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. |
| --- | --- | --- | --- | --- |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.x Clause 5.2.3.2.x |  |

    - Nokia: As long as every company agrees that the performance does not differ, we are fine with not having the extension.

  - Do not include test with NWA in scenario 1 (Huawei, CMCC, CTC, ZTE, E///)

Simuation assumption and CR

| tdoc number | Title/Content | CRs/TPs Status update recommendation |
| --- | --- | --- |
| R4-2209404, CTC | Simulation assumptions for CRS-IM (for 15kHz FDD and TDD) | Revised to capture the new agreements. |
| R4-2209738, Nokia | Draft CR 38_101-4 Abbreviations section | Endorsed |
| R4-2208261, Nokia | Draft CR General and applicability sections | Revised |
| R4-2209411, CTC | Draft CR Requirements for FDD Scenario 1 | Revised |
| R4-2209695, Ericsson | Draft CR Requirements for TDD Scenario 1 | Revised |
| R4-2209817, Huawei, HiSilicon | Draft CR Requirements for FDD Scenario 2 | Revised |
| R4-2208420, CMCC | Draft CR Requirements for TDD Scenario 2 | Revised |
| R4-2209410, CTC | Draft CR FRC for15kHz SCS | Revised |
| R4-2209795, MediaTek | Draft CR interference model | Revised |

Discussion on 2nd round

Way forward

| tdoc number | Title/Content | Comment collection |
| --- | --- | --- |
| R4-2210659, Huawei, HiSilicon | WF on the test with only inter-RAT MO configured for scenario 2 |  |

Issue 1: Extra time for CHBW information detection in the test with only inter-RAT MO configured (if introduced)

–   Option 1 2s

o  Follow the RRM definition and consider the whole timing for UE to acquire neighboring LTE cells CBW to be the TMeasure, E-UTRAN FDD = 2s

–   Option 2: 4.34 s

o  PDSCH is scheduled after TIdentify, E-UTRAN FDD+500ms for FDD and TIdentify, E-UTRAN TDD+ 500ms for FDD, where 500ms is the time for UE to decode cell 2 PBCH within 5 coherent times (100ms for TDLA30-10) to reach 99.99% (1 - 0.145) PBCH accuracy

o  According to the RRM requirements, TIdentify, E-UTRAN FDD is 3840ms for gap pattern 0 for FDD and TDD.

o  With the above, PDSCH is scheduled after 3840 +500ms = 4340ms for FDD and TDD.

–   Other options are not precluded

| Company | Comments |
| --- | --- |
| Apple | We support Option 2. We are also checking internally to confirm the numbers, especially for TDD. The gap pattern would also need to be specified as part of the test setup. For PBCH decoding time we would need time to check and come back in next meeting since its based on 1 company’s simulation result. |
| Huawei | We support option 2. We think the time of identifying neighboring cells should be considered since tested UEs don’t have any information of neighboring cell at the beginning of test, After checking content in TS 38.133, we can conclude that TIdentify, E-UTRAN FDD =3840ms for FDD. However, for TDD, TIdentify, E-UTRAN FDD depends on measurement configuration: <image002.jpg> <image003.jpg> Hence, we should also define TDD measurement configuration in Table 9.4.3.2-1. We propose to use configuration 0. Since we use DDDSU as TDD pattern, so UE has more chance (3DL slots per 5ms) to do measurement compared to 2 DL slots per 5ms. So it is feasible to use TIdentify, E-UTRAN TDD =3840ms which can be calculated use the same formula as FDD. For PBCH decoding time, we are OK to collect more companies’ simulations. Can we set 500ms as baseline to move forward? It can be changed if some companies has technical concerns. |
| CMCC | Fine with Option 2 + undetected neighbouring cells. 500ms can be the baseline. |
| China Telecom | Fine with option 2, longer time for testing is safer. |
| Ericsson | We are fine with either option 1 or 2. Our intention for option 1 is to just consider the measurement time period which is clearly defined by RRM session. Such value of TMeasure, E-UTRAN FDD can be directly reused. But, we are also fine with option 2, since it gives more room for UE to do the PBCH decoding, which is apparently safer. We also propose to capture when TE start PDSCH scheduling in TS38.101-4. |
| Qualcomm | Ok with Option 2. |
| MediaTek | OK with Option 2. |

Issue 2: Whether the inter-RAT MO is only configured during the beginning of the test or throughout the test

o  Option 1: Inter-RAT MO is configured during the beginning of the test, and applied throughout the test. The measurement gap exists throughout the test, and PDSCH is not transmitted in DL slots in the measurements gap.

o  Option 2: Inter-RAT MO is configured during the beginning of the test. Meanwhile, not configure measurement gap (although inter-RAT MO is always configured) for the period for PDSCH throughput statistics.

o  Other options are not precluded

| Company | Comments |
| --- | --- |
| Apple | We support option 1. A longer time is configured in the beginning before PDSCH transmission starts. But inter-RAT MO is configured throughout the test |
| Huawei | Support option 1. Similar views as 1 round |
| CMCC | We are fine with Option 1. The measurement gap should cover PBCH from neighbouring cells, and we have already agreed no PDSCH scheduling in PBCH/SS slots. Therefore, no PDSCH scheduling in DL slots in the measurement gap is feasible. |
| China Telecom | Our preference is option 2, with simpler setup and PDSCH throughput statistics for the second period. Can compromise to option 1 if it is the majority view. |
| Ericsson | Similar view as CMCC. We are fine with option 1. |
| Qualcomm | Support Option 1. |
| MediaTek | OK with Option 1. |

Issue 3: Inter-RAT MO gap configuration

  Option 1: Use Measurement gap 0 (MGL=6ms, MGRP=40ms) in Table 9.1.2-1 to make measurement gap overlap LTE’s PBCH

  Option 1a:

o  Other options are not precluded

| Company | Comments |
| --- | --- |
| Apple | Option 1 is fine. With gap pattern 0 - the initial measurement time is 3840ms, but there will be a MG every 40ms. If we use gap pattern 1, the initial measurement time is 7680ms and the MG will be every 80ms. |
| Huawei | Option 1. In TS 38.133, RRM part defined 26 gap patterns which has been captured in Table 9.1.2-1 (See in appendix). Considering the scenario2 is NR standalone operation in serving cell, only pattern 0,1,2,3 can be used which is specified in Table 9.1.2-3 (See in appendix).However, gap pattern 2 and 3 need UE capability signalling according to TS 38.306. Meanwhile, comparing to gap pattern #0, gap# 1 need longer period which may need more absolute time for inter-RAT measurement. Hence, we propose to use Gap pattern 0 for CRS-IM scenario 2 requirements definition. |
| CMCC | OK with Option 1. |
| China Telecom | Fine with Option 1 |
| Ericsson | Fine with option 1. |
| Qualcomm | Ok with Option 1. |
| MediaTek | OK with Option 1. |

Issue 4: Test applicability if two test setups are introduced for scenario 2

o  Option 1: If a UE supports both InterRAT MO and NWA based CRS-IM (i.e., Capability #2 and #3 for 15kHz SCS, Capability #4 and #5 for 30kHz SCS), it is only required that the UE pass the requirements for one of the test cases. Otherwise, UE pass the requirements based on the corresponding capability

o  Other options are not precluded

| Company | Comments |
| --- | --- |
| Apple | If there is a possibility that UE signals support for both Capability #2 and Capability #3, the requirement with NWA should be applicable. In the actual deployment it may ve subject to what NWA is provided to it. But for the demod requirement, it should be verified with NWA provided. |
| Huawei | We prefer option 1. |
| CMCC | To move forward, we can compromise to Option 1. we propose Capability #2 and Capability# 4 to be tested if UE support both Capability #2 and #3 , Capability #4 and #5. |
| China Telecom | Option 1. We propose Capability #2 and Capability# 4 to be tested if UE support both Capability #2 and #3 , Capability #4 and #5. |
| Ericsson | Fine with option 1. Fine with CMCC and CTC’s proposal. |
| Qualcomm | Ok with Option 1. |
| MediaTek | OK with Option 1. |
| Nokia | Similar to Apple, we would prefer to have NWA verified in case both capabilities are reported. We are however open for compromise. |

Issue 5: Test configuration for 30kHz SCS

o  Option 1: Apply all agreements in issue1~4 for test with 30kHz SCS

o  Other options are not precluded

| Company | Comments |
| --- | --- |
| Apple | Fine as a baseline. Might need to check if anything is missing for 30KHz SCS case, since we only have TDD. |
| Huawei | We can set option 1 as baseline. |
| CMCC | We can set Option 1 as baseline, and further check if anything is missing. |
| China Telecom | Ok with option 1 |
| Ericsson | OK with option 1 |
| Qualcomm | Ok to consider Option 1 as baseline. Need to further check in next meeting if any issues with TDD. |
| MediaTek | We share the same view as Qualcomm. |

Draft CRs and Simulation assumptions

| tdoc number | Title/Content | Comment collection |
| --- | --- | --- |
| R4-2210923, CTC | Simulation assumptions for CRS-IM (for 15kHz FDD and TDD) |  |
| R4-2210919, Nokia | Draft CR General and applicability sections | Qualcomm: Corresponding draft CRs for these requirements have clause numbers as .X instead of .16 or .17. It will be good to keep it as .X for now and update it later at all the places together. |
|  |  | CMCC: We think the wording “Capability #X” in brackets can be deleted, since it is not the common name among specs. |
| R4-2210925, CTC | Draft CR Requirements for FDD Scenario 1 | Qualcomm: Similar to Apple’s comment for other draft CRs, for LTE parameters, include TM, precoding granularity, a and b settings. |
|  |  | CTC: Thanks for the comment. Revised in v01. Please note that LTE TM has already been included in v00. |
|  |  | CMCC2: According to the TS 36.101, the precoding granularity 6 PRBs may more suitable for BW 10MHz. Highly appreciated if other companies can help to check. |
|  |  | Qualcomm2: Based on 36.101, typical values for a and b are -3dB for 2Tx for TM4. Please correct it. |
| R4-2210927, Ericsson | Draft CR Requirements for TDD Scenario 1 | Apple: For LTE parameters, include precoding granularity, a and b settings. |
|  |  | Qualcomm: Suggest to align the test parameter tables Table 5.2.2.2.16-2 and Table 5.2.2.2.16-3 with FDD draft CR. There are some parameters missing compared to FDD draft CR. Also, position of 1st DMRS should be 3. Also, please set the clause number to 5.2.2.2.X for now since it needs to be aligned with other requirements. |
|  |  | CTC: 1) Suggest using ‘Network-based CRS interference mitigation’ defined in the LTE spec instead of ‘CRS muting’.2) TDD UL DL pattern should be captured in the minimum performance table, not in the test parameter table. 3) Use ‘INR’ instead of ‘Interference power level’ and add reference definition note. 4) Format of Table 5.2.2.2.16-2 and Table 5.2.3.2.16-2 seems not aligned with others. 5) Missing of PDCCH symbol number config in the serving cell test parameter table. |
|  |  | Qualcomm2: Serving cell rate matching configuration is missing in test parameters tables. Also, please change the clause number 5.2.3.2.16 to 5.2.3.2.X. Based on 36.101, typical values for a and b are -6dB for 4Tx for TM4. Please correct it. |
| R4-2210929, Huawei, HiSilicon | Draft CR Requirements for FDD Scenario 2 | Qualcomm: As commented in the 1st round, please add a note that LTE Nbr cells will not have MBSFN and CRS Muting configured. Also,Tables 5.2.y.1.X-4 and 5.2.y.1.X-5 have the same titles. Pleas update the titles to distinguish the requirements. |
|  |  | Apple: For LTE parameters, include TM, precoding granularity, a and b settings. |
|  |  | CMCC: It may be better to align the order of ‘requirement Table’ and corresponding descriptions between our CRs. We are open to discuss how to revise our CRs and any suggestions are appreciated. |
|  |  | CTC: 1) Use ‘INR’ instead of ‘’ and add reference definition note. 2) Remove the square brackets for LTE CRS ports Antenna ports [0,1,2,3]; 3) Align the requirement table title and the wording above the requirement table with CR on TDD Sce2. CMCC’s wording is clearer from our view. |
|  |  | CTC2: We would like to check other companies’ view that whether we should specify the NWA content (only CHBW information for each cell in our understanding based on the LS?) in the test requirement for capability#3. Otherwise, RAN5 may not know what should be signalled to the UE. |
| R4-2210921, CMCC | Draft CR Requirements for TDD Scenario 2 | Apple: In Table 5.2.2.2.X-1: Tests purpose – it should be for 2 receive antenna. For LTE parameters, include precoding granularity, a and b settings. |
|  |  | Qualcomm: Tables 5.2.y.2.X-4 and 5.2.y.2.X-5 have the same titles. Pleas update the titles to distinguish the requirements. |
|  |  | CMCC: Thanks for all comments! @Apple: the number of receive antenna has been fixed. The LTE parameters has been added. @Qualcomm: We change the titles to ‘ Minimum performance for Rank 1 without Rel-17 new network assistant signaling on LTE channel bandwidth’ and ‘Minimum performance for Rank 1 with Rel-17 new network assistant signaling on LTE channel bandwidth’ in order to distinguish the requirements, please help to check is it ok. |
|  |  | CTC: 1) For the requirement title, suggest to align with the wording to be specified in TS38.306. As far as I know, it will be specified in this RAN2 meeting as ‘with/without the assistance of network signaling on LTE channel bandwidth.’ So the suggested title is: Minimum performance for Rank 1 ‘with/without the assistance of network signaling on LTE channel bandwidth. 2) Suggest using ‘Network-based CRS interference mitigation’ defined in the LTE spec instead of ‘CRS muting’ |
|  |  | CMCC2: @CTC your comments have been captured in the v02. Thank you! Besides, we add another Note under LTE parameter’s Table. Please check is it needed or not. Note 4: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame |
|  |  | CTC2: 1) We would like to check other companies’ view that whether we should specify the NWA content (only CHBW information for each cell in our understanding based on the LS?) in the test requirement for capability#3. Otherwise, RAN5 may not know what should be signalled to the UE. 2) TDD UL-DL pattern should be captured in the Minimum performance requirement table. 3) Pls also update the extra sentence above Table 5.2.3.2.X-4 and Table 5.2.3.2.X-5 with the revised title. 4) We think Note 4 is necessary to be included. |
|  |  | CMCC3: For NWA content, we are fine with add it in RAN4 spec. However, which content should be covered may need discussion. We are fine with only cover CBW information. Other comments have been captured in v03. |
|  |  | Qualcomm2: Based on 36.101, typical values for a and b are -6dB for 4Tx for TM4. Please correct it. |
| R4-2210924, CTC | Draft CR FRC for15kHz SCS | Qualcomm: For TDD RMCs, Slot indices for number of DMRS REs need to be same as the slots with grants. |
|  |  | CTC: Does QC comment to also set the DMRS REs and PDSCH symbols as‘N/A’ for the LTE PSS/SSS/PBCH slots? (Like below) In that way we will also need to revise the existing FRCs for LTE-NR coexistence tests. |
|  |  | Qualcomm: Yes. As per our understanding, there will be no PDSCH DMRS scheduled on slots without PDSCH. So, we wanted to align the slot numbers. Do you see any issue with that? |
|  |  | CTC: Thanks for the comment. TDD FRCs are updated in v02. |
| R4-2210928, MediaTek | Draft CR interference model | Apple: Wording suggestion – “…associated interferer to noise ratio (INR) value as defined in [B.Y.1].” Is the last sentence required? The EPRE ratio of LTE CRS of i-th dominant interferer to NR SSS of serving cell shall be defined as INR-i/SNR, where Noc is same for serving cell and all interfering cells. What is SNR and Noc. They haven’t been defined or referenced here. |

Topic #3: CRS-IM for 30 kHz SCS scenario

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207803 | Apple | Observation #5: We don’t observe measurable gain with CRS-IM for 30KHz SCS. Table 1: Results in Non-DSS Scenario with 30KHz SCS Ant. Conf.; LTE Cell Loading; MCS; Disable CRS-IM; Enable CRS-IM; Gain with CRS-IM. 4x2; 10%; 13; 9.8; 9.0; 0.6. 4x4; 10%; 13; 6; 5.0; 0.8. Based on the results we observe that the gain with CRS-IM with 30KHz SCS for NR is ≤ 0.8 dB with 10% loading, with assumption that all parameters a known to the UE and not detected. Proposal #2: Do not introduce CRS-IM requirements for 30KHz SCS. |
| R4-2208050 | Intel Corporation | Observations #2: For Scenario 2 with 30 kHz SCS and 1+1 DMRS configuration - 10% loading: CRS-IM provides 1.4-1.8 dB performance improvement Proposal 1: Define CRS-IM requirements for scenario with 30 kHz SCS using 10% interference loading assumptions and 1+1 DMRS configuration. Proposal 2: Assume that the following information about interference structure is required for CRS-IM processing in scenario with 30 kHz SCS: LTE carrier frequency, bandwidth and CRS port number Proposal 3: For scenario with 30 kHz SCS use the same as for 15 kHz SCS procedure for obtaining the information about interference parameters required for CRS-IM processing. |
| R4-2208416 | CMCC | Discussion on the CRS-IM for NR 30kHz SCS Observation 1: When using CRS-IM receiver with LLR weighting algorithm, the performance gain is larger than 1dB under all simulation cases comparing with MMSE-IRC receiver. Proposal 1: Define CRS-IM requirements for scenario with 30 kHz SCS assuming 10% interference loading, 4 CRS ports and 1+1 DMRS configuration. Proposal 2: Approve the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario. Proposal 3: For the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario, reuse the configuration proposed in last meeting. |
| R4-2208417 | CMCC | Simulation results collection for 30kHz SCS CRS-IM |
| R4-2208418 | CMCC | LS on UE capability and network assistant signalling for CRS interference mitigation in the scenario with overlapping spectrum for LTE and NR with 30kHz SCS |
| R4-2208419 | CMCC | Draft CR for introduction of general applicability section of CRS-IM with serving cell 30kHz SCS in TS38.101-4 |
| R4-2209148 | ZTE Corporation | Draft CR for TS38.101-4 PDSCH Reference Channel for CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR |
| R4-2209693 | Ericsson | Observation 1: There is a 1.19dB gain from LLR weighting over the reference at PDSCH loading level 10%. Proposal 1: Define requirements for TDD 30kHz SCS scenario. Proposal 2: Consider following test cases for defining requirements: Test case; MCS; Channel; PDSCH loading level; Antenna configuration. 1; MCS13; TDLA30-10; 10%; 4x2. 2; 4x4. |
| R4-2209818 | Huawei, HiSilicon | Discussion on CRS-IM with 30kHz SCS Proposal 1: Define the performance requirements for 30 kHz SCS with LLR weighting. Proposal 2: Use option 1. I.e. S=6D+4G+4U and it is up to UE implementation whether perform CRS-IM in S slot. If simulation is not aligned, we can unify the UE behaviour in S slot. Table 2-1: Summary of simulation results for scenario 2 CRS port; Duplex mode; Antenna configuration; SNR (dB)@ 70% of max TP; Gain (dB). LLR weighting; Baseline. 4 CRS pots; TDD; 4T2R; 9.4; 7.8; 1.6. 4T4R; 6.4; 5.3; 1.1. |
| R4-2209819 | Huawei, HiSilicon | draftCR: Introduction of PDSCH requirements for CRS-IM scenario 2 with 30kHz |
| R4-2210003 | ZTE Corporation | Simulation results for 30kHz SCS CRS-IM Number of UE Rx antenna; Baseline receiver; CRS-IM using LLR weighting; Performance gain. 2 Rx; 9.8; 8.7; 1.11. 4 Rx; 6.2; 5.5; 0.7. |

Open issues summary

Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario

- Agreements in RAN4 #102e in the WF R4-2207240

  - Only define CRS-IM requirements for scenario with 30 kHz SCS assuming [10%] interference loading, 4 CRS ports and 1+1 DMRS configuration under the condition with enough performance discrimination between CRS-IM on and CRS-IM off i.e. at least 1dB performance difference observed

    - Channel BW: 20MHz

    - FFS on special slot configuration

    - Interested companies can bring results with Rel-15 rate matching (symbol level)

- Summary of simulation results

  - 4T2R, 10% LTE Cell Loading, MCS 13

    - 6 companies provided simulation results. More than 1 dB gain is achieved in the results from 5 companies, and more than 1 dB gain is achieved in the average of CRS-IM gain.

|  | Baseline receiver (dB) | LLR weighting (dB) | Gain with CRS-IM (dB) |
| --- | --- | --- | --- |
| Apple | 9.8 | 9.0 | 0.6 |
| Intel |  |  | ~1.4 |
| CMCC | 8.7 | 7.1 | 1.6 |
| E/// |  |  | 1.19 |
| Huawei | 9.4 | 7.8 | 1.6 |
| ZTE | 9.8 | 8.7 | 1.11 |
| Average |  |  | 1.25 |

  - 4T4R, 10% LTE Cell Loading, MCS 13

    - 5 companies provided simulation results. More than 1 dB gain is achieved in the results from 3 companies, and more than 1 dB gain is achieved in the average of CRS-IM gain.

|  | Baseline receiver (dB) | LLR weighting (dB) | Gain with CRS-IM (dB) |
| --- | --- | --- | --- |
| Apple | 6 | 5.0 | 0.8 |
| Intel |  |  | ~1.5 |
| CMCC | 6.0 | 3.5 | 2.5 |
| Huawei | 6.4 | 5.3 | 1.1 |
| ZTE | 6.2 | 5.5 | 0.7 |
| Average |  |  | 1.32 |

- Proposals on whether to define CRS-IM requirements for 30kHz SCS

  - Yes (Intel, CMCC, E///, HW)

  - No (Apple)

- Recommended WF

  - Considering the simulation results and positions from companies, is it agreeable to define CRS-IM requirements for 30 kHz SCS?

  - Meanwhile, companies are encouraged to discuss whether the performance difference is caused by different assumptions (e.g., whether PDSCH is scheduled in the special slot) or implementation (e.g., whether CRS-IM is enabled if PDSCH is scheduled in the special slot) in the special slot.

Issue 3-2: Special slot configuration for target cell with 30kHz SCS

- Agreements in RAN4 #102e in the WF R4-2207240

  - Use 7DS2U for the target cell with TDD 30kHz SCS

  - FFS the configuration in the special slot

    - Option 1: S=6D+4G+4U

    - Other Options are not precluded

- Proposals

  - Option 1: Use option 1, i.e. S=6D+4G+4U and it is up to UE implementation whether perform CRS-IM in S slot. If simulation is not aligned, we can unify the UE behavior in S slot. (HW)

- Recommended WF

  - Based on the FRC for Rel-16 TDD LTE-NR coexistence test, as well as the CRS-IM FRC submitted in this meeting, NR PDSCH is not scheduled in the special slot. So can we assume no PDSCH scheduling in the specials slot?

Issue 3-3: UE feature for 30 kHz CRS-IM

- Proposals

  - Option 1: Approve the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario. For the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario, reuse the configuration proposed in last meeting WF in R4-2207239. (CMCC)

| NR_perf_enh2_Demod | X-4 | CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_perf_enh2_Demod | X-5 | CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |

- Recommended WF

  - Is option 1 agreeable?

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario Issue 3-2: Special slot configuration for target cell with 30kHz SCS Issue 3-3: UE feature for 30 kHz CRS-IM |
|  | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario Issue 3-2: Special slot configuration for target cell with 30kHz SCS Issue 3-3: UE feature for 30 kHz CRS-IM |
| CMCC | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario We support to define CRS-IM requirements for 30kHz SCS. For performance of special slot, MMSE-IRC and CRS-IM algorithm perform both worse in our simulation results. For the CRS-IM implementation, we do power estimation for each subcarrier with whole bandwidth granularity. Besides, since the max gap between companies is not larger than 2dB, we think the results are valid. Issue 3-2: Special slot configuration for target cell with 30kHz SCS We prefer to use Option 1, which is typical configuration in real network. We are fine that whether to perform CRS-IM in S slot is up to UE implementation. Issue 3-3: UE feature for 30 kHz CRS-IM Support Option 1, which was discussed extensively in last meeting. |
| Apple | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario We don’t support to define requirements with CRS-IM for 30KHz SCS. We had agreement in last meeting for companies to submit results with and without CRS-IM to compare. Based on the collected simulation results, we see a large span in results with CRS-IM with 2RX. We don’t think there is a measurable gain with CRS-IM in 30KHx SCS and don’t see the benefit of defining requirements. Issue 3-2: Special slot configuration for target cell with 30kHz SCS We are fine with the recommended WF if we agree to define requirements. Issue 3-3: UE feature for 30 kHz CRS-IM FFS based on Issue 3-1. But we already have it in “[ ]” in LS to RAN2. |
| China Telecom | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario We support option 1. Issue 3-2: Special slot configuration for target cell with 30kHz SCS It is ok to assume no PDSCH scheduling in the specials slot. Issue 3-3: UE feature for 30 kHz CRS-IM We support option 1, and we need to send LS to RAN2 in the first week of the meeting. |
| Qualcomm | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario We only see ~0.6dB gain due to CRS-IM in our simulations. So, we prefer not to define such requirements. Issue 3-2: Special slot configuration for target cell with 30kHz SCS Ok with recommended WF. Issue 3-3: UE feature for 30 kHz CRS-IM If we agree to define the requirements for 30kHz, Option 1 is ok. |
| ZTE | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario We support option 1. Issue 3-2: Special slot configuration for target cell with 30kHz SCS We are ok with recommended WF. Issue 3-3: UE feature for 30 kHz CRS-IM We support option 1. |
| Nokia, Nokia Shanghai Bell | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario Looking at the contributed simulations from other companies, the gain seems to average around 1dB for both 2R an 4R. In our opinion, this gain is close to the threshold where we would usually start to consider inclusion on performance distinction alone. For this specific topic we don’t have a strong opinion either way. Issue 3-2: Special slot configuration for target cell with 30kHz SCS Currently no strong opinion. Issue 3-3: UE feature for 30 kHz CRS-IM If decided to introduce 30kHz SCS scenario requirements, we agree to option 1. |
| MediaTek | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario The average performance gain is closed to 1dB. We think the gain of LLR weighting for 30kHz is limited. Hence, we prefer not to define CRS-IM requirements for 30kHz SCS scenario. |
| Ericsson | Issue 3-2: Special slot configuration for target cell with 30kHz SCS We are fine with option 1 if introduced. Issue 3-3: UE feature for 30 kHz CRS-IM We are fine with option 1 if introduced. |
| Intel | Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario We support to define CRS-IM requirements for 30kHz SCS. An average of over 1dB gain is seen among the simulation results and 30kHz SCS is one of the important features for field network deployment. In the case that the UE is able to choose to support CRS-IM, we don’t see any reason not to specify requirement. Issue 3-3: UE feature for 30 kHz CRS-IM We are OK with option 1. |

CR/LS comments collection

| tdoc number | Title/Content | Comment collection |
| --- | --- | --- |
| R4-2208418, CMCC | LS for 30kHz SCS CRS-IM | Apple: We already sent LS to RAN2 in last meeting with most of the same content. We don’t see the necessity to send the same information again, but suggest to only convey new information based on agreements in RAN4. |
|  |  | CTC: We support the content of the LS. We have one editorial update suggestion: Since we are trying to send this LS within this week and RAN2 will have to update their CRs for the next week, we suggest simply state something like ‘RAN4 have reached same agreement on scenario 2 under 30kHz SCS, with scenario 2 under 15kHz SCS in terms of default network configuration assumptions, the content of the Rel-17 new RRC network assistant signalling, which is captured in last meeting LS R4-22xxxxx’, instead of repeating all the agreements, to try to reduce RAN2’s workload. |
|  |  | - Qualcomm: We suggest to add “, and should be signalled to UE supporting Capability #5 but not supporting Capability #4.” to the following. - Meanwhile, new RRC based network assistant signalling is agreed to be introduced to assist CRS-IM in Rel-17, and the Rel-17 new RRC signalling are optionally to be indicated to UE supporting Capability #4. |
|  |  | CMCC: Thanks for all comments @Apple @CTC: Our original consideration is providing more information to RAN2 since they don’t know our discussion details. However, your consideration also makes sense, and we are fine with it. we will delete the information which had been captured in last meeting LS. @QC: We prefer don’t add such sentence, since this sentence have been added under the sub-bullet “channel bandwidth”. We prefer to keep the same structure as last meeting, to avoid potential ambiguity in RAN2. |
|  |  | Ericsson: We share similar view with Apple. Perhaps, we can only include the additional content (30kHz SCS related) into the LS? Meanwhile, we propose to add the necessary information and description of the timer discussed in issue 2-1-3. |
| R4-2208419, CMCC | Draft CR for general applicability section | Apple: If agreed to define requirements, we suggest to add new rows to the end of the table. |
|  |  | Nokia: Agree with Apple’s comment, or is there a specific reason for this placement? |
|  |  | Nokia2: In the draftCR we have provided for the 15kHz scenarios, we have the following observation From the LS: The above capabilities are applicable for FR1 only without FDD/TDD difference. The above information from LS states no difference between FDD/TDD, however since there are different clauses for FDD and TDD, we have kept the FDD and TDD separate. We see here in the 30kHz draftCR, that only TDD is specified. We need to align on this and would appreciate feedback on our proposal of separating FDD and TDD. |
| R4-2209819, Huawei, HiSilicon | draft CR for PDSCH TDD requirements | CMCC: The test num in Table 5.2.2.1.19-5 and Table 5.2.3.1.19-5 should be 2-1 |
|  |  | Apple: Pending decision on introducing requirements. |
|  |  | Qualcomm: Need to mention that LTE Nbr cells will not have MBSFN and CRS Muting configured. Also, requirement tables need to be shrunken to fit within the Clause frame. |
| R4-2209148, ZTE | draft CR for FRC | Apple: Pending decision on introducing requirements. |
|  |  | Qualcomm: Allocated slots per frame is incorrect. It also depends on whether we need to skip scheduling for LTE PBCH/PSS/SSS. Peak thpt will change based on that allocation. Also, reference channel number should follow the existing pattern for other RMCs. |

Summary for 1st round

Open issues

Issue 3-1: Whether to define CRS-IM requirements for 30 kHz SCS scenario

- Agreements in RAN4 #102e in the WF R4-2207240

  - Only define CRS-IM requirements for scenario with 30 kHz SCS assuming [10%] interference loading, 4 CRS ports and 1+1 DMRS configuration under the condition with enough performance discrimination between CRS-IM on and CRS-IM off i.e. at least 1dB performance difference observed

    - Channel BW: 20MHz

    - FFS on special slot configuration

    - Interested companies can bring results with Rel-15 rate matching (symbol level)

- Agreement:

Define CRS-IM requirements for 30kHz SCS scenario

- Separate UE capability introducing for 30kHz SCS scenario (Capability #4, Capability #5)

- RAN4 will continue the effort on the alignment of simulation results from companies.

Issue 3-2: Special slot configuration for target cell with 30kHz SCS

- Proposals

  - Option 1: Use option 1 for the special slot configuration, i.e. S=6D+4G+4U (HW, CMCC, Apple, China Telecom, QC, ZTE, E///)

    - Option 1A: it is up to UE implementation whether perform CRS-IM in S slot. If simulation is not aligned, we can unify the UE behavior in S slot. (HW, CMCC, E///)

    - Option 1B: assume no PDSCH scheduling in the specials slot (Apple, China Telecom, QC, ZTE)

      - Based on the FRC for Rel-16 TDD LTE-NR coexistence test, as well as the CRS-IM FRC submitted in this meeting, NR PDSCH is not scheduled in the special slot.

- Agreement: Option 1B agreed.

Issue 3-3: UE feature for 30 kHz CRS-IM

- Proposals

  - Option 1: Approve the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario. For the features of CRS-IM in non-DSS and 30 kHz NR SCS scenario, reuse the configuration proposed in last meeting WF in R4-2207239. (CMCC, China Telecom, ZTE, QC & Nokia - if agreed to define requirements for 30kHz SCS)

| NR_perf_enh2_Demod | X-4 | CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_perf_enh2_Demod | X-5 | CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |

- Agreement: Option 1 agreed.

CR/LS

| tdoc number | Title/Content | CRs/TPs Status update recommendation |
| --- | --- | --- |
| R4-2208418, CMCC | LS for 30kHz SCS CRS-IM | Already revised and approved |
| R4-2208419, CMCC | draft CR for general applicability section | Revised |
| R4-2209819, Huawei, HiSilicon | draft CR for PDSCH TDD requirements | Revised |
| R4-2209148, ZTE | draft CR for FRC | Revised |

Discussion on 2nd round

Draft CRs and Simulation assumptions

| tdoc number | Title/Content | Comment collection |
| --- | --- | --- |
| R4-2210658, CMCC | Simulation assumptions for CRS-IM (30 kHz SCS TDD) | Apple: Suggest adding that no PDSCH scheduling in SS/PBCH slots as well. We should also include a, b, precoding granularity in LTE parameters. |
|  |  | CMCC: Thanks for comments. ‘no PDSCH scheduling in SS/PBCH’ has already existed in the Table 2-1, last row as the Note. We add the LTE parameters in Table 2-2. |
| R4-2210920, CMCC | Draft CR for general applicability section |  |
| R4-2210930, Huawei, HiSilicon | draft CR for PDSCH TDD requirements | Apple: Why do we need new section for these requirements? Can’t we add them as another test following 15KHz SCS requirements in TDD? We don’t use defined in this spec. Suggest using INR and referencing the Annex. Add the following in LTE parameters : PDSCH loading, TM, a & b, Precoder granularity. |
|  |  | Qualcomm: Tables 5.2.y.1.X-4 and Tables 5.2.y.1.X-5 have the same titles. Please update them to distinguish the requirements. |
|  |  | CMCC: a. The ‘Number of HARQ Processes’ should be 8 for TDD. b. The ‘The number of slots between PDSCH and corresponding HARQ-ACK information’ should be ‘Specific to each TDD UL-DL pattern and as defined in Annex A.1.2’ c. The square brackets for ‘Antenna ports [0,1,2,3]’ can be removed. d. According to the WF R4-2120705, TDD UL-DL pattern for LTE cells should be ‘DSUDDDSUDD with S = 10D+2G+2U’. and a Note is needed to clarify that ‘The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame’ |
|  |  | CTC: For the requirement title, suggest to align with the wording to be specified in TS38.306. As far as I know, it will be specified in this RAN2 meeting as ‘with/without the assistance of network signaling on LTE channel bandwidth.’ So the suggested title is: Minimum performance for Rank 1 ‘with/without the assistance of network signaling on LTE channel bandwidth. For the wording above the titles, the wording from CR on TDD Sce 2 for 15kHz is clearer. |
| R4-2210922, ZTE | draft CR for FRC | CMCC: a. Based on our understanding, for Slot 14, 15, 34, and 35, they won’t conflict with LTE PBCH/SS; b. Number of consecutive PDSCH symbols should be revised. Such as For Slot i, if mod(i, 20) = {4,5} for i from {0,…,39}, the value should be N/A; And For Slot i, if mod(i, 10) = {7} for i from {0,…,39}, the value is N/A |
|  |  | CTC: 1) Create new table in the spec in Table A.3.2.2.2-19. Please name the Reference channel as ‘R.PDSCH.2-19.1 TDD’. 2) It has been clearly specified that TBS = 0 on S slots, we think we do not need to additionally add note to say that ‘no PDSCH scheduling on the specials slot.’. Similar as all the existing FRCs without PDSCH on S slots. |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | Simulation assumptions for CRS-IM (30 kHz SCS TDD) | CMCC |  |
|  | WF on the test with only inter-RAT MO configured for scenario 2 | Huawei, HiSilicon | Cover the issues in Sub-topic 2-1, for both 15kHz and 30kHz SCS |
|  | draft Big CR for CRS-IM | Ericsson | The big CR is for post-meeting endorsement, since the test setup for capability #2 and #4 will be concluded in the next meeting. |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2208418 | R4-2210435 | LS on UE capability and network assistant signalling for CRS interference mitigation in the scenario with overlapping spectrum for LTE and NR with 30kHz SCS | CMCC | R4-2210435 is Approved |  |
| R4-2207803 |  | Simulation results for CRS-IM requirements | Apple | Noted |  |
| R4-2208050 |  | Discussion on CRS-IM requirements for 30 kHz SCS case | Intel Corporation | Noted |  |
| R4-2208259 |  | On General for CRS-IM | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2208260 |  | On Test Setup for CRS-IM | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2208261 |  | draftCR to 38_101-4: NR CRS-IM 15KHz SCS Scenario - General and applicability sections | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2208416 |  | Discussion on the CRS-IM for NR 30kHz SCS | CMCC | Noted |  |
| R4-2208417 |  | Simulation results collection for 30kHz SCS CRS-IM | CMCC | Return to |  |
| R4-2208419 |  | Draft CR for introduction of general applicability section of CRS-IM with serving cell 30kHz SCS in TS38.101-4 | CMCC | Revised |  |
| R4-2208420 |  | Draft CR on TDD PDSCH CRS-IM demod requirements for Scenario2 with overlapping spectrum for LTE and NR 15kHz SCS | CMCC | Revised |  |
| R4-2208421 |  | Discussion on the test setup for CRS-IM 15kHz | CMCC | Noted |  |
| R4-2209147 |  | CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR | ZTE Corporation | Noted |  |
| R4-2209148 |  | Draft CR for TS38.101-4 PDSCH Reference Channel for CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR | ZTE Corporation | Revised |  |
| R4-2209404 |  | Simulation assumptions for CRS-IM (for 15kHz FDD and TDD) | China Telecom | Revised |  |
| R4-2209405 |  | Summary of CRS-IM simulation results (15 kHz SCS FDD and TDD) | China Telecom | Return to |  |
| R4-2209408 |  | Simulation results for CRS-IM for 15kHz SCS scenario | China Telecom | Noted |  |
| R4-2209409 |  | Discussion on the test setup for CRS-IM requirement definition | China Telecom | Noted |  |
| R4-2209410 |  | Draft CR on adding FRC for CRS-IM 15kHz SCS test requirements | China Telecom | Revised |  |
| R4-2209411 |  | Draft CR on FDD PDSCH CRS-IM demod requirements for DSS Scenario | China Telecom | Revised |  |
| R4-2209417 |  | Updated work plan for Further enhancement on NR demodulation performance WI | China Telecom | Revised |  |
| R4-2209527 |  | Summary of CRS-IM simulation results (15 kHz SCS FDD and TDD) | ZTE Corporation | Noted |  |
| R4-2209692 |  | Discussion on the test setup for CRS-IM | Ericsson | Noted |  |
| R4-2209693 |  | Discussion on the 30kHz SCS scenario for CRS-IM | Ericsson | Noted |  |
| R4-2209694 |  | Simulation results for CRS-IM | Ericsson | Noted |  |
| R4-2209695 |  | draft CR to TS 38.101-4: TDD PDSCH CRS-IM demod requirements for DSS Scenario | Ericsson | Revised |  |
| R4-2209738 |  | Draft CR to 38_101-4: Abbreviations section | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2209794 |  | Simulation results and discussion on PDSCH requirements for CRS-IM receiver | MediaTek inc. | Noted |  |
| R4-2209795 |  | Draft CR to TS38.101-4, interference model for CRS-IM receiver | MediaTek inc. | Revised |  |
| R4-2209815 |  | Discussion on test setup for CSI-IM with 15kHz SCS | Huawei, HiSilicon | Noted |  |
| R4-2209816 |  | Simulation results for CRS-IM with 15kHz SCS | Huawei, HiSilicon | Noted |  |
| R4-2209817 |  | draftCR: Introduction of PDSCH requirements for FDD CRS-IM scenario 2 with 15kHz | Huawei, HiSilicon | Revised |  |
| R4-2209818 |  | Discussion on CRS-IM with 30kHz SCS | Huawei, HiSilicon | Noted |  |
| R4-2209819 |  | draftCR: Introduction of PDSCH requirements for CRS-IM scenario 2 with 30kHz | Huawei, HiSilicon | Revised |  |
| R4-2210003 |  | Simulation results collection for 30kHz SCS CRS-IM | ZTE Corporation | Noted |  |
| R4-2210191 |  | Views on Test Setup for CRS Interference Mitigation in NR | Qualcomm Incorporated | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2210926 |  | Updated work plan for Further enhancement on NR demodulation performance WI | China Telecom | Agreeable |  |
| R4-2210659 |  | WF on the test with only inter-RAT MO configured for scenario 2 | Huawei, HiSilicon | Agreeable |  |
| R4-2210923, |  | Simulation assumptions for CRS-IM (for 15kHz FDD and TDD) | China Telecom | Agreeable |  |
| R4-2210919 |  | draftCR to 38_101-4: NR CRS-IM 15KHz SCS Scenario - General and applicability sections | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2210925 |  | Draft CR on FDD PDSCH CRS-IM demod requirements for DSS Scenario | China Telecom | Endorsed |  |
| R4-2210927 |  | draft CR to TS 38.101-4: TDD PDSCH CRS-IM demod requirements for DSS Scenario | Ericsson | Endorsed |  |
| R4-2210929 |  | draftCR: Introduction of PDSCH requirements for FDD CRS-IM scenario 2 with 15kHz | Huawei, HiSilicon | Endorsed |  |
| R4-2210921 |  | Draft CR on TDD PDSCH CRS-IM demod requirements for Scenario2 with overlapping spectrum for LTE and NR 15kHz SCS | CMCC | Endorsed | Capture in Chairman Notes that: Whether to define the CRS-IM requirements for TDD scenario 2 with 15kHz SCS and 30kHz SCS in the same or different sections can be further discussed in the next meeting. |
| R4-2210924 |  | Draft CR on adding FRC for CRS-IM 15kHz SCS test requirements | China Telecom | Endorsed |  |
| R4-2210928 |  | Draft CR to TS38.101-4, interference model for CRS-IM receiver | MediaTek inc. | Endorsed |  |
| R4-2210658 |  | Simulation assumptions for CRS-IM (30 kHz SCS TDD) | CMCC | Agreeable |  |
| R4-2210920 |  | Draft CR for introduction of general applicability section of CRS-IM with serving cell 30kHz SCS in TS38.101-4 | CMCC | Endorsed |  |
| R4-2210930 |  | draftCR: Introduction of PDSCH requirements for CRS-IM scenario 2 with 30kHz | Huawei, HiSilicon | Endorsed | Capture in Chairman Notes that: Whether to define the CRS-IM requirements for TDD scenario 2 with 15kHz SCS and 30kHz SCS in the same or different sections can be further discussed in the next meeting. |
| R4-2210922 |  | Draft CR for TS38.101-4 PDSCH Reference Channel for CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR | ZTE Corporation | Endorsed |  |
| R4-2210660 |  | draft Big CR for CRS-IM | Ericsson | The big CR is for post-meeting endorsement |  |
| R4-2209405 |  | Summary of CRS-IM simulation results (15 kHz SCS FDD and TDD) | China Telecom | Noted |  |
| R4-2208417 |  | Simulation results collection for 30kHz SCS CRS-IM | CMCC | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |
| Nokia, Nokia Shanghai Bell | Karsten Petersen | Karsten.petersen@nokia-bell-labs.com |
| Ericsson | Jiakai Shi | Jiakai.Shi@ericsson.com |
| Apple | Manasa Raghavan | manasa.raghavan@apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)