3GPP TSG-RAN WG4 Meeting # 103-e 										R4-2210617

Electronic Meeting, May 9th – 20th, 2022

Agenda item:			9.18.3

Source:	Samsung

Title:	WF on FeMIMO RRM performance requirements

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM performance requirements in RAN4 #103-e meeting [103-e][229] NR_feMIMO_RRM_2.

Agreements

Agreements during the meeting

Unified TCI state switching

- No test case will be introduced for DCI-based unified TCI state switching

- MAC-CE-based unified TCI state switching test cases will be defined in FR2 EN-DC and NR SA operation mode.

- Define the MAC-CE based TCI state switching test cases:

  - For serving cells, define the test cases for Joint TCI and UL TCI state

  - For cells with different PCI, define the test case for DL TCI state

- Configurations for TCI state switching test case

  - Specify test cases for known TCI state only

    - For the known DL or Joint TCI test cases, the TCI is not in the active TCI list

    - For the known UL TCI test cases, the PL-RS is not maintained.

L1-RSRP measurement on Cells with different PCI

- No new section for accuracy requirement of L1-RSRP Measurement on Cells with different PCI, revise the applicability rule of the current accuracy requirement to support the cells with different PCI.

- No need to introduce separated test cases for measurement accuracy.

- Scenarios for the test case of L1-RSRP measurement on Cells with different PCI

  - Define test cases for

    - FR1 NR SA

    - FR1-FR2 CA operation where FR2 SCell is to be tested

  - For CA scenario, FR2 NR Cell is to be tested in the test cases.

- DRX configuration for the L1-RSRP measurement procedure test cases

  - For each test case, UE is configured either with DRX or without DRX

TRP specific BFD and LR

- Define the TRP specific BFR and LR test case for both FR1 and FR2 and both EN-DC and NR SA.

- Scenarios and configurations for defining TRP specific BFD and LR test case

  - RAN4 aims to agree a reasonable test case list to minimize test effort

  - The table below can be taking starting point of scenarios and configurations of the test for further discussion

| 1 | EN-DC | FR1 | PSCell | SSB based | Non-DRX |
| --- | --- | --- | --- | --- | --- |
| 2 | EN-DC | FR2 | PSCell | CSI-RS based | DRX |
| 3 | EN-DC | FR1 | SCell | CSI-RS-based BFD and SSB-based LR | Non-DRX |
| 4 | NR SA | FR1 | PCell | CSI-RS based | DRX |
| 5 | NR SA | FR2 | PCell | SSB based | Non-DRX |
| 6 | NR SA | FR2 | SCell | CSI-RS based | DRX |

  - Other options not precluded

- DRX configuration for the TRP specific BFD and LR test cases

  - For each test case, UE is configured either with DRX or without DRX

Remaining Open Issues

- Further study whether the following test cases are needed

  - Scheduling availability restriction during TRP specific Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

  - Test case for MAC-CE based active TCI state list update