3GPP TSG-RAN WG4 Meeting # 103-e 													R4-2210666

Electronic Meeting, May 9-20, 2022

Agenda item:	9.17.2.1

Source:	China Telecom

Title:	WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement

Document for:	Approval

Introduction

This document provides the way forward on PUSCH demodulation performance of Rel-17 NR coverage enhancement.

The previous approved WFs are listed as below:

  - R4-2203030 - WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement, China Telecom, RAN4# 101-bis-e

  - R4-2207210 - WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement, China Telecom, RAN4# 102-e

Test requirement discussion scope for FR2

Issue 1-1-1: Test requirement discussion scope for FR2

- Candidate options:

  - Option 1: Limit the discussion scope of Rel-17 NR coverage enhancement demodulation to FR1 and FR2-1

  - Option 2: Do not limit the scope in this WI

  - Option 3: Following the decision in the RF core part

- Recommended WF

  - FFS

PUSCH TB over Multi Slots (TBoMS)

Issue 1-2-1: Physical/available slots for BS requirements for PUSCH TBoMS

- Agreement in the second round:

  - 4 slots for FDD and 2 slots for TDD.

Issue 1-2-2: PRB number for BS requirements for PUSCH TBoMS

  - 5 PRB

Issue 1-2-3: TDD UL-DL pattern and test applicability for BS requirements for PUSCH TBoMS

- Agreement in the second round:

  - For 15/60/120kHz SCS, use 3D1S1U, S=10D:2G:2U.

  - Keep the existing test applicability rule for different TDD UL-DL patterns as baseline and interested companies can bring TBoMS simulation results between different TDD patterns.

  - FFS whether Manufacturer declaration can be introduced for TBoMS

Issue 1-2-4: Transform precoding for BS requirements for PUSCH TBoMS

  - CP-OFDM only

Issue 1-2-5: Test applicability rule for different channel bandwidths for PUSCH TBoMS requirements

  - The test applicability rule for different channel bandwidths defined in 8.1.2.1.2 in TS 38.141-1/2 for FR1/FR2 can be used

Issue 1-2-6: MCS for TBoMS PUSCH demod test

- Agreement in the second round:

  - MCS2

Issue 1-2-7: PUSCH mapping type for TBoMS PUSCH demod test

  - Cover both PUSCH mapping type A and type B for FR1

Issue 1-2-8: Antenna configuration for TBoMS PUSCH demod test

- Candidate options

  - Option 1: 1T2R

  - Option 2: Cover 2Rx, 4Rx and 8Rx

- Recommended in the second round

  - As a compromise for all companies, can we agree the following:

    - Only cover 1T2R for TBoMS

    - Cover 2/4/8 Rx for PUSCH JCE

Issue 1-2-9: Test metric for TBoMS PUSCH demod test

  - Test SNR at which the PUSCH achieves 70% of throughput

Issue 1-2-10: Other parameters for BS requirements for PUSCH TBoMS for FR2

- Agreement in the second round:

  - DMRS 1+1 only and no PT-RS configured

PUSCH demodulation with Joint Channel Estimation (JCE)

Issue 1-3-1: Actual TDW length for JCE in BS PUSCH demod requirements

- Agreement on the aTDW length

  - For TDD:

    - Confirm 2 consecutive slots for TDD 30kHz SCS.

  - For FDD:

    - 8 slots

- Agreement for aTDW length for TDD 15/60/120 kHz SCS:

  - 2 slots

Issue 1-3-2: PUSCH repetition number for BS PUSCH demod requirements with JCE

- Agreement:

  - For TDD: same as aTDW length 2 for 30kHz

  - For FDD: same as aTDW length, i.e., 8

- Agreement in the second round:

  - Use the same agreement for TDD with 15/60/120 kHz SCS

Issue 1-3-3: Configured TDW number for JCE in BS PUSCH demod requirements

- Agreement in the second round:

  - For TDD:

    - cTDW length is configured same as the aTDW length

  - For FDD:

    - cTDW length is configured same as the aTDW length.

Issue 1-3-4: Inter-slot frequency hopping for BS PUSCH demod requirements with JCE

  - Inter-slot frequency hopping is disabled for PUSCH JCE for TDD and FDD

Issue 1-3-5: TDD UL-DL pattern for BS PUSCH demod requirements with JCE

- GTW agreement:

  - For FR1 15KHz SCS, define new TDD pattern with multiple contiguous UL slots and further discuss the exact TDD pattern

  - For FR2 60/120 kHz SCS, define new TDD pattern with multiple contiguous UL slots and further discuss the exact TDD pattern

  - Manufacture declaration can be introduced for supporting JCE with corresponding SCS

- Candidate options for new TDD patterns:

  - For FR1 15KHz SCS

    - Option 1A: DSUUU

    - Option 1B: 7D1S2U, S=6D:4G:4U

    - Option 1C: DDSUU, S=10G:2G:2U

  - For FR2 60/120 kHz SCS:

    - Option 1A: DSUUU

    - Option 1B: 5D1S4U

    - Option 1C: DDSUU, S=10G:2G:2U

- Proposals for manufacture declaration for support of JCE:

  - Proposal 1:

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.yyy | SCS for PUSCH JCE and PUCCH JCE | Declaration of supported SCS for PUSCH JCE and PUCCH JCE, i.e. {15kHz, 30kHz, 60kHz 120kHz} |

- Proposals on JCE test applicability rule for different TDD patterns for each SCS:

  - Proposal 1: the requirement defined for each SCS can be applicable for other TDD patterns with same number of physical consecutive slots (aTDW length)

  - Proposal 2: The same requirements are applicable to TDD with different UL-DL patterns, as long as those patterns allow for the mandated aTDW

- Agreement in the second round:

  - Make decision on the exact TDD pattern for 15/60/120kHz in the next meeting.

  - Use the below TDD patterns for simulation purpose only, and the same requirement can be reused if other patterns with same number of UL consecutive slot number is agreed:

    - 7D1S2U, S=6D:4G:4U for 15kHz

    - DDSUU, S=10G:2G:2U for 60/120kHz

  - Further discuss the test applicability rule for different TDD patterns for each SCS.

  - Further discuss the details for manufacture declaration for PUSCH JCE.

Issue 1-3-6: Transform precoding for BS PUSCH demod requirements with JCE

  - CP-OFDM only

Issue 1-3-7: PRB number for BS PUSCH demod requirements with JCE

- Agreement in the second round:

  - Cover the minimum bandwidth for each SCS

  - FFS on whether to cover the largest CHBW for each SCS and interested companies can bring simulation results for the largest CHBW

Issue 1-3-8: MCS for BS PUSCH demod requirements with JCE

- Agreement in the second round:

  - MCS4

Issue 1-3-9: Antenna configuration for BS PUSCH demod requirements with JCE

- Candidate options:

  - Option 1: 1T2R for FR1 and FR2

  - Option 2: Cover 2Rx 4Rx and 8Rx for FR1

- Recommended in the second round

  - As a compromise for all companies, can we agree the following

    - Only cover 1T2R for TBoMS

    - Cover 2/4/8 Rx for PUSCH JCE

Issue 1-3-10: Propagation condition for FR2 BS PUSCH demod requirements with JCE

- Agreement in the second round:

  - TDLA30-75 for FR2

Issue 1-3-11: Additional DM-RS position for BS PUSCH demod requirements with JCE (for FR1)

- Candidate options:

  - Option 1: DMRS 1+1

  - Option 2: Use both DMRS 1+1 and DMRS 1+0

  - Option 3: DMRS 1+0

- Agreement in the second round:

  - Make decision on this issue in the next meeting.

  - Encourage companies to bring simulation results for both DMRS1+1 and DMRS1+0 for the next meeting

Issue 1-3-12: Phase offset modelling for BS PUSCH demod requirements with JCE

  - Use ideal phase modelling for the PUSCH JCE test and choose one of the following

    - Option 1: companies can consider the phase offset in the impairment results

    - Option 2: Phase offset model will be covered by TE side in the test uncertainty

Issue 1-3-13: Receiver implementation for BS PUSCH demod requirements with JCE

- Agreement in the second round:

  - Further discuss the detailed simulation assumption about the receiver implementation if the simulation results are not aligned well

Issue 1-3-14: Test metric for BS PUSCH demod requirements with JCE

  - Test SNR at which the PUSCH achieves 70% of throughput

Issue 1-3-15: Other parameters for BS PUSCH demod requirements with JCE for FR2

- Agreement in the second round:

  - Follow the same config for DMRS and PT-RS for TBoMS

  - Use full slot allocation (14 symbols) for PUSCH JCE requirements

CFO modeling for the JCE simulation

- Agreement in the second round:

  - Do not consider CFO modelling for the PUSCH JCE test and choose one of the following

    - Option 1: companies can consider the CFO impact in the impairment results.

    - Option 2: CFO impact will be covered by TE side in the test uncertaint