3GPP TSG-RAN WG4 Meeting #103-e	R4-2210670

# Electronic Meeting, 09 – 20 May 2022

Title:	WF on demodulation requirement for Enhancement on HST-SFN deployment

Source:	Ericsson

Agenda item:			9.18.4.2.1

Document for:	Approval

# Introduction

This WF capture all agreements and open issues for the following topics in [103-e][328] NR_FeMIMO_Demod:

- Topic #2: Demodulation requirement for Enhancement on HST-SFN scenario

  - Sub-topic 2-1: Test scope

  - Sub-topic 2-2: Test setup for PDSCH requirement for SFN scheme A with Single Carrier

  - Sub-topic 2-3: Test setup for PDSCH requirement for SFN scheme B with Single Carrier If introduced

The agreed WFs on demodulation performance requirements for enhancement on HST-SFN in the previous meetings:

- R4-2203091, “WF on demodulation requirement for Enhancement on HST-SFN deployment”, Intel, RAN4#101-bis-e

- R4-2207208, “WF on demodulation requirement for Enhancement on HST-SFN deployment”, Intel, RAN4#102-e

# Topic #2: Demodulation requirement for Enhancement on HST-SFN scenario

## Sub-topic 2-1: Test scope

Issue 2-1-1: Whether to define PDSCH requireemnt with HST-SFN scheme B

- Option 1: Introduced the PDSCH requirement with HST-SFN B under test applicability rule

  - FFS on the pre-compensation modeling

- Option 2: Not to introduce the PDSCH requirement with HST-SFN B

## Sub-topic 2-2: Test setup for PDSCH requirement for SFN scheme A with Single Carrier

Issue 2-2-1: Maximum Doppler shift

- RAN4 will make final decision about the value of Maximum Doppler shift in RAN4#104-e meeting based on the simulation results, e.g. compare the required SNR to achieve 70% of peak rate with MCS17 (or MCS13) with rank 2.

  - Option 1: 870Hz (baseline)

  - Option 2: 972Hz

- Encourage interested companies provide the simulation results for both Options in RAN4#104-e meeting.

Issue 2-2-2: MCS & Rank

- MCS17 rank 2 as the baseline.

- RAN4 may consider MCS13 rank 2 according to the simulation results

Issue 2-2-3: UE capability

- The PDSCH demodulation requirements for HST-SFN Scheme A should be applicable for UE capable of “23-6-1 SFN scheme A (scheme 1) for PDSCH and PDCCH”.

## Sub-topic 2-3: Test setup for PDSCH requirement for SFN scheme B with Single Carrier If introduced

Issue 2-3-1: Modeling of TRP pre-compensation

- Assume perfect modeling of TRP pre-compensation as baseline.

- Interested companies can evaluate the following model:

  - The frequency shift FD,k (Hz) from kth RRH is given by:

FD,k=fc×real-v×y-xky-xk×C for k*Ds≤a<k*Ds+Ds

and

FD,k+1=FD,k+F∆,k+1 for k*Ds≤a<k*Ds+Ds

where

F∆,k=+0.1 ppm*fc ,  k mod 2=0-0.1 ppm*fc ,  k mod 2=1

# Reference

- R4-2210531, Email discussion summary for [103-e][328] NR_FeMIMO_Demod, RAN4#103-e, Samsung