3GPP RAN4 Meeting #103-e		R4-2210773

# Electronic meeting, May 9 - 20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | draft | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.101-3 on new column for mandatory simultaneous RxTx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_Simult_RxTx-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The applicability of mandatory simultaneous Rx/Tx requirements is captured via notes to the configuration tables. It is not correct to capture normative aspects as informative notes. Furthermore, the notes' notation is not easily machine parseable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are introduced: - Inter-band CA between FR1 and FR2: add a new column "Mandatory simultaneousRx/Tx" to Tables 5.2A.1-1, 5.2A.1-2, 5.2A.1-4, 5.2A.1-5 - Inter-band EN-DC within FR1: add a new column "Mandatory simultaneousRx/Tx" to Tables 5.5B.4.1-1, 5.5B.4.2-1, 5.5B.4.3-1, 5.5B.4.4-1, 5.5B.4.5-1 - Inter-band NE-DC within FR1: add a new column "Mandatory simultaneousRx/Tx" to Tables 5.5B.4a.1-1, 5.5B.4a.2-1, Table 5.5B.4a.3-1, Table 5.5B.4a.4-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mandatory simultaneous Rx/Tx requirements can be incorrectly understood. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.1, 5.5B.4.1, 5.5B.4.2, 5.5B.4.3, 5.5B.4.4, 5.5B.4.5, 5.5B.4a.1, 5.5B.4a.2, 5.5B.4a.3, 5.5B.4a.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  |  |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS38.521-3 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<start of change 1>

## 5.2A	Operating bands for CA

### 5.2A.1	Inter-band CA between FR1 and FR2

NR carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1 and Table 5.2A.1-2. The band combinations include at least one FR1 operating band and one FR2 operating band.

Operating bands for CA including Band n90 are defined by the corresponding operating bands for CA including Band n41 with Band n90 replacing Band n41. For brevity the said operating bands for CA including Band n90 are not listed in the tables below but are covered by this specification.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, the mandatory simultaneous Rx/Tx capability also applies for the band combination when the applicable band combination is a subset of a higher order band combination.

Table 5.2A.1-1: Band combinations for inter-band CA between FR1 and FR2 (two bands)

| NR CA Band | NR Band | Mandatory simultaneous Rx/Tx |
| --- | --- | --- |
| CA_n1-n257 | n1, n257 | Yes |
| CA_n1-n258 | n1, n258 | Yes |
| CA_n2-n260 | n2, n260 | Yes |
| CA_n2-n261 | n2, n261 | Yes |
| CA_n3-n257 | n3, n257 | Yes |
| CA_n3-n258 | n3, n258 | Yes |
| CA_n5-n260 | n5, n260 | Yes |
| CA_n5-n261 | n5, n261 | Yes |
| CA_n7-n258 | n7, n258 | Yes |
| CA_n8-n257 | n8, n257 |  |
| CA_n8-n258 | n8, n258 | Yes |
| CA_n12-n260 | n12, n260 | Yes |
| CA_n14-n260 | n14, n260 | Yes |
| CA_n30-n260 | n30, n260 | Yes |
| CA_n25-n258 | n25, n258 | Yes |
| CA_n25-n260 | n25, n260 | Yes |
| CA_n25-n261 | n25, n261 | Yes |
| CA_n28-n257 | n28, n257 | Yes |
| CA_n34-n258 | n34, n258 | Yes |
| CA_n39-n257 | n39, n257 | Yes |
| CA_n39-n258 | n39, n258 | Yes |
| CA_n40-n257 | n40, n257 | Yes |
| CA_n40-n258 | n40, n258 | Yes |
| CA_n41-n257 | n41, n257 | Yes |
| CA_n41-n258 | n41, n258 | Yes |
| CA_n41-n260 | n41, n260 | Yes |
| CA_n41-n261 | n41, n261 | Yes |
| CA_n48-n260 | n48, n260 | Yes |
| CA_n48-n261 | n48, n261 | Yes |
| CA_n66-n258 | n66, n258 | Yes |
| CA_n66-n260 | n66, n260 |  |
| CA_n66-n261 | n66, n261 |  |
| CA_n71-n257 | n71, n257 | Yes |
| CA_n71-n260 | n71, n260 | Yes |
| CA_n71-n261 | n71, n261 | Yes |
| CA_n77-n257 | n77, n257 | Yes |
| CA_n77-n258 | n77, n258 | Yes |
| CA_n77-n260 | n77, n260 | Yes |
| CA_n77-n261 | n77, n261 | Yes |
| CA_n78-n257 | n78, n257 | Yes |
| CA_n78-n258 | n78, n258 | Yes |
| CA_n79-n257 | n79, n257 | Yes |
| CA_n79-n258 | n79, n258 | Yes |
| NOTE 1: Void. |  |  |

Table 5.2A.1-2: Band combinations for inter-band CA between FR1 and FR2 (three bands)

| NR CA Band | NR Band | Mandatory simultaneous Rx/Tx |
| --- | --- | --- |
| CA_n1-n3-n257 | n1, n3, n257 |  |
| CA_n1-n8-n257 | n1, n8, n257 |  |
| CA_n1-n28-n257 | n1, n28, n257 | Yes |
| CA_n1-n41-n257 | n1, n41, n257 | Yes |
| CA_n1-n77-n257 | n1, n77, n257 | Yes |
| CA_n1-n78-n257 | n1, n78, n257 | Yes |
| CA_n1-n79-n257 | n1, n79, n257 | Yes |
| CA_n2-n77-n260 | n2, n77, n260 |  |
| CA_n2-n77-n261 | n2, n77, n261 |  |
| CA_n3-n8-n257 | n3, n8, n257 |  |
| CA_n3-n28-n257 | n3, n28, n257 | Yes |
| CA_n3-n41-n257 | n3, n41, n257 |  |
| CA_n3-n77-n257 | n3, n77, n257 | Yes |
| CA_n3-n78-n257 | n3, n78, n257 | Yes |
| CA_n3-n79-n257 | n3, n79, n257 | Yes |
| CA_n5-n77-n260 | n5, n77, n260 |  |
| CA_n5-n77-n261 | n5, n77, n261 |  |
| CA_n7-n78-n258 | n7, n78, n258 |  |
| CA_n8-n77-n257 | n8, n77, n257 |  |
| CA_n28-n41-n257 | n28, n41, n257 |  |
| CA_n28-n77-n257 | n28, n77, n257 | Yes |
| CA_n28-n78-n257 | n28, n78, n257 | Yes |
| CA_n28-n79-n257 | n28, n79, n257 | Yes |
| CA_n40-n41-n258 | n40, n41, n258 |  |
| CA_n41-n66-n260 | n41, n66, n260 |  |
| CA_n41-n77-n257 | n41, n77, n257 |  |
| CA_n41-n78-n257 | n41, n78, n257 |  |
| CA_n41-n79-n258 | n41, n79, n258 |  |
| CA_n66-n77-n260 | n66, n77, n260 |  |
| CA_n66-n77-n261 | n66, n77, n261 |  |
| CA_n77-n79-n257 | n77, n79, n257 |  |
| CA_n78-n79-n257 | n78, n79, n257 |  |
| NOTE 1: Void. |  |  |

Table 5.2A.1-3: Band combinations for inter-band CA between FR1 and FR2 (four bands)

| NR CA Band | NR Band | Mandatory simultaneous Rx/Tx |
| --- | --- | --- |
| CA_n1-n3-n8-n257 | n1, n3, n8, n257 |  |
| CA_n1-n3-n28-n257 | n1, n3, n28, n257 |  |
| CA_n1-n3-n77-n257 | n1, n3, n77, n257 |  |
| CA_n1-n3-n79-n257 | n1, n3, n79, n257 |  |
| CA_n1-n8-n77-n257 | n1, n8, n77, n257 |  |
| CA_n1-n28-n77-n257 | n1, n28, n77, n257 |  |
| CA_n1-n28-n79-n257 | n1, n28, n79, n257 |  |
| CA_n1-n77-n79-n257 | n1, n77, n79, n257 |  |
| CA_n1-n78-n79-n257 | n1, n78, n79, n257 |  |
| CA_n3-n7-n78-n258 | n3, n7, n78, n258 |  |
| CA_n3-n8-n77-n257 | n3, n8, n77, n257 |  |
| CA_n3-n28-n41-n257 | n3, n28, n41, n257 |  |
| CA_n3-n28-n77-n257 | n3, n28, n77, n257 | Yes |
| CA_n3-n28-n78-n257 | n3, n28, n78, n257 | Yes |
| CA_n3-n41-n77-n257 | n3, n41, n77, n257 |  |
| CA_n3-n77-n79-n257 | n3, n77, n79, n257 |  |
| CA_n3-n28-n79-n257 | n3, n28, n79, n257 |  |
| CA_n28-n41-n77-n257 | n28, n41, n77, n257 |  |
| CA_n28-n77-n79-n257 | n28, n77, n79, n257 |  |
| CA_n28-n78-n79-n257 | n28, n78, n79, n257 |  |
| NOTE 1: Void. |  |  |

Table 5.2A.1-4: Band combinations for inter-band CA between FR1 and FR2 (five bands)

| NR CA Band | NR Band | Mandatory simultaneous Rx/Tx |
| --- | --- | --- |
| CA_n1-n3-n8-n77-n257 | n1, n3, n8, n77, n257 |  |

<end of change 1>

<start of change 2>

## 5.5B	Configuration for DC

### 5.5B.1	General

The operating bands and bandwidth classes are specified for operation with EN-DC, NGEN-DC, NE-DC or NR-DC configured. The EN-DC, NGEN-DC or NE-DC band combinations include at least one E-UTRA operating band.

For EN-DC or NE-DC configurations indicated by column "Single Uplink allowed" (e.g., problematic band combinations as defined in TS 38.306 [11]) in tables in this clause the UE may indicate capability of not supporting simultaneous dual and triple uplink operation due to possible intermodulation interference to its own primary downlink channel bandwidth of PCell or PSCell if the intermodulation order is 2 or if the intermodulation order is 3 for the combinations when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz. When LTE and NR transmissions collide, simultaneous dual transmissions may not be supported by UE for these EN-DC band combinations for which only single switched UL is supported.

In the case for EN-DC or NE-DC configurations listed in tables in this clause for which the intermodulation products caused by the dual and triple uplink operation fall into the receive band but do not interfere with its own primary downlink channel bandwidth of PCell or PSCell as defined in Annex I the UE is mandated to operate in dual and triple uplink mode. Single Uplink is also allowed for certain band combinations where intermodulation or reverse intermodulation products could create difficulty for meeting emission requirementsFor EN-DC combinations of order 3 or higher, "Single Uplink allowed" UL configurations captured in Table 5.5B.2-1, Table 5.5B.3-1, and Table 5.5B.4-1 apply.

If multiple UL DC configurations are listed for multiple DL DC configurations, valid uplink configurations are such that uplink does not have more carriers than downlink.

The configurations for operating bands for DC including Band n41 also apply for the corresponding operating bands for DC with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for operating bands for DC with Band n90 are not listed in the tables below but are covered by this specification.

Non contiguous resource allocation and almost contiguous allocation are not applicable for E UTRA or NR carrier part of intra band EN DC configuration.

If the mandatory simultaneous Rx/Tx capability applies for a DC configuration, the mandatory simultaneous Rx/Tx capability also applies for the DC configuration when the applicable DC configuration is a subset of a higher order DC configuration.

### 5.5B.2	Intra-band contiguous EN-DC

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)5AA | DC_(n)5AA6 | Yes6 |
| DC_(n)12AA | DC_(n)12AA6 | Yes6 |
| DC_(n)38AA5 | DC_(n)38AA6 | Yes6 |
| DC_(n)41AA5 DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_(n)41AA | Yes3 |
| DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_41A_n41A | Yes3 |
| DC_(n)48AA5 | DC_(n)48AA6 | Yes6 |
| DC_(n)48CA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)48DA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)66AA | DC_(n)66AA6 | Yes |
| DC_(n)71AA2 | DC_(n)71AA | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. Uplink DC_(n)71AA is not applicable to BCS2. NOTE 5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 6: Only single switched UL is supported |  |  |

### 5.5B.3	Intra-band non-contiguous EN-DC

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_2A_n2A | DC_2A_n2A5 | Yes5 |
| DC_3A_n3A | DC_3A_n3A | Yes7 |
| DC_5A_n5A | DC_5A_n5A5 | Yes5 |
| DC_7A_n7A6 | DC_7A_n7A5 | Yes5 |
| DC_41A_n41A3 DC_41C_n41A3 DC_41D_n41A3 | DC_41A_n41A | Yes4 |
| DC_48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48A-(n)48AA3 | DC_(n)48AA5 DC_48A_n48A5 | Yes5 |
| DC_48A-48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48C_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48D_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_66A_n66A | DC_66A_n66A5 | Yes5 |
| DC_71A_n71A3 | DC_71A_n71A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void. NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 7: Single UL allowed due to potential emission issues and self-interference. |  |  |

### 5.5B.4	Inter-band EN-DC within FR1

#### 5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) | Mandatory simultaneous Rx/Tx |
| --- | --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |  |  |
| DC_1A_n5A | DC_1A_n5A | No |  |  |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A | No |  |  |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |  |  |
| DC_1A_n8A | DC_1A_n8A | No |  |  |
| DC_1A_n20A | DC_1A_n20A | No |  |  |
| DC_1A_n28A | DC_1A_n28A | No |  |  |
| DC_1A-1A_n28A | DC_1A_n28A | No |  |  |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |  |  |
| DC_1A_n40A DC_1A_n40B | DC_1A_n40A | No |  |  |
| DC_1A_n41A7 | DC_1A_n41A | No |  |  |
| DC_1A_n50A | DC_1A_n50A | No |  |  |
| DC_1A_n51A | DC_1A_n51A | No |  |  |
| DC_1A_n71A DC_1A_n71B | DC_1A_n71A | No |  |  |
| DC_1A_n77A DC_1A_n77C | DC_1A_n77A | DC_1_n77 | No | Yes |
| DC_1A_n77(2A) DC_1A_n77(3A) | DC_1A_n77A | DC_1_n77 | No | Yes |
| DC_1A_n78A DC_1A_n78C | DC_1A_n78A | No | No | Yes |
| DC_1A_n78(2A) | DC_1A_n78A | No | No | Yes |
| DC_1A-1A_n78A | DC_1A_n78A | No | No |  |
| DC_1A_n79A DC_1A_n79C | DC_1A_n79A | No | No | Yes |
| DC_2A_n5A | DC_2A_n5A | No |  |  |
| DC_2A-2A_n5A | DC_2A_n5A | No |  |  |
| DC_2A_n7A | DC_2A_n7A | No |  |  |
| DC_2A_n7(2A) | DC_2A_n7A | No |  |  |
| DC_2A_n12A | DC_2A_n12A | No |  |  |
| DC_2A_n25A11, 13, 20 | n25A | DC_2_n25 |  |  |
| DC_2A_n28A | DC_2A_n28A | No |  |  |
| DC_2A_n30A | DC_2A_n30A | No |  |  |
| DC_2A-2A_n30A | DC_2A_n30A | No |  |  |
| DC_2A_n38A | DC_2A_n38A | No |  |  |
| DC_2A-2A_n38A | DC_2A_n38A | No |  |  |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |  |  |
| DC_2A_n41(2A) | DC_2A_n41A | No |  |  |
| DC_2A-2A_n41A | DC_2A_n41A | No |  |  |
| DC_2A_n46A | DC_2A_n46A | No |  |  |
| DC_2A_n48A DC_2A_n48B | DC_2A_n48A | No |  |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |  |
| DC_2A_n66(2A) | DC_2A_n66A | DC_2_n66 |  |  |
| DC_2A-2A_n66A | DC_2A_n66A | DC_2_n66 |  |  |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A | DC_2A_n71A DC_2C_n71A | No |  |  |
| DC_2A-2A_n71A | DC_2A_n71A | No |  |  |
| DC_2A_n77A DC_2A_n77C | DC_2A_n77A | DC_2_n77 |  |  |
| DC_2A-2A_n77A DC_2A-2A_n77C | DC_2A_n77A | DC_2_n77 |  |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |  |
| DC_2A-2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |  |
| DC_2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |  |
| DC_2A-2A_n78A | DC_2A_n78A | DC_2_n78 |  |  |
| DC_3A_n1A DC_3C_n1A | DC_3A_n1A DC_3C_n1A | DC_3_n1 |  |  |
| DC_3A-3A_n1A | DC_3A_n1A | DC_3_n1 |  |  |
| DC_3A_n5A DC_3C_n5A | DC_3A_n5A DC_3C_n5A | DC_3_n5 |  |  |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | DC_3A_n7A DC_3A_n7B DC_3C_n7A | No |  |  |
| DC_3A-3A_n7A DC_3A-3A_n7B | DC_3A_n7A | No |  |  |
| DC_3A_n8A | DC_3A_n8A | No |  |  |
| DC_3A-3A_n8A | DC_3A_n8A | No |  |  |
| DC_3A_n20A DC_3C_n20A | DC_3A_n20A | No |  |  |
| DC_3A_n28A DC_3C_n28A | DC_3A_n28A DC_3C_n28A | No |  |  |
| DC_3A_n34A | DC_3A_n34A | No |  |  |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A | No |  |  |
| DC_3A_n40A DC_3A_n40B | DC_3A_n40A | No |  |  |
| DC_3A_n41A7 DC_3C_n41A | DC_3A_n41A DC_3C_n41A | DC_3_n41 | No |  |
| DC_3A_n50A | DC_3A_n50A | No |  |  |
| DC_3A_n51A | DC_3A_n51A | No |  |  |
| DC_3A_n71A DC_3A_n71B | DC_3A_n71A | No |  |  |
| DC_3A_n77A DC_3A_n77C DC_3C_n77A | DC_3A_n77A DC_3C_n77A | DC_3_n77 | No | Yes |
| DC_3A_n77(2A) DC_3A_n77(3A) DC_3C_n77(2A) | DC_3A_n77A DC_3C_n77A | DC_3_n77 | No | Yes |
| DC_3A-3A_n77A | DC_3A_n77A | DC_3_n77 | No | Yes |
| DC_3A_n78A DC_3A_n78C DC_3C_n78A | DC_3A_n78A DC_3C_n78A | DC_3_ n78 | No | Yes |
| DC_3A_n78(2A) DC_3C_n78(2A) | DC_3A_n78A | DC_3_n78 | No | Yes |
| DC_3A-3A_n78A | DC_3A_n78A | DC_3_n78 | No | Yes |
| DC_3A_n79A DC_3A_n79C DC_3C_n79A | DC_3A_n79A DC_3C_n79A | No | No | Yes |
| DC_4A_n2A | DC_4A_n2A | No |  |  |
| DC_4A_n5A | DC_4A_n5A | DC_4_n5 |  |  |
| DC_4A_n7A | DC_4A_n7A | No |  |  |
| DC_4A_n28A | DC_4A_n28A | No |  |  |
| DC_4A_n38A | DC_4A_n38A | No |  |  |
| DC_4A_n41A | DC_4A_n41A | No |  |  |
| DC_4A_n78A | DC_4A_n78A | No |  |  |
| DC_4A_n78(2A) | DC_4A_n78A | No |  |  |
| DC_5A_n1A | DC_5A_n1A | No |  |  |
| DC_5A_n2A DC_5B_n2A | DC_5A_n2A | No |  |  |
| DC_5A-5A_n2A | DC_5A_n2A | No |  |  |
| DC_5A_n3A | DC_5A_n3A | DC_5_n3 |  |  |
| DC_5A_n7A | DC_5A_n7A | DC_5_n7 |  |  |
| DC_5A_n7(2A) | DC_5A_n7A | DC_5_n7 |  |  |
| DC_5A_n12A | DC_5A_n12A | No |  |  |
| DC_5A_n30A | DC_5A_n30A | No |  |  |
| DC_5A_n38A | DC_5A_n38A | DC_5_n38 |  |  |
| DC_5A_n40A | DC_5A_n40A | No |  |  |
| DC_5A_n48A DC_5A_n48B | DC_5A_n48A | No |  |  |
| DC_5A_n66A DC_5B_n66A | DC_5A_n66A | DC_5_n66 |  |  |
| DC_5A-5A_n66A | DC_5A_n66A | DC_5_n66 |  |  |
| DC_5A_n77A DC_5A_n77C | DC_5A_n77A | No |  |  |
| DC_5A_n77(2A) | DC_5A_n77A | No |  |  |
| DC_5A_n71A | DC_5A_n71A | No |  |  |
| DC_5A_n78A DC_5A_n78C | DC_5A_n78A | No | No | Yes |
| DC_5A_n78(2A) | DC_5A_n78A | No | No | Yes |
| DC_5A_n79A | DC_5A_n79A | No | No |  |
| DC_7A_n1A DC_7C_n1A | DC_7A_n1A DC_7C_n1A | No |  |  |
| DC_7A-7A_n1A | DC_7A_n1A | No |  |  |
| DC_7A_n2A DC_7C_n2A | DC_7A_n2A | No |  |  |
| DC_7A_n3A DC_7C_n3A | DC_7A_n3A DC_7C_n3A | No |  |  |
| DC_7A_n5A DC_7C_n5A | DC_7A_n5A DC_7C_n5A | DC_7_n5 |  |  |
| DC_7A-7A_n5A | DC_7A_n5A | DC_7_n5 |  |  |
| DC_7A_n8A | DC_7A_n8A | No |  |  |
| DC_7A-7A_n8A | DC_7A_n8A | No |  |  |
| DC_7A-7A_n78A DC_7A-7A_n78C | DC_7A_n78A | No |  | Yes |
| DC_7A-7A_n78(2A) | DC_7A_n78A | No |  | Yes |
| DC_7A_n20A | DC_7A_n20A | No |  |  |
| DC_7A_n25A DC_7C_n25A | DC_7A_n25A | No |  |  |
| DC_7A-7A_n25A | DC_7A_n25A | No |  |  |
| DC_7A_n28A DC_7C_n28A | DC_7A_n28A DC_7C_n28A | No |  |  |
| DC_7A_n40A | DC_7A_n40A | Yes |  |  |
| DC_7A_n51A | DC_7A_n51A | No |  |  |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |  |  |
| DC_7A-7A_n66A | DC_7A_n66A | No |  |  |
| DC_7A_n71A | DC_7A_n71A | No |  |  |
| DC_7A_n77A | DC_7A_n77A | No |  | Yes |
| DC_7C_n77A | DC_7A_n77A | No |  |  |
| DC_7A_n77(2A) DC_7C_n77(2A) | DC_7A_n77A | No |  |  |
| DC_7A-7A_n77A | DC_7A_n77A | No |  | Yes |
| DC_7A-7A_n77(2A) | DC_7A_n77A | No |  |  |
| DC_7A_n78A DC_7C_n78A DC_7A_n78C | DC_7A_n78A DC_7C_n78A | No |  | Yes |
| DC_7A_n78(2A) DC_7C_n78(2A) | DC_7A_n78A DC_7C_n78A | No |  | Yes |
| DC_7A_n79A DC_7A_n79C | DC_7A_n79A | No |  |  |
| DC_8A_n1A | DC_8A_n1A | No |  |  |
| DC_8A_n2A | DC_8A_n2A | DC_8_n2 |  |  |
| DC_8A_n3A | DC_8A_n3A | No |  |  |
| DC_8A_n7A | DC_8A_n7A | No |  |  |
| DC_8A_n20A | DC_8A_n20A | Yes |  |  |
| DC_8A_n28A | DC_8A_n28A | No |  |  |
| DC_8A_n34A | DC_8A_n34A | No |  |  |
| DC_8A_n39A | DC_8A_n39A | No |  |  |
| DC_8A_n40A | DC_8A_n40A | No |  | Yes |
| DC_8A_n41A | DC_8A_n41A | No | No | Yes |
| DC_8A_n41C | DC_8A_n41A | No | No |  |
| DC_8A_n41(2A) | DC_8A_n41A | No | No |  |
| DC_8A_n77A | DC_8A_n77A | No | No | Yes |
| DC_8A_n77(2A) DC_8A_n77(3A) | DC_8A_n77A | No | No | Yes |
| DC_8A_n78A | DC_8A_n78A | No | No | Yes |
| DC_8A_n78(2A) | DC_8A_n78A | No | No | Yes |
| DC_8A_n79A | DC_8A_n79A DC_8A_n79C | No | No | Yes |
| DC_8A_n79C | DC_8A_n79A DC_8A_n79C | No | No |  |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |  |  |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |  |  |
| DC_11A_n1A | DC_11A_n1A | No |  |  |
| DC_11A_n3A | DC_11A_n3A | No |  |  |
| DC_11A_n28A | DC_11A_n28A | No |  |  |
| DC_11A_n41A | DC_11A_n41A | No |  | Yes |
| DC_11A_n77A | DC_11A_n77A | No | No | Yes |
| DC_11A_n77(2A) DC_11A_n77(3A) | DC_11A_n77A | No | No | Yes |
| DC_11A_n78A | DC_11A_n78A | No | No | Yes |
| DC_11A_n79A | DC_11A_n79A | No |  | Yes |
| DC_12A_n2A | DC_12A_n2A | No |  |  |
| DC_12A_n5A | DC_12A_n5A | No |  |  |
| DC_12A_n7A | DC_12A_n7A | No |  |  |
| DC_12A_n7(2A) | DC_12A_n7A | No |  |  |
| DC_12A_n25A | DC_12A_n25A | No |  |  |
| DC_12A_n30A | DC_12A_n30A | No |  |  |
| DC_12A_n38A | DC_12A_n38A | No |  |  |
| DC_12A_n41A | DC_12A_n41A | No |  |  |
| DC_12A_n66A | DC_12A_n66A | No |  |  |
| DC_12A_n66(2A) | DC_12A_n66A | No |  |  |
| DC_12A_n71A | DC_12A_n71A18,19 | DC_12_n71 |  |  |
| DC_12A_n77A | DC_12A_n77A | DC_12_n77 |  |  |
| DC_12A_n78A | DC_12A_n78A | DC_12_n78 |  |  |
| DC_12A_n78(2A) | DC_12A_n78A | DC_12_n78 |  |  |
| DC_13A_n2A | DC_13A_n2A | No |  |  |
| DC_13A_n5A | DC_13A_n5A | DC_13_n5 |  |  |
| DC_13A_n7A | DC_13A_n7A | No |  |  |
| DC_13A_n7(2A) | DC_13A_n7A | No |  |  |
| DC_13A_n25A | DC_13A_n25A | No |  |  |
| DC_13A_n48A DC_13A_n48B | DC_13A_n48A | No |  |  |
| DC_13A_n66A | DC_13A_n66A | No |  |  |
| DC_13A_n71A | DC_13A_n71A | No |  |  |
| DC_13A_n77A DC_13A_n77C | DC_13A_n77A | No |  |  |
| DC_13A_n78A | DC_13A_n78A | No |  |  |
| DC_13A_n78(2A) | DC_13A_n78A | No |  |  |
| DC_14A_n2A | DC_14A_n2A | No |  |  |
| DC_14A_n5A | DC_14A_n5A | DC_14_n5 |  |  |
| DC_14A_n30A | DC_14A_n30A | No |  |  |
| DC_14A_n66A | DC_14A_n66A | No |  |  |
| DC_14A_n77A | DC_14A_n77A | No |  |  |
| DC_18A_n3A | DC_18A_n3A | No |  |  |
| DC_18A_n28A | DC_18A_n28A | No |  |  |
| DC_18A_n41A16 | DC_18A_n41A | No |  |  |
| DC_18A_n77A DC_18A_n77(2A) | DC_18A_n77A | No | No | Yes |
| DC_18A_n78A | DC_18A_n78A | No | No | Yes |
| DC_18A_n78(2A) | DC_18A_n78A | No | No | Yes |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |  |  |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |  |  |
| DC_18A_n79A | DC_18A_n79A | No |  | Yes |
| DC_19A_n1A | DC_19A_n1A | No |  |  |
| DC_19A_n77A DC_19A_n77C | DC_19A_n77A | No |  | Yes |
| DC_19A_n77(2A) | DC_19A_n77A | No |  | Yes |
| DC_19A_n78A DC_19A_n78C | DC_19A_n78A | No | No | Yes |
| DC_19A_n78(2A) | DC_19A_n78A | No | No | Yes |
| DC_19A_n79A DC_19A_n79C | DC_19A_n79A | No | No | Yes |
| DC_20A_n1A | DC_20A_n1A | No |  |  |
| DC_20A_n3A | DC_20A_n3A | No |  |  |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |  |  |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |  |  |
| DC_20A_n28A8, 11,13 | DC_20A_n28A | No |  |  |
| DC_20A_n38A | DC_20A_n38A | No |  |  |
| DC_20A_n41A | DC_20A_n41A | DC_20_n41 |  |  |
| DC_20A_n50A | DC_20A_n50A | No |  |  |
| DC_20A_n51A | DC_20A_n51A | No |  |  |
| DC_20A_n77A | DC_20A_n77A | No |  | Yes |
| DC_20A_n78A DC_20A_n78C | DC_20A_n78A | No |  | Yes |
| DC_20A_n78(2A) | DC_20A_n78A | No |  | Yes |
| DC_21A_n1A | DC_21A_n1A | No |  |  |
| DC_21A_n28A17 | DC_21A_n28A | DC_21_n28 |  |  |
| DC_21A_n77A DC_21A_n77C | DC_21A_n77A | No |  | Yes |
| DC_21A_n77(2A) | DC_21A_n77A | No |  | Yes |
| DC_21A_n78A DC_21A_n78C | DC_21A_n78A | No | No | Yes |
| DC_21A_n78(2A) | DC_21A_n78A | No | No | Yes |
| DC_21A_n79A DC_21A_n79C | DC_21A_n79A | No | No | Yes |
| DC_25A_n41A | DC_25A_n41A | No |  |  |
| DC_25A-25A_n41A | DC_25A_n41A | No |  |  |
| DC_25A_n77A | DC_25A_n77A | DC_25_n77 |  |  |
| DC_25A-25A_n77A | DC_25A_n77A | DC_25_n77 |  |  |
| DC_25A_n78A | DC_25A_n78A | DC_25_n78 |  |  |
| DC_25A-25A_n78A | DC_25A_n78A | DC_25_n78 |  |  |
| DC_26A_n25A | DC_26A_n25A | No |  |  |
| DC_26A_n41A | DC_26A_n41A | No |  |  |
| DC_26A_n77A | DC_26A_n77A | No |  | Yes |
| DC_26A_n78A | DC_26A_n78A | No |  | Yes |
| DC_26A_n79A | DC_26A_n79A | No |  | Yes |
| DC_28A_n1A | DC_28A_n1A | No |  |  |
| DC_28A_n2A | DC_28A_n2A | No |  |  |
| DC_28A_n3A | DC_28A_n3A | No |  |  |
| DC_28A_n5A8 | DC_28A_n5A | No |  |  |
| DC_28A_n7A DC_28A_n7B | DC_28A_n7A DC_28A_n7B | No |  |  |
| DC_28A_n51A | DC_28A_n51A | No |  |  |
| DC_28A_n8A | DC_28A_n8A | No |  |  |
| DC_28A_n40A | DC_28A_n40A | No |  |  |
| DC_28A_n41A | DC_28A_n41A | No |  | Yes |
| DC_28A_n50A | DC_28A_n50A | No |  |  |
| DC_28A_n66A | DC_28A_n66A | No |  |  |
| DC_28A_n77A DC_28A_n77C | DC_28A_n77A | No | No | Yes |
| DC_28A_n77(2A) | DC_28A_n77A | No | No | Yes |
| DC_28A_n78A DC_28A_n78C | DC_28A_n78A | No | No | Yes |
| DC_28A_n78(2A) | DC_28A_n78A | No | No | Yes |
| DC_28A_n79A DC_28A_n79C | DC_28A_n79A | No |  | Yes |
| DC_30A_n2A | DC_30A_n2A | No |  |  |
| DC_30A_n5A | DC_30A_n5A | No |  |  |
| DC_30A_n66A | DC_30A_n66A | No |  |  |
| DC_30A_n77A | DC_30A_n77A | No |  |  |
| DC_38A_n1A | DC_38A_n1A | No |  |  |
| DC_38A_n3A | DC_38A_n3A | No |  |  |
| DC_38A_n8A | DC_38A_n8A | No |  |  |
| DC_38A_n28A | DC_38A_n28A | No |  |  |
| DC_38A_n78A | DC_38A_n78A | No |  | Yes |
| DC_38A_n79A DC_38A_n79C | DC_38A_n79A | No |  |  |
| DC_39A_n40A3 | DC_39A_n40A | No |  |  |
| DC_39A_n41A3 DC_39C_n41A3 | DC_39A_n41A DC_39C_n41A | No | No |  |
| DC_39A_n78A5 | DC_39A_n78A | No |  | Yes |
| DC_39A_n79A DC_39A_n79C | DC_39A_n79A | No | No | Yes |
| DC_40A_n1A DC_40C_n1A | DC_40A_n1A | No |  |  |
| DC_40A_n41A3 DC_40A_n41C3 DC_40C_n41A3 | DC_40A_n41A | No |  |  |
| DC_40A_n41(2A)3 | DC_40A_n41A | No |  |  |
| DC_40A_n77A | DC_40A_n77A | No |  |  |
| DC_40A_n78A DC_40C_n78A | DC_40A_n78A DC_40C_n78A | No |  |  |
| DC_40A_n78(2A) DC_40C_n78(2A) | DC_40A_n78A DC_40C_n78A | No |  |  |
| DC_40A_n79A12 DC_40A_n79C12 DC_40C_n79A12 | DC_40A_n79A | No | No | Yes |
| DC_41A_n1A DC_41C_n1A | DC_41A_n1A DC_41C_n1A | No | DC_41A_n1A DC_41C_n1A |  |
| DC_41A_n3A DC_41C_n3A | DC_41A_n3A DC_41C_n3A | No |  | Yes |
| DC_41A_n28A DC_41C_n28A | DC_41A_n28A DC_41C_n28A | No |  | Yes |
| DC_41A_n77A DC_41C_n77A | DC_41A_n77A DC_41C_n77A | No |  |  |
| DC_41A_n77(2A) DC_41C_n77(2A) | DC_41A_n77A DC_41C_n77A | No |  |  |
| DC_41A_n78A DC_41C_n78A DC_41D_n78A | DC_41A_n78A DC_41C_n78A | No |  |  |
| DC_41A_n78(2A) DC_41C_n78(2A) | DC_41A_n78A DC_41C_n78A | No |  |  |
| DC_41A_n79A6 DC_41A_n79C6 DC_41C_n79A6 | DC_41A_n79A DC_41C_n79A | No | No | Yes |
| DC_42A_n1A DC_42C_n1A | DC_42A_n1A DC_42C_n1A | No |  | Yes |
| DC_42A_n3A DC_42C_n3A | DC_42A_n3A DC_42C_n3A | DC_42_n3 |  | Yes |
| DC_42A_n28A DC_42C_n28A | DC_42A_n28A DC_42C_n28A | No |  | Yes |
| DC_42A_n51A | DC_42A_n51A | No |  |  |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A | N/A |  |  |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 | N/A | N/A |  |  |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C3,4,9,11 DC_42E_n78A3,4,9,11 DC_42E_n78C3,4,9,11 | N/A | N/A |  |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42D_n79C9,15 DC_42E_n79A9,15 DC_42E_n79C9,15 | N/A | N/A |  |  |
| DC_46A_n77A2 | N/A | N/A |  |  |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 | N/A | N/A |  |  |
| DC_48A_n5A DC_48C_n5A DC_48D_n5A DC_48E_n5A | DC_48A_n5A | No |  |  |
| DC_48A_n12A | DC_48A_n12A | No |  |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | DC_48A_n25A | No |  |  |
| DC_48A_n46A DC_48B_n46A DC_48C_n46A DC_48D_n46A DC_48E_n46A DC_48A_n46B DC_48B_n46B DC_48C_n46B DC_48D_n46B DC_48E_n46B DC_48A_n46C DC_48B_n46C DC_48C_n46C DC_48D_n46C DC_48E_n46C DC_48A_n46D DC_48B_n46D DC_48C_n46D DC_48D_n46D DC_48E_n46D DC_48A_n46E DC_48B_n46E DC_48C_n46E DC_48D_n46E DC_48E_n46E | DC_48A_n46A DC_48B_n46A | No |  |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A DC_48E_n66A | DC_48A_n66A | No |  |  |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A | DC_48A_n71A | No |  |  |
| DC_48A-48A_n71A DC_48A-48A-48A_n71A | DC_48A_n71A | No |  |  |
| DC_48A_n77A3. 4. 9, 11 DC_48C_n77A3. 4. 9, 11 DC_48A_n77C3. 4. 9, 11 DC_48C_n77C3. 4. 9, 11 DC_48D_n77A3. 4. 9, 11 DC_48D_n77C3. 4. 9, 11 DC_48E_n77A3. 4. 9, 11 | N/A | N/A |  |  |
| DC_66A_n2A DC_66B_n2A DC_66C_n2A | DC_66A_n2A | DC_66_n2 |  |  |
| DC_66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |  |
| DC_66A-66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |  |
| DC_66A_n5A DC_66B_n5A DC_66C_n5A | DC_66A_n5A | DC_66_n5 |  |  |
| DC_66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |  |
| DC_66A-66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |  |
| DC_66A_n7A | DC_66A_n7A | No |  |  |
| DC_66A_n7(2A) | DC_66A_n7A | No |  |  |
| DC_66A-66A_n7A | DC_66A_n7A | No |  |  |
| DC_66A-66A_n7(2A) | DC_66A_n7A | No |  |  |
| DC_66A_n12A | DC_66A_n12A | No |  |  |
| DC_66A_n25A | DC_66A_n25A | DC_66_n25 |  |  |
| DC_66A_n28A | DC_66A_n28A | No |  |  |
| DC_66A_n30A | DC_66A_n30A | No |  |  |
| DC_66A-66A_n30A | DC_66A_n30A | No |  |  |
| DC_66A_n38A | DC_66A_n38A | No |  |  |
| DC_66A-66A_n38A | DC_66A_n38A | No |  |  |
| DC_66A_n41A DC_66A_n41C | DC_66A_n41A | No |  |  |
| DC_66A_n41(2A) | DC_66A_n41A | No |  |  |
| DC_66A_n46A | DC_66A_n46A | No |  |  |
| DC_66A_n48A DC_66A_n48B | DC_66A_n48A | No |  |  |
| DC_66A-66A_n48A DC_66A-66A_n48B | DC_66A_n48A | No |  |  |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B | DC_66A_n71A | No |  |  |
| DC_66A-66A_n71A | DC_66A_n71A | No |  |  |
| DC_66A_n77A DC_66A_n77C | DC_66A_n77A | DC_66_n77 |  |  |
| DC_66A-66A_n77A DC_66A-66A_n77C DC_66A-66A-66A_n77C | DC_66A_n77A | DC_66_n77 |  |  |
| DC_66A-66A-66A_n77A | DC_66A_n77A | DC_66_n77 |  |  |
| DC_66A_n78A | DC_66A_n78A | No |  |  |
| DC_66A_n78(2A) | DC_66A_n78A | No |  |  |
| DC_66A-66A_n78A | DC_66A_n78A | No |  |  |
| DC_66A-66A_n78(2A) | DC_66A_n78A | No |  |  |
| DC_71A_n2A | DC_71A_n2A | No |  |  |
| DC_71A_n5A | DC_71A_n5A | No |  |  |
| DC_71A_n38A | DC_71A_n38A | No |  |  |
| DC_71A_n41A | DC_71A_n41A | No |  |  |
| DC_71A_n48A | DC_71A_n48A | No |  |  |
| DC_71A_n66A | DC_71A_n66A | No |  |  |
| DC_71A_n78A | DC_71A_n78A | No |  |  |
| DC_71A_n78(2A) | DC_71A_n78A | No |  |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Void. NOTE 8: The frequency range in band n28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 10: Void. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16, the power imbalance requirement defined in clause 7.6B.2.6 apply. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 18: Only single switched UL is supported. NOTE 19: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 20: The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. |  |  |  |  |

#### 5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Mandatory simultaneous Rx/Tx |
| --- | --- | --- |
| DC_1A-3A_n3A | DC_1A_n3A DC_3A_n3A2 |  |
| DC_1A-3A_n5A DC_1A-3C_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A |  |
| DC_1A-3A_n7A DC_1A-3A_n7B DC_1A-3C_n7A DC_1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |  |
| DC_1A-1A-3A_n7A DC_1A-1A-3A_n7B DC_1A-1A-3C_n7A DC_1A-1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |  |
| DC_1A-3A-3A_n7A DC_1A-3A-3A_n7B | DC_1A_n7A DC_3A_n7A |  |
| DC_1A-1A-3A-3A_n7A | DC_1A_n7A DC_3A_n7A |  |
| DC_1A-3A_n8A | DC_1A_n8A DC_3A_n8A |  |
| DC_1A-3A_n28A DC_1A-3C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |  |
| DC_1A-1A-3A_n28A DC_1A-1A-3C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |  |
| DC_1A_n3A-n28A | DC_1A_n3A DC_1A_n28A |  |
| DC_1A-3A_n38A | DC_1A_n38A DC_3A_n38A |  |
| DC_1A-3A_n40A | DC_1A_n40A DC_3A_n40A |  |
| DC_1A-3A_n41A | DC_1A_n41A DC_3A_n41A | Yes |
| DC_1A-3C_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A |  |
| DC_1A_n3A-n41A | DC_1A_n3A DC_1A_n41A | Yes |
| DC_1A-3A_n71A DC_1A-3A_n71B | DC_1A_n71A DC_3A_n71A |  |
| DC_1A-3A_n77A DC_1A-3A_n77C DC_1A-3C_n77A | DC_1A_n77A DC_3A_n77A DC_3C_n77A | Yes |
| DC_1A-3A_n77(2A) | DC_1A_n77A DC_3A_n77A | Yes |
| DC_1A-3C_n77(2A) | DC_1A_n77A DC_3A_n77A DC_3C_n77A |  |
| DC_1A-3A_n77(3A) | DC_1A_n77A DC_3A_n77A | Yes |
| DC_1A-3A_n78A DC_1A-3A_n78C DC_1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A | Yes |
| DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_3C_n78A | Yes |
| DC_1A-1A-3A_n78A DC_1A-1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |  |
| DC_1A_n3A-n8A | DC_1A_n3A DC_1A_n8A |  |
| DC_1A_n3A-n77A | DC_1A_n3A DC_1A_n77A | Yes |
| DC_1A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A | Yes |
| DC_1A_n3A-n78A | DC_1A_n3A DC_1A_n78A | Yes |
| DC_1A_n3A-n79A | DC_1A_n3A DC_1A_n79A |  |
| DC_1A-3A_n79A DC_1A-3A_n79C | DC_1A_n79A DC_3A_n79A | Yes |
| DC_1A-5A_n77A | DC_1A_n77A DC_5A_n77A |  |
| DC_1A-5A_n77(2A) | DC_1A_n77A DC_5A_n77A |  |
| DC_1A-5A_n78A DC_1A-5A_n78C | DC_1A_n78A DC_5A_n78A | Yes |
| DC_1A-5A_n78(2A) | DC_1A_n78A DC_5A_n78A | Yes |
| DC_1A-1A-5A_n78A | DC_1A_n78A DC_5A_n78A |  |
| DC_1A-5A_n79A | DC_1A_n79A DC_5A_n79A |  |
| DC_1A_n5A-n78A | DC_1A_n5A DC_1A_n78A | Yes |
| DC_1A-7A_n3A DC_1A-7C_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A |  |
| DC_1A-7A_n5A DC_1A-7C_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A |  |
| DC_1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |  |
| DC_1A-1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |  |
| DC_1A-7A_n8A | DC_1A_n8A DC_7A_n8A |  |
| DC_1A-7A_n28A | DC_1A_n28A DC_7A_n28A | Yes |
| DC_1A-7C_n28A | DC_1A_n28A DC_7A_n28A DC_7C_n28A |  |
| DC_1A-1A-7A_n28A | DC_1A_n28A DC_7A_n28A |  |
| DC_1A-7A_n38A17,18 | N/A |  |
| DC_1A-7A_n40A | DC_1A_n40A DC_7A_n40A |  |
| DC_1A-7A_n77A | DC_1A_n77A DC_7A_n77A |  |
| DC_1A-7A_n77(2A) | DC_1A_n77A DC_7A_n77A |  |
| DC_1A-7A-7A_n77A | DC_1A_n77A DC_7A_n77A |  |
| DC_1A-7A-7A_n77(2A) | DC_1A_n77A DC_7A_n77A |  |
| DC_1A-7A_n78A DC_1A-7A_n78C | DC_1A_n78A DC_7A_n78A | Yes |
| DC_1A-7C_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A |  |
| DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_7C_n78A | Yes |
| DC_1A-7A-7A_n78A DC_1A-7A-7A_n78C | DC_1A_n78A DC_7A_n78A | Yes |
| DC_1A-7A-7A_n78(2A) | DC_1A_n78A DC_7A_n78A | Yes |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | DC_1A_n7A DC_1A_n78A |  |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |  |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A |  |
| DC_1A_n8A-n40A | DC_1A_n8A DC_1A_n40A |  |
| DC_1A-8A_n77A | DC_1A_n77A DC_8A_n77A | Yes |
| DC_1A-8A_n77(2A) | DC_1A_n77A DC_8A_n77A | Yes |
| DC_1A-8A_n77(3A) | DC_1A_n77A DC_8A_n77A | Yes |
| DC_1A-8A_n78A | DC_1A_n78A DC_8A_n78A | Yes |
| DC_1A-8A_n78(2A) | DC_1A_n78A DC_8A_n78A | Yes |
| DC_1A_n8A-n78A | DC_1A_n8A DC_1A_n78A | Yes |
| DC_1A-8A_n79A | DC_1A_n79A DC_8A_n79A | Yes |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A |  |
| DC_1A-11A_n28A | DC_1A_n28A DC_11A_n28A |  |
| DC_1A-11A_n41A | DC_1A_n41A DC_11A_n41A | Yes |
| DC_1A-11A_n77A | DC_1A_n77A DC_11A_n77A | Yes |
| DC_1A-11A_n77(2A) | DC_1A_n77A DC_11A_n77A | Yes |
| DC_1A-11A_n77(3A) | DC_1A_n77A DC_11A_n77A | Yes |
| DC_1A-11A_n78A | DC_1A_n78A DC_11A_n78A | Yes |
| DC_1A-11A_n79A | DC_1A_n79A DC_11A_n79A | Yes |
| DC_1A-18A_n3A | DC_1A_n3A DC_18A_n3A |  |
| DC_1A-18A_n28A | DC_1A_n28A DC_18A_n28A |  |
| DC_1A-18A_n41A | DC_1A_n41A DC_18A_n41A |  |
| DC_1A-18A_n77A | DC_1A_n77A DC_18A_n77A | Yes |
| DC_1A-18A_n77(2A) | DC_1A_n77A DC_18A_n77A | Yes |
| DC_1A-18A_n78A | DC_1A_n78A DC_18A_n78A | Yes |
| DC_1A-18A_n78(2A) | DC_1A_n78A DC_18A_n78A | Yes |
| DC_1A-18A_n79A | DC_1A_n79A DC_18A_n79A |  |
| DC_1A-19A_n77A DC_1A-19A_n77C | DC_1A_n77A DC_19A_n77A | Yes |
| DC_1A-19A_n77(2A) | DC_1A_n77A DC_19A_n77A | Yes |
| DC_1A-19A_n78A DC_1A-19A_n78C | DC_1A_n78A DC_19A_n78A | Yes |
| DC_1A-19A_n78(2A) | DC_1A_n78A DC_19A_n78A | Yes |
| DC_1A-19A_n79A DC_1A-19A_n79C | DC_1A_n79A DC_19A_n79A | Yes |
| DC_1A-20A_n3A DC_1C-20A_n3A | DC_1A_n3A DC_20A_n3A |  |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |  |
| DC_1A-20A_n28A | DC_1A_n28A DC_20A_n28A |  |
| DC_1A-20A_n38A | DC_1A_n38A DC_20A_n38A |  |
| DC_1A-20A_n41A | DC_1A_n41A DC_20A_n41A |  |
| DC_1A-20A_n78A | DC_1A_n78A DC_20A_n78A | Yes |
| DC_1A-21A_n28A13 | DC_1A_n28A DC_21A_n28A |  |
| DC_1A-21A_n77A DC_1A-21A_n77C | DC_1A_n77A DC_21A_n77A | Yes |
| DC_1A-21A_n77(2A) | DC_1A_n77A DC_21A_n77A | Yes |
| DC_1A-21A_n78A DC_1A-21A_n78C | DC_1A_n78A DC_21A_n78A | Yes |
| DC_1A-21A_n78(2A) | DC_1A_n78A DC_21A_n78A | Yes |
| DC_1A-21A_n79A DC_1A-21A_n79C | DC_1A_n79A DC_21A_n79A | Yes |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |  |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |  |
| DC_1A-28A_n7A DC_1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |  |
| DC_1A-1A-28A_n7A DC_1A-1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |  |
| DC_1A_n28A-n40A | DC_1A_n28A DC_1A_n40A |  |
| DC_1A-28A_n40A | DC_1A_n40A DC_28A_n40A |  |
| DC_1A_n28A-n41A | DC_1A_n28A DC_1A_n41A | Yes |
| DC_1A_n28A-n75A | DC_1A_n28A DC_1A_n75A |  |
| DC_1A-28A_n77A DC_1A-28A_n77C | DC_1A_n77A DC_28A_n77A | Yes |
| DC_1A-28A_n78A DC_1A-28A_n78C | DC_1A_n78A DC_28A_n78A | Yes |
| DC_1A-1A-28A_n78A | DC_1A_n78A DC_28A_n78A |  |
| DC_1A_n28A-n77A | DC_1A_n28A DC_1A_n77A | Yes |
| DC_1A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A | Yes |
| DC_1A_n28A-n78A | DC_1A_n28A DC_1A_n78A | Yes |
| DC_1A-28A_n79A DC_1A-28A_n79C | DC_1A_n79A DC_28A_n79A | Yes |
| DC_1A_n28A-n79A | DC_1A_n28A DC_1A_n79A | Yes |
| DC_1A-32A_n3A | DC_1A_n3A |  |
| DC_1A-32A_n8A | DC_1A_n8A |  |
| DC_1A-32A_n28A | DC_1A_n28A |  |
| DC_1A-32A_n78A DC_1A-32A_n78C | DC_1A_n78A |  |
| DC_1A-32A_n78(2A) | DC_1A_n78A |  |
| DC_1A-38A_n3A | DC_1A_n3A |  |
| DC_1A-38A_n8A | DC_1A_n8A DC_38A_n8A |  |
| DC_1A-38A_n28A | DC_1A_n28A DC_38A_n28A |  |
| DC_1A-(n)38AA | DC_1A_n38A |  |
| DC_1A_n38A-n78A | DC_1A_n78A |  |
| DC_1A-38A_n78A | DC_1A_n78A |  |
| DC_1A-40A_n78A DC_1A-40C_n78A | DC_1A_n78A DC_40A_n78A |  |
| DC_1A-40A_n78(2A) DC_1A-40C_n78(2A) | DC_1A_n78A DC_40A_n78A |  |
| DC_1A_n40A-n78A | DC_1A_n40A DC_1A_n78A |  |
| DC_1A_n40A-n78(2A) | DC_1A_n40A DC_1A_n78A |  |
| DC_1A-41A_n3A DC_1A-41C_n3A | DC_1A_n3A DC_41A_n3A DC_41C_n3A | Yes |
| DC_1A-41A_n28A DC_1A-41C_n28A | DC_1A_n28A DC_41A_n28A DC_41C_n28A | Yes |
| DC_1A-(n)41AA DC_1A-(n)41CA DC_1A-(n)41DA | DC_1A_n41A |  |
| DC_1A-41A_n41A DC_1A-41C_n41A | DC_1A_n41A |  |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A DC_41C_n77A |  |
| DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | DC_1A_n77A DC_41A_n77A DC_41C_n77A |  |
| DC_1A_n41A-n77A | DC_1A_n41A DC_1A_n77A |  |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A DC_41C_n78A |  |
| DC_1A_n41A-n78A | DC_1A_n41A DC_1A_n78A |  |
| DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | DC_1A_n78A DC_41A_n78A DC_41C_n78A |  |
| DC_1A-41A_n79A DC_1A-41C_n79A | DC_1A_n79A | Yes |
| DC_1A-42A_n3A | DC_1A_n3A DC_42A_n3A | Yes |
| DC_1A-42C_n3A | DC_1A_n3A DC_42A_n3A DC_42C_n3A | Yes |
| DC_1A-42A_n28A | DC_1A_n28A DC_42A_n28A | Yes |
| DC_1A-42C_n28A | DC_1A_n28A DC_42A_n28A DC_42C_n28A | Yes |
| DC_1A-42A_n77A15,16 DC_1A-42A_n77C15,16 DC_1A-42C_n77A15,16 DC_1A-42C_n77C15,16 DC_1A-42D_n77A15,16 DC_1A-42D_n77C15,16 DC_1A-42E_n77A15,16 DC_1A-42E_n77C15,16 | DC_1A_n77A |  |
| DC_1A-42A_n77(2A)15,16 DC_1A-42C_n77(2A)15,16 | DC_1A_n77A |  |
| DC_1A-42A_n78A15,16 DC_1A-42A_n78C15,16 DC_1A-42C_n78A15,16 DC_1A-42C_n78C15,16 DC_1A-42D_n78A15,16 DC_1A-42D_n78C15,16 DC_1A-42E_n78A15,16 DC_1A-42E_n78C15,16 | DC_1A_n78A |  |
| DC_1A-42A_n79A DC_1A-42A_n79C DC_1A-42C_n79A DC_1A-42C_n79C DC_1A-42D_n79A DC_1A-42D_n79C DC_1A-42E_n79A DC_1A-42E_n79C | DC_1A_n79A |  |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | DC_1A_n78A |  |
| DC_1A_n77A-n79A DC_1A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A |  |
| DC_1A_SUL_n77A-n80A | DC_1A_n77A DC_1A_n80A |  |
| DC_1A_SUL_n77A-n84A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |  |
| DC_1A_n78A-n79A | DC_1A_n78A DC_1A_n79A |  |
| DC_1A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A |  |
| DC_1A_SUL_n78A-n84A | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A | Yes |
| DC_1A_SUL_n79A-n84A | DC_1A_n79A, DC_1A_n84A_ULSUP-TDM_n79A |  |
| DC_2A_n2A-n38A | DC_2A_n38A |  |
| DC_2A_n2A-n41A | DC_2A_n41A |  |
| DC_2A_n2A-n66A | DC_2A_n66A |  |
| DC_2A_n2A-n71A | DC_2A_n71A |  |
| DC_2A_n2A-n77A14 DC_2A_n2A-n77C14 | DC_2A_n77A14 |  |
| DC_2A_n2A-n78A | DC_2A_n78A |  |
| DC_2A-4A_n28A | DC_2A_n28A DC_4A_n28A |  |
| DC_2A-4A_n38A | DC_2A_n38A DC_4A_n38A |  |
| DC_2A-4A_n41A | DC_2A_n41A DC_4A_n41A |  |
| DC_2A-5A_n2A | DC_5A_n2A |  |
| DC_2A-5B_n2A | DC_5A_n2A |  |
| DC_2A-5A-5A_n2A | DC_5A_n2A |  |
| DC_2A-5A_n5A | DC_2A_n5A |  |
| DC_2A-2A-5A_n5A | DC_2A_n5A |  |
| DC_2A-(n)5AA | DC_2A_n5A DC_(n)5AA2 |  |
| DC_2A-5A_n7A | DC_2A_n7A DC_5A_n7A |  |
| DC_2A-5A_n12A | DC_2A_n12A DC_5A_n12A |  |
| DC_2A-5A_n30A | DC_2A_n30A DC_5A_n30A |  |
| DC_2A-2A-5A_n30A | DC_2A_n30A DC_5A_n30A |  |
| DC_2A-5A_n48A DC_2A-5A_n48B | DC_2A_n48A DC_5A_n48A |  |
| DC_2A-5A_n66A DC_2A-5B_n66A | DC_2A_n66A DC_5A_n66A |  |
| DC_2A-5A-5A_n66A DC_2A-2A-5A_n66A | DC_2A_n66A DC_5A_n66A |  |
| DC_2A-5A_n71A | DC_2A_n71A DC_5A_n71A |  |
| DC_2A-5A_n77A14 DC_2A-5A_n77C14 DC_2A-2A-5A_n77C14 | DC_2A_n77A14 DC_5A_n77A14 |  |
| DC_2A-2A-5A_n77A14 | DC_2A_n77A14 DC_5A_n77A14 |  |
| DC_2A-5A_n78A | DC_2A_n78A DC_5A_n78A |  |
| DC_2A-5A_n78(2A) | DC_2A_n78A DC_5A_n78A |  |
| DC_2A-7A_n5A DC_2A-7C_n5A | DC_2A_n5A DC_7A_n5A |  |
| DC_2A-7A-7A_n5A | DC_2A_n5A DC_7A_n5A |  |
| DC_2A-7A_n7A | DC_2A_n7A DC_7A_n7A2 |  |
| DC_2A-7A_n28A DC_2A-7C_n28A | DC_2A_n28A DC_7A_n28A |  |
| DC_2A_n5A-n77A14 DC_2A-2A_n5A-n77A14 DC_2A_n5A-n77C14 DC_2A-2A_n5A-n77C14 | DC_2A_n5A DC_2A_n77A14 |  |
| DC_2A-7A_n38A | 2A8 |  |
| DC_2A-2A-7A_n38A | 2A8 |  |
| DC_2A-7A_n66A DC_2A-7C_n66A | DC_2A_n66A DC_7A_n66A |  |
| DC_2A-2A-7C_n66A | DC_2A_n66A DC_7A_n66A |  |
| DC_2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |  |
| DC_2A-2A-7A_n66A | DC_2A_n66A DC_7A_n66A |  |
| DC_2A-2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |  |
| DC_2A_n7A-n66A | DC_2A_n7A DC_7A_n66A |  |
| DC_2A_n7(2A)-n66A | DC_7A_n66A |  |
| DC_2A-7A_n71A | DC_2A_n71A DC_7A_n71A |  |
| DC_2A-2A-7A_n71A | DC_2A_n71A DC_7A_n71A |  |
| DC_2A-7A_n77A DC_2A-7C_n77A | DC_2A_n77A DC_7A_n77A |  |
| DC_2A-7A-7A_n77A | DC_2A_n77A DC_7A_n77A |  |
| DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) | DC_2A_n77A DC_7A_n77A |  |
| DC_2A-7A-7A_n77(2A) | DC_2A_n77A DC_7A_n77A |  |
| DC_2A-7A_n78A DC_2A-7C_n78A | DC_2A_n78A DC_7A_n78A DC_7C_n78A |  |
| DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) | DC_2A_n78A DC_7A_n78A DC_7C_n78A |  |
| DC_2A-2A-7A_n78A | DC_2A_n78A DC_7A_n78A |  |
| DC_2A_n7A-n78A | DC_2A_n7A DC_2A_n78A |  |
| DC_2A_n7(2A)-n78A | DC_2A_n7A DC_2A_n78A |  |
| DC_2A_n7A-n78(2A) | DC_2A_n7A DC_2A_n78A |  |
| DC_2A_n7(2A)-n78(2A) | DC_2A_n7A DC_2A_n78A |  |
| DC_2A-7A-7A_n78A | DC_2A_n78A DC_7A_n78A |  |
| DC_2A-7A-7A_n78(2A) | DC_2A_n78A DC_7A_n78A |  |
| DC_2A-8A_n2A | DC_2A_n2A2 DC_8A_n2A |  |
| DC_2A-12A_n2A | DC_12A_n2A |  |
| DC_2A-12A_n5A | DC_2A_n5A DC_12A_n5A |  |
| DC_2A-12A_n7A | DC_2A_n7A DC_12A_n7A |  |
| DC_2A-12A_n7(2A) | DC_2A_n7A DC_12A_n7A |  |
| DC_2A-(n)12AA | DC_2A_n12A DC_(n)12AA2 |  |
| DC_2A-12A_n30A | DC_2A_n30A DC_12A_n30A |  |
| DC_2A-2A-12A_n30A | DC_2A_n30A DC_12A_n30A |  |
| DC_2A-12A_n41A | DC_2A_n41A DC_12A_n41A |  |
| DC_2A-2A-12A_n41A | DC_2A_n41A DC_12A_n41A |  |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |  |
| DC_2A-2A-12A_n66A | DC_2A_n66A DC_12A_n66A |  |
| DC_2A-12A_n77A14 DC_2A-2A-12A_n77A14 | DC_2A_n77A14 DC_12A_n77A14 |  |
| DC_2A-13A_n2A | DC_13A_n2A |  |
| DC_2A-12A_n78A | DC_2A_n78A DC_12A_n78A |  |
| DC_2A-12A_n78(2A) | DC_2A_n78A DC_12A_n78A |  |
| DC_2A-2A-12A_78A | DC_2A_n78A DC_12A_n78A |  |
| DC_2A-13A_n5A | DC_2A_n5A |  |
| DC_2A-2A-13A_n5A | DC_2A_n5A |  |
| DC_2A-13A_n25A16,20 | DC_13A_n25A |  |
| DC_2A-13A_n48A DC_2A-13A_n48B | DC_2A_n48A DC_13A_n48A |  |
| DC_2A-13A_n66A | DC_2A_n66A DC_13A_n66A |  |
| DC_2A-2A-13A_n66A | DC_2A_n66A DC_13A_n66A |  |
| DC_2A-13A_n77A14 DC_2A-13A_n77C14 DC_2A-2A-13A_n77C14 | DC_2A_n77A14 DC_13A_n77A14 |  |
| DC_2A-2A-13A_n77A | DC_2A_n77A14 DC_13A_n77A14 |  |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |  |
| DC_2A-14A_n30A | DC_2A_n30A DC_14A_n30A |  |
| DC_2A-2A-14A_n30A | DC_2A_n30A DC_14A_n30A |  |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |  |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |  |
| DC_2A-14A_n77A14 DC_2A-2A-14A_n77A14 | DC_2A_n77A14 DC_14A_n77A14 |  |
| DC_2A_n25A-n66A | DC_2A_n66A |  |
| DC_2A-28A_n7A | DC_2A_n7A DC_28A_n7A |  |
| DC_2A-28A_n66A | DC_2A_n66A DC_28A_n66A |  |
| DC_2A-28A_n78A | DC_2A_n78A DC_28A_n78A |  |
| DC_2A-29A_n30A | DC_2A_n30A |  |
| DC_2A-2A-29A_n30A | DC_2A_n30A |  |
| DC_2A-29A_n66A | DC_2A_n66A |  |
| DC_2A-2A-29A_n66A | DC_2A_n66A |  |
| DC_2A-29A_n77A14 DC_2A-2A-29A_n77A14 | DC_2A_n77A14 |  |
| DC_2A-29A_n78A | DC_2A_n78A |  |
| DC_2A-30A_n5A | DC_2A_n5A DC_30A_n5A |  |
| DC_2A-30A_n2A | DC_2A_n2A2 DC_30A_n2A |  |
| DC_2A-2A-30A_n5A | DC_2A_n5A DC_30A_n5A |  |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |  |
| DC_2A-2A-30A_n66A | DC_2A_n66A DC_30A_n66A |  |
| DC_2A-30A_n77A14 DC_2A-2A-30A_n77A14 | DC_2A_n77A14 DC_30A_n77A14 |  |
| DC_2A_n38A-n66A | DC_2A_n38A DC_2A_n66A |  |
| DC_2A_n38A-n71A | DC_2A_n38A DC_2A_n71A |  |
| DC_2A-38A_n78A | DC_2A_n78A DC_38A_n78A |  |
| DC_2A_n38A-n78A | DC_2A_n38A DC_2A_n78A |  |
| DC_2A_n41A-n66A DC_2A_n41C-n66A | DC_2A_n41A DC_2A_n66A |  |
| DC_2A_n41(2A)-n66A | DC_2A_n41A DC_2A_n66A |  |
| DC_2A_n41A-n71A DC_2A_n41C-n71A | DC_2A_n41A DC_2A_n71A |  |
| DC_2A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |  |
| DC_2A-46A_n2A3 DC_2A-46C_n2A3 DC_2A-46D_n2A3 DC_2A-46E_n2A3 | DC_2A_n2A2 |  |
| DC_2A-46A_n5A3 DC_2A-46C_n5A3 DC_2A-46D_n5A3 DC_2A-46E_n5A3 DC_2A-2A-46A_n5A3 DC_2A-2A-46C_n5A3 DC_2A-2A-46D_n5A3 | DC_2A_n5A |  |
| DC_2A-46A_n41A DC_2A-46C_n41A DC_2A-46D_n41A | DC_2A_n41A |  |
| DC_2A-46A_n41(2A) DC_2A-46C_n41(2A) DC_2A-46D_n41(2A) | DC_2A_n41A |  |
| DC_2A-46A_n66A DC_2A-46C_n66A DC_2A-46D_n66A DC_2A-46E_n66A | DC_2A_n66A |  |
| DC_2A-46A_n71A DC_2A-46C_n71A DC_2A-46D_n71A | DC_2A_n71A |  |
| DC_2A-46A_n77A | DC_2A_n77A |  |
| DC_2A-46A-46A_n77A | DC_2A_n77A |  |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | DC_2A_n2A2 |  |
| DC_2A-48A_n5A | DC_2A_n5A DC_48A_n5A |  |
| DC_2A-48C_n5A DC_2A-48D_n5A DC_2A-48E_n5A | DC_2A_n5A |  |
| DC_2A_n48A-n66A | DC_2A_n48A DC_2A_n66A |  |
| DC_2A-48A_n71A | DC_2A_n71A DC_48A_n71A |  |
| DC_2A-48A_n12A | DC_2A_n12A DC_48A_n12A |  |
| DC_2A-48A_n48A | DC_2A_n48A |  |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A DC_2A-48E_n66A | DC_2A_n66A DC_48A_n66A |  |
| DC_2A-48A_n77A14,15,16 | DC_2A_n77A14 |  |
| DC_2A-48A-48A_n77A14,15,16 | DC_2A_n77A DC_48A_n77A |  |
| DC_2A-48A-48A-48A_n77A14,15,16 | DC_2A_n77A DC_48A_n77A |  |
| DC_2A-48C_n77A14,15,16 DC_2A-48D_n77A14,15,16 DC_2A-48E_n77A14,15,16 DC_2A-48A_n77C14,15,16 DC_2A-48C_n77C14,15,16 DC_2A-48D_n77C14,15,16 | DC_2A_n77A14 |  |
| DC_2A-66A_n2A | DC_2A_n2A2 DC_66A_n2A |  |
| DC_2A-66A-66A_n2A | DC_66A_n2A |  |
| DC_2A-66A_n5A DC_2A-66B_n5A | DC_2A_n5A DC_66A_n5A |  |
| DC_2A-2A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |
| DC_2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |
| DC_2A-2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |
| DC_2A-66A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |
| DC_2A-66A_n7A | DC_2A_n7A DC_66A_n7A |  |
| DC_2A-66A-66A_n7A | DC_2A_n7A DC_66A_n7A |  |
| DC_2A-66A_n12A | DC_2A_n12A DC_66A_n12A |  |
| DC_2A-66A_n25A16,20 | DC_66A_n25A |  |
| DC_2A-66A_n28A | DC_2A_n28A DC_66A_n28A |  |
| DC_2A-66A_n30A | DC_2A_n30A DC_66A_n30A |  |
| DC_2A-2A-66A_n30A | DC_2A_n30A DC_66A_n30A |  |
| DC_2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |  |
| DC_2A-2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |  |
| DC_2A-66A_n38A | DC_2A_n38A DC_66A_n38A |  |
| DC_2A-2A-66A_n38A | DC_2A_n38A DC_66A_n38A |  |
| DC_2A-66A-66A_n38A | DC_2A_n38A DC_66A_n38A |  |
| DC_2A-66A_n41A14 DC_2A-66A_n41C DC_2C-66A_n41A | DC_2A_n41A DC_66A_n41A14 |  |
| DC_2A-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |  |
| DC_2A-2A-66A_n41A | DC_2A_n41A DC_66A_n41A |  |
| DC_2A-66A_n48A | DC_2A_n48A DC_66A_n48A |  |
| DC_2A-66A_n48B | DC_2A_n48A DC_66A_n48A |  |
| DC_2A-66A-66A_n48A | DC_2A_n48A DC_66A_n48A |  |
| DC_2A-66A-66A_n48B | DC_2A_n48A DC_66A_n48A |  |
| DC_2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |  |
| DC_2A-66A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |  |
| DC_2A-(n)66AA | DC_2A_n66A |  |
| DC_2A-2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |  |
| DC_2A-2A-66A-66A_n66A | DC_2A_n66A |  |
| DC_2A-66A_n71A DC_2A-66A_n71B DC_2A-66C_n71A DC_2C-66A_n71A | DC_2A_n71A DC_66A_n71A |  |
| DC_2A-2A-66A_n71A | DC_2A_n71A DC_66A_n71A |  |
| DC_2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |  |
| DC_2A-2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |  |
| DC_2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |  |
| DC_2A-66A_n77A14 DC_2A-66A_n77C14 DC_2A-2A-66A_n77C14 DC_2A-66A-66A_n77C14 DC_2A-2A-66A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |  |
| DC_2A-2A-66A_n77A14 | DC_2A_n77A14 DC_66A_n77A14 |  |
| DC_2A-66A-66A_n77A14 | DC_2A_n77A14 DC_66A_n77A14 |  |
| DC_2A-2A-66A-66A_n77A14 | DC_2A_n77A14 DC_66A_n77A14 |  |
| DC_2A_n66A-n77A14 DC_2A_n66A-n77C14 DC_2A-2A_n66A-n77A14 DC_2A-2A_n66A-n77C14 | DC_2A_n77A14 DC_2A_n66A |  |
| DC_2A-66A_n78A | DC_2A_n78A DC_66A_n78A |  |
| DC_2A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |  |
| DC_2A_n66A-n78A DC_2A-2A_n66A-n78A | DC_2A_n66A DC_2A_n78A |  |
| DC_2A_n66A-n78(2A) | DC_2A_n66A DC_2A_n78A |  |
| DC_2A_n66(2A)-n78A | DC_2A_n66A DC_2A_n78A |  |
| DC_2A_n66(2A)-n78(2A) | DC_2A_n66A DC_2A_n78A |  |
| DC_2A-66A-66A_n78A | DC_2A_n78A DC_66A_n78A |  |
| DC_2A-66A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |  |
| DC_2A-71A_n38A | DC_71A_n38A DC_2A_n38A |  |
| DC_2A-2A-71A_n38A | DC_71A_n38A DC_2A_n38A |  |
| DC_2A-71A_n41A | DC_2A_n41A DC_71A_n41A |  |
| DC_2A-2A-71A_n41A | DC_2A_n41A DC_71A_n41A |  |
| DC_2A-71A_n66A | DC_2A_n66A DC_71A_n66A |  |
| DC_2A-2A-71A_n66A | DC_2A_n66A DC_71A_n66A |  |
| DC_2A-71A_n71A | DC_2A_n71A |  |
| DC_2A-71A_n78A DC_2A-71A_n78(2A) | DC_71A_n78A DC_2A_n78A |  |
| DC_2A-71A_n78A | DC_71A_n78A DC_2A_n78A |  |
| DC_2A-2A-71A_n78A | DC_71A_n78A DC_2A_n78A |  |
| DC_2A_n71A-n78A | DC_2A_n71A DC_2A_n78A |  |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |  |
| DC_3A_n1A-n7A | DC_3A_n1A DC_3A_n7A |  |
| DC_3C_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_3C_n1A DC_3C_n7A |  |
| DC_3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |  |
| DC_3A-3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |  |
| DC_3A_n1A-n28A | DC_3A_n1A DC_3A_n28A |  |
| DC_3C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_3C_n28A |  |
| DC_3A_n1A-n38A | DC_3A_n1A DC_3A_n38A |  |
| DC_3A_n1A-n40A | DC_3A_n1A DC_3A_n40A |  |
| DC_3A_n1A-n41A | DC_3A_n1A DC_3A_n41A |  |
| DC_3A_n1A-n77A | DC_3A_n1A DC_3A_n77A | Yes |
| DC_3A_n1A-n78A DC_3C_n1A-n78A | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A | Yes |
| DC_3A_n1A-n78(2A) DC_3C_n1A-n78(2A) | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A | Yes |
| DC_3A-3A_n1A-n78A | DC_3A_n1A DC_3A_n78A | Yes |
| DC_3A_n1A-n79A | DC_3A_n1A DC_3A_n79A | Yes |
| DC_3A_n3A-n41A | DC_3A_n41A DC_3A_n3A2 |  |
| DC_3A_n3A-n77A | DC_3A_n77A DC_3A_n3A2 | Yes |
| DC_3A_n3A-n78A | DC_3A_n78A DC_3A_n3A2 | Yes |
| DC_3A-5A_n77A | DC_3A_n77A DC_5A_n77A |  |
| DC_3A-5A_n77(2A) | DC_3A_n77A DC_5A_n77A |  |
| DC_3A-5A_n78A DC_3A-5A_n78C | DC_3A_n78A DC_5A_n78A | Yes |
| DC_3C-5A_n78A | DC_3A_n78A DC_5A_n78A |  |
| DC_3A-5A_n78(2A) | DC_3A_n78A DC_5A_n78A | Yes |
| DC_3A_n5A-n78A514 DC_3C_n5A-n78A514 | DC_3A_n5A DC_3A_n78A14 DC_3C_n5A DC_3C_n78A14 | Yes |
| DC_3A-5A_n79A | DC_3A_n79A DC_5A_n79A | Yes |
| DC_3A-7A_n1A DC_3A-7C_n1A DC_3C-7A_n1A DC_3C-7C_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A |  |
| DC_3A-3A-7A_n1A | DC_3A_n1A DC_7A_n1A |  |
| DC_3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |  |
| DC_3A-3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |  |
| DC_3A-7A_n3A DC_3A-7C_n3A | DC_3A_n3A2 DC_7A_n3A |  |
| DC_3A-7A_n5A DC_3C-7A_n5A DC_3A-7C_n5A DC_3C-7C_n5A | DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A |  |
| DC_3A-7A_n7A DC_3C-7A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A2 |  |
| DC_3A-3A-7A_n7A | DC_3A_n7A DC_7A_n7A2 |  |
| DC_3A-7A_n8A | DC_3A_n8A DC_7A_n8A |  |
| DC_3A-3A-7A_n8A | DC_3A_n8A DC_7A_n8A |  |
| DC_3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |  |
| DC_3A-3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |  |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |  |
| DC_3A-7A_n38A17,18 | N/A |  |
| DC_3A-7A_n40A | DC_3A_n40A DC_7A_n40A |  |
| DC_3A-7A_n77A | DC_3A_n77A DC_7A_n77A | Yes |
| DC_3A-3A-7A_n77A | DC_3A_n77A DC_7A_n77A | Yes |
| DC_3A-7A-7A_n77A | DC_3A_n77A DC_7A_n77A | Yes |
| DC_3A-3A-7A-7A_n77A | DC_3A_n77A DC_7A_n77A | Yes |
| DC_3A-7A_n77(2A) | DC_3A_n77A DC_7A_n77A |  |
| DC_3A-7A-7A_n77(2A) | DC_3A_n77A DC_7A_n77A |  |
| DC_3A-7A_n78A14 DC_3C-7A_n78A14 DC_3A-7C_n78A14 DC_3C-7C_n78A14 DC_3A-7A_n78C | DC_3A_n78A14 DC_3C_n78A14 DC_7A_n78A14 DC_7C_n78A14 | Yes |
| DC_3A_n7A-n28A DC_3C_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_3C_n28A |  |
| DC_3A-7A_n78(2A) DC_3C-7A_n78(2A) DC_3A-7C_n78(2A) DC_3C-7C_n78(2A) DC_3A_n7A-n78(2A) DC_3C_n7A-n78(2A) | DC_3A_n78A DC_7A_n78A DC_3C_n78A DC_7C_n78A | Yes |
| DC_3A-3A-7A_n78A | DC_3A_n78A DC_7A_n78A | Yes |
| DC_3A-7A-7A_n78A DC_3A-7A-7A_n78C | DC_3A_n78A DC_7A_n78A | Yes |
| DC_3A-7A-7A_n78(2A) | DC_3A_n78A DC_7A_n78A | Yes |
| DC_3A-3A-7A-7A_n78A | DC_3A_n78A DC_7A_n78A | Yes |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | DC_3A_n7A DC_3C_n7A DC_3A_n78A DC_3C_n78A | Yes |
| DC_3A-3A_n7A-n78A DC_3A-3A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_3A_n78A | Yes |
| DC_3A-8A_n1A DC_3C-8A_n1A | DC_3A_n1A DC_8A_n1A |  |
| DC_3A-3A-8A_n1A | DC_3A_n1A DC_8A_n1A |  |
| DC_3A-3A_n8A-n78A | DC_3A_n8A DC_3A_n78A | Yes |
| DC_3A_n8A-n40A | DC_3A_n8A DC_3A_n40A |  |
| DC_3A-8A_n28A | DC_3A_n28A DC_8A_n28A |  |
| DC_3A-8A_n40A | DC_3A_n40A DC_8A_n40A |  |
| DC_3A-8A_n77A | DC_3A_n77A DC_8A_n77A | Yes |
| DC_3C-8A_n77A | DC_3A_n77A DC_3C_n77A DC_8A_n77A |  |
| DC_3A-8A_n77(2A) | DC_3A_n77A DC_8A_n77A | Yes |
| DC_3C-8A_n77(2A) | DC_3A_n77A DC_3C_n77A DC_8A_n77A |  |
| DC_3A-8A_n77(3A) | DC_3A_n77A DC_8A_n77A | Yes |
| DC_3A-8A_n78A DC_3C-8A_n78A | DC_3A_n78A DC_8A_n78A | Yes |
| DC_3A-8A_n78(2A) | DC_3A_n78A DC_8A_n78A |  |
| DC_3A-3A-8A_n78A | DC_3A_n78A DC_8A_n78A | Yes |
| DC_3A-8A_n79A | DC_3A_n79A DC_8A_n79A | Yes |
| DC_3A_n8A-n78A | DC_3A_n8A DC_3A_n78A | Yes |
| DC_3A-11A_n28A | DC_3A_n28A DC_11A_n28A |  |
| DC_3A-11A_n77A | DC_3A_n77A DC_11A_n77A | Yes |
| DC_3A-11A_n77(2A) | DC_3A_n77A DC_11A_n77A | Yes |
| DC_3A-11A_n77(3A) | DC_3A_n77A DC_11A_n77A | Yes |
| DC_3A-18A_n3A | DC_3A_n3A2 DC_18A_n3A |  |
| DC_3A-18A_n28A | DC_3A_n28A DC_18A_n28A |  |
| DC_3A-18A_n41A | DC_3A_n41A DC_18A_n41A |  |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A |  |
| DC_3A-18A_n77(2A) | DC_3A_n77A DC_18A_n77A |  |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |  |
| DC_3A-18A_n78(2A) | DC_3A_n78A DC_18A_n78A |  |
| DC_3A-18A_n79A | DC_3A_n79A DC_18A_n79A |  |
| DC_3A-19A_n1A | DC_3A_n1A DC_19A_n1A |  |
| DC_3A-19A_n77A DC_3A-19A_n77C | DC_3A_n77A DC_19A_n77A | Yes |
| DC_3A-19A_n77(2A) | DC_3A_n77A DC_19A_n77A | Yes |
| DC_3A-19A_n78A DC_3A-19A_n78C | DC_3A_n78A DC_19A_n78A | Yes |
| DC_3A-19A_n78(2A) | DC_3A_n78A DC_19A_n78A | Yes |
| DC_3A-19A_n79A DC_3A-19A_n79C | DC_3A_n79A DC_19A_n79A | Yes |
| DC_3A-20A_n1A DC_3C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |  |
| DC_3A-20A_n7A DC_3C-20A_n7A | DC_3A_n7A DC_3C_n7A DC_20A_n7A |  |
| DC_3A-20A_n8A | DC_3A_n8A DC_20A_n8A |  |
| DC_3A-20A_n28A6,16,20 DC_3C-20A_n28A6,16,20 | DC_3A_n28A DC_3C_n28A DC_20A_n28A | Yes |
| DC_3A-20A_n41A | DC_3A_n41A DC_20A_n41A |  |
| DC_3C-20A_n41A | DC_3C_n41A DC_20A_n41A |  |
| DC_3A-20A_n38A | DC_3A_n38A DC_20A_n38A |  |
| DC_3C_n20A-n67A | DC_3A_n20A |  |
| DC_3A-20A_n78A DC_3C-20A_n78A | DC_3A_n78A DC_3C_n78A DC_20A_n78A | Yes |
| DC_3A-20A_n78(2A) | DC_3A_n78A DC_20A_n78A | Yes |
| DC_3A_n20A-n78A | DC_3A_n20A DC_3A_n78A |  |
| DC_3A-21A_n1A10,11 | DC_3A_n1A DC_21A_n1A |  |
| DC_3A-21A_n28A | DC_3A_n28A DC_21A_n28A |  |
| DC_3A-21A_n77A DC_3A-21A_n77C | DC_3A_n77A DC_21A_n77A | Yes |
| DC_3A-21A_n77(2A) | DC_3A_n77A DC_21A_n77A | Yes |
| DC_3A-21A_n78A DC_3A-21A_n78C | DC_3A_n78A DC_21A_n78A | Yes |
| DC_3A-21A_n78(2A) | DC_3A_n78A DC_21A_n78A | Yes |
| DC_3A-21A_n79A DC_3A-21A_n79C | DC_3A_n79A DC_21A_n79A | Yes |
| DC_3A-28A_n1A | DC_28A_n1A DC_3A_n1A |  |
| DC_3A-28A_n3A | DC_3A_n3A2 DC_28A_n3A |  |
| DC_3A-28A_n5A DC_3C-28A_n5A | DC_3A_n5A DC_3C_n5A DC_28A_n5A |  |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |  |
| DC_3A-28A_n40A | DC_3A_n40A DC_28A_n40A |  |
| DC_3A-3A-28A_n7A DC_3A-3A-28A_n7B | DC_3A_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |  |
| DC_3A_n28A-n40A | DC_3A_n28A DC_3A_n40A |  |
| DC_3A_n28A-n41A | DC_3A_n28A DC_3A_n41A | Yes |
| DC_3A-28A_n41A | DC_3A_n41A DC_28A_n41A | Yes |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | DC_3A_n28A DC_3A_n75A DC_3C_n28A DC_3C_n75A |  |
| DC_3A-28A_n77A DC_3A-28A_n77C | DC_3A_n77A DC_28A_n77A | Yes |
| DC_3A-28A_n77(2A) | DC_3A_n77A DC_28A_n77A | Yes |
| DC_3A_n28A-n77A | DC_3A_n28A DC_3A_n77A | Yes |
| DC_3A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A | Yes |
| DC_3A-28A_n78A514 DC_3C-28A_n78A514 DC_3A-28A_n78C | DC_3A_n78A14 DC_3C_n78A14 DC_28A_n78A14 | Yes |
| DC_3A-3A-28A_n78A | DC_3A_n78A DC_28A_n78A |  |
| DC_3A_n28A-n78A514 DC_3C_n28A-n78A514 | DC_3A_n28A DC_3A_n78A14 DC_3C_n28A DC_3C_n78A14 | Yes |
| DC_3A_n28A-n78(2A) DC_3C_n28A-n78(2A) | DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A | Yes |
| DC_3A-28A_n79A DC_3A-28A_n79C | DC_3A_n79A DC_28A_n79A | Yes |
| DC_3A_n28A-n79A | DC_3A_n28A DC_3A_n79A | Yes |
| DC_3A-32A_n1A DC_3C-32A_n1A | DC_3A_n1A DC_3C_n1A |  |
| DC_3A-32A_n28A DC_3C-32A_n28A | DC_3A_n28A DC_3C_n28A |  |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C | DC_3A_n78A DC_3C_n78A |  |
| DC_3A-32A_n78(2A) | DC_3A_n78A DC_3C_n78A |  |
| DC_3A-38A_n28A DC_3C-38A_n28A | DC_3A_n28A DC_3C_n28A DC_38A_n28A |  |
| DC_3A-38A_n78A | DC_3A_n78A |  |
| DC_3A_n38A-n78A | DC_3A_n78A |  |
| DC_3A-40A_n1A DC_3A-40C_n1A | DC_3A_n1A DC_40A_n1A |  |
| DC_3A_n40A-n41A | DC_3A_n40A DC_3A_n41A |  |
| DC_3A-40A_n78A DC_3A-40C_n78A | DC_3A_n78A DC_40A_n78A |  |
| DC_3A-40A_n78(2A) DC_3A-40C_n78(2A) | DC_3A_n78A DC_40A_n78A |  |
| DC_3A_n40A-n78A | DC_3A_n40A DC_3A_n78A |  |
| DC_3A_n40A-n79A | DC_3A_n40A DC_3A_n79A |  |
| DC_3A-41A_n3A DC_3A-41C_n3A | DC_3A_n3A2 DC_41A_n3A DC_41C_n3A |  |
| DC_3A-41A_n28A | DC_3A_n28A DC_41A_n28A | Yes |
| DC_3A-41C_n28A | DC_3A_n28A DC_41A_n28A DC_41C_n28A | Yes |
| DC_3A-41A_n41A DC_3A-41C_n41A DC_3A-41D_n41A | DC_3A_n41A |  |
| DC_3A-(n)41AA DC_3A-(n)41CA DC_3A-(n)41DA | DC_3A_n41A DC_(n)41AA |  |
| DC_3A-41A_n77A DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A DC_41C_n77A |  |
| DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | DC_3A_n77A DC_41A_n77A DC_41C_n77A |  |
| DC_3A-41A_n78A DC_3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |  |
| DC_3A_n41A-n78A | DC_3A_n41A DC_3A_n78A |  |
| DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | DC_3A_n78A DC_41A_n78A DC_41C_n78A |  |
| DC_3A-42A_n1A DC_3A-42C_n1A | DC_3A_n1A DC_42A_n1A | Yes |
| DC_3A-42A_n28A | DC_3A_n28A DC_42A_n28A | Yes |
| DC_3A-42C_n28A | DC_3A_n28A DC_42A_n28A DC_42C_n28A | Yes |
| DC_3A-41A_n79A DC_3A-41C_n79A | DC_3A_n79A DC_41A_n79A | Yes |
| DC_3A_n41A-n77A | DC_3A_n41A DC_3A_n77A |  |
| DC_3A_n41A-n79A | DC_3A_n41A DC_3A_n79A | Yes |
| DC_3A_SUL_n41A-n80A DC_3C_SUL_n41A-n80A | DC_3A_n41A DC_3C_n41A DC_3A_n80A_ULSUP-TDM_n41A DC_3C_n80A_ULSUP-TDM_n41A |  |
| DC_3A-42A_n77A15,16 DC_3A-42A_n77C15,16 DC_3A-42C_n77A15,16 DC_3A-42C_n77C15,16 DC_3A-42D_n77A15,16 DC_3A-42D_n77C15,16 DC_3A-42E_n77A15,16 DC_3A-42E_n77C15,16 | DC_3A_n77A |  |
| DC_3A-42A_n77(2A)15,16 DC_3A-42C_n77(2A)15,16 | DC_3A_n77A |  |
| DC_3A-42A_n78A15,16 DC_3A-42A_n78C15,16 DC_3A-42C_n78A15,16 DC_3A-42C_n78C15,16 DC_3A-42D_n78A15,16 DC_3A-42D_n78C15,16 DC_3A-42E_n78A15,16 DC_3A-42E_n78C15,16 | DC_3A_n78A |  |
| DC_3A-42A_n79A DC_3A-42A_n79C DC_3A-42C_n79A DC_3A-42C_n79C DC_3A-42D_n79A DC_3A-42D_n79C DC_3A-42E_n79A DC_3A-42E_n79C | DC_3A_n79A |  |
| DC_3A_n75A-n78A | DC_3A_n78A |  |
| DC_3A_n75A-n78(2A) | DC_3A_n78A |  |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A |  |
| DC_3A_n78A-n79A DC_3A_n78A-n79C | DC_3A_n78A DC_3A_n79A |  |
| DC_3A_SUL_n77A-n80A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A |  |
| DC_3A_SUL_n77A-n84A | DC_3A_n77A DC_3A_n84A |  |
| DC_3A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A | Yes |
| DC_3C_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |  |
| DC_3A_SUL_n78A-n82A | DC_3A_n78A DC_3A_n82A | Yes |
| DC_3A_SUL_n78A-n84A | DC_3A_n78A DC_3A_n84A |  |
| DC_3A_SUL_n79A-n80A | DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A | Yes |
| DC_4A-7A_n28A | DC_4A_n28A DC_7A_n28A |  |
| DC_5A_n1A-n78A | DC_5A_n1A DC_5A_n78A |  |
| DC_5A_n2A-n77A14 DC_5A_n2A-n77C14 | DC_5A_n77A14 DC_5A_n2A |  |
| DC_5A_n5A-n77A14 DC_5A_n5A-n77C14 | DC_5A_n77A14 |  |
| DC_5A-7A_n7A | DC_5A_n7A DC_7A_n7A2 |  |
| DC_5A-7A_n66A DC_5A-7C_n66A | DC_5A_n66A DC_7A_n66A |  |
| DC_5A-7A-7A_n66A | DC_5A_n66A DC_7A_n66A |  |
| DC_5A-7A-7A_n77A | DC_5A_n71A DC_7A_n71A |  |
| DC_5A-7A_n77(2A) | DC_5A_n77A DC_7A_n77A |  |
| DC_5A-7A-7A_n77(2A) | DC_5A_n77A DC_7A_n77A |  |
| DC_5A-7A_n77(2A) | DC_5A_n77A DC_7A_n77A |  |
| DC_5A-7A-7A-n77(2A) | DC_5A_n77A DC_7A_n77A |  |
| DC_5A-7A-7A_n78A DC_5A-7A-7A_n78C | DC_5A_n78A DC_7A_n78A |  |
| DC_5A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |  |
| DC_5A_n7A-n78A | DC_5A_n7A DC_5A_n78A |  |
| DC_5A_n7(2A)-n78A | DC_5A_n7A DC_5A_n78A |  |
| DC_5A_n7A-n78(2A) | DC_5A_n7A DC_5A_n78A |  |
| DC_5A_n7(2A)-n78(2A) | DC_5A_n7A DC_5A_n78A |  |
| DC_5A-7A-7A_n78ADC_5A-7A-7A_n78C | DC_5A_n78A DC_7A_n78A |  |
| DC_5A-7A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |  |
| DC_5A-(n)12AA | DC_5A_n12A DC_(n)12AA2 |  |
| DC_5A-13A_n2A | DC_5A_n2A DC_13A_n2A |  |
| DC_5A-13A_n66A | DC_5A_n66A DC_13A_n66A |  |
| DC_5A-13A_n77A DC_5A-13A_n77C | DC_5A_ n77A DC_13A_ n77A |  |
| DC_5A-30A_n2A | DC_5A_n2A DC_30A_n2A |  |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |  |
| DC_5A-30A_n77A14 | DC_5A_n77A14 DC_30A_n77A14 |  |
| DC_5A_n38A-n66A | DC_5A_n38A DC_5A_n66A |  |
| DC_5A-41A_n79A | DC_5A_n79A DC_41A_n79A |  |
| DC_5A-46A_n66A | DC_5A_n66A DC_46A_n66A |  |
| DC_5A-48A_n12A | DC_5A_n12A DC_48A_n12A |  |
| DC_5A-48A_n71A | DC_5A_n71A DC_48A_n71A |  |
| DC_5A-48A_n77A14,15,16 DC_5A-48C_n77A14,15,16 DC_5A-48D_n77A14,15,16 DC_5A-48A_n77C14,15,16 DC_5A-48C_n77C14,15,16 DC_5A-48D_n77C14,15,16 | DC_5A_n77A14 |  |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-66B_n2A | DC_5A_n2A DC_66A_n2A |  |
| DC_5A-5A-66A_n2A | DC_5A_n2A DC_66A_n2A |  |
| DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |  |
| DC_5A-5A-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |  |
| DC_5A-66A_n5A | DC_66A_n5A |  |
| DC_5A-66A-66A_n5A | DC_66A_n5A |  |
| DC_5A-66A_n7A | DC_5A_n7A DC_66A_n7A |  |
| DC_5A-66A-66A_n7A | DC_5A_n7A DC_66A_n7A |  |
| DC_5A-66A_n12A | DC_5A_n12A DC_66A_n12A |  |
| DC_5A-66A_n30A | DC_5A_n30A DC_66A_n30A |  |
| DC_5A-66A-66A_n30A | DC_5A_n30A DC_66A_n30A |  |
| DC_5A-66A_n48A DC_5A-66A_n48B | DC_5A_n48A DC_66A_n48A |  |
| DC_5A-66A-66A_n48A DC_5A-66A-66A_n48B | DC_5A_n48A DC_66A_n48A |  |
| DC_5A-66A_n66A DC_5B-66A_n66A | DC_5A_n66A |  |
| DC_5A-5A-66A_n66A | DC_5A_n66A |  |
| DC_5A-66A-66A_n66A DC_5B-66A-66A_n66A | DC_5A_n66A |  |
| DC_5A-5A-66A-66A_n66A | DC_5A_n66A |  |
| DC_5A-66A_n71A | DC_5A_n71A DC_66A_n71A |  |
| DC_5A-66A_n77A14 DC_5A-66A_n77C14 DC_5A-66A-66A_n77C14 | DC_5A_n77A14 DC_66A_n77A14 |  |
| DC_5A-66A-66A_n77A14 | DC_5A_n77A14 DC_66A_n77A14 |  |
| DC_5A_n66A-n77A14 DC_5A_n66A-n77C14 | DC_5A_n66A DC_5A_n77A14 |  |
| DC_5A-66A_n78A | DC_5A_n78A DC_66A_n78A |  |
| DC_5A-66A_n78(2A) | DC_5A_n78A DC_66A_n78A |  |
| DC_5A_n66A-n78A | DC_5A_n66A DC_5A_n78A |  |
| DC_5A-66A-66A_n78A | DC_5A_n78A DC_66A_n78A |  |
| DC_5A-13A_n77A14 DC_5A-13A_n77C14 | DC_5A_n77A14 DC_13A_n77A14 |  |
| DC_7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |  |
| DC_7A-7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |  |
| DC_7A_n1A-n40A | DC_7A_n1A DC_7A_n40A |  |
| DC_7A_n1A-n78A DC_7C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A | Yes |
| DC_7A-7A_n1A-n78A | DC_7A_n1A DC_7A_n78A | Yes |
| DC_7A_n2A-n66A | DC_7A_n2A DC_7A_n66A |  |
| DC_7A_n2A-n71A | DC_7A_n2A DC_7A_n71A |  |
| DC_7A_n2A-n78A | DC_7A_n2A DC_7A_n78A |  |
| DC_7A_n3A-n78A DC_7C_n3A-n78A | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |  |
| DC_7A_n5A-n78A14 DC_7C_n5A-n78A14 | DC_7A_n5A DC_7C_n5A DC_7A_n78A14 DC_7C_n78A14 |  |
| DC_7A_n7A-n78A | DC_7A_n78A DC_7A_n7A2 | Yes |
| DC_7A_n7A-n78(2A) | DC_7A_n78A DC_7A_n7A2 |  |
| DC_7A-8A_n1A | DC_7A_n1A DC_8A_n1A |  |
| DC_7A-7A-8A_n1A | DC_7A_n1A DC_8A_n1A |  |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |  |
| DC_7A-8A_n28A | DC_7A_n28A DC_8A_n28A |  |
| DC_7A-8A_n40A | DC_7A_n40A DC_8A_n40A |  |
| DC_7A_n8A-n40A | DC_7A_n8A DC_7A_n40A |  |
| DC_7A-8A_n77A | DC_7A_n77A DC_8A_n77A | Yes |
| DC_7A-8A_n78A | DC_7A_n78A DC_8A_n78A | Yes |
| DC_7A-8A_n78(2A) | DC_7A_n78A DC_8A_n78A |  |
| DC_7A-7A-8A_n78A | DC_7A_n78A DC_8A_n78A | Yes |
| DC_7A-7A_n8A-n78A | DC_7A_n8A DC_7A_n78A | Yes |
| DC_7A_n8A-n78A | DC_7A_n8A DC_7A_n78A | Yes |
| DC_7A-12A_n66A | DC_7A_n66A DC_12A_n66A |  |
| DC_7A-12A_n78A DC_7A-12A_n78(2A) | DC_7A_n78A DC_12A_n78A |  |
| DC_7A-12A_n78A | DC_7A_n78A DC_12A_n78A |  |
| DC_7A-13A_n25A DC_7C-13A_n25A | DC_7A_n25A DC_13A_n25A |  |
| DC_7A-7A-13A_n25A | DC_7A_n25A DC_13A_n25A |  |
| DC_7A-13A_n66A DC_7C-13A_n66A | DC_7A_n66A DC_13A_n66A |  |
| DC_7A-7A-13A_n66A | DC_7A_n66A DC_13A_n66A |  |
| DC_7A-20A_n1A DC_7C-20A_n1A | DC_7A_n1A DC_7C_n1A DC_20A_n1A |  |
| DC_7A-20A_n3A DC_7C-20A_n3A | DC_7A_n3A DC_7C_n3A DC_20A_n3A |  |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |  |
| DC_7A-20A_n28A6,16,20 | DC_7A_n28A DC_20A_n28A |  |
| DC_7A-20A_n38A17,18 | N/A |  |
| DC_7A-20A_n78A | DC_7A_n78A DC_20A_n78A | Yes |
| DC_7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |  |
| DC_7A-7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |  |
| DC_7C_n25A-n66A | DC_7A_n25A DC_7A_n66A |  |
| DC_7A-25A_n77A DC_7C-25A_n77A | DC_7A_n77A DC_25A_n77A |  |
| DC_7A-7A-25A_n77A | DC_7A_n77A DC_25A_n77A |  |
| DC_7A-25A-25A_n77A DC_7C-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |  |
| DC_7A-7A-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |  |
| DC_7A-25A_n78A DC_7C-25A_n78A | DC_7A_n78A DC_25A_n78A |  |
| DC_7A-7A-25A_n78A | DC_7A_n78A DC_25A_n78A |  |
| DC_7A-25A-25A_n78A DC_7C-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |  |
| DC_7A-7A-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |  |
| DC_7A-28A_n1A | DC_28A_n1A DC_7A_n1A |  |
| DC_7A-7A-28A_n1A | DC_28A_n1A DC_7A_n1A |  |
| DC_7A-28A_n2A | DC_7A_n2A DC_28A_n2A |  |
| DC_7A-28A_n3A DC_7C-28A_n3A | DC_7A_n3A DC_7C_n3A DC_28A_n3A |  |
| DC_7A-28A_n5A6 DC_7C-28A_n5A6 | DC_7A_n5A DC_7C_n5A DC_28A_n5A |  |
| DC_7A-28A_n7A | DC_7A_n7A2 DC_28A_n7A |  |
| DC_7A_n28A-n40A | DC_7A_n28A DC_7A_n40A |  |
| DC_7A-28A_n40A | DC_7A_n40A DC_28A_n40A |  |
| DC_7A-28A_n66A DC_7C-28A_n66A | DC_7A_n66A DC_28A_n66A |  |
| DC_7A-28A_n78A14 DC_7C-28A_n78A14 | DC_7A_n78A14 DC_7C_n78A14 DC_28A_n78A14 | Yes |
| DC_7A_n28A-n78A14 | DC_7A_n28A DC_7A_n78A14 | Yes |
| DC_7C_n28A-n78A14 | DC_7A_n28A DC_7A_n78A14 DC_7C_n28A DC_7C_n78A14 |  |
| DC_7A-29A_n78A DC_7C-29A_n78A | DC_7A_n78A |  |
| DC_7A-7A-29A_n78A | DC_7A_n78A |  |
| DC_7A-32A_n1A | DC_7A_n1A |  |
| DC_7A-32A_n3A | DC_7A_n3A |  |
| DC_7A-32A_n8A | DC_7A_n8A |  |
| DC_7A-32A_n28A | DC_7A_n28A |  |
| DC_7A-32A_n78A | DC_7A_n78A |  |
| DC_7A-38A_n3A17,18 | N/A |  |
| DC_7A-38A_n78A17,18 | N/A |  |
| DC_7A_n38A-n78A17,18 | N/A |  |
| DC_7A-40A_n1A DC_7A-40C_n1A | DC_7A_n1A DC_40A_n1A |  |
| DC_7A-40A_n78A DC_7A-40C_n78A | DC_7A_n78A DC_40A_n78A |  |
| DC_7A-40A_n78(2A) DC_7A-40C_n78(2A) | DC_7A_n78A DC_40A_n78A |  |
| DC_7A_n40A-n78A | DC_7A_n40A DC_7A_n78A |  |
| DC_7A-46A_n78A3 DC_7A-46C_n78A3 DC_7A-46D_n78A3 DC_7A-46E_n78A3 | DC_7A_n78A |  |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | DC_7A_n5A DC_66A_n5A |  |
| DC_7A-7A-66A_n5A | DC_7A_n5A DC_66A_n5A |  |
| DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A | DC_7A_n5A DC_66A_n5A |  |
| DC_7A-7A-66A-66A_n5A | DC_7A_n5A DC_66A_n5A |  |
| DC_7A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |  |
| DC_7A-66A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |  |
| DC_7A-66A_n25A DC_7C-66A_n25A | DC_7A_n25A DC_66A_n25A |  |
| DC_7A-7A-66A_n25A | DC_7A_n25A DC_66A_n25A |  |
| DC_7A-66A_n28A | DC_7A_n28A DC_66A_n28A |  |
| DC_7A-66A_n38A | 66A9 |  |
| DC_7A-66A_n66A DC_7C-66A_n66A | DC_7A_n66A DC_66A_n66A2 |  |
| DC_7A-7A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |  |
| DC_7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |  |
| DC_7A-7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |  |
| DC_7A-66A_n71A | DC_7A_n71A DC_66A_n71A |  |
| DC_7A-66A-66A_n71A | DC_7A_n71A DC_66A_n71A |  |
| DC_7A_n66A-n71A | DC_7A_n66A DC_7A_n71A |  |
| DC_7A-66A_n77A DC_7C-66A_n77A | DC_7A_n77A DC_66A_n77A |  |
| DC_7A-7A-66A_n77A | DC_7A_n66A DC_66A_n77A |  |
| DC_7A-7A-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |  |
| DC_7A-66A_n77(2A) DC_7C-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |  |
| DC_7A_n66A-n77A DC_7C_n66A-n77A | DC_7A_n66A DC_7A_n77A |  |
| DC_7A-7A_n66A-n77A | DC_7A_n66A DC_7A_n77A |  |
| DC_7A_n66A-n78A DC_7C_n66A-n78A | DC_7A_n66A DC_7A_n78A |  |
| DC_7A-7A_n66A-n78A | DC_7A_n66A DC_7A_n78A |  |
| DC_7A-66A_n78A DC_7C-66A_n78A | DC_7A_n78A DC_7C_n78A DC_66A_n78A |  |
| DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) | DC_7A_n78A DC_7C_n78A DC_66A_n78A |  |
| DC_7A-7A-66A_n78A | DC_7A_n78A DC_66A_n78A |  |
| DC_7A-7A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |  |
| DC_7A-7A-66A-66A_n78A | DC_7A_n78A DC_66A_n78A |  |
| DC_7A-7A-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |  |
| DC_7A-66A-66A_n78A DC_7C-66A-66A_n78A | DC_7A_n78A DC_66A_n78A |  |
| DC_7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |  |
| DC_7A-71A_n66A | DC_7A_n66A DC_71A_n66A |  |
| DC_7A-71A_n78A DC_7A-71A_n78(2A) | DC_7A_n78A DC_71A_n78A |  |
| DC_7A_n71A-n78A | DC_7A_n71A DC_7A_n78A |  |
| DC_7A_n78A-n79A DC_7A_n78A-n79C | DC_7A_n78A DC_7A_n79A |  |
| DC_7A_SUL_n78A-n80A | DC_7A_n78A DC_7A_n80A |  |
| DC_8A_n1A-n3A | DC_8A_n1A DC_8A_n3A |  |
| DC_8A_n1A-n28A | DC_8A_n1A DC_8A_n28A |  |
| DC_8A_n1A-n40A | DC_8A_n1A DC_8A_n40A |  |
| DC_8A_n1A-n77A DC_8A_n1A-n77(2A) | DC_8A_n1A DC_8A_n77A | Yes |
| DC_8A_n1A-n78A | DC_8A_n1A DC_8A_n78A | Yes |
| DC_8A-(n)3AA | DC_(n)3AA DC_8A_n3A |  |
| DC_8A_n3A-n28A | DC_8A_n3A DC_8A_n28A |  |
| DC_8A_n3A-n77A | DC_8A_n3A DC_8A_n77A | Yes |
| DC_8A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A | Yes |
| DC_8A_n3A-n79A | DC_8A_n3A DC_8A_n79A |  |
| DC_8A-11A_n1A | DC_8A_n1A DC_11A_n1A |  |
| DC_8A-11A_n3A | DC_8A_n3A DC_11A_n3A |  |
| DC_8A-11A_n28A | DC_8A_n28A DC_11A_n28A |  |
| DC_8A-11A_n77A | DC_8A_n77A DC_11A_n77A | Yes |
| DC_8A-11A_n77(2A) | DC_8A_n77A DC_11A_n77A | Yes |
| DC_8A-11A_n77(3A) | DC_8A_n77A DC_11A_n77A | Yes |
| DC_8A-11A_n78A | DC_8A_n78A DC_11A_n78A | Yes |
| DC_8A-11A_n79A | DC_8A_n79A DC_11A_n79A | Yes |
| DC_8A-20A_n1A | DC_8A_n1A DC_20A_n1A |  |
| DC_8A-20A_n3A | DC_8A_n3A DC_20A_n3A |  |
| DC_8A-20A_n28A6,16,19,20 | DC_8A_n28A DC_20A_n28A |  |
| DC_8A-20A_n78A | DC_8A_n78A DC_20A_n78A |  |
| DC_8A_n28A-n77A | DC_8A_n28A DC_8A_n77A | Yes |
| DC_8A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A | Yes |
| DC_8A_n28A-n78A | DC_8A_n28A DC_8A_n78A | Yes |
| DC_8A-32A_n1A | DC_8A_n1A |  |
| DC_8A-32A_n3A | DC_8A_n3A |  |
| DC_8A-38A_n1A | DC_8A_n1A DC_38A_n1A |  |
| DC_8A_n39A-n40A | DC_8A_n39A DC_8A_n40A |  |
| DC_8A_n39A-n79A | DC_8A_n39A DC_8A_n79A |  |
| DC_8A-40A_n1A DC_8A-40C_n1A | DC_8A_n1A DC_40A_n1A |  |
| DC_8A_n40A-n41A | DC_8A_n40A DC_8A_n41A |  |
| DC_8A-40A_n78A DC_8A-40C_n78A | DC_8A_n78A DC_40A_n78A |  |
| DC_8A-40A_n78(2A) DC_8A-40C_n78(2A) | DC_8A_n78A DC_40A_n78A |  |
| DC_8A_n40A-n78A | DC_8A_n40A DC_8A_n78A |  |
| DC_8A_n40A-n79A | DC_8A_n40A DC_8A_n79A |  |
| DC_8A-41A_n1A DC_8A-41C_n1A | DC_8A_n1A DC_41A_n1A |  |
| DC_8A-41A_n3A DC_8A-41C_n3A | DC_8A_n3A DC_41A_n3A DC_41C_n3A | Yes |
| DC_8A-41A_n77A DC_8A-41C_n77A | DC_8A_n77A DC_41A_n77A DC_41C_n77A |  |
| DC_8A_n41A-n79A | DC_8A_n41A DC_8A_n79A | Yes |
| DC_8A-42A_n1A DC_8A-42C_n1A | DC_8A_n1A DC_42A_n1A DC_42C_n1A | Yes |
| DC_8A-42A_n3A | DC_8A_n3A DC_42A_n3A | Yes |
| DC_8A-42C_n3A | DC_8A_n3A DC_42A_n3A DC_42C_n3A | Yes |
| DC_8A-42A_n28A | DC_8A_n28A DC_42A_n28A | Yes |
| DC_8A-42C_n28A | DC_8A_n28A DC_42A_n28A DC_42C_n28A | Yes |
| DC_8A-42A_n77A15,16 DC_8A-42C_n77A15,16 | DC_8A_n77A |  |
| DC_8A-42A_n77(2A) 15,16 DC_8A-42C_n77(2A) 15,16 | DC_8A_n77A |  |
| DC_8A_SUL_n41A-n81A | DC_8A_n41A, DC_8A_n81A_ULSUP-TDM_n41A |  |
| DC_8A_n77A-n79A DC_8A_n77(2A)-n79A | DC_8A_n77A DC_8A_n79A |  |
| DC_8A_SUL_n78A-n80A | DC_8A_n78A DC_8A_n80A |  |
| DC_8A_SUL_n78A-n81A | DC_8A_n78A, DC_8A_n81A_ULSUP-TDM_n78A | Yes |
| DC_8A_SUL_n79A-n81A | DC_8A_n79A, DC_8A_n81A_ULSUP-TDM_n79A | Yes |
| DC_11A_n1A-n77A | DC_11A_n1A DC_11A_n77A | Yes |
| DC_11A_n1A-n77(2A) | DC_11A_n1A DC_11A_n77A | Yes |
| DC_11A_n3A-n28A | DC_11A_n3A DC_11A_n28A |  |
| DC_11A_n3A-n77A | DC_11A_n3A DC_11A_n77A |  |
| DC_11A_n3A-n77(2A) | DC_11A_n3A DC_11A_n77A |  |
| DC_11A_n3A-n79A | DC_11A_n3A DC_11A_n79A | Yes |
| DC_11A-18A_n77A | DC_11A_n77A DC_18A_n77A |  |
| DC_11A-18A_n78A | DC_11A_n78A DC_18A_n78A |  |
| DC_11A_n28A-n77A | DC_11A_n28A DC_11A_n77A | Yes |
| DC_11A_n28A-n77(2A) | DC_11A_n28A DC_11A_n77A | Yes |
| DC_11A_n77A-n79A DC_11A_n77(2A)-n79A | DC_11A_n77A DC_11A_n79A |  |
| DC_12A_n2A-n38A | DC_12A_n2A DC_12A_n38A |  |
| DC_12A_n2A-n41A | DC_12A_n2A DC_12A_n41A |  |
| DC_12A_n2A-n78A | DC_12A_n2A DC_12A_n78A |  |
| DC_12A-(n)5AA | DC_12A_n5A DC_(n)5AA2 |  |
| DC_12A_n7A-n66A | DC_12A_n7A DC_12A_n66A |  |
| DC_12A_n7(2A)-n66A | DC_12A_n7A DC_12A_n66A |  |
| DC_12A_n7A-n78A | DC_12A_n7A DC_12A_n78A |  |
| DC_12A_n7(2A)-n78A | DC_12A_n7A DC_12A_n78A |  |
| DC_12A_n7A-n78(2A) | DC_12A_n7A DC_12A_n78A |  |
| DC_12A_n7(2A)-n78(2A) | DC_12A_n7A DC_12A_n78A |  |
| DC_12A-30A_n2A | DC_12A_n2A DC_30A_n2A |  |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |  |
| DC_12A-30A_n77A14 | DC_12A_n77A14 DC_30A_n77A14 |  |
| DC_12A-48A_n5A | DC_12A_n5A DC_48A_n5A |  |
| DC_12A-66A_n2A | DC_12A_n2A DC_66A_n2A |  |
| DC_12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |  |
| DC_12A-66A_n5A | DC_12A_n5A DC_66A_n5A |  |
| DC_12A-66A_n25A | DC_12A_n25A DC_66A_n25A |  |
| DC_12A-66A_n30A | DC_12A_n30A DC_66A_n30A |  |
| DC_12A-66A-66A_n30A | DC_12A_n30A DC_66A_n30A |  |
| DC_12A-66A_n41A | DC_12A_n41A DC_66A_n41A |  |
| DC_12A-66A_n66A | DC_12A_n66A DC_66A_n66A2 |  |
| DC_12A-66A_n77A14 DC_12A-66A-66A_n77A14 | DC_12A_n77A14 DC_66A_n77A14 |  |
| DC_12A-66A_n78A DC_12A-66A_n78(2A) | DC_12A_n78A DC_66A_n78A |  |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | DC_12A_n66A DC_12A_n78A |  |
| DC_12A_n66(2A)-n78A | DC_12A_n66A DC_12A_n78A |  |
| DC_12A_n66A-n78(2A) | DC_12A_n66A DC_12A_n78A |  |
| DC_12A_n66(2A)-n78(2A) | DC_12A_n66A DC_12A_n78A |  |
| DC_13A_n2A-n77A14 DC_13A_n2A-n77C14 | DC_13A_n2A DC_13A_n77A14 |  |
| DC_13A_n5A-n48A | DC_13A_n48A |  |
| DC_13A_n5A-n77A14 DC_13A_n5A-n77C14 | DC_13A_n77A14 |  |
| DC_13A_n7A-n78A | DC_13A_n7A DC_13A_n78A |  |
| DC_13A_n25A-n66A | DC_13A_n25A DC_13A_n66A |  |
| DC_13A-46A_n2A3 | DC_13A_n2A |  |
| DC_13A-46A_n5A | DC_13A_n5A |  |
| DC_13A-46A_n66A3 | DC_13A_n66A |  |
| DC_13A-46A_n77A DC_13A-46A-46A_n77A | DC_13A_n77A |  |
| DC_13A_n48A-n66A | DC_13A_n48A DC_13A_n66A |  |
| DC_13A-66A_n2A DC_13A-66B_n2A DC_13A-66C_n2A | DC_13A_n2A DC_66A_n2A |  |
| DC_13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |  |
| DC_13A-66A_n5A DC_13A-66A-66A_n5A | DC_13A_n5A DC_66A_n5A |  |
| DC_13A-66A_n48A DC_13A-66A_n48B | DC_13A_n48A DC_66A_n48A |  |
| DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | DC_13A_n48A DC_66A_n48A |  |
| DC_13A-66A_n66A | DC_13A_n66A |  |
| DC_13A-66A-66A_n66A | DC_13A_n66A |  |
| DC_13A-66A_n77A14 DC_13A-66A_n77C14 DC_13A-66A-66A_n77C14 | DC_13A_n77A14 DC_66A_n77A14 |  |
| DC_13A-66A-66A_n77A | DC_13A_n77A14 DC_66A_n77A14 |  |
| DC_13A_n66A-n77A14 DC_13A_n66A-n77C14 | DC_13A_n66A DC_13A_n77A14 |  |
| DC_13A-48A_n2A DC_13A-48B_n2A DC_13A-48C_n2A DC_13A-48D_n2A DC_13A-48E_n2A | DC_13A_n2A |  |
| DC_13A-48A_n66A DC_13A-48B_n66A DC_13A-48C_n66A DC_13A-48D_n66A DC_13A-48E_n66A | DC_13A_n66A |  |
| DC_13A-48A_n77A14,15,16 DC_13A-48A_n77C14,15,16 DC_13A-48C_n77A14,15,16 DC_13A-48C_n77C14,15,16 DC_13A-48D_n77A14,15,16 DC_13A-48D_n77C14,15,16 DC_13A-48A-48A_n77A14,15,16 | DC_13A_n77A14 |  |
| DC_14A-30A_n2A | DC_14A_n2A DC_30A_n2A |  |
| DC_14A-30A_n66A | DC_14A_n66A DC_30A_n66A |  |
| DC_14A-30A_n77A14 | DC_14A_n77A14 DC_30A_n77A14 |  |
| DC_14A-66A_n2A | DC_14A_n2A DC_66A_n2A |  |
| DC_14A-66A-66A_n2A | DC_14A_n2A DC_66A_n2A |  |
| DC_14A-66A_n30A DC_14A-66A-66A_n30A | DC_14A_n30A DC_66A_n30A |  |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A2 |  |
| DC_14A-66A_n77A14 DC_14A-66A-66A_n77A14 | DC_14A_n77A14 DC_66A_n77A14 |  |
| DC_18A_n3A-n41A | DC_18A_n3A DC_18A_n41A |  |
| DC_18A_n3A-n77A | DC_18A_n3A DC_18A_n77A |  |
| DC_18A_n3A-n78A | DC_18A_n3A DC_18A_n78A |  |
| DC_18A_n28A-n41A | DC_18A_n28A DC_18A_n41A |  |
| DC_18A-28A_n77A | DC_18A_n77A DC_28A_n77A | Yes |
| DC_18A_n28A-n77A | DC_18A_n28A DC_18A_n77A | Yes |
| DC_18A-28A_n78A | DC_18A_n78A DC_28A_n78A | Yes |
| DC_18A_n28A-n78A | DC_18A_n28A DC_18A_n78A | Yes |
| DC_18A-28A_n79A | DC_18A_n79A DC_28A_n79A | Yes |
| DC_18A-41A_n3A DC_18A-41C_n3A | DC_18A_n3A DC_41A_n3A DC_41C_n3A |  |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |  |
| DC_18A_n41A-n77A | DC_18A_n41A DC_18A_n77A |  |
| DC_18A-42A_n77A15,16 DC_18A-42C_n77A15,16 | DC_18A_n77A |  |
| DC_18A_n41A-n78A | DC_18A_n41A DC_18A_n78A |  |
| DC_18A-42A_n78A15,16 DC_18A-42C_n78A15,16 | DC_18A_n78A |  |
| DC_18A-42A_n79A DC_18A-42C_n79A | DC_18A_n79A |  |
| DC_19A-21A_n1A | DC_19A_n1A DC_21A_n1A |  |
| DC_19A_n1A-n77A | DC_19A_n1A DC_19A_n77A | Yes |
| DC_19A_n1A-n78A | DC_19A_n1A DC_19A_n78A | Yes |
| DC_19A_n1A-n79A | DC_19A_n1A DC_19A_n79A | Yes |
| DC_19A-21A_n77A DC_19A-21A_n77C | DC_19A_n77A DC_21A_n77A | Yes |
| DC_19A-21A_n77(2A) | DC_19A_n77A DC_21A_n77A | Yes |
| DC_19A-21A_n78A DC_19A-21A_n78C | DC_19A_n78A DC_21A_n78A | Yes |
| DC_19A-21A_n78(2A) | DC_19A_n78A DC_21A_n78A | Yes |
| DC_19A-21A_n79A DC_19A-21A_n79C | DC_19A_n79A DC_21A_n79A | Yes |
| DC_19A-42A_n1A10,12 DC_19A-42C_n1A10,12 | DC_19A_n1A DC_42A_n1A | Yes |
| DC_19A-42A_n77A15,16 DC_19A-42A_n77C15,16 DC_19A-42C_n77A15,16 DC_19A-42C_n77C15,16 DC_19A-42D_n77A15,16 DC_19A-42D_n77C15,16 | DC_19A_n77A |  |
| DC_19A-42A_n78A15,16 DC_19A-42A_n78C15,16 DC_19A-42C_n78A15,16 DC_19A-42C_n78C15,16 DC_19A-42D_n78A15,16 DC_19A-42D_n78C15,16 | DC_19A_n78A |  |
| DC_19A-42A_n79A DC_19A-42A_n79C DC_19A-42C_n79A DC_19A-42C_n79C DC_19A-42D_n79A DC_19A-42D_n79C | DC_19A_n79A |  |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |  |
| DC_19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |  |
| DC_20A_n1A-n7A | DC_20A_n1A DC_20A_n7A |  |
| DC_20A_n1A-n28A16,20 | DC_20A_n1A DC_20A_n28A |  |
| DC_20A_n1A-n67A | DC_20A_n1A |  |
| DC_20A_n1A-n78A | DC_20A_n1A DC_20A_n78A |  |
| DC_20A_n3A-n67A | DC_20A_n3A |  |
| DC_20A_n3A-n78A | DC_20A_n3A DC_20A_n78A |  |
| DC_20A_n7A-n28A, 16, 20 | DC_20A_n7A DC_20A_n28A |  |
| DC_20A_n8A-n75A6 | DC_20A_n8A |  |
| DC_20A_n8A-n78A | DC_20A_n78A DC_20A_n8A |  |
| DC_20A-28A_n1A | DC_20A_n1A DC_28A_n1A |  |
| DC_20A-28A_n3A | DC_20A_n3A DC_28A_n3A |  |
| DC_20A_n28A-n75A6,16,20 | DC_20A_n28A |  |
| DC_20A_n28A-n78A6,16,20 | DC_20A_n28A DC_20A_n78A | Yes |
| DC_20A-32A_n1A | DC_20A_n1A |  |
| DC_20A-32A_n3A | DC_20A_n3A |  |
| DC_20A-32A_n8A | DC_20A_n8A |  |
| DC_20A-32A_n28A16,20 | DC_20A_n28A |  |
| DC_20A-32A_n78A DC_20A-32A_n78C | DC_20A_n78A |  |
| DC_20A-32A_n78(2A) | DC_20A_n78A |  |
| DC_20A-38A_n1A | DC_20A_n1A DC_38A_n1A |  |
| DC_20A-38A_n3A | DC_20A_n3A |  |
| DC_20A-(n)38AA | DC_20A_n38A |  |
| DC_20A-38A_n78A | DC_20A_n78A DC_38A_n78A |  |
| DC_20A_n38A-n78A | DC_20A_n78A |  |
| DC_20A-40A_n1A DC_20A-40C_n1A | DC_20A_n1A DC_40A_n1A |  |
| DC_20A-40A_n78A | DC_20A_n78A DC_40A_n78A |  |
| DC_20A_n41A-n78A | DC_20A_n41A DC_20A_n78A |  |
| DC_20A-(n)41AA DC_20A-(n)41CA DC_20A-(n)41DA | DC_20A_n41A |  |
| DC_20A_n75A-n78A | DC_20A_n78A | Yes |
| DC_20A_n76A-n78A5 | DC_20A_n78A | Yes |
| DC_20A_SUL_n78A-n80A | DC_20A_n78A DC_20A_n80A |  |
| DC_20A_SUL_n78A-n82A | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A | Yes |
| DC_20A_SUL_n78A-n83A | DC_20A_n78A DC_20A_n83A | Yes |
| DC_20A_n78A-n92A DC_20A_n78(2A)-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |  |
| DC_21A_n1A-n77A | DC_21A_n1A DC_21A_n77A | Yes |
| DC_21A_n1A-n78A | DC_21A_n1A DC_21A_n78A | Yes |
| DC_21A_n1A-n79A | DC_21A_n1A DC_21A_n79A | Yes |
| DC_21A-28A_n77A | DC_21A_n77A DC_28A_n77A | Yes |
| DC_21A-28A_n77C | DC_21A_n77A DC_28A_n77A |  |
| DC_21A_n28A-n77A | DC_21A_n28A DC_21A_n77A | Yes |
| DC_21A-28A_n78A | DC_21A_n78A DC_28A_n78A | Yes |
| DC_21A-28A_n78C | DC_21A_n78A DC_28A_n78A |  |
| DC_21A_n28A-n78A | DC_21A_n28A DC_21A_n78A | Yes |
| DC_21A-28A_n79A | DC_21A_n79A DC_28A_n79A | Yes |
| DC_21A-28A_n79C | DC_21A_n79A DC_28A_n79A |  |
| DC_21A_n28A-n79A | DC_21A_n28A DC_21A_n79A | Yes |
| DC_21A-42A_n1A10,12 DC_21A-42C_n1A10,12 | DC_21A_n1A DC_42A_n1A | Yes |
| DC_21A-42A_n77A15,16 DC_21A-42A_n77C15,16 DC_21A-42C_n77A15,16 DC_21A-42C_n77C15,16 DC_21A-42D_n77A15,16 DC_21A-42D_n77C15,16 DC_21A-42E_n77A15,16 DC_21A-42E_n77C15,16 | DC_21A_n77A |  |
| DC_21A-42A_n78A15,16 DC_21A-42A_n78C15,16 DC_21A-42C_n78A15,16 DC_21A-42C_n78C15,16 DC_21A-42D_n78A15,16 DC_21A-42D_n78C15,16 DC_21A-42E_n78A15,16 DC_21A-42E_n78C15,16 | DC_21A_n78A |  |
| DC_21A-42A_n79A DC_21A-42A_n79C DC_21A-42C_n79A DC_21A-42C_n79C DC_21A-42D_n79A DC_21A-42D_n79C DC_21A-42E_n79A DC_21A-42E_n79C | DC_21A_n79A |  |
| DC_28A-32A_n1A | DC_28A_n1A |  |
| DC_28A-32A_n3A | DC_28A_n3A |  |
| DC_28A-38A_n1A | DC_28A_n1A DC_38A_n1A |  |
| DC_28A-66A_n7A | DC_28A_n7A DC_66A_n7A |  |
| DC_28A-66A_n66A | DC_28A_n66A DC_66A_n66A2 |  |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A |  |
| DC_21A_n78A-n79A | DC_21A_n78A DC_21A_n79A |  |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | DC_25A_n41A DC_41A_n41A |  |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | DC_25A_n41A DC_41A_n41A |  |
| DC_25A-(n)41AA | DC_25A_n41A DC_(n)41AA |  |
| DC_25A-25A-(n)41AA | DC_25A_n41A DC_(n)41AA |  |
| DC_25A-(n)41CA DC_25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |  |
| DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |  |
| DC_25A-66A_n77A | DC_25A_n77A DC_66A_n77A |  |
| DC_25A-25A-66A_n77A | DC_25A_n77A DC_66A_n77A |  |
| DC_25A-66A_n78A | DC_25A_n78A DC_66A_n78A |  |
| DC_25A-25A-66A_n78A | DC_25A_n78A DC_66A_n78A |  |
| DC_28A-40A_n78A DC_28A-40C_n78A | DC_28A_n78A DC_40A_n78A |  |
| DC_28A-41A_n77A DC_28A-41C_n77A | DC_28A_n77A DC_41A_n77A |  |
| DC_28A-41A_n78A DC_28A-41C_n78A | DC_28A_n78A DC_41A_n78A |  |
| DC_28A-41A_n79A DC_28A-41C_n79A | DC_28A_n79A DC_41A_n79A | Yes |
| DC_28A_n1A-n40A | DC_28A_n1A DC_28A_n40A |  |
| DC_28A_n1A-n78A | DC_28A_n1A DC_28A_n78A | Yes |
| DC_28A_n3A-n77A | DC_28A_n3A DC_28A_n77A | Yes |
| DC_28A_n3A-n78A | DC_28A_n3A DC_28A_n78A | Yes |
| DC_28A_n5A-n78A | DC_28A_n5A DC_28A_n78A | Yes |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |  |
| DC_28A_n7B-n78A | DC_28A_n7A DC_28A_n7B DC_28A_n78A |  |
| DC_28A_n8A-n78A | DC_28A_n8A DC_28A_n78A | Yes |
| DC_28A_n40A-n78A | DC_28A_n40A DC_28A_n78A |  |
| DC_28A_SUL_n41A-n83A | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41A | Yes |
| DC_28A-42A_n77A15,16 DC_28A-42A_n77C15,16 DC_28A-42C_n77A15,16 | DC_28A_n77A |  |
| DC_28A-42A_n78A15,16 DC_28A-42A_n78C15,16 DC_28A-42C_n78A15,16 | DC_28A_n78A |  |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A | DC_28A_n79A |  |
| DC_28A_SUL_n78A-n83A | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A | Yes |
| DC_29A-30A_n2A | DC_30A_n2A |  |
| DC_29A-30A_n66A | DC_30A_n66A |  |
| DC_29A-30A_n77A14 | DC_30A_n77A14 |  |
| DC_29A-66A_n2A | DC_66A_n2A |  |
| DC_29A-66A-66A_n2A | DC_66A_n2A |  |
| DC_29A-66A_n30A | DC_66A_n30A |  |
| DC_29A-66A-66A_n30A | DC_66A_n30A |  |
| DC_29A-66A_n77A14 DC_29A-66A-66A_n77A14 | DC_66A_n77A14 |  |
| DC_29A-66A_n78A | DC_66A_n78A |  |
| DC_30A-(n)5AA | DC_30A_n5A DC_(n)5AA2 |  |
| DC_30A-66A_n2A | DC_30A_n2A DC_66A_n2A |  |
| DC_30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |  |
| DC_30A-66A_n5A | DC_30A_n5A DC_66A_n5A |  |
| DC_30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |  |
| DC_30A-66A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |  |
| DC_30A-66A_n66A | DC_30A_n66A DC_66A_n66A2 |  |
| DC_30A-66A_n77A14 DC_30A-66A-66A_n77A14 | DC_30A_n77A14 DC_66A_n77A14 |  |
| DC_32A-38A_n1A | DC_38A_n1A |  |
| DC_38A_n3A-n78A | DC_38A_n3A DC_38A_n78A |  |
| DC_39A_n40A-n41A | DC_39A_n40A DC_39A_n41A |  |
| DC_39A_n40A-n79A | DC_39A_n40A DC_39A_n79A |  |
| DC_39A_n41A-n79A | DC_39A_n41A DC_39A_n79A |  |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | DC_40A_n1A DC_40A_n78A |  |
| DC_40A_n41A-n79A | DC_40A_n41A DC_40A_n79A |  |
| DC_41A_n1A-n77A | DC_41A_n1A DC_41A_n77A |  |
| DC_41C_n1A-n77A | DC_41A_n1A DC_41A_n77A DC_41C_n77A |  |
| DC_41A_n3A-n41A | DC_41A_n3A DC_41A_n41A |  |
| DC_41A_n3A-n77A | DC_41A_n3A DC_41A_n77A |  |
| DC_41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |  |
| DC_41A_n3A-n78A | DC_41A_n3A DC_41A_n78A |  |
| DC_41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |  |
| DC_41A_n28A-n41A | DC_41A_n28A |  |
| DC_41A_n28A-n77A | DC_41A_n28A DC_41A_n77A |  |
| DC_41C_n28A-n77A | DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |  |
| DC_41A_n28A-n78A | DC_41A_n28A DC_41A_n78A |  |
| DC_41C_n28A-n78A | DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |  |
| DC_(n)41AA-n78A DC_(n)41CA-n78A DC_(n)41DA-n78A | DC_41A_n78A |  |
| DC_41A_n41A-n77A | DC_41A_n77A |  |
| DC_41A_n41A-n78A | DC_41A_n78A |  |
| DC_41A-42A_n77A15,16 DC_41A-42C_n77A15,16 DC_41C-42A_n77A15,16 DC_41C-42C_n77A15,16 | DC_41A_n77A |  |
| DC_41A-42A_n77(2A)15,16 DC_41A-42C_n77(2A)15,16 | DC_41A_n77A |  |
| DC_41A-42A_n78A15,16 DC_41A-42C_n78A15,16 DC_41C-42A_n78A15,16 DC_41C-42C_n78A15,16 | DC_41A_n78A |  |
| DC_41A-42A_n79A DC_41A-42C_n79A DC_41C-42A_n79A DC_41C-42C_n79A | DC_41A_n79A |  |
| DC_42A_n1A-n3A | DC_42A_n1A DC_42A_n3A | Yes |
| DC_42C_n1A-n3A | DC_42A_n1A DC_42A_n3A DC_42C_n1A DC_42C_n3A | Yes |
| DC_42A_n1A-n77A15,16 | DC_42A_n1A |  |
| DC_42C_n1A-n77A15,16 | DC_42A_n1A DC_42C_n1A |  |
| DC_42A_n1A-n78A15,16 DC_42C_n1A-n78A15,16 | N/A |  |
| DC_42A_n1A-n79A DC_42C_n1A-n79A | N/A |  |
| DC_42A_n3A-n28A | DC_42A_n3A DC_42A_n28A |  |
| DC_42C_n3A-n28A | DC_42A_n3A DC_42A_n28A DC_42C_n28A |  |
| DC_42A_n3A-n77A15,16 | DC_42A_n3A |  |
| DC_42A_n3A-n77(2A)15,16 | DC_42A_n3A |  |
| DC_42C_n3A-n77A15,16 | DC_42A_n3A DC_42C_n3A |  |
| DC_42C_n3A-n77(2A)15,16 | DC_42A_n3A DC_42C_n3A |  |
| DC_42A_n28A-n77A15,16 | DC_42A_n28A |  |
| DC_42A_n28A-n77(2A)15,16 | DC_42A_n28A |  |
| DC_42C_n28A-n77A15,16 | DC_42A_n28A DC_42C_n28A |  |
| DC_42C_n28A-n77(2A)15,16 | DC_42A_n28A DC_42C_n28A |  |
| DC_46A-48A_n5A3 DC_46C-48A_n5A3 DC_46D-48A_n5A3 DC_46E-48A_n5A3 | DC_48A_n5A |  |
| DC_46A-48A_n66A3 DC_46C-48A_n66A3 DC_46D-48A_n66A3 DC_46E-48A_n66A3 | DC_48A_n66A |  |
| DC_46A-66A_n5A DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | DC_66A_n5A |  |
| DC_46A-66A_n25A DC_46C-66A_n25A DC_46D-66A_n25A | DC_66A_n25A |  |
| DC_46A-66A_n41A DC_46C-66A_n41A DC_46D-66A_n41A | DC_66A_n41A |  |
| DC_46A-66A_n41(2A) DC_46C-66A_n41(2A) DC_46D-66A_n41(2A) | DC_66A_n41A |  |
| DC_46A-66A_n71A DC_46C-66A_n71A DC_46D-66A_n71A | DC_66A_n71A |  |
| DC_46A-66A_n77A DC_46A-46A-66A_n77A | DC_66A_n77A |  |
| DC_48A-(n)5AA | DC_48A_n5A DC_(n)5AA2 |  |
| DC_48A-(n)12AA | DC_48A_n12A DC_(n)12AA2 |  |
| DC_48A_n25A-n48A | DC_48A_n25A |  |
| DC_48A_n48A-n66A | DC_48A_n66A |  |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | DC_66A_n2A |  |
| DC_48A-66A_n5A DC_48B-66A_n5A DC_48C-66A_n5A DC_48D-66A_n5A DC_48E-66A_n5A | DC_66A_n5A |  |
| DC_48A-66A_n12A | DC_48A_n12A DC_66A_n12A |  |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | DC_48A_n25A DC_66A_n25A |  |
| DC_48A-66A_n48A | DC_66A_n48A |  |
| DC_48A-66A_n66A DC_48C-66A_n66A DC_48D-66A_n66A DC_48E-66A_n66A | DC_66A_n66A2 DC_48A_n66A |  |
| DC_48A-66A_n71A | DC_48A_n71A DC_66A_n71A |  |
| DC_48A-66A_n77A14,15,16 DC_48A-66A_n77C14,15,16 DC_48C-66A_n77A14,15,16 DC_48C-66A_n77C14,15,16 DC_48D-66A_n77A14,15,16 DC_48D-66A_n77C14,15,16 DC_48E-66A_n77A14,15,16 | DC_66A_n77A14 |  |
| DC_48A-48A-66A_n77A14,15,16 | DC_66A_n77A |  |
| DC_66A-(n)5AA | DC_66A_n5A DC_(n)5AA2 |  |
| DC_66A_n2A-n38A | DC_66A_n2A DC_66A_n38A |  |
| DC_66A_n2A-n66A | DC_66A_n2A |  |
| DC_66A_n2A-n71A | DC_66A_n2A DC_66A_n71A |  |
| DC_66A_n2A-n77A14 DC_66A_n2A-n77C14 DC_66A-66A_n2A-n77C14 | DC_66A_n2A DC_66A_n77A14 |  |
| DC_66A-66A_n2A-n77A14 | DC_66A_n2A DC_66A_n77A14 |  |
| DC_66A_n2A-n78A | DC_66A_n2A DC_66A_n78A |  |
| DC_66A_n5A-n48A | DC_66A_n5A DC_66A_n48A |  |
| DC_66A_n5A-n77A14 DC_66A_n5A-n77C14 DC_66A-66A_n5A-n77C14 | DC_66A_n5A DC_66A_n77A14 |  |
| DC_66A-66A_n5A-n77A14 | DC_66A_n5A DC_66A_n77A14 |  |
| DC_66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |  |
| DC_66A-66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |  |
| DC_66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |  |
| DC_66A-66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |  |
| DC_66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |  |
| DC_66A-66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |  |
| DC_66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |  |
| DC_66A-66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |  |
| DC_66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |  |
| DC_66A_n38A-n66A | DC_66A_n38A DC_66A_n66A2 |  |
| DC_66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |  |
| DC_66A_n66A-n77A14 DC_66A_n66A-n77C14 | DC_66A_n77A14 |  |
| DC_66A_n66A-n78A | DC_66A_n66A2 DC_66A_n78A |  |
| DC_66A-(n)12AA | DC_66A_n12A DC_(n)12AA2 |  |
| DC_66A-(n)71AA DC_66C-(n)71AA | DC_66A_n71A DC_(n)71AA |  |
| DC_66A_n25A-n41A DC_66A_n25A-n41C | DC_66A_n25A DC_66A_n41A |  |
| DC_66A_n25A-n41(2A) | DC_66A_n25A DC_66A_n41A |  |
| DC_66A_n25A-n48A | DC_66A_n25A DC_66A_n48A |  |
| DC_66A_n25A-n66A | DC_66A_n25A DC_66A_n66A2 |  |
| DC_66A_n38A-n71A | DC_66A_n38A DC_66A_n71A |  |
| DC_66A_n41A-n71A DC_66A_n41C-n71A | DC_66A_n41A DC_66A_n71A |  |
| DC_66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |  |
| DC_66A_n66A-n71A | DC_66A_n66A DC_66A_n71A |  |
| DC_66A-71A_n38A | DC_71A_n38A DC_66A_n38A |  |
| DC_66A-71A_n41A | DC_66A_n41A DC_71A_n41A |  |
| DC_66A-71A_n66A | DC_71A_n66A DC_66A_n66A2 |  |
| DC_66A-71A_n71A | DC_66A_n71A |  |
| DC_66A-71A_n78A DC_66A-71A_n78(2A) | DC_71A_n78A DC_66A_n78A |  |
| DC_66A_n71A-n78A | DC_66A_n71A DC_66A_n78A |  |
| DC_66A_SUL_n78A-n86A | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A | Yes |
| DC_66A_SUL_n78(2A)-n86A | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A | Yes |
| DC_71A_n2A-n41A | DC_71A_n2A DC_71A_n41A |  |
| DC_71A_n2A-n66A | DC_71A_n2A DC_71A_n66A |  |
| DC_71A_n2A-n78A | DC_71A_n2A DC_71A_n78A |  |
| DC_71A_n38A-n66A | DC_71A_n38A DC_71A_n66A |  |
| DC_71A_n38A-n78A | DC_71A_n38A DC_71A_n78A |  |
| DC_71A_n66A-n78A | DC_71A_n66A DC_71A_n78A |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Void NOTE 6: N/A NOTE 7: Void. NOTE 8: UL carrier shall be supported in Band 2 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 9: UL carrier shall be supported in Band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 10: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE 11: The frequency range in band 3 is restricted for this band combination to 1765 - 1785 MHz for the UL and 1860-1880 MHz for the DL. NOTE 12: The frequency range in band 42 is restricted for this band combination to 3440 - 3520 MHz. NOTE 13: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 19: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 20: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |  |

#### 5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |  | Mandatory simultaneous Rx/Tx |  |
| --- | --- | --- | --- | --- |
| DC_1A-3A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A |  |  |  |
| DC_1A-3A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A |  | Yes |  |
| DC_1A-3A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A |  | Yes |  |
| DC_1A-3A-5A_n77A DC_1A-3A-5A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A |  |  |  |
| DC_1A-3A-5A_n78A DC_1A-3A-5A_n78C | DC_1A_n78A DC_3A_n78A DC_5A_n78A |  | Yes |  |
| DC_1A-3C-5A_n78A DC_1A-1A-3A-5A_n78A DC_1A-1A-3C-5A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A |  |  |  |
| DC_1A-3A-5A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A |  |  |  |
| DC_1A-1A-3A-5A_n78A DC_1A-1A-3C-5A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A |  |  |  |
| DC_1A-3A_n5A-n78A DC_1A-3C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A |  | Yes |  |
| DC_1A-3A-5A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A |  | Yes |  |
| DC_1A-3A-7A_n3A DC_1A-3A-7C_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A |  |  |  |
| DC_1A-3A-7A_n5A DC_1A-3A-7C_n5A DC_1A-3C-7A_n5A DC_1A-3C-7C_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A |  |  |  |
| DC_1A-3A-7A_n7A DC_1A-3C-7A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 |  |  |  |
| DC_1A-1A-3A-7A_n7A DC_1A-1A-3C-7A_n7A DC_1A-3A-3A-7A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 |  |  |  |
| DC_1A-3A-7A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A |  |  |  |
| DC_1A-3A-7A_n28A DC_1A-3A-7C_n28A DC_1A-3C-7A_n28A DC_1A-3C-7C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |  |  |  |
| DC_1A-1A-3C-7A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_7A_n28A |  |  |  |
| DC_1A-3A-7A_n38A12,13 | CA_1A-3A |  |  |  |
| DC_1A-3A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A |  |  |  |
| DC_1A-3A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |  |  |  |
| DC_1A-3A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |  |  |  |
| DC_1A-3A-7A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |  |  |  |
| DC_1A-3A-7A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |  |  |  |
| DC_1A-3A-7A_n78A DC_1A-3C-7A_n78A DC_1A-3A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |  | Yes |  |
| DC_1A-3A-7C_n78A DC_1A-3C-7C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |  |  |  |
| DC_1A-3A-7A_n78(2A) DC_1A-3C-7A_n78(2A) DC_1A-3A-7C_n78(2A) DC_1A-3C-7C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |  |  |  |
| DC_1A-1A-3A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |  |  |  |
| DC_1A-3A_n7A-n78A DC_1A-3A_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A |  |  |  |
| DC_1A-3A_n7A-n78(2A) DC_1A-3C_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A |  |  |  |
| DC_1A-3C_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A |  |  |  |
| DC_1A-3A-7A-7A_n78A DC_1A-3A-7A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_7A_n78A |  | Yes |  |
| DC_1A-1A-3C-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |  |  |  |
| DC_1A-3A-7A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |  |  |  |
| DC_1A-3A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A |  |  |  |
| DC_1A-3A-8A_n77A | DC_1A_n77A DC_3A_n77A DC_8A_n77A |  | Yes |  |
| DC_1A-3C-8A_n77A | DC_1A_n77A DC_3A_n77A DC_3C_n77A DC_8A_n77A |  |  |  |
| DC_1A-3A-8A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_8A_n77A |  | Yes |  |
| DC_1A-3C-8A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_3C_n77A DC_8A_n77A |  |  |  |
| DC_1A-3A-8A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_8A_n77A |  | Yes |  |
| DC_1A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A |  | Yes |  |
| DC_1A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A |  | Yes |  |
| DC_1A-3A-8A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A |  | Yes |  |
| DC_1A-3C-8A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A |  |  |  |
| DC_1A-3A-8A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_8A_n78A |  | Yes |  |
| DC_1A-3A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A |  |  |  |
| DC_1A-3A-8A_n79A | DC_1A_n79A DC_3A_n79A DC_8A_n79A |  | Yes |  |
| DC_1A-3A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_11A_n28A |  |  |  |
| DC_1A-3A-11A_n77A | DC_1A_n77A DC_3A_n77A DC_11A_n77A |  | Yes |  |
| DC_1A-3A-11A_n77(2A) DC_1A-3A-11A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_11A_n77A |  | Yes |  |
| DC_1A-3A-18A_n3A | DC_1A_n3A DC_3A_n3A4 DC_18A_n3A |  |  |  |
| DC_1A-3A-18A_n28A | DC_1A_n28A DC_3A_n28A DC_18A_n28A |  |  |  |
| DC_1A-3A-18A_n41A | DC_1A_n41A DC_3A_n41A DC_18A_n41A |  |  |  |
| DC_1A-3A-18A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |  |  |  |
| DC_1A-3A-18A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_18A_n77A |  |  |  |
| DC_1A-3A-18A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |  |  |  |
| DC_1A-3A-18A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_18A_n78A |  |  |  |
| DC_1A-3A-18A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |  |  |  |
| DC_1A-3A-19A_n77A DC_1A-3A-19A_n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A |  | Yes |  |
| DC_1A-3A-19A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_19A_n77A | Yes |  |
| DC_1A-3A-19A_n78A DC_1A-3A-19A_n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78A |  | Yes |  |
| DC_1A-3A-19A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_19A_n78A | Yes |  |
| DC_1A-3A-19A_n79A DC_1A-3A-19A_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A |  | Yes |  |
| DC_1A-3A-20A_n7A | DC_1A_n7A DC_3A_n7A DC_20A_n7A |  |  |  |
| DC_1A-3A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_20A_n8A |  |  |  |
| DC_1A-3A-20A_n28A3,8,14 DC_1A-3C-20A_n28A3,8,14 | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_20A_n28A |  |  |  |
| DC_1A-3A-20A_n38A | DC_3A_n38A DC_20A_n38A |  |  |  |
| DC_1A-3A-20A_n41A DC_1A-3C-20A_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A DC_20A_n41A |  |  |  |
| DC_1A-3A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |  | Yes |  |
| DC_1A-3A-21A_n77A DC_1A-3A-21A_n77C | DC_1A_n77A DC_3A_n77A DC_21A_n77A |  | Yes |  |
| DC_1A-3A-21A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_21A_n77A | Yes |  |
| DC_1A-3A-21A_n78A DC_1A-3A-21A_n78C | DC_1A_n78A DC_3A_n78A DC_21A_n78A |  | Yes |  |
| DC_1A-3A-21A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_21A_n78A | Yes |  |
| DC_1A-3A-21A_n79A DC_1A-3A-21A_n79C | DC_1A_n79A DC_3A_n79A DC_21A_n79A |  | Yes |  |
| DC_1A-3A-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A |  |  |  |
| DC_1A-3A-28A_n5A DC_1A-3C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A DC_28A_n5A |  |  |  |
| DC_1A-3A-28A_n7A DC_1A-3C-28A_n7A DC_1A-3A-28A_n7B DC_1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |  |  |  |
| DC_1A-3A-3A-28A_n7A DC_1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_28A_n7A |  |  |  |
| DC_1A-1A-3A-28A_n7A DC_1A-1A-3C-28A_n7A DC_1A-1A-3A-28A_n7B DC_1A-1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |  |  |
| DC_1A-1A-3A-3A-28A_n7A DC_1A-1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |  |  |
| DC_1A-3A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_28A_n40A |  |  |  |
| DC_1A-3A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A |  | Yes |  |
| DC_1A-3A_n28A-n75A | DC_1A_n28A DC_3A_n28A |  |  |  |
| DC_1A-3C_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |  |  |  |
| DC_1A-3A-28A_n77A DC_1A-3A-28A_n77C | DC_1A_n77A DC_3A_n77A DC_28A_n77A |  | Yes |  |
| DC_1A-3A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |  | Yes |  |
| DC_1A-3A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |  | Yes |  |
| DC_1A-3A-28A_n78A DC_1A-3C-28A_n78A DC_1A-3A-28A_n78C | DC_1A_n78A DC_3A_n78A DC_28A_n78A |  | Yes |  |
| DC_1A-1A-3A-28A_n78A DC_1A-1A-3C-28A_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A |  |  |  |
| DC_1A-3A-28A_n79A DC_1A-3A-28A_n79C | DC_1A_n79A DC_3A_n79A DC_28A_n79A |  | Yes |  |
| DC_1A-3A_n28A-n79A | DC_1A_n28A DC_1A_n79A DC_3A_n28A DC_3A_n79A |  | Yes |  |
| DC_1A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A |  |  |
| DC_1A-3A_n28A-n78A DC_1A-3C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A |  | Yes |  |
| DC_1A-3A-32A_n28A DC_1A-3C-32A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |  |  |
| DC_1A-3A-32A_n78A DC_1A-3A-32A_n78C | DC_1A_n78A DC_3A_n78A |  |  |  |
| DC_1A-3A-32A_n78(2A) | DC_1A_n78A DC_3A_n78A |  |  |
| DC_1A-3C-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |  |  |
| DC_1A-3A-38A_n28A DC_1A-3C-38A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_38A_n28A |  |  |  |
| DC_1A-3A-38A_n78A | DC_1A_n78A DC_3A_n78A |  |  |  |
| DC_1A-3A_n38A-n78A | DC_3A_n38A DC_3A_n78A |  |  |  |
| DC_1A-3A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A |  |  |  |
| DC_1A-3A-40A_n78A DC_1A-3A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_40A_n78A |  |  |  |
| DC_1A-3A-40A_n78(2A) DC_1A-3A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_40A_n78A |  |  |
| DC_1A-3A-41A_n3A DC_1A-3A-41C_n3A | DC_1A_n3A DC_3A_n3A4 DC_41A_n3A DC_41C_n3A |  |  |  |
| DC_1A-3A-41A_n28A DC_1A-3A-41C_n28A | DC_1A_n28A DC_3A_n28A DC_41A_n28A DC_41C_n28A |  | Yes |  |
| DC_1A-3A-41A_n41A | DC_1A_n41A DC_3A_n41A |  |  |  |
| DC_1A-3A-(n)41AA | DC_1A_n41A DC_3A_n41A |  |  |  |
| DC_1A-3A-41A_n77A DC_1A-3A-41C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |  |  |  |
| DC_1A-3A-41A_n77(2A) DC_1A-3A-41C_n77(2A) | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |  |  |  |
| DC_1A-3A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A |  |  |  |
| DC_1A-3A-41A_n78A DC_1A-3A-41C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |  |  |  |
| DC_1A-3A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |  |  |  |
| DC_1A-3A-41A_n78(2A) DC_1A-3A-41C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |  |  |  |
| DC_1A-3A-41A_n79A DC_1A-3A-41C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |  | Yes |  |
| DC_1A-3A-42A_n28A DC_1A-3A-42C_n28A | DC_1A_n28A DC_3A_n28A DC_42A_n28A DC_42C_n28A |  | Yes |  |
| DC_1A-3A-42A_n77A7,8 DC_1A-3A-42A_n77C7,8 DC_1A-3A-42C_n77A7,8 DC_1A-3A-42C_n77C7,8 DC_1A-3A-42D_n77A7,8 | DC_1A_n77A DC_3A_n77A |  |  |  |
| DC_1A-3A-42A_n77(2A) 7,8 DC_1A-3A-42C_n77(2A) 7,8 | DC_1A_n77A DC_3A_n77A |  |  |  |
| DC_1A-3A-42A_n78A7,8 DC_1A-3A-42A_n78C7,8 DC_1A-3A-42C_n78A7,8 DC_1A-3A-42C_n78C7,8 DC_1A-3A-42D_n78A7,8 | DC_1A_n78A DC_3A_n78A |  |  |  |
| DC_1A-3A-42A_n79A DC_1A-3A-42A_n79C DC_1A-3A-42C_n79A DC_1A-3A-42C_n79C DC_1A-3A-42D_n79A | DC_1A_n79A DC_3A_n79A |  |  |  |
| DC_1A-3A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_3A_n77A DC_3A_n79A |  |  |  |
| DC_1A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |  |  |
| DC_1A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |  |  |  |
| DC_1A-3A_n78A-n79A | DC_1A_n78A DC_1A_n79A DC_3A_n78A DC_3A_n79A |  |  |  |
| DC_1A-3A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |  |  |  |
| DC_1A-5A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |  |  |  |
| DC_1A-5A-7A_n77(2A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_1A-5A-7A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_1A-5A-7A-7A_n77(2A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_1A-5A-7A_n78A DC_1A-5A-7A_n78C | DC_1A_n78A DC_5A_n78A DC_7A_n78A |  |  |  |
| DC_1A-5A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_1A-5A-7A-7A_n78A DC_1A-5A-7A-7A_n78C | DC_1A_n78A DC_5A_n78A DC_7A_n78A |  |  |  |
| DC_1A-5A-7A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_1A-5A-41A_n79A | DC_1A_n79A DC_5A_n79A DC_41A_n79A |  |  |  |
| DC_1A-7A_n3A-n38A | DC_1A_n3A |  |  |  |
| DC_1A-7A_n3A-n78A DC_1A-7C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_7A_n3A DC_7C_n3A DC_7A_n78A DC_7C_n78A |  |  |  |
| DC_1A-7A_n5A-n78A DC_1A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A |  |  |  |
| DC_1A-7A-8A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A |  |  |  |
| DC_1A-7A-8A_n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A |  |  |  |
| DC_1A-7A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_1A_n78A DC_7A_n78A |  |  |  |
| DC_1A-7A-8A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A |  |  |  |
| DC_1A-7A-8A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A |  |  |
| DC_1A-7A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_7A_n8A DC_7A_n78A |  |  |  |
| DC_1A-7A-20A_n3A DC_1A-7C-20A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_20A_n3A |  |  |  |
| DC_1A-7A-20A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |  |  |  |
| DC_1A-7A-20A_n28A3,8,14 | DC_1A_n28A DC_7A_n28A DC_20A_n28A |  |  |  |
| DC_1A-7A-20A_n38A12,13 | CA_1A-20A |  |  |
| DC_1A-7A-20A_n78A | DC_1A_n78A DC_7A_n78A DC_20A_n78A |  | Yes |  |
| DC_1A-7A-28A_n3A DC_1A-7C-28A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_28A_n3A |  |  |  |
| DC_1A-7A-28A_n5A DC_1A-7C-28A_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |  |  |  |
| DC_1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |  |  |  |
| DC_1A-1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |  |  |  |
| DC_1A-7A-28A_n40A | DC_1A_n40A DC_7A_n40A DC_28A_n40A |  |  |  |
| DC_1A-7A-28A_n78A DC_1A-7C-28A_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |  |  |  |
| DC_1A-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A |  | Yes |  |
| DC_1A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |  |  |  |
| DC_1A-7A-32A_n3A | DC_1A_n3A DC_7A_n3A |  |  |  |
| DC_1A-7A-32A_n8A | DC_1A_n8A DC_7A_n8A |  |  |  |
| DC_1A-7A-32A_n28A | DC_1A_n28A DC_7A_n28A |  |  |  |
| DC_1A-7A-32A_n78A | DC_1A_n78A DC_7A_n78A |  |  |  |
| DC_1A-7A-38A_n3A | DC_1A_n3A |  |  |  |
| DC_1A-7A-38A_n8A | DC_1A_n8A |  |  |  |
| DC_1A-7A-38A_n28A10 | DC_1A_n28A |  |  |  |
| DC_1A-7A-38A_n78A10 | DC_1A_n78A |  |  |  |
| DC_1A-7A-40A_n78A DC_1A-7A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_40A_n78A |  |  |  |
| DC_1A-7A-40A_n78(2A) DC_1A-7A-40C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_40A_n78A |  |  |  |
| DC_1A-7A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A |  |  |  |
| DC_1A-8A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A |  |  |  |
| DC_1A-8A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A |  | Yes |  |
| DC_1A-8A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A | Yes |  |
| DC_1A-8A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A |  |  |  |
| DC_1A-8A-11A_n3A | DC_1A_n3A DC_8A_n3A DC_11A_n3A |  |  |  |
| DC_1A-8A-11A_n28A | DC_1A_n28A DC_8A_n28A DC_11A_n28A |  |  |  |
| DC_1A-8A-11A_n77A | DC_1A_n77A DC_8A_n77A DC_11A_n77A |  | Yes |  |
| DC_1A-8A-11A_n77(2A) DC_1A-8A-11A_n77(3A) | DC_1A_n77A DC_8A_n77A DC_11A_n77A |  | Yes |  |
| DC_1A-8A-11A_n78A | DC_1A_n78A DC_8A_n78A DC_11A_n78A |  | Yes |  |
| DC_1A-8A-11A_n79A | DC_1A_n79A DC_8A_n79A DC_11A_n79A |  | Yes |  |
| DC_1A-8A-20A_n3A | DC_1A_n3A DC_8A_n3A DC_20A_n3A |  |  |  |
| DC_1A-8A-20A_n28A3,8,11,14 | DC_1A_n28A DC_8A_n28A DC_20A_n28A |  |  |
| DC_1A-8A-20A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A |  |  |  |
| DC_1A-8A-28A_n3A | DC_1A_n3A DC_8A_n3A DC_28A_n3A |  |  |  |
| DC_1A-8A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |  | Yes |  |
| DC_1A-8A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |  | Yes |  |
| DC_1A-8A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_28A_n78A |  |  |  |
| DC_1A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_8A_n28A DC_8A_n78A |  | Yes |  |
| DC_1A-8A_n28A-n79A | DC_1A_n28A DC_1A_n79A DC_8A_n28A DC_8A_n79A |  | Yes |  |
| DC_1A-8A-32A_n3A | DC_1A_n3A DC_8A_n3A |  |  |  |
| DC_1A-8A-32A_n78A | DC_1A_n78A DC_8A_n78A |  |  |  |
| DC_1A-8A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_8A_n40A DC_8A_n78A |  |  |  |
| DC_1A-8A-40A_n78A DC_1A-8A-40C_n78A | DC_1A_n78A DC_8A_n78A DC_40A_n78A |  |  |  |
| DC_1A-8A-40A_n78(2A) DC_1A-8A-40C_n78(2A) | DC_1A_n78A DC_8A_n78A DC_40A_n78A |  |  |  |
| DC_1A-8A-42A_n3A DC_1A-8A-42C_n3A | DC_1A_n3A DC_8A_n3A DC_42A_n3A DC_42C_n3A |  | Yes |  |
| DC_1A-8A-42A_n28A DC_1A-8A-42C_n28A | DC_1A_n28A DC_8A_n28A DC_42A_n28A DC_42C_n28A |  | Yes |  |
| DC_1A-8A-42A_n77A7,8 DC_1A-8A-42C_n77A7,8 | DC_1A_n77A DC_8A_n77A |  |  |  |
| DC_1A-8A-42A_n77(2A) 7,8 DC_1A-8A-42C_n77(2A) 7,8 | DC_1A_n77A DC_8A_n77A |  |  |  |
| DC_1A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A |  |  |  |
| DC_1A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_11A_n3A DC_11A_n28A |  |  |  |
| DC_1A-11A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |  | Yes |  |
| DC_1A-11A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |  | Yes |  |
| DC_1A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_11A_n3A DC_11A_n79A |  |  |  |
| DC_1A-11A-18A_n3A | DC_1A_n3A DC_11A_n3A DC_18A_n3A |  |  |  |
| DC_1A-11A-18A_n28A | DC_1A_n28A DC_11A_n28A DC_18A_n28A |  |  |  |
| DC_1A-11A-18A_n41A | DC_1A_n41A DC_11A_n41A DC_18A_n41A |  |  |  |
| DC_1A-11A-18A_n77A | DC_1A_n77A DC_11A_n77A DC_18A_n77A |  |  |  |
| DC_1A-11A-18A_n77(2A) | DC_1A_n77A DC_11A_n77A DC_18A_n77A |  |  |
| DC_1A-11A-18A_n78A | DC_1A_n78A DC_11A_n78A DC_18A_n78A |  |  |  |
| DC_1A-11A-18A_n78(2A) | DC_1A_n78A DC_11A_n78A DC_18A_n78A |  |  |
| DC_1A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |  | Yes |  |
| DC_1A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |  | Yes |  |
| DC_1A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |  |  |  |
| DC_1A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |  |  |  |
| DC_1A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_18A_n3A DC_18A_n41A |  |  |  |
| DC_1A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_18A_n3A DC_18A_n77A |  |  |  |
| DC_1A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_18A_n3A DC_18A_n78A |  |  |  |
| DC_1A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_18A_n28A DC_18A_n41A |  |  |  |
| DC_1A-18A-28A_n77A | DC_1A_n77A DC_18A_n77A DC_28A_n77A |  |  |  |
| DC_1A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |  |  |  |
| DC_1A-18A-28A_n78A | DC_1A_n78A DC_18A_n78A DC_28A_n78A |  |  |  |
| DC_1A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |  |  |  |
| DC_1A-18A-28A_n79A | DC_1A_n79A DC_18A_n79A DC_28A_n79A |  | Yes |  |
| DC_1A-18A-41A_n3A DC_1A-18A-41C_n3A | DC_1A_n3A DC_18A_n3A DC_41A_n3A DC_41C_n3A |  |  |  |
| DC_1A-18A-41A_n77A DC_1A-18A-41C_n77A | DC_1A_n77A DC_18A_n77A DC_41A_n77A DC_41C_n77A |  |  |  |
| DC_1A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |  |  |  |
| DC_1A-18A-41A_n78A DC_1A-18A-41C_n78A | DC_1A_n78A DC_18A_n78A DC_41A_n78A DC_41C_n78A |  |  |  |
| DC_1A-18A-42A_n77A7,8 DC_1A-18A-42C_n77A7,8 | DC_1A_n77A DC_18A_n77A |  |  |  |
| DC_1A-18A-42A_n78A7,8 DC_1A-18A-42C_n78A7,8 | DC_1A_n78A DC_18A_n78A |  |  |  |
| DC_1A-18A-42A_n79A DC_1A-18A-42C_n79A | DC_1A_n79A DC_18A_n79A |  |  |  |
| DC_1A-19A-21A_n77A DC_1A-19A-21A_n77C | DC_1A_n77A DC_19A_n77A DC_21A_n77A |  | Yes |  |
| DC_1A-19A-21A_n77(2A) | DC_1A_n77A DC_19A_n77A DC_21A_n77A |  | Yes |  |
| DC_1A-19A-21A_n78A DC_1A-19A-21A_n78C | DC_1A_n78A DC_19A_n78A DC_21A_n78A | Yes |  |
| DC_1A-19A-21A_n78(2A) | DC_1A_n78A DC_19A_n78A DC_21A_n78A |  | Yes |  |
| DC_1A-19A-21A_n79A DC_1A-19A-21A_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A | Yes |  |
| DC_1A-19A-42A_n77A7,8 DC_1A-19A-42A_n77C7,8 DC_1A-19A-42C_n77A7,8 DC_1A-19A-42C_n77C7,8 | DC_1A_n77A DC_19A_n77A |  |  |  |
| DC_1A-19A-42A_n78A7,8 DC_1A-19A-42A_n78C7,8 DC_1A-19A-42C_n78A7,8 DC_1A-19A-42C_n78C7,8 | DC_1A_n78A DC_19A_n78A |  |  |  |
| DC_1A-18A-42A_n77A7,8 DC_1A-18A-42C_n77A7,8 | DC_1A_n77A DC_18A_n77A |  |  |  |
| DC_1A-19A-42A_n79A DC_1A-19A-42A_n79C DC_1A-19A-42C_n79A DC_1A-19A-42C_n79C | DC_1A_n79A DC_19A_n79A |  |  |  |
| DC_1A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |  |  |  |
| DC_1A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |  |  |  |
| DC_1A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A DC_1A_n38A DC_20A_n38A |  |  |  |
| DC_1A-20A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |  |  |  |
| DC_1A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_20A_n8A DC_20A_n78A |  |  |  |
| DC_1A-20A-28A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |  |  |  |
| DC_1A-20A_n28A-n75A | DC_1A_n28A DC_20A_n28A |  |  |  |
| DC_1A-20A-28A_n78A | DC_1A_n78A DC_20A_n78A DC_28A_n78A |  |  |  |
| DC_1A-20A_n28A-n78A3,8,14 | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A |  | Yes |  |
| DC_1A-20A-32A_n3A | DC_1A_n3A DC_20A_n3A |  |  |  |
| DC_1A-20A-32A_n8A | DC_1A_n8A DC_20A_n8A |  |  |  |
| DC_1A-20A-32A_n28A8,14 | DC_1A_n28A DC_20A_n28A |  |  |  |
| DC_1A-20A-32A_n78A | DC_1A_n78A DC_20A_n78A |  |  |  |
| DC_1A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |  |  |  |
| DC_1A-20A-(n)38AA | DC_1A_n38A DC_20A_n38A |  |  |  |
| DC_1A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A DC_38A_n8A |  |  |  |
| DC_1A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A |  |  |  |
| DC_1A-20A-40A_n78A | DC_1A_n78A DC_20A_n78A DC_40A_n78A |  |  |  |
| DC_1A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_20A_n41A DC_20A_n78A |  |  |  |
| DC_1A-21A-28A_n77A | DC_1A_n77A DC_21A_n77A DC_28A_n77A |  | Yes |  |
| DC_1A-21A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_21A_n28A DC_21A_n77A |  | Yes |  |
| DC_1A-21A-28A_n78A | DC_1A_n78A DC_21A_n78A DC_28A_n78A |  | Yes |  |
| DC_1A-21A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_21A_n28A DC_21A_n78A |  | Yes |  |
| DC_1A-21A-28A_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A |  | Yes |  |
| DC_1A-21A_n28A-n79A | DC_1A_n28A DC_1A_n79A DC_21A_n28A DC_21A_n79A |  | Yes |  |
| DC_1A-21A-42A_n77A7,8 DC_1A-21A-42A_n77C7,8 DC_1A-21A-42C_n77A7,8 DC_1A-21A-42C_n77C7,8 DC_1A-21A-42D_n77A7,8 DC_1A-21A-42D_n77C7,8 | DC_1A_n77A DC_21A_n77A |  |  |  |
| DC_1A-21A-42A_n78A7,8 DC_1A-21A-42A_n78C7,8 DC_1A-21A-42C_n78A7,8 DC_1A-21A-42C_n78C7,8 DC_1A-21A-42D_n78A7,8 DC_1A-21A-42D_n78C7,8 | DC_1A_n78A DC_21A_n78A |  |  |  |
| DC_1A-21A-42A_n79A DC_1A-21A-42A_n79C DC_1A-21A-42C_n79A DC_1A-21A-42C_n79C DC_1A-21A-42D_n79A DC_1A-21A-42D_n79C | DC_1A_n79A DC_21A_n79A |  |  |  |
| DC_1A-21A_n77A-n79A | DC_1A_n77A DC_1A_n79A |  |  |  |
| DC_1A-21A_n78A-n79A | DC_1A_n78A DC_1A_n79A |  |  |  |
| DC_1A-28A_n3A-n77A | DC_28A_n3A DC_28A_n77A |  | Yes |  |
| DC_1A-28A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_28A_n3A DC_28A_n78A |  | Yes |  |
| DC_1A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_28A_n5A DC_28A_n78A |  | Yes |  |
| DC_1A-28A_n7A-n78A | DC_1A_n7A DC_28A_n7A DC_1A_n78A DC_28A_n78A |  |  |  |
| DC_1A-28A_n7B-n78A | DC_1A_n7A DC_1A_n7B DC_28A_n7A DC_28A_n7B DC_1A_n78A DC_28A_n78A |  |  |  |
| DC_1A-28A-32A_n3A | DC_1A_n3A DC_28A_n3A |  |  |  |
| DC_1A-28A-40A_n78A | DC_1A_n78A DC_28A_n78A DC_40A_n78A |  |  |  |
| DC_1A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_28A_n40A DC_28A_n78A |  |  |  |
| DC_1A-28A-42A_n77A7,8 DC_1A-28A-42C_n77A7,8 | DC_1A_n77A DC_28A_n77A |  |  |  |
| DC_1A-28A-42A_n78A7,8 DC_1A-28A-42C_n78A7,8 | DC_1A_n78A DC_28A_n78A |  |  |  |
| DC_1A-28A-42A_n79A DC_1A-28A-42C_n79A | DC_1A_n79A DC_28A_n79A |  |  |  |
| DC_1A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_41A_n3A |  |  |  |
| DC_1A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A |  |  |  |
| DC_1A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A |  |  |  |
| DC_1A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_38A_n3A DC_38A_n78A |  |  |  |
| DC_1A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_41A_n3A DC_41A_n77A |  |  |  |
| DC_1A-41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |  |  |  |
| DC_1A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_41A_n3A DC_41A_n78A |  |  |  |
| DC_1A-41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |  |  |  |
| DC_1A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_41A_n28A |  |  |  |
| DC_1A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A |  |  |  |
| DC_1A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |  |  |  |
| DC_1A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A |  |  |  |
| DC_1A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |  |  |  |
| DC_1A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_41A_n77A |  |  |  |
| DC_1A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_41A_n78A |  |  |  |
| DC_1A-42A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A |  | Yes |  |
| DC_1A-42C_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A DC_42C_n3A DC_42C_n28A |  | Yes |  |
| DC_1A-42A_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |  |  |  |
| DC_1A-42A_n3A-n77(2A) 7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |  |  |  |
| DC_1A-42C_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |  |  |  |
| DC_1A-42C_n3A-n77(2A)7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |  |  |  |
| DC_1A-42A_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |  |  |  |
| DC_1A-42A_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |  |  |  |
| DC_1A-42C_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |  |  |  |
| DC_1A-42C_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |  |  |  |
| DC_1A-41A-42A_n77A7,8 DC_1A-41A-42C_n77A7,8 DC_1A-41C-42A_n77A7,8 DC_1A-41C-42C_n77A7,8 | DC_1A_n77A DC_41A_n77A |  |  |  |
| DC_1A-41A-42A_n77(2A)7,8 DC_1A-41A-42C_n77(2A)7,8 | DC_1A_n77A DC_41A_n77A |  |  |  |
| DC_1A-41A-42A_n78A7,8 DC_1A-41A-42C_n78A7,8 DC_1A-41C-42A_n78A7,8 DC_1A-41C-42C_n78A7,8 | DC_1A_n78A DC_41A_n78A |  |  |  |
| DC_1A-41A-42A_n79A DC_1A-41A-42C_n79A DC_1A-41C-42A_n79A DC_1A-41C-42C_n79A | DC_1A_n79A DC_41A_n79A |  |  |  |
| DC_1A-42A_n77A-n79A7,8 DC_1A-42C_n77A-n79A7,8 | DC_1A_n77A DC_1A_n79A |  |  |  |
| DC_1A-42A_n78A-n79A7,8 DC_1A-42C_n78A-n79A7,8 | DC_1A_n78A DC_1A_n79A |  |  |  |
| DC_2A-4A-7A_n28A | DC_2A_n28A DC_4A_n28A DC_7A_n28A |  |  |  |
| DC_2A-5A_n2A-n77A DC_2A-5A_n2A-n77C | DC_2A_n77A DC_5A_n2A DC_5A_n77A |  |  |  |
| DC_2A-5A_n2A-n78A | DC_5A_n2A DC_2A_n78A DC_5A_n78A |  |  |  |
| DC_2A-5A_n5A-n77A DC_2A-5A_n5A-n77C | DC_2A_n5A DC_2A_n77A DC_5A_n77A |  |  |  |
| DC_2A-5A_n5A-n77A9 DC_2A-5A_n5A-n77C9 | DC_2A_n77A DC_5A_n77A |  |  |
| DC_2A-5A-7A_n2A | DC_5A_n2A DC_7A_n2A |  |  |  |
| DC_2A-5A-7A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 |  |  |  |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |  |  |  |
| DC_2A-5A-7A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A |  |  |  |
| DC_2A-2A-5A-7A_n66A DC_2A-5A-7A-7A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |  |  |  |
| DC_2A-5A-(n)12AA | DC_5A_n12A DC_2A_n12A DC_(n)12AA4 |  |  |  |
| DC_2A-12A-(n)5AA | DC_2A_n5A DC_12A_n5A DC_(n)5AA4 |  |  |  |
| DC_2A-5A-30A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A |  |  |  |
| DC_2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |  |  |  |
| DC_2A-2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |  |  |
| DC_2A-5A-30A_n77A9 DC_2A-2A-5A-30A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |  |  |
| DC_2A-5A-48A_n12A | DC_2A_n12A DC_5A_n12A DC_48A_n12A |  |  |  |
| DC_2A-5A-48A_n77A7,8,9 DC_2A-5A-48C_n77A7,8,9 DC_2A-5A-48C_n77C7,8,9 | DC_2A_n77A DC_5A_n77A |  |  |  |
| DC_2A-5A-48A_n77A9 DC_2A-5A-48C_n77A9 DC_2A-5A-48C_n77C9 | DC_2A_n77A DC_5A_n77A |  |  |
| DC_2A-5A-66A_n2A DC_2A-5B-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |  |  |  |
| DC_2A-5A-5A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |  |  |  |
| DC_2A-5A-66A-66A_n2A DC_2A-5B-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |  |  |
| DC_2A-5A-5A-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |  |  |
| DC_2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |  |
| DC_2A-2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |  |
| DC_2A-2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |
| DC_2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |
| DC_2A-5A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |  |  |  |
| DC_2A-5A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |  |  |
| DC_2A-5A-66A_n12A | DC_2A_n12A DC_5A_n12A DC_66A_n12A |  |  |  |
| DC_2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |  |  |  |
| DC_2A-2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |  |  |
| DC_2A-5A-66A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |  |  |
| DC_2A-5A-66A_n48A DC_2A-5A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |  |  |  |
| DC_2A-5A-66A-66A_n48A DC_2A-5A-66A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |  |  |
| DC_2A-5A-66A_n66A DC_2A-5B-66A_n66A | DC_2A_n66A DC_5A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-5A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |  |  |  |
| DC_2A-2A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |  |  |
| DC_2A-5A-66A-66A_n66A DC_2A-5B-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |  |  |
| DC_2A-2A-5A-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |  |  |
| DC_2A-5A-5A-66A-66A_n66A |  |  |  |
| DC_2A-5A-66A_n71A | DC_2A_n71A DC_5A_n71A DC_66A_n71A |  |  |  |
| DC_2A-5A-66A_n77A9 DC_2A-5A-66A_n77C9 DC_2A-2A-5A-66A_n77C9 DC_2A-5A-66A-66A_n77C9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |  |  |  |
| DC_2A-2A-5A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |  |  |
| DC_2A-5A-66A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |  |  |
| DC_2A-5A_n66A-n77A DC_2A-5A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A |  |  |  |
| DC_2A-5A_n66A-n78A | DC_2A_n66A DC_5A_n66A DC_2A_n78A DC_5A_n78A |  |  |  |
| DC_2A-7A_n2A-n78A | DC_7A_n2A DC_2A_n78A DC_7A_n78A |  |  |  |
| DC_2A-7A-12A_n2A | DC_7A_n2A DC_12A_n2A |  |  |  |
| DC_2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |  |  |  |
| DC_2A-2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |  |  |
| DC_2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |  |  |  |
| DC_2A-2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |  |  |
| DC_2A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |  |  |  |
| DC_2A-7A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |  |  |  |
| DC_2A-7C-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |  |  |  |
| DC_2A-7A-13A_n66A DC_2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |  |  |  |
| DC_2A-2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |  |  |
| DC_2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |  |  |
| DC_2A-2A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |  |  |  |
| DC_2A-2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |  |  |
| DC_2A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |  |  |  |
| DC_2A-7A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |  |  |  |
| DC_2A-7C_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |  |  |  |
| DC_2A-7A-28A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A |  |  |  |
| DC_2A-7A-28A_n66A DC_2A-7C-28A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A |  |  |  |
| DC_2A-7A-28A_n78A DC_2A-7C-28A_n78A | DC_2A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |  |  |  |
| DC_2A-7A_n38A-n66A DC_2A-7C_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A |  |  |  |
| DC_2A-7A-7A_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A |  |  |
| DC_2A-7A-29A_n78A DC_2A-7C-29A_n78A | DC_2A_n78A DC_7A_n78A |  |  |
| DC_2A-7A-7A-29A_n78A | DC_2A_n78A DC_7A_n78A |  |  |
| DC_2A-7A_n38A-n78A DC_2A-7C_n38A-n78A | DC_2A_n78A |  |  |  |
| DC_2A-7A-7A_n38A-n78A | DC_2A_n78A |  |  |
| DC_2A-7A-66A_n2A | DC_7A_n2A DC_66A_n2A |  |  |  |
| DC_2A-7A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |  |  |  |
| DC_2A-7A-66A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |  |  |
| DC_2A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |  |  |  |
| DC_2A-7A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |  |  |  |
| DC_2A-7C-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |  |  |  |
| DC_2A-7A-66A_n28A | DC_2A_n28A DC_7A_n28A DC_66A_n28A |  |  |  |
| DC_2A-7A-66A_n38A | 2A5 66A5 |  |  |  |
| DC_2A-2A-7A-66A_n38A | 2A5 66A5 |  |  |
| DC_2A-7A-66A_n66A DC_2A-7C-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-7A-7A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |  |  |
| DC_2A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |  |  |
| DC_2A-7A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |  |  |
| DC_2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |  |  |  |
| DC_2A-2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |  |  |
| DC_2A-7A-66A_n77A DC_2A-7C-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |  |  |  |
| DC_2A-7A-66A_n77(2A) DC_2A-7C-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |  |  |
| DC_2A-7A-7A-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |  |  |
| DC_2A-7A-7A-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |  |  |
| DC_2A-7A_n66A-n77A DC_2A-7C_n66A-n77A DC_2A-7A-7A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A |  |  |  |
| DC_2A-7A-66A_n78A DC_2A-7C-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |  |
| DC_2A-2A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |  |
| DC_2A-7A_n66A-n78A DC_2A-7C_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |  |  |  |
| DC_2A-7A-66A_n78(2A) DC_2A-7C-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |
| DC_2A-7A-7A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |  |  |
| DC_2A-7A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |  |
| DC_2A-7A-66A-66A_n78A DC_2A-7C-66A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |
| DC_2A-7A-66A-66A_n78(2A) DC_2A-7C-66A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |
| DC_2A-7A-7A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |
| DC_2A-7A-7A-66A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |
| DC_2A-7A-7A-66A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |
| DC_2A-7A-71A_n2A | DC_7A_n2A DC_71A_n2A |  |  |  |
| DC_2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |  |  |  |
| DC_2A-2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |  |  |
| DC_2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |  |  |  |
| DC_2A-2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |  |  |
| DC_2A-7A_n71A-n78A | DC_2A_n71A DC_7A_n71A DC_2A_n78A DC_7A_n78A |  |  |
| DC_2A-12A_n2A-n78A | DC_12A_n2A DC_2A_n78A DC_7A_n78A |  |  |
| DC_2A-12A-30A_n2A | DC_12A_n2A DC_30A_n2A |  |  |  |
| DC_2A-12A-48A_n5A | DC_2A_n5A DC_12A_n5A DC_48A_n5A |  |  |  |
| DC_2A-12A-66A_n5A | DC_2A_n5A DC_12A_n5A DC_66A_n5A |  |  |  |
| DC_2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |  |  |  |
| DC_2A-2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |  |  |
| DC_2A-12A-30A_n77A9 DC_2A-2A-12A-30A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_30A_n77A9 |  |  |
| DC_2A-12A-66A_n2A | DC_12A_n2A DC_66A_n2A |  |  |  |
| DC_2A-12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |  |  |  |
| DC_2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |  |  |  |
| DC_2A-2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |  |  |
| DC_2A-12A-66A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |  |  |
| DC_2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |  |  |  |
| DC_2A-2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |  |  |
| DC_2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-12A-66A_n77A9 DC_2A-2A-12A-66A_n77A9 DC_2A-12A-66A-66A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_66A_n77A9 |  |  |
| DC_2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |  |  |  |
| DC_2A-2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |  |  |
| DC_2A-12A_n66A-n78A | DC_2A_n66A DC_12A_n66A DC_2A_n78A DC_12A_n78A |  |  |
| DC_2A-13A_n2A-n77A DC_2A-13A_n2A-n77C | DC_2A_n77A DC_13A_n2A DC_13A_n77A |  |  |  |
| DC_2A-13A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A |  |  |  |
| DC_2A-2A-13A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A |  |  |  |
| DC_2A-13A_n2A-n77A9 DC_2A-13A_n2A-n77C9 | DC_2A_n77A DC_13A_n77A |  |  |
| DC_2A-13A_n5A-n77A9 DC_2A-2A-13A_n5A-n77A9 DC_2A-13A_n5A-n77C9 | DC_2A_n77A DC_13A_n77A |  |  |
| DC_2A-13A_n25A-n66A8,14 | DC_2A_n66A DC_13A_n25A DC_13A_n66A |  |  |  |
| DC_2A-13A-48A_n77A7,8,9 DC_2A-13A-48A_n77C7,8,9 DC_2A-13A-48C_n77A7,8,9 DC_2A-13A-48C_n77C7,8,9 | DC_2A_n77A DC_13A_n77A |  |  |  |
| DC_2A-13A-66A_n2A | DC_13A_n2A |  |  |  |
| DC_2A-13A-66A-66A_n2A | DC_13A_n2A |  |  |  |
| DC_2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |  |
| DC_2A-2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |
| DC_2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |
| DC_2A-2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |
| DC_2A-13A-66A_n48A DC_2A-13A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |  |  |  |
| DC_2A-13A-66A-66A_n48A DC_2A-13A-66A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |  |  |  |
| DC_2A-13A-66A_n66A DC_2A-2A-13A-66A_n66A DC_2A-13A-66A-66A_n66A DC_2A-2A-13A-66A-66A_n66A | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-2A-13A-66A_n66A | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 |  |  |
| DC_2A-13A-66A-66A_n66A | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 |  |  |
| DC_2A-2A-13A-66A-66A_n66A | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 |  |  |
| DC_2A-13A-66B_n66A | DC_13A_n66A |  |  |  |
| DC_2A-13A-66A_n77A9 DC_2A-13A-66A_n77C9 DC_2A-2A-13A-66A_n77C9 DC_2A-2A-13A-66A-66A_n77A DC_2A-13A-66A-66A_n77C9 | DC_2A_n66A DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |  |  |  |
| DC_2A-2A-13A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |  |  |
| DC_2A-13A-66A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |  |  |
| DC_2A-13A_n66A-n77A9 DC_2A-13A_n66A-n77C9 DC_2A-2A-13A_n66A-n77A9 | DC_2A_n66A DC_2A_n77A9 DC_13A_n66A DC_13A_n77A9 |  |  |  |
| DC_2A-14A-30A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A |  |  |  |
| DC_2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |
| DC_2A-14A-30A_n77A9 DC_2A-2A-14A-30A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_30A_n77A9 |  |  |
| DC_2A-14A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |  |  |  |
| DC_2A-14A-66A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |  |  |  |
| DC_2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |  |  |  |
| DC_2A-2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |  |  |
| DC_2A-14A-66A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |  |  |
| DC_2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-14A-66A_n77A9 DC_2A-2A-14A-66A_n77A9 DC_2A-14A-66A-66A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_66A_n77A9 |  |  |
| DC_2A-28A-66A_n7A | DC_2A_n7A DC_28A_n7A DC_66A_n7A |  |  |  |
| DC_2A-28A-66A_n66A | DC_2A_n66A DC_28A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-29A-30A_n2A | DC_2A_n2A4 DC_30A_n2A |  |  |  |
| DC_2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |  |  |  |
| DC_2A-2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |  |  |
| DC_2A-29A-30A_n77A9 DC_2A-2A-29A-30A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 |  |  |
| DC_2A-29A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |  |  |  |
| DC_2A-29A-66A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |  |  |  |
| DC_2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |  |  |  |
| DC_2A-2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |  |  |
| DC_2A-29A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |  |  |
| DC_2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |  |  |
| DC_2A-29A-66A_n77A9 | DC_2A_n77A9 DC_66A_n77A9 |  |  |
| DC_2A-29A-66A_n78A | DC_2A_n78A DC_66A_n78A |  |  |  |
| DC_2A-30A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |  |  |  |
| DC_2A-30A-66A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |  |  |  |
| DC_2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |  |  |  |
| DC_2A-2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |  |  |
| DC_2A-30A-66A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |  |  |
| DC_2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |  |
| DC_2A-2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |
| DC_2A-30A-66A_n77A9 DC_2A-2A-30A-66A_n77A9 DC_2A-30A-66A-66A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |  |  |
| DC_2A-46A_n41A-n66A DC_2A-46C_n41A-n66A DC_2A-46D_n41A-n66A | DC_2A_n41A DC_2A_n66A |  |  |  |
| DC_2A-46A_n41A-n71A DC_2A-46C_n41A-n71A DC_2A-46D_n41A-n71A | DC_2A_n41A DC_2A_n71A |  |  |  |
| DC_2A-46A_n41(2A)-n71A DC_2A-46C_n41(2A)-n71A DC_2A-46D_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |  |  |  |
| DC_2A-46A-48A_n2A DC_2A-46C-48A_n2A DC_2A-46D-48A_n2A DC_2A-46E-48A_n2A | DC_2A_n2A4 DC_48A_n2A |  |  |  |
| DC_2A-46A-48A_n5A DC_2A-46C-48A_n5A DC_2A-46D-48A_n5A DC_2A-46E-48A_n5A | DC_2A_n5A DC_48A_n5A |  |  |  |
| DC_2A-46A-48A_n66A DC_2A-46C-48A_n66A DC_2A-46D-48A_n66A DC_2A-46E-48A_n66A | DC_2A_n66A DC_48A_n66A |  |  |  |
| DC_2A-46A-66A_n5A DC_2A-46C-66A_n5A DC_2A-46D-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |  |
| DC_2A-46A-66A_n41A DC_2A-46C-66A_n41A DC_2A-46D-66A_n41A | DC_2A_n41A DC_66A_n41A |  |  |  |
| DC_2A-46A-66A_n41(2A) DC_2A-46C-66A_n41(2A) DC_2A-46D-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |  |  |  |
| DC_2A-46A-66A_n71A DC_2A-46C-66A_n71A DC_2A-46D-66A_n71A | DC_2A_n71A DC_66A_n71A |  |  |  |
| DC_2A-48A-(n)5AA | DC_2A_n5A DC_48A_n5A DC_(n)5AA4 |  |  |  |
| DC_2A-46A_n66A-n71A DC_2A-46C_n66A-n71A DC_2A-46D_n66A-n71A | DC_2A_n66A DC_2A_n71A |  |  |  |
| DC_2A-48A_n48A-n66A | DC_2A_n48A DC_2A_n66A DC_48A_n66A |  |  |  |
| DC_2A-48A-66A_n2A DC_2A-48C-66A_n2A DC_2A-48D-66A_n2A DC_2A-48E-66A_n2A | DC_66A_n2A DC_48A_n2A DC_2A_n2A4 |  |  |  |
| DC_2A-48A-66A_n5A | DC_2A_n5A DC_48A_n5A DC_66A_n5A |  |  |  |
| DC_2A-48C-66A_n5A DC_2A-48D-66A_n5A DC_2A-48E-66A_n5A | DC_2A_n5A DC_66A_n5A |  |  |  |
| DC_2A-48A-66A_n12A | DC_2A_n12A DC_48A_n12A DC_66A_n12A |  |  |  |
| DC_2A-48A-66A_n66A DC_2A-48C-66A_n66A DC_2A-48D-66A_n66A DC_2A-48E-66A_n66A | DC_66A_n66A4 DC_48A_n66A DC_2A_n66A |  |  |  |
| DC_2A-48A-66A_n71A | DC_2A_n71A DC_48A_n71A DC_66A_n71A |  |  |  |
| DC_2A-48A-66A_n77A7,8,9 DC_2A-48C-66A_n77A7,8,9 DC_2A-48A-66A_n77C7,8,9 DC_2A-48C-66A_n77C7,8,9 DC_2A-48D-66A_n77A7,8,9 DC_2A-48E-66A_n77A7,8,9 | DC_2A_n77A DC_66A_n77A |  |  |  |
| DC_2A-66A_n2A-n77A DC_2A-66A_n2A-n77C | DC_2A_n77A DC_66A_n2A DC_66A_n77A |  |  |  |
| DC_2A-66A-66A_n2A-n77A | DC_2A_n77A DC_66A_n2A DC_66A_n77A |  |  |  |
| DC_2A-66A-(n)5AA | DC_2A_n5A DC_66A_n5A DC_(n)5AA4 |  |  |  |
| DC_2A-66A_n2A-n77A9 DC_2A-66A-66A_n2A-n77A9 DC_2A-66A_n2A-n77C9 | DC_2A_n77A DC_66A_n77A |  |  |
| DC_2A-66A_n2A-n78A | DC_66A_n2A DC_2A_n78A DC_66A_n78A |  |  |
| DC_2A-66A_n5A-n77A9 DC_2A-2A-66A_n5A-n77A9 DC_2A-66A-66A_n5A-n77A9 DC_2A-66A_n5A-n77C9 | DC_2A_n5A DC_2A_n77A9 DC_5A_n77A DC_66A_n5A DC_66A_n77A9 |  |  |  |
| DC_2A-66A_n25A-n66A7,8 | DC_2A_n66A DC_66A_n25A |  |  |  |
| DC_2A-66A_n38A-n78A | DC_2A_n38A DC_2A_n78A DC_66A_n38A DC_66A_n78A |  |  |  |
| DC_2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |  |  |  |
| DC_2A-2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |  |  |
| DC_2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |  |  |  |
| DC_2A-2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |  |  |
| DC_2A-66A-71A_n66A | DC_2A_n66A DC_66A_n66A4 DC_71A_n66A |  |  |  |
| DC_2A-66A-71A_n71A | DC_2A_n71A DC_66A_n71A |  |  |  |
| DC_2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |  |  |  |
| DC_2A-2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |  |  |
| DC_2A-66A-(n)71AA DC_2A-66C-(n)71AA | DC_2A_n71A DC_66A_n71A DC_(n)71AA |  |  |  |
| DC_2A-66A_n41A-n71A DC_2A-66A_n41C-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |  |  |  |
| DC_2A-66A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |  |  |  |
| DC_2A-66A_n66A-n77A9 DC_2A-2A-66A_n66A-n77A9 DC_2A-66A_n66A-n77C9 | DC_2A_n66A DC_2A_n77A9 DC_66A_n77A9 |  |  |  |
| DC_2A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_66A_n66A4 |  |  |  |
| DC_2A-66A-71A_n2A | DC_66A_n2A DC_71A_n2A |  |  |  |
| DC_2A-66A_n71A-n78A | DC_2A_n71A DC_66A_n71A DC_2A_n78A DC_66A_n78A |  |  |  |
| DC_2A-71A_n2A-n78A | DC_71A_n2A DC_2A_n78A DC_7A_n78A |  |  |  |
| DC_2A-71A_n66A-n78A | DC_2A_n66A DC_71A_n66A DC_2A_n78A DC_71A_n78A |  |  |  |
| DC_3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |  |  |  |
| DC_3A-3A_n1A-n8A-n78A | DC_3A_n1A DC_3A_n8A DC_3A_n78A |  | Yes |  |
| DC_3A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A |  |  |  |
| DC_3A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A |  |  |  |
| DC_3A-5A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |  |  |  |
| DC_3A-5A-7A_n77(2A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_3A-5A-7A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_3A-5A-7A-7A_n77(2A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_3A-5A-7A_n78A DC_3C-5A-7A_n78A DC_3A-5A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |  |  |  |
| DC_3A-5A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_3A-5A-7A-7A_n78A DC_3A-5A-7A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_3A-5A-7A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |  |  |  |
| DC_3A-3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |  |  |
| DC_3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |  |  |
| DC_3A-3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |  |  |
| DC_3A-7A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A |  |  |  |
| DC_3A-7A_n1A-n78A DC_3C-7A_n1A-n78A | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A DC_7A_n1A DC_7A_n78A |  | Yes |  |
| DC_3A-3A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A | Yes |  |
| DC_3A-7A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A | Yes |  |
| DC_3A-3A-7A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A | Yes |  |
| DC_3A-7C_n1A-n78A DC_3C-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |  |  |  |
| DC_3A-5A-41A_n79A | DC_3A_n79A DC_5A_n79A DC_41A_n79A |  |  |  |
| DC_3A-7A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_3A_n78A DC_7A_n78A |  |  |  |
| DC_3A-7C_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_3A_n78A DC_7C_n78A DC_7A_n78A |  |  |  |
| DC_3A-7A_n5A-n78A9 DC_3A-7C_n5A-n78A9 DC_3C-7A_n5A-n78A9 DC_3C-7C_n5A-n78A9 | DC_3A_n5A DC_3C_n5A DC_3A_n78A9 DC_3C_n78A9 DC_7A_n5A DC_7C_n5A DC_7A_n78A9 DC_7C_n78A9 |  |  |  |
| DC_3A-7A_n7A-n78A | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |  | Yes |  |
| DC_3A-3A-7A_n7A-n78A | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A | Yes |  |
| DC_3C-7A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_3A_n78A DC_3C_n78A DC_7A_n78A |  |  |  |
| DC_3A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |  |  |  |
| DC_3A-3A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |  |  |  |
| DC_3A-7A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |  |  |
| DC_3A-3A-7A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |  |  |
| DC_3A-7A-8A_n28A | DC_3A_n28A DC_7A_n28A DC_8A_n28A |  |  |  |
| DC_3A-7A-8A_n40A | DC_3A_n40A DC_7A_n40A DC_8A_n40A |  |  |  |
| DC_3A-7A-8A_n77A | DC_3A_n77A DC_7A_n77A DC_8A_n77A |  | Yes |  |
| DC_3A-7A-8A_n78A | DC_3A_n78A, DC_7A_n78A, DC_8A_n78A |  | Yes |  |
| DC_3A-7A-8A_n78(2A) | DC_3A_n78A, DC_7A_n78A, DC_8A_n78A |  |  |
| DC_3A-3A-7A-8A_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A |  | Yes |  |
| DC_3A-7A-7A-8A_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A | Yes |  |
| DC_3A-3A-7A-7A-8A_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A | Yes |  |
| DC_3A-7A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |  | Yes |  |
| DC_3A-3A-7A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A | Yes |  |
| DC_3A-7A-7A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A | Yes |  |
| DC_3A-3A-7A-7A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A | Yes |  |
| DC_3A-7A-20A_n1A DC_3C-7A-20A_n1A DC_3A-7C-20A_n1A DC_3C-7C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A DC_20A_n1A |  |  |  |
| DC_3A-7A-20A_n8A | DC_3A_n8A DC_7A_n8A DC_20A_n8A |  |  |  |
| DC_3A-7A-20A_n28A3,8,14 DC_3C-7A-20A_n28A3 | DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_20A_n28A |  |  |  |
| DC_3A-7A-20A_n38A12,13 | CA_3A-20A |  |  |
| DC_3A-7A-20A_n78A DC_3C-7A-20A_n78A | DC_3A_n78A DC_3C_n78A DC_20A_n78A DC_7A_n78A |  | Yes |  |
| DC_3A-7A-28A_n1A DC_3C-7A-28A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_28A_n1A |  |  |  |
| DC_3A-7A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |  |  |
| DC_3A-7A-28A_n3A DC_3A-7C-28A_n3A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |  |  |  |
| DC_3A-7A-28A_n5A DC_3A-7C-28A_n5A DC_3C-7A-28A_n5A DC_3C-7C-28A_n5A | DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |  |  |  |
| DC_3A-7A-28A_n7A DC_3C-7A-28A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |  |  |  |
| DC_3A-3A-7A-28A_n7A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |  |  |  |
| DC_3A-7A-28A_n40A | DC_3A_n40A DC_7A_n40A DC_28A_n40A |  |  |  |
| DC_3A-7A-28A_n78A9 DC_3A-7C-28A_n78A9 | DC_3A_n78A9 DC_7A_n78A9 DC_7C_n78A9 DC_28A_n78A9 |  | Yes |  |
| DC_3C-7A-28A_n78A9 DC_3C-7C-28A_n78A9 | DC_3A_n78A9 DC_3C_n78A9 DC_7A_n78A9 DC_7C_n78A9 DC_28A_n78A9 |  |  |  |
| DC_3A-7A_n28A-n78A9 | DC_3A_n28A DC_3A_n78A9 DC_7A_n28A DC_7A_n78A9 |  | Yes |  |
| DC_3A-7C_n28A-n78A9 DC_3C-7A_n28A-n78A9 DC_3C-7C_n28A-n78A9 | DC_3A_n28A DC_3A_n78A9 DC_3C_n28A9 DC_7A_n28A DC_7A_n78A9 DC_7C_n28A DC_7C_n78A9 |  |  |  |
| DC_3A-7A-32A_n1A | DC_3A_n1A DC_7A_n1A |  |  |  |
| DC_3A-7A-32A_n28A DC_3C-7A-32A_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A |  |  |
| DC_3A-7A-32A_n78A DC_3C-7A-32A_n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A |  |  |  |
| DC_3A-7A-38A_n28A10 DC_3C-7A-38A_n28A10 | DC_3A_n28A DC_3C_n28A |  |  |  |
| DC_3A-7A-40A_n1A DC_3A-7A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_40A_n1A |  |  |  |
| DC_3A-7A-40A_n78A DC_3A-7A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_40A_n78A |  |  |  |
| DC_3A-7A-40A_n78(2A) DC_3A-7A-40C_n78(2A) | DC_3A_n78A DC_7A_n78A DC_40A_n78A |  |  |  |
| DC_3A-7A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |  |  |  |
| DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_7A_n78A DC_7A_n80A |  |  |  |
| DC_3A-8A_n1A-n28A | DC_3A_n1A DC_8A_n1A DC_3A_n28A DC_8A_n28A |  |  |  |
| DC_3A-8A_n1A-n40A | DC_3A_n1A DC_8A_n1A DC_3A_n40A DC_8A_n40A |  |  |  |
| DC_3A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A |  | Yes |  |
| DC_3A-3A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A | Yes |  |
| DC_3A-8A-11A_n28A | DC_3A_n28A DC_8A_n28A DC_11A_n28A |  |  |  |
| DC_3A-8A-11A_n77A | DC_3A_n77A DC_8A_n77A DC_11A_n77A |  | Yes |  |
| DC_3A-8A-11A_n77(2A) DC_3A-8A-11A_n77(3A) | DC_3A_n77A DC_8A_n77A DC_11A_n77A |  | Yes |  |
| DC_3A-8A-20A_n1A | DC_3A_n1A DC_8A_n1A DC_20A_n1A |  |  |  |
| DC_3A-8A-20A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A |  |  |  |
| DC_3A-8A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |  | Yes |  |
| DC_3A-8A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |  | Yes |  |
| DC_3A-8A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_28A_n78A |  |  |  |
| DC_3A-8A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_8A_n28A DC_8A_n78A |  | Yes |  |
| DC_3A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_8A_n40A DC_8A_n78A |  |  |  |
| DC_3A-8A-40A_n1A DC_3A-8A-40C_n1A | DC_3A_n1A DC_8A_n1A DC_40A_n1A |  |  |  |
| DC_3A-8A-40A_n78A DC_3A-8A-40C_n78A | DC_3A_n78A DC_8A_n78A DC_40A_n78A |  |  |  |
| DC_3A-8A-40A_n78(2A) DC_3A-8A-40C_n78(2A) | DC_3A_n78A DC_8A_n78A DC_40A_n78A |  |  |  |
| DC_3A-8A-42A_n77A7,8 DC_3A-8A-42C_n77A7,8 | DC_3A_n77A DC_8A_n77A |  |  |  |
| DC_3A-8A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A |  |  |  |
| DC_3A-8A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_8A_n78A DC_8A_n80A |  |  |  |
| DC_3A-11A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |  | Yes |  |
| DC_3A-11A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |  | Yes |  |
| DC_3A-18A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_18A_n3A DC_18A_n41A |  |  |  |
| DC_3A-18A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_18A_n3A DC_18A_n77A |  |  |  |
| DC_3A-18A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_18A_n3A DC_18A_n78A |  |  |  |
| DC_3A-18A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |  |  |  |
| DC_3A-18A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |  |  |  |
| DC_3A-18A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |  |  |  |
| DC_3A-18A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |  |  |  |
| DC_3A-18A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |  |  |  |
| DC_3A-18A-42A_n77A7,8 DC_3A-18A-42C_n77A7,8 | DC_3A_n77A DC_18A_n77A |  |  |  |
| DC_3A-18A-42A_n78A7,8 DC_3A-18A-42C_n78A7,8 | DC_3A_n78A DC_18A_n78A |  |  |  |
| DC_3A-18A-42A_n79A DC_3A-18A-42C_n79A | DC_3A_n79A DC_18A_n79A |  |  |  |
| DC_3A-19A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A |  | Yes |  |
| DC_3A-19A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A |  | Yes |  |
| DC_3A-19A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A |  | Yes |  |
| DC_3A-19A-21A_n77A DC_3A-19A-21A_n77C | DC_3A_n77A DC_19A_n77A DC_21A_n77A |  | Yes |  |
| DC_3A-19A-21A_n78A DC_3A-19A-21A_n78C | DC_3A_n78A DC_19A_n78A DC_21A_n78A |  | Yes |  |
| DC_3A-19A-21A_n79A DC_3A-19A-21A_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A |  | Yes |  |
| DC_3A-19A-42A_n1A DC_3A-19A-42C_n1A | DC_3A_n1A DC_19A_n1A DC_42A_n1A |  | Yes |  |
| DC_3A-19A-42A_n77A7,8 DC_3A-19A-42A_n77C7,8 DC_3A-19A-42C_n77A7,8 DC_3A-19A-42C_n77C7,8 DC_3A-19A-42D_n77A7,8 DC_3A-19A-42D_n77C7,8 | DC_3A_n77A DC_19A_n77A |  |  |  |
| DC_3A-19A-42A_n78A7,8 DC_3A-19A-42A_n78C7,8 DC_3A-19A-42C_n78A7,8 DC_3A-19A-42C_n78C7,8 DC_3A-19A-42D_n78A7,8 DC_3A-19A-42D_n78C7,8 | DC_3A_n78A DC_19A_n78A |  |  |  |
| DC_3A-19A-42A_n79A DC_3A-19A-42A_n79C DC_3A-19A-42C_n79A DC_3A-19A-42C_n79C | DC_3A_n79A DC_19A_n79A |  | Yes |  |
| DC_3A-19A-42D_n79A DC_3A-19A-42D_n79C | DC_3A_n79A DC_19A_n79A |  |  |  |
| DC_3A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |  |  |  |
| DC_3A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |  |  |  |
| DC_3A-20A_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_20A_n1A DC_20A_n7A |  |  |  |
| DC_3C-20A_n1A-n7A | DC_3A_n1A DC_3C_n1A DC_3A_n7A DC_3C_n7A DC_20A_n1A DC_20A_n7A |  |  |  |
| DC_3A-20A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A |  |  |  |
| DC_3A-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A |  |  |  |
| DC_3C-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_3C_n1A DC_3C_n28A DC_20A_n28A |  |  |  |
| DC_3A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A |  |  |  |
| DC_3C-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_3C_n1A DC_3C_n78A |  |  |  |
| DC_3A-20A_n7A-n28A8,14 | DC_3A_n7A DC_3A_n28A DC_20A_n7A DC_20A_n28A |  |  |  |
| DC_3A-20A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_20A_n8A DC_20A_n78A |  |  |  |
| DC_3A-20A-28A_n1A | DC_3A_n1A DC_20A_n1A DC_28A_n1A |  |  |  |
| DC_3A-20A_n28A-n75A | DC_3A_n28A DC_20A_n28A |  |  |  |
| DC_3C-20A_n28A-n75A | DC_20A_n28A DC_3A_n28A DC_3C_n28A |  |  |  |
| DC_3A-20A-28A_n78A | DC_3A_n78A DC_20A_n78A DC_28A_n78A |  |  |  |
| DC_3A-20A_n28A-n78A3,8,14 DC_3C-20A_n28A-n78A3,8,14 | DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |  | Yes |  |
| DC_3A-20A-32A_n1A DC_3C-20A-32A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |  |  |  |
| DC_3A-20A-32A_n28A8,14 DC_3C-20A-32A_n28A8,14 | DC_3A_n28A DC_3C_n28A DC_20A_n28A |  |  |
| DC_3A-20A-32A_n78A | DC_3A_n78A DC_20A_n78A |  |  |  |
| DC_3A-20A-38A_n78A | DC_3A_n78A DC_20A_n78A |  |  |  |
| DC_3A-20A_n38A-n78A | DC_3A_n78A DC_20A_n78A DC_3A_n38A DC_20A_n38A |  |  |  |
| DC_3A-20A-40A_n78A DC_3A-20A-40C_n78A | DC_3A_n78A DC_20A_n78A DC_40A_n78A |  |  |  |
| DC_3A-20A-40A_n78(2A) DC_3A-20A-40C_n78(2A) | DC_3A_n78A DC_20A_n78A DC_40A_n78A |  |  |  |
| DC_3A-20A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |  |  |  |
| DC_3A-20A_SUL_n78A-n80A DC_3C-20A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_20A_n78A DC_20A_n80A |  |  |  |
| DC_3A-21A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_21A_n28A DC_21A_n77A |  |  |  |
| DC_3A-21A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_21A_n28A DC_21A_n78A |  |  |  |
| DC_3A-21A_n28A-n79A | DC_3A_n28A DC_3A_n79A DC_21A_n28A DC_21A_n79A |  | Yes |  |
| DC_3A-21A-42A_n1A DC_3A-21A-42C_n1A | DC_3A_n1A DC_21A_n1A DC_42A_n1A |  | Yes |  |
| DC_3A-21A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A |  | Yes |  |
| DC_3A-21A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A |  | Yes |  |
| DC_3A-21A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A |  | Yes |  |
| DC_3A-21A-42A_n77A7,8 DC_3A-21A-42A_n77C7,8 DC_3A-21A-42C_n77A7,8 DC_3A-21A-42C_n77C7,8 DC_3A-21A-42D_n77A7,8 DC_3A-21A-42D_n77C7,8 | DC_3A_n77A DC_21A_n77A |  |  |  |
| DC_3A-21A-42A_n78A7,8 DC_3A-21A-42A_n78C7,8 DC_3A-21A-42C_n78A7,8 DC_3A-21A-42C_n78C7,8 DC_3A-21A-42D_n78A7,8 DC_3A-21A-42D_n78C7,8 | DC_3A_n78A DC_21A_n78A |  |  |  |
| DC_3A-21A-42A_n79A DC_3A-21A-42A_n79C DC_3A-21A-42C_n79A DC_3A-21A-42C_n79C DC_3A-21A-42D_n79A DC_3A-21A-42D_n79C | DC_3A_n79A DC_21A_n79A |  |  |  |
| DC_3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_21A_n77A DC_21A_n79A |  |  |  |
| DC_3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_21A_n78A DC_21A_n79A |  |  |  |
| DC_3A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_28A_n1A DC_28A_n40A |  |  |  |
| DC_3A-28A_n1A-n78A | DC_3A_n1A DC_28A_n1A DC_3A_n78A DC_28A_n78A |  | Yes |  |
| DC_3A-28A_n3A-n78A | DC_3A_n3A4 DC_28A_n3A DC_3A_n78A DC_28A_n78A |  | Yes |  |
| DC_3A-28A_n5A-n78A DC_3C-28A_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |  | Yes |  |
| DC_3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |  |  |  |
| DC_3A-3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |  |  |
| DC_3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |  |  |  |
| DC_3A-3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |  |  |
| DC_3C-28A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_28A_n78A |  |  |  |
| DC_3C-28A_n7B-n78A | DC_3A_n7A DC_3C_n7A DC_3A_n7B DC_3C-n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_3C_n78A DC_28A_n78A |  |  |  |
| DC_3A-28A-32A_n1A | DC_3A_n1A DC_28A_n1A |  |  |
| DC_3A-28A-40A_n78A | DC_3A_n78A DC_28A_n78A DC_40A_n78A |  |  |  |
| DC_3A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |  |  |  |
| DC_3A-28A-41A_n78A DC_3A-28A-41C_n78A | DC_3A_n78A DC_28A_n78A DC_41A_n78A DC_41C_n78A |  |  |  |
| DC_3A-28A-42A_n77A7,8 DC_3A-28A-42C_n77A7,8 | DC_3A_n77A DC_28A_n77A |  |  |  |
| DC_3A-28A-42A_n78A7,8 DC_3A-28A-42C_n78A7,8 | DC_3A_n78A DC_28A_n78A |  |  |  |
| DC_3A-28A-42A_n79A DC_3A-28A-42C_n79A | DC_3A_n79A DC_28A_n79A |  |  |  |
| DC_3A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A |  |  |  |
| DC_3A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A |  |  |  |
| DC_3A-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A |  |  |  |
| DC_3C-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_3C_n28A |  |  |  |
| DC_3A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |  |  |  |
| DC_3A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |  |  |  |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |  |  |  |
| DC_3A-41A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A |  |  |  |
| DC_3A-41C_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |  |  |  |
| DC_3A-41A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A |  |  |  |
| DC_3A-41C_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |  |  |  |
| DC_3A-41A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_41A_n28A |  |  |  |
| DC_3A-41A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |  |  |  |
| DC_3A-41C_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |  |  |  |
| DC_3A-41A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |  |  |  |
| DC_3A-41C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |  |  |  |
| DC_3A-41A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_41A_n77A |  |  |  |
| DC_3A-41A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_41A_n78A |  |  |  |
| DC_3A-41A-42A_n77A7,8 DC_3A-41A-42C_n77A7,8 DC_3A-41C-42A_n77A7,8 DC_3A-41C-42C_n77A7,8 | DC_3A_n77A DC_41A_n77A |  |  |  |
| DC_3A-41A-42A_n77(2A)7,8 DC_3A-41A-42C_n77(2A)7,8 | DC_3A_n77A DC_41A_n77A |  |  |  |
| DC_3A-41A-42A_n78A7,8 DC_3A-41A-42C_n78A7,8 DC_3A-41C-42A_n78A7,8 DC_3A-41C-42C_n78A7,8 | DC_3A_n78A DC_41A_n78A |  |  |  |
| DC_3A-41A-42A_n79A DC_3A-41A-42C_n79A DC_3A-41C-42A_n79A DC_3A-41C-42C_n79A | DC_3A_n79A DC_41A_n79A |  |  |  |
| DC_3A-42A_n1A-n77A7,8 DC_3A-42C_n1A-n77A7,8 | DC_3A_n1A DC_3A_n77A |  |  |  |
| DC_3A-42A_n1A-n78A7,8 DC_3A-42C_n1A-n78A7,8 | DC_3A_n1A DC_3A_n78A |  |  |  |
| DC_3A-42A_n1A-n79A DC_3A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A |  |  |  |
| DC_3A-42A_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |  |  |  |
| DC_3A-42A_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |  |  |  |
| DC_3A-42C_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |  |  |  |
| DC_3A-42C_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |  |  |  |
| DC_3A-42A_n77A-n79A7,8 DC_3A-42C_n77A-n79A7,8 | DC_3A_n77A DC_3A_n79A |  |  |  |
| DC_3A-42A_n78A-n79A7,8 DC_3A-42C_n78A-n79A7,8 | DC_3A_n78A DC_3A_n79A |  |  |  |
| DC_5A-7A_n2A-n78A | DC_5A_n2A DC_7A_n2A DC_5A_n78A DC_7A_n78A |  |  |  |
| DC_5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |  |  |  |
| DC_5A-7A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |  |  |  |
| DC_5A-7A-66A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |  |  |
| DC_5A-7A-66A_n66A DC_5A-7C-66A_n66A DC_5A-7A-7A-66A_n66A | DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |  |  |  |
| DC_5A-7A-7A-66A_n66A | DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |  |  |
| DC_5A-7A-66A_n78A DC_5A-7C-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |  |  |
| DC_5A-7A-66A-66A_n78A DC_5A-7C-66A-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |  |  |
| DC_5A-7A_n66A-n78A | DC_5A_n66A DC_7A_n66A DC_5A_n78A DC_7A_n78A |  |  |
| DC_5A-30A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A |  |  |  |
| DC_5A-30A-66A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A |  |  |
| DC_5A-30A-66A_n66A | DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |  |
| DC_5A-30A-66A_n77A9 DC_5A-30A-66A-66A_n77A9 | DC_5A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |  |  |
| DC_5A-48A-(n)12AA | DC_5A_n12A DC_48A_n12A DC_(n)12AA4 |  |  |  |
| DC_5A-48A-66A_n12A | DC_5A_n12A DC_48A_n12A DC_66A_n12A |  |  |  |
| DC_5A-48A-66A_n71A | DC_5A_n71A DC_48A_n71A DC_66A_n71A |  |  |  |
| DC_5A-66A_n2A-n77A DC_5A-66A_n2A-n77C | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |  |  |  |
| DC_5A-66A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |  |  |  |
| DC_5A-66A_n5A-n77A DC_5A-66A_n5A-n77C | DC_5A_n77A DC_66A_n5A DC_66A_n77A |  |  |  |
| DC_5A-66A-66A_n5A-n77A | DC_5A_n77A DC_66A_n5A DC_66A_n77A |  |  |  |
| DC_5A-48A-66A_n77A7,8,9 DC_5A-48C-66A_n77A7,8,9 DC_5A-48C-66A_n77C7,8,9 | DC_5A_n77A DC_66A_n77A |  |  |
| DC_5A-66A_n2A-n77A9 DC_5A-66A-66A_n2A-n77A9 DC_5A-66A_n2A-n77C9 | DC_5A_n77A9 DC_66A_n77A9 |  |  |
| DC_5A-66A_n2A-n78A | DC_5A_n2A DC_66A_n2A DC_5A_n78A DC_66A_n78A |  |  |
| DC_5A-66A_n5A-n77A9 DC_5A-66A-66A_n5A-n77A9 DC_5A-66A_n5A-n77C9 | DC_5A_n77A, DC_66A_n77A |  |  |
| DC_5A-66A-(n)12AA | DC_5A_n12A DC_66A_n12A DC_(n)12AA4 |  |  |  |
| DC_5A-66A_n66A-n77A9 DC_5A-66A_n66A-n77C9 | DC_5A_n66A DC_5A_n77A9 DC_66A_n77A9 |  |  |  |
| DC_7A_n1A-n8A-n78A | DC_7A_n1A DC_7A_n8A DC_7A_n78A |  | Yes |  |
| DC_7A-7A_n1A-n8A-n78A | DC_7A_n1A DC_7A_n8A DC_7A_n78A |  | Yes |  |
| DC_7A-8A_n1A-n40A | DC_7A_n1A DC_8A_n1A DC_7A_n40A DC_8A_n40A |  |  |  |
| DC_7A-8A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |  | Yes |  |
| DC_7A-7A-8A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A | Yes |  |
| DC_7A-8A-20A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |  |  |  |
| DC_7A-8A-20A_n3A | DC_7A_n3A DC_8A_n3A DC_20A_n3A |  |  |  |
| DC_7A-8A-32A_n1A | DC_7A_n1A DC_8A_n1A |  |  |  |
| DC_7A-8A-32A_n78A | DC_7A_n78A DC_8A_n78A |  |  |  |
| DC_7A-8A-38A_n1A | DC_8A_n1A |  |  |  |
| DC_7A-8A_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |  |  |  |
| DC_7A-8A-40A_n1A DC_7A-8A-40C_n1A | DC_7A_n1A DC_8A_n1A DC_40A_n1A |  |  |  |
| DC_7A-8A-40A_n78A DC_7A-8A-40C_n78A | DC_7A_n78A DC_8A_n78A DC_40A_n78A |  |  |  |
| DC_7A-8A-40A_n78(2A) DC_7A-8A-40C_n78(2A) | DC_7A_n78A DC_8A_n78A DC_40A_n78A |  |  |  |
| DC_7A-8A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A |  |  |  |
| DC_7A-12A_n2A-n78A | DC_7A_n2A DC_12A_n2A DC_7A_n78A DC_12A_n78A |  |  |  |
| DC_7A-12A-66A_n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |  |  |  |
| DC_7A-12A-66A_n78A | DC_7A_n78A DC_12A_n78A DC_66A_n78A |  |  |  |
| DC_7A-12A_n66A-n78A | DC_7A_n66A DC_12A_n66A DC_7A_n78A DC_12A_n78A |  |  |  |
| DC_7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |  |  |  |
| DC_7A-7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |  |  |  |
| DC_7C-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |  |  |  |
| DC_7A-13A-66A_n66A DC_7C-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |  |  |  |
| DC_7A-7A-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |  |  |  |
| DC_7A-20A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |  |  |  |
| DC_7A-20A_n3A-n38A | DC_20A_n3A |  |  |  |
| DC_7A-20A_n3A-n78A | DC_7A_n3A DC_20A_n3A DC_7A_n78A DC_20A_n78A |  |  |  |
| DC_7A-20A_n8A-n78A | DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |  |  |  |
| DC_7A-20A-28A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |  |  |  |
| DC_7A-20A-28A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |  |  |  |
| DC_7A-20A_n28A-n78A3 | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |  | Yes |  |
| DC_7A-20A-32A_n1A | DC_7A_n1A DC_20A_n1A |  |  |  |
| DC_7A-20A-32A_n3A | DC_7A_n3A DC_20A_n3A |  |  |  |
| DC_7A-20A-32A_n8A | DC_7A_n8A DC_20A_n8A |  |  |  |
| DC_7A-20A-32A_n28A | DC_7A_n28A DC_20A_n28A |  |  |  |
| DC_7A-20A-32A_n78A | DC_7A_n78A DC_20A_n78A |  |  |  |
| DC_7A-20A-38A_n1A | DC_20A_n1A |  |  |  |
| DC_7A-20A-38A_n3A | DC_20A_n3A |  |  |  |
| DC_7A-20A-38A_n8A | DC_20A_n8A |  |  |  |
| DC_7A-20A-38A_n78A10 | DC_20A_n78A |  |  |  |
| DC_7A-25A-66A_n77A DC_7C-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |  |  |  |
| DC_7A-7A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |  |  |
| DC_7A-25A-25A-66A_n77A DC_7C-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |  |  |
| DC_7A-7A-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |  |  |
| DC_7A-25A-66A_n78A DC_7C-25A-66A_n78A DC_7C-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |  |  |  |
| DC_7A-7A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |  |  |
| DC_7A-25A-25A-66A_n78A DC_7C-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |  |  |
| DC_7A-7A-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |  |  |
| DC_7A-28A_n1A-n40A | DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A |  |  |  |
| DC_7A-28A_n1A-n78A | DC_7A_n1A DC_28A_n1A DC_7A_n78A DC_28A_n78A |  |  |  |
| DC_7A-28A_n3A-n78A | DC_7A_n3A DC_28A_n3A DC_7A_n78A DC_28A_n78A |  |  |  |
| DC_7C-28A_n3A-n78A | DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_7A_n78A DC_7C_n78A DC_28A_n78A |  |  |  |
| DC_7A-28A_n5A-n78A DC_7C-28A_n5A-n78A | DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |  |  |  |
| DC_7A-28A_n7A-n78A | DC_7A_n7A4 DC_28A_n7A DC_7A_n78A DC_28A_n78A |  |  |  |
| DC_7A-28A-32A_n1A | DC_7A_n1A DC_28A_n1A |  |  |  |
| DC_7A-28A-32A_n3A | DC_7A_n3A DC_28A_n3A |  |  |  |
| DC_7A-28A-38A_n1A | DC_28A_n1A |  |  |  |
| DC_7A-28A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |  |  |  |
| DC_7A-66A_n38A-n78A DC_7C-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |  |  |  |
| DC_7A-7A-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |  |  |
| DC_7A-28A-66A_n7A | DC_7A_n7A4 DC_78A_n7A DC_66A_n7A |  |  |  |
| DC_7A-28A-66A_n66A DC_7C-28A-66A_n66A | DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |  |  |  |
| DC_7A-29A-66A_n78A DC_7C-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |  |  |  |
| DC_7A-7A-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |  |  |  |
| DC_7A-38A_n3A-n78A10 | N/A |  |  |  |
| DC_7A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |  |  |  |
| DC_7A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |  |  |  |
| DC_7A-66A_n2A-n78A | DC_7A_n2A DC_66A_n2A DC_7A_n78A DC_66A_n78A |  |  |  |
| DC_7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |  |  |  |
| DC_7A-7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |  |  |  |
| DC_7C-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |  |  |  |
| DC_7A-66A_n66A-n77A DC_7C-66A_n66A-n77A DC_7A-7A-66A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_66A_n77A |  |  |  |
| DC_7A-66A_n66A-n78A DC_7C-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |  |  |  |
| DC_7A-7A-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |  |  |
| DC_7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |  |  |  |
| DC_7A-66A-71A_n78A | DC_7A_n78A DC_66A_n78A DC_71A_n78A |  |  |  |
| DC_7A-66A_n71A-n78A | DC_7A_n71A DC_66A_n71A DC_7A_n78A DC_66A_n78A |  |  |  |
| DC_7A-71A_n2A-n78A | DC_7A_n2A DC_71A_n2A DC_7A_n78A DC_71A_n78A |  |  |  |
| DC_7A-71A_n66A-n78A | DC_7A_n66A DC_71A_n66A DC_7A_n78A DC_71A_n78A |  |  |  |
| DC_8A_n1A-n3A-n77A | DC_8A_n1A DC_8A_n3A DC_8A_n77A |  |  |  |
| DC_8A_n3A-n28A-n77A | DC_8A_n3A DC_8A_n28A DC_8A_n77A |  | Yes |  |
| DC_8A_n3A-n28A-n77(2A) | DC_8A_n3A DC_8A_n28A DC_8A_n77A |  | Yes |  |
| DC_8A_n3A-n28A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n79A |  |  |
| DC_8A_n3A-n77A-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |  |  |
| DC_8A_n3A-n77(2A)-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |  |  |  |
| DC_8A_n40A-n41A-n79A | DC_8A_n40A DC_8A_n41A DC_8A_n79A |  |  |  |
| DC_8A-11A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A |  |  |  |
| DC_8A-11A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |  | Yes |  |
| DC_8A-11A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |  | Yes |  |
| DC_8A-11A_n3A-n79A | DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A |  |  |  |
| DC_8A-11A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |  | Yes |  |
| DC_8A-11A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |  | Yes |  |
| DC_8A-11A_n77A-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |  |  |  |
| DC_8A-11A_n77(2A)-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |  |  |  |
| DC_8A-20A-28A_n78A | DC_8A_n78A DC_20A_n78A DC_28A_n78A |  |  |  |
| DC_8A-20A-32A_n1A | DC_8A_n1A DC_20A_n1A |  |  |  |
| DC_8A_n28A-n77A-n79A | DC_8A_n28A DC_8A_n77A DC_8A_n79A |  |  |  |
| DC_8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |  |  |  |
| DC_8A-32A-38A_n1A | DC_8A_n1A DC_38A_n1A |  |  |  |
| DC_8A_n39A-n40A-n41A | DC_8A_n39A DC_8A_n40A DC_8A_n41A |  |  |  |
| DC_8A_n39A-n40A-n79A | DC_8A_n39A DC_8A_n40A DC_8A_n79A |  |  |  |
| DC_8A-41A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |  |  |  |
| DC_8A-41C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |  |  |  |
| DC_8A-40A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |  |  |  |
| DC_8A-40C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |  |  |  |
| DC_8A-41A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41A_n77A |  |  |  |
| DC_8A-41C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41C_n3A DC_41A_n77A DC_41C_n77A |  |  |  |
| DC_8A-42A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42A_n3A |  |  |  |
| DC_8A-42C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42C_n1A DC_42A_n3A DC_42C_n3A |  |  |  |
| DC_8A-42A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A |  |  |  |
| DC_8A-42C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A DC_42C_n1A |  |  |  |
| DC_8A-42A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A |  | Yes |  |
| DC_8A-42C_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |  | Yes |  |
| DC_8A-42A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |  |  |  |
| DC_8A-42A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |  |  |  |
| DC_8A-42C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |  |  |  |
| DC_8A-42C_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |  |  |  |
| DC_8A-42A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A |  |  |  |
| DC_8A-42A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A |  |  |  |
| DC_8A-42C_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |  |  |  |
| DC_8A-42C_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |  |  |  |
| DC_12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |  |  |  |
| DC_12A-30A-66A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |  |  |
| DC_11A_n3A-n28A-n77A | DC_11A_n3A DC_11A_n28A DC_11A_n77A |  | Yes |  |
| DC_11A_n3A-n28A-n77(2A) | DC_11A_n3A DC_11A_n28A DC_11A_n77A |  | Yes |  |
| DC_11A_n3A-n77A-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |  |  |  |
| DC_11A_n3A-n77(2A)-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |  |  |  |
| DC_12A-30A-66A_n66A | DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |  |
| DC_12A-30A-66A_n77A9 DC_12A-30A-66A-66A_n77A9 | DC_12A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |  |  |
| DC_12A-48A-(n)5AA | DC_12A_n5A DC_48A_n5A DC_(n)5AA4 |  |  |  |
| DC_12A-48A-66A_n5A | DC_12A_n5A DC_48A_n5A DC_66A_n5A |  |  |  |
| DC_12A-66A-(n)5AA | DC_12A_n5A DC_66A_n5A DC_(n)5AA4 |  |  |  |
| DC_12A-66A_n2A-n78A | DC_12A_n2A DC_66A_n2A DC_12A_n78A DC_66A_n78A |  |  |  |
| DC_13A-48A-66A_n77A9 DC_13A-48C-66A_n77A9 DC_13A-48A-66A_n77C9 DC_13A-48C-66A_n77C9 | DC_13A_n77A DC_66A_n77A |  |  |  |
| DC_13A-66A_n2A-n77A9 DC_13A-66A-66A_n2A-n77A9 DC_13A-66A_n2A-n77C9 | DC_13A_n2A DC_13A_n77A9 DC_66A_n2A DC_66A_n77A9 |  |  |  |
| DC_13A-66A_n5A-n48A | DC_13A_n48A DC_66A_n5A DC_66A_n48A |  |  |  |
| DC_13A-66A_n5A-n77A DC_13A-66A_n5A-n77C | DC_13A_n77A DC_66A_n5A DC_66A_n77A |  |  |  |
| DC_13A-66A-66A_n5A-n77A | DC_13A_n77A DC_66A_n5A DC_66A_n77A |  |  |  |
| DC_13A-66A_n5A-n77A9 DC_13A-66A-66A_n5A-n77A9 DC_13A-66A_n5A-n77C9 DC_13A-66A-66A_n5A-n77C9 | DC_66A_n5A DC_13A_n77A DC_66A_n77A |  |  |
| DC_13A-66A_n66A-n77A9 DC_13A-66A_n66A-n77C | DC_13A_n66A DC_13A_n77A9 DC_66A_n77A9 |  |  |  |
| DC_14A-30A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |  |  |  |
| DC_14A-30A-66A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |  |  |
| DC_14A-30A-66A_n66A | DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |  |
| DC_14A-30A-66A_n77A9 DC_14A-30A-66A-66A_n77A9 | DC_14A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |  |  |
| DC_18A-41A_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A |  |  |  |
| DC_18A-41C_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |  |  |  |
| DC_18A-41A_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A |  |  |  |
| DC_18A-41C_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |  |  |  |
| DC_19A_n1A-n77A-n79A | DC_19A_n1A DC_19A_n77A DC_19A_n79A |  |  |  |
| DC_19A_n1A-n78A-n79A | DC_19A_n1A DC_19A_n78A DC_19A_n79A |  |  |  |
| DC_19A-21A_n1A-n77A | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A |  | Yes |  |
| DC_19A-21A_n1A-n78A | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A |  | Yes |  |
| DC_19A-21A_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A |  | Yes |  |
| DC_19A-21A-42A_n1A DC_19A-21A-42C_n1A | DC_19A_n1A DC_21A_n1A DC_42A_n1A |  | Yes |  |
| DC_19A-21A-42A_n77A DC_19A-21A-42A_n77C DC_19A-21A-42C_n77A DC_19A-21A-42C_n77C | DC_19A_n77A DC_21A_n77A |  |  |  |
| DC_19A-21A-42A_n78A DC_19A-21A-42A_n78C DC_19A-21A-42C_n78A DC_19A-21A-42C_n78C | DC_19A_n78A DC_21A_n78A |  |  |  |
| DC_19A-21A-42A_n79A DC_19A-21A-42A_n79C DC_19A-21A-42C_n79A DC_19A-21A-42C_n79C | DC_19A_n79A DC_21A_n79A |  |  |  |
| DC_19A-21A_n77A-n79A | DC_19A_n77A DC_19A_n79A |  |  |  |
| DC_19A-21A_n78A-n79A | DC_19A_n78A DC_19A_n79A |  |  |  |
| DC_19A-42A_n1A-n77A DC_19A-42C_n1A-n77A | DC_19A_n1A DC_19A_n77A |  |  |  |
| DC_19A-42A_n1A-n78A DC_19A-42C_n1A-n78A | DC_19A_n1A DC_19A_n78A |  |  |  |
| DC_19A-42A_n1A-n79A DC_19A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A |  |  |  |
| DC_19A-42A_n77A-n79A DC_19A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A |  |  |  |
| DC_19A-42A_n78A-n79A DC_19A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A |  |  |  |
| DC_20A-28A-32A_n1A | DC_20A_n1A DC_28A_n1A |  |  |  |
| DC_20A-28A-32A_n3A | DC_20A_n3A DC_28A_n3A |  |  |  |
| DC_20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |  |  |  |
| DC_20A-32A_n1A-n28A | DC_20A_n1A DC_20A_n28A |  |  |  |
| DC_20A-32A-38A_n1A | DC_20A_n1A DC_38A_n1A |  |  |  |
| DC_20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A DC_38A_n3A DC_38A_n78A |  |  |  |
| DC_21A_n1A-n77A-n79A | DC_21A_n1A DC_21A_n77A DC_21A_n79A |  |  |  |
| DC_21A_n1A-n78A-n79A | DC_21A_n1A DC_21A_n78A DC_21A_n79A |  |  |  |
| DC_21A-28A-42A_n77A DC_21A-28A-42C_n77A | DC_21A_n77A DC_28A_n77A |  |  |  |
| DC_21A-28A-42A_n78A DC_21A-28A-42C_n78A | DC_21A_n78A DC_28A_n78A |  |  |  |
| DC_21A-28A-42A_n79A DC_21A-28A-42C_n79A | DC_21A_n79A DC_28A_n79A |  |  |  |
| DC_21A_n28A-n77A-n79A | DC_21A_n28A DC_21A_n77A DC_21A_n79A |  |  |  |
| DC_21A_n28A-n78A-n79A | DC_21A_n28A DC_21A_n78A DC_21A_n79A |  |  |  |
| DC_21A-42A_n1A-n77A DC_21A-42C_n1A-n77A | DC_21A_n1A DC_21A_n77A |  |  |  |
| DC_21A-42A_n1A-n78A DC_21A-42C_n1A-n78A | DC_21A_n1A DC_21A_n78A |  |  |  |
| DC_21A-42A_n1A-n79A DC_21A-42C_n1A-n79A | DC_21A_n1A DC_21A_n79A |  |  |  |
| DC_21A-42A_n77A-n79A DC_21A-42C_n77A-n79A | DC_21A_n77A DC_21A_n79A |  |  |  |
| DC_21A-42A_n78A-n79A DC_21A-42C_n78A-n79A | DC_21A_n78A DC_21A_n79A |  |  |  |
| DC_28A-32A-38A_n1A | DC_28A_n1A DC_38A_n1A |  |  |  |
| DC_28A-41A-42A_n78A DC_28A-41C-42A_n78A DC_28A-41A-42C_n78A DC_28A-41C-42C_n78A | DC_28A_n78A DC_41A_n78A DC_41C_n78A |  |  |  |
| DC_29A-30A-66A_n2A | DC_30A_n2A DC_66A_n2A |  |  |  |
| DC_29A-30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |  |  |  |
| DC_29A-30A-66A_n66A | DC_30A_n66A DC_66A_n66A4 |  |  |  |
| DC_29A-30A-66A_n77A9 | DC_30A_n77A9 DC_66A_n77A9 |  |  |
| DC_42A_n1A-n77A-n79A7,8 | N/A |  |  |  |
| DC_42A_n1A-n78A-n79A7,8 | N/A |  |  |  |
| DC_42A_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42A_n28A |  |  |  |
| DC_42A_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42A_n28A |  |  |  |
| DC_42C_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |  |  |  |
| DC_42C_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |  |  |  |
| DC_46A-66A_n25A-n41A DC_46C-66A_n25A-n41A DC_46D-66A_n25A-n41A | DC_66A_n25A DC_66A_n41A |  |  |  |
| DC_46A-66A_n25A-n71A DC_46C-66A_n25A-n71A DC_46D-66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |  |  |  |
| DC_46A-66A_n41A-n71A DC_46C-66A_n41A-n71A DC_46D-66A_n41A-n71A | DC_66A_n41A DC_66A_n71A |  |  |  |
| DC_46A-66A_n41(2A)-n71A DC_46C-66A_n41(2A)-n71A DC_46D-66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |  |  |  |
| DC_48A-66A_n25A-n48A | DC_48A_n25A DC_66A_n25A DC_66A_n48A |  |  |  |
| DC_66A-71A_n2A-n78A | DC_66A_n2A DC_71A_n2A DC_66A_n78A DC_71A_n78A |  |  |  |
| OTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: UL carrier shall be supported in Band 2 or band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 6: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 8: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 9: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 10: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 11: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 12: The combination is not used alone as fall back mode of other band combinations. NOTE 13: Power imbalance between downlink carriers on Band 7 and band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |  |  |  |

#### 5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Mandatory simultaneous Rx/Tx |  |
| --- | --- | --- | --- |
| DC_1A-3A-5A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_1A-3A-5A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_1A-3A-5A-7A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_1A-3A-5A-7A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |  |  |
| DC_1A-3A-5A-7A_n78A DC_1A-3C-5A-7A_n78A DC_1A-3A-5A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_1A-3A-5A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_1A-3A-5A-7A-7A_n78A DC_1A-3A-5A-7A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_1A-3A-5A-7A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_1A-1A-3A-5A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |  |  |
| DC_1A-3A-5A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A DC_41A_n79A |  |  |
| DC_1A-3A-7A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A |  |  |
| DC_1A-3A-7C_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A |  |  |
| DC_1A-3A-7A_n5A-n78A DC_1A-3C-7A_n5A-n78A DC_1A-3A-7C_n5A-n78A DC_1A-3C-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A |  |  |
| DC_1A-3A-7A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_7A_n78A |  |  |
| DC_1A-3C-7A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |  |  |
| DC_1A-3A-7A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_8A_n28A |  |  |
| DC_1A-3A-7A-8A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |  |  |
| DC_1A-3A-7A-8A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |  |  |
| DC_1A-3A-7A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |  |  |
| DC_1A-3A-7A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A DC_20A_n8A |  |  |
| DC_1A-3A-7A-20A_n28A3 | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_20A_n28A |  |  |
| DC_1A-3A-7A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A | Yes |  |
| DC_1A-3A-7A-20A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A | Yes |  |
| DC_1A-3A-7A-20A_n38A7,10 | N/A |  |  |
| DC_1A-3A-7A-28A_n3A DC_1A-3A-7C-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |  |  |
| DC_1A-3A-7A-28A_n5A DC_1A-3C-7A-28A_n5A DC_1A-3A-7C-28A_n5A DC_1A-3C-7C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |  |  |
| DC_1A-3A-7A-28A_n7A DC_1A-3C-7A-28A_n7A DC_1A-3A-3A-7A-28A_n7A DC_1A-1A-3C-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |  |  |
| DC_1A-1A-3A-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |  |  |
| DC_1A-1A-3A-3A-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |  |  |
| DC_1A-3A-7A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A DC_28A_n40A |  |  |
| DC_1A-3A-7A-28A_n78A DC_1A-3A-7C-28A_n78A DC_1A-3C-7A-28A_n78A DC_1A-3C-7C-28A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |  |  |
| DC_1A-1A-3A-7A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_1A-3A-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A | Yes |  |
| DC_1A-3A-7C_n28A-n78A DC_1A-3C-7A_n28A-n78A DC_1A-3C-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |  |
| DC_1A-3A-7A-32A_n28A DC_1A-3C-7A-32A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_7A_n28A |  |  |
| DC_1A-3A-7A-32A_n78A DC_1A-3C-7A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |  |  |
| DC_1A-3A-7A-38A_n28A7 DC_1A-3C-7A-38A_n28A7 | DC_1A_n28A DC_3A_n28A DC_3C_n28A |  |  |
| DC_1A-3A-7A-40A_n78A DC_1A-3A-7A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_40A_n78A |  |  |
| DC_1A-3A-7A-40A_n78(2A) DC_1A-3A-7A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_40A_n78A |  |  |
| DC_1A-3A-8A-40A_n78A DC_1A-3A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_40A_n78A |  |  |
| DC_1A-3A-8A-40A_n78(2A) DC_1A-3A-8A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_40A_n78A |  |  |
| DC_1A-3A-7A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |  |  |
| DC_1A-3A-8A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A DC_11A_n28A |  |  |
| DC_1A-3A-8A-11A_n77A DC_1A-3A-8A-11A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_8A_n77A DC_11A_n77A | Yes |  |
| DC_1A-3A-8A-11A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_8A_n77A DC_11A_n77A | Yes |  |
| DC_1A-3A-8A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_20A_n78A |  |  |
| DC_1A-3A-8A_n28A-n77A DC_1A-3A-8A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A | Yes |  |
| DC_1A-3A-8A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A | Yes |  |
| DC_1A-3A-8A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_28A_n78A |  |  |
| DC_1A-3A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_8A_n28A DC_8A_n78A | Yes |  |
| DC_1A-3A-8A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A |  |  |
| DC_1A-3A-8A-42A_n77A DC_1A-3A-8A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_8A_n77A |  |  |
| DC_1A-3A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A |  |  |
| DC_1A-3A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-3A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-3A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A DC_3A_n41A DC_18A_n3A DC_18A_n41A |  |  |
| DC_1A-3A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A DC_18A_n3A DC_18A_n77A |  |  |
| DC_1A-3A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A DC_18A_n3A DC_18A_n78A |  |  |
| DC_1A-3A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |  |  |
| DC_1A-3A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A1 DC_3A_n77A DC_18A_n28A DC_18A_n77A |  |  |
| DC_1A-3A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A1 DC_3A_n77A DC_18A_n28A DC_18A_n77A |  |  |
| DC_1A-3A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A1 DC_3A_n78A DC_18A_n28A DC_18A_n78A |  |  |
| DC_1A-3A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_3A_n28A1 DC_3A_n78A DC_18A_n28A DC_18A_n78A |  |  |
| DC_1A-3A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |  |  |
| DC_1A-3A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |  |  |
| DC_1A-3A-18A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |  |  |
| DC_1A-3A-18A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |  |  |
| DC_1A-3A-18A-42A_n77A DC_1A-3A-18A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |  |  |
| DC_1A-3A-18A-42A_n78A DC_1A-3A-18A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |  |  |
| DC_1A-3A-18A-42A_n79A DC_1A-3A-18A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |  |  |
| DC_1A-3A-19A-21A_n77A DC_1A-3A-19A-21A_n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A DC_21A_n77A | Yes |  |
| DC_1A-3A-19A-21A_n78A DC_1A-3A-19A-21A_n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78A DC_21A_n78A | Yes |  |
| DC_1A-3A-19A-21A_n79A DC_1A-3A-19A-21A_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_21A_n79A | Yes |  |
| DC_1A-3A-19A-42A_n77A5,6 DC_1A-3A-19A-42A_n77C5,6 DC_1A-3A-19A-42C_n77A5,6 DC_1A-3A-19A-42C_n77C5,6 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |  |  |
| DC_1A-3A-19A-42A_n78A5,6 DC_1A-3A-19A-42A_n78C5,6 DC_1A-3A-19A-42C_n78A5,6 DC_1A-3A-19A-42C_n78C5,6 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |  |  |
| DC_1A-3A-19A-42A_n79A DC_1A-3A-19A-42A_n79C DC_1A-3A-19A-42C_n79A DC_1A-3A-19A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A |  |  |
| DC_1A-3A-20A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_20A_n7A DC_1A_n78A DC_3A_n78A DC_20A_n78A |  |  |
| DC_1A-3A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_20A_n8A DC_20A_n78A |  |  |
| DC_1A-3A-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_20A_n28A |  |  |
| DC_1A-3C-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_20A_n28A |  |  |
| DC_1A-3A-20A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_28A_n78A |  |  |
| DC_1A-3A-20A_n28A-n78A3,6,11 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A | Yes |  |
| DC_1A-3A-20A-32A_n28A6,11 DC_1A-3C-20A-32A_n28A,6,11 | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_20A_n28A |  |  |
| DC_1A-3A-20A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |  |  |
| DC_1A-3A-20A-38A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |  |  |
| DC_1A-3A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_1A_n38A DC_3A_n38A DC_20A_n38A |  |  |
| DC_1A-3A-20A-38A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_20A_n78A |  |  |
| DC_1A-3A-20A-40A_n78A DC_1A-3A-20A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_40A_n78A |  |  |
| DC_1A-3A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |  |  |
| DC_1A-3A-21A-42A_n77A5,6 DC_1A-3A-21A-42A_n77C5,6 DC_1A-3A-21A-42C_n77A5,6 DC_1A-3A-21A-42C_n77C5,6 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |  |  |
| DC_1A-3A-21A-42A_n78A5,6 DC_1A-3A-21A-42A_n78C5,6 DC_1A-3A-21A-42C_n78A5,6 DC_1A-3A-21A-42C_n78C5,6 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |  |  |
| DC_1A-3A-21A-42A_n79A DC_1A-3A-21A-42A_n79C DC_1A-3A-21A-42C_n79A DC_1A-3A-21A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_21A_n79A |  |  |
| DC_1A-3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A |  |  |
| DC_1A-3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A |  |  |
| DC_1A-3A-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_28A_n78A | Yes |  |
| DC_1A-3A-28A_n5A-n78A DC_1A-3C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A | Yes |  |
| DC_1A-3A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_28A_n78A |  |  |
| DC_1A-3A-28A_n7B-n78A | DC_1A_n7A DC_3A_n7A DC_28A_n7A DC_1A_n7B DC_3A_n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_28A_n78A |  |  |
| DC_1A-3C-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |  |  |
| DC_1A-3C-28A_n7B-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_1A_n7B DC_3A_n7B DC_3C_n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |  |  |
| DC_1A-3A-28A-40A_n78A DC_1A-3A-28A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A DC_40A_n78A |  |  |
| DC_1A-3A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |  |  |
| DC_1A-3A-28A-42A_n77A DC_1A-3A-28A-42A_n77C DC_1A-3A-28A-42C_n77A DC_1A-3A-28A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_28A_n77A |  |  |
| DC_1A-3A-28A-42A_n78A DC_1A-3A-28A-42A_n78C DC_1A-3A-28A-42C_n78A DC_1A-3A-28A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_28A_n78A |  |  |
| DC_1A-3A-28A-42A_n79A DC_1A-3A-28A-42A_n79C DC_1A-3A-28A-42C_n79A DC_1A-3A-28A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_28A_n79A |  |  |
| DC_1A-3A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_3A_n28A DC_3A_n77A DC_3A_n79A |  |  |
| DC_1A_n3A-n28A-n77A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_1A_n79A |  |  |
| DC_1A-3A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_3A_n28A DC_3A_n78A DC_3A_n79A |  |  |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A1 DC_3A_n41A DC_41A_n3A |  |  |
| DC_1A-3A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A |  |  |
| DC_1A-3A-41C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |  |  |
| DC_1A-3A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A |  |  |
| DC_1A-3A-41C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |  |  |
| DC_1A-3A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_41A_n28A |  |  |
| DC_1A-3A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |  |  |
| DC_1A-3A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |  |  |
| DC_1A-3A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |  |  |
| DC_1A-3A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |  |  |
| DC_1A-3A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_41A_n77A |  |  |
| DC_1A-3A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_41A_n78A |  |  |
| DC_1A-3A-41A-42A_n77A5,6 DC_1A-3A-41A-42C_n77A5,6 DC_1A-3A-41C-42A_n77A5,6 DC_1A-3A-41C-42C_n77A5,6 | DC_1A_n77A DC_3A_n77A DC_41A_n77A |  |  |
| DC_1A-3A-41A-42A_n77(2A)5,6 DC_1A-3A-41A-42C_n77(2A)5,6 | DC_1A_n77A DC_3A_n77A DC_41A_n77A |  |  |
| DC_1A-3A-41A-42A_n78A5,6 DC_1A-3A-41A-42C_n78A5,6 DC_1A-3A-41C-42A_n78A5,6 DC_1A-3A-41C-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_41A_n78A |  |  |
| DC_1A-3A-41A-42A_n79A DC_1A-3A-41A-42C_n79A DC_1A-3A-41C-42A_n79A DC_1A-3A-41C-42C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |  |  |
| DC_1A-3A-42A_n28A-n77A5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A |  |  |
| DC_1A-3A-42A_n28A-n77(2A)5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A |  |  |
| DC_1A-3A-42C_n28A-n77A5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |  |  |
| DC_1A-3A-42C_n28A-n77(2A)5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |  |  |
| DC_1A-7A-8A-20A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A DC_20A_n3A |  |  |
| DC_1A-7A-8A-20A_n28A3,9 | DC_1A_n28A DC_7A_n28A DC_8A_n28A DC_20A_n28A |  |  |
| DC_1A-7A-8A-20A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_20A_n78A |  |  |
| DC_1A-7A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |  |  |
| DC_1A-7A-8A-32A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A |  |  |
| DC_1A-7A-8A-40A_n78A DC_1A-7A-8A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |  |  |
| DC_1A-7A-8A-40A_n78(2A) DC_1A-7A-8A-40C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |  |  |
| DC_1A-7A-20A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_20A_n3A DC_20A_n78A |  |  |
| DC_1A-7A-20A_n3A-n38A | DC_1A-n3A DC_20A_n3A |  |  |
| DC_1A-7A-20A_n3A-n78A | DC_1A_n3A |  |  |
| DC_1A-7A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |  |  |
| DC_1A-7A-20A-28A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A DC_28A_n3A |  |  |
| DC_1A-7A-20A_n28A-n78A3 | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | Yes |  |
| DC_1A-7A-20A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A |  |  |
| DC_1A-7A-20A-32A_n28A | DC_1A_n28A DC_7A_n28A DC_20A_n28A |  |  |
| DC_1A-7A-20A-32A_n78A | DC_1A_n78A DC_7A_n78A DC_20A_n78A |  |  |
| DC_1A-7A-20A-32A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |  |  |
| DC_1A-7A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |  |  |
| DC_1A-7A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A |  |  |
| DC_1A-7A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A |  |  |
| DC_1A-7A-28A_n3A-n78A | DC_1A_n3A DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_1A-7A-28A_n5A-n78A DC_1A-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |  |  |
| DC_1A-7A-28A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_1A-7A-28A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_28A_n3A |  |  |
| DC_1A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |  |  |
| DC_1A-7A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A |  |  |
| DC_1A-8A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A | Yes |  |
| DC_1A-8A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A | Yes |  |
| DC_1A-8A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A DC_8A_n3A DC_8A_n28A DC_8A_n79A |  |  |
| DC_1A-8A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |  |  |
| DC_1A-8A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |  |  |
| DC_1A-8A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A |  |  |
| DC_1A-8A-11A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | Yes |  |
| DC_1A-8A-11A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | Yes |  |
| DC_1A-8A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-8A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-8A-20A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |  |  |
| DC_1A-8A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_8A_n28A DC_8A_n77A DC_8A_n79A |  |  |
| DC_1A-8A-42A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A | Yes |  |
| DC_1A-8A-42C_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A | Yes |  |
| DC_1A-8A-42A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A |  |  |
| DC_1A-8A-42A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A |  |  |
| DC_1A-8A-42C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A |  |  |
| DC_1A-8A-42C_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A |  |  |
| DC_1A-8A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A |  |  |
| DC_1A-8A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A |  |  |
| DC_1A-8A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |  |  |
| DC_1A-8A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |  |  |
| DC_1A-11A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-11A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-19A-21A-42A_n77A5,6 DC_1A-19A-21A-42A_n77C5,6 DC_1A-19A-21A-42C_n77A5,6 DC_1A-19A-21A-42C_n77C5,6 | DC_1A_n77A DC_19A_n77A DC_21A_n77A |  |  |
| DC_1A-19A-21A-42A_n78A5,6 DC_1A-19A-21A-42A_n78C5,6 DC_1A-19A-21A-42C_n78A5,6 DC_1A-19A-21A-42C_n78C5,6 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |  |  |
| DC_1A-19A-21A-42A_n79A DC_1A-19A-21A-42A_n79C DC_1A-19A-21A-42C_n79A DC_1A-19A-21A-42C_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A |  |  |
| DC_1A-19A-42A_n77A-n79A5,6 DC_1A-19A-42C_n77A-n79A5,6 | DC_19A_n77A DC_19A_n79A |  |  |
| DC_1A-19A-42A_n78A-n79A5,6 DC_1A-19A-42C_n78A-n79A5,6 | DC_19A_n78A DC_19A_n79A |  |  |
| DC_1A-20A-28A-32A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |  |  |
| DC_1A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_38A_n3A DC_1A_n78A DC_20A_n78A DC_38A_n78A |  |  |
| DC_1A-21A-28A-42A_n77A5,6 DC_1A-21A-28A-42C_n77A5,6 | DC_1A_n77A DC_21A_n77A DC_28A_n77A |  |  |
| DC_1A-21A-28A-42A_n78A5,6 DC_1A-21A-28A-42C_n78A5,6 | DC_1A_n78A DC_21A_n78A DC_28A_n78A |  |  |
| DC_1A-21A-28A-42A_n79A DC_1A-21A-28A-42C_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A |  |  |
| DC_1A-21A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |  |  |
| DC_1A-21A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |  |  |
| DC_1A-21A-42A_n77A-n79A5,6 DC_1A-21A-42C_n77A-n79A5,6 | DC_1A_n77A DC_1A_n79A |  |  |
| DC_1A-21A-42A_n78A-n79A5,6 DC_1A-21A-42C_n78A-n79A5,6 | DC_1A_n78A DC_1A_n79A |  |  |
| DC_1A-42A_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |  |  |
| DC_1A-42A_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |  |  |
| DC_1A-42C_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |  |  |
| DC_1A-42C_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |  |  |
| DC_2A-5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |  |  |
| DC_2A-5A-7A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |  |  |
| DC_2A-5A-7A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |  |  |
| DC_2A-5A-7A-66A_n66A DC_2A-5A-7C-66A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |  |  |
| DC_2A-5A-7A-7A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |  |  |
| DC_2A-5A-7A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A DC_66A_n78A |  |  |
| DC_2A-5A-30A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A DC_66A_n2A |  |  |
| DC_2A-5A-30A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |
| DC_2A-5A-30A-66A_n77A8 | DC_2A_n77A8 DC_5A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |  |  |
| DC_2A-5A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A |  |  |
| DC_2A-5A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A |  |  |
| DC_2A-5A-66A_n2A-n77A8 DC_2A-5A-66A-66A_n2A-n77A8 | DC_2A_n77A8 DC_5A_n77A8 DC_66A_n77A8 |  |  |
| DC_2A-5A-66A_n66A-n77A8 | DC_2A_n77A8 DC_5A_n77A8 DC_66A_n77A8 |  |  |
| DC_2A-7A-12A-66A_n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |  |  |
| DC_2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |  |  |
| DC_2A-2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |  |  |
| DC_2A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |  |  |
| DC_2A-7A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |  |  |
| DC_2A-7C-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |  |  |
| DC_2A-7A-13A-66A_n66A DC_2A-7C-13A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |  |  |
| DC_2A-7A-7A-13A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |  |  |
| DC_2A-7A-28A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A DC_66A_n7A |  |  |
| DC_2A-7A-28A-66A_n66A DC_2A-7C-28A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |  |  |
| DC_2A-7A-29A-66A_n78A DC_2A-7C-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |
| DC_2A-7A-7A-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |  |  |
| DC_2A-7A-66A_n25A-n66A6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |  |  |
| DC_2A-7A-7A-66A_n25A-n66A6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |  |  |
| DC_2A-7C-66A_n25A-n66A,6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |  |  |
| DC_2A-7A-66A_n66A-n77A DC_2A-7A-7A-66A_n66A-n77A DC_2A-7C-66A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A DC_66A_n77A |  |  |
| DC_2A-7A-66A_n66A-n78A DC_2A-7C-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |  |  |
| DC_2A-7A-7A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |  |  |
| DC_2A-7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |  |  |
| DC_2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |  |  |
| DC_2A-2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |  |  |
| DC_2A-12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |  |  |
| DC_2A-12A-30A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |
| DC_2A-12A-30A-66A_n77A8 | DC_2A_n77A8 DC_12A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |  |  |
| DC_2A-13A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A |  |  |
| DC_2A-13A-66A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A |  |  |
| DC_2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |  |  |
| DC_2A-2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |  |  |
| DC_2A-13A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |  |  |
| DC_2A-13A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A DC_66A_n77A |  |  |
| DC_2A-13A-66A_n66A-n77A8 DC_2A-2A-13A-66A_n66A-n77A8 | DC_2A_n66A DC_2A_n77A8 DC_13A_n66A DC_13A_n77A8 DC_66A_n77A8 |  |  |
| DC_2A-5A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A DC_66A_n77 |  |  |
| DC_2A-14A-30A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A DC_66A_n2A |  |  |
| DC_2A-14A-30A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |
| DC_2A-14A-30A-66A_n77A8 | DC_2A_n77A8 DC_14A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |  |  |
| DC_2A-29A-30A-66A_n2A | DC_2A_n2A DC_30A_n2A DC_66A_n2A |  |  |
| DC_2A-29A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |  |  |
| DC_2A-29A-30A-66A_n77A8 | DC_2A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |  |  |
| DC_2A-46A-66A_n41A-n71A DC_2A-46C-66A_n41A-n71A DC_2A-46D-66A_n41A-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |  |  |
| DC_3A-7A-8A_n1A-n40A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_3A_n40A DC_7A_n40A DC_8A_n40A |  |  |
| DC_3A-7A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A | Yes |  |
| DC_3A-3A-7A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A | Yes |  |
| DC_3A-7A-7A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A | Yes |  |
| DC_3A-3A-7A-7A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A | Yes |  |
| DC_3A-7A-8A-20A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_20A_n1A |  |  |
| DC_3A-7A-8A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |  |  |
| DC_3A-7A-8A-32A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |  |  |
| DC_3A-7A-8A-32A_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A |  |  |
| DC_3A-7A-8A-40A_n1A DC_3A-7A-8A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_40A_n1A |  |  |
| DC_3A-7A-8A-40A_n78A DC_3A-7A-8A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |  |  |
| DC_3A-7A-8A-40A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |  |  |
| DC_3A-7A-8A-40C_n78(2A) | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |  |  |
| DC_3A-7A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A |  |  |
| DC_3A-7A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |  |  |
| DC_3C-7A-20A_n1A-n78A | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |  |  |
| DC_3A-7A-20A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |  |  |
| DC_3A-7A-20A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A DC_28A_n1A |  |  |
| DC_3A-7A-20A_n28A-n78A3,6,11 DC_3C-7A-20A_n28A-n78A3,6,11 | DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | Yes |  |
| DC_3A-7A-20A-32A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A |  |  |
| DC_3A-7A-20A-32A_n78A | DC_3A_n78A DC_7A_n78A DC_20A_n78A |  |  |
| DC_3A-7A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A |  |  |
| DC_3A-7A-28A_n1A-n78A | DC_3A_n1A DC_7A_n1A DC_28A_n1A DC_3A_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_3A-7A-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_3A-7C-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |  |  |
| DC_3A-7A-28A_n5A-n78A DC_3C-7A-28A_n5A-n78A DC_3A-7C-28A_n5A-n78A DC_3C-7C-28A_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |  |  |
| DC_3A-7A-28A_n7A-n78A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A DC_3A_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_3C-7A-28A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_3A-7A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |  |  |
| DC_3A-7A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |  |  |
| DC_3A-7A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |  |  |
| DC_3A-8A-11A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_3A-8A-11A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_3A-8A-20A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |  |  |
| DC_3A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |  |  |
| DC_3A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |  |  |
| DC_3A-19A-21A-42A_n77A5,6 DC_3A-19A-21A-42A_n77C5,6 DC_3A-19A-21A-42C_n77A5,6 DC_3A-19A-21A-42C_n77C5,6 | DC_3A_n77A DC_19A_n77A DC_21A_n77A |  |  |
| DC_3A-19A-21A-42A_n78A5,6 DC_3A-19A-21A-42A_n78C5,6 DC_3A-19A-21A-42C_n78A5,6 DC_3A-19A-21A-42C_n78C5,6 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |  |  |
| DC_3A-19A-21A-42A_n79A DC_3A-19A-21A-42A_n79C DC_3A-19A-21A-42C_n79A DC_3A-19A-21A-42C_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A |  |  |
| DC_3A-19A-42A_n1A-n77A5,6 DC_3A-19A-42C_n1A-n77A5,6 | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A |  |  |
| DC_3A-19A-42A_n1A-n78A5,6 DC_3A-19A-42C_n1A-n78A5,6 | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A |  |  |
| DC_3A-19A-42A_n1A-n79A DC_3A-19A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A |  |  |
| DC_3A-20A-32A_n1A-n28A | DC_3A_n1A DC_20A_n1A DC_3A_n28A DC_20A_n28A |  |  |
| DC_3C-20A-32A_n1A-n28A | DC_3A_n1A DC_3C_n1A DC_20A_n1A DC_3A_n28A DC_3C_n28A DC_20A_n28A |  |  |
| DC_3A-21A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A DC_21A_n1A DC_21A_n77A DC_21A_n79A |  |  |
| DC_3A-21A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A DC_21A_n1A DC_21A_n78A DC_21A_n79A |  |  |
| DC_3A-21A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |  |  |
| DC_2A-5A-66A_n5A-n77A8 DC_2A-5A-66A-66A_n5A-n77A8 | DC_2A_n77A DC_5A_n77A DC_66A_n77A |  |  |
| DC_2A-13A-66A_n2A-n77A8 DC_2A-13A-66A-66A_n2A-n77A8 | DC_2A_n77A DC_13A_n77A DC_66A_n77A |  |  |
| DC_2A-13A-66A_n5A-n77A8 DC_2A-2A-13A-66A_n5A-n77A8 DC_2A-13A-66A-66A_n5A-n77A8 | DC_2A_n77A DC_13A_n77A DC_66A_n77A |  |  |
| DC_3A-7A_n1A-n8A-n78A | DC_3A_n1A DC_3A_n8A DC_3A_n78A DC_7A_n1A DC_7A_n8A DC_7A_n78A | Yes |  |
| DC_3A-3A-7A_n1A-n8A-n78A DC_3A-7A-7A_n1A-n8A-n78A DC_3A-3A-7A-7A_n1A-n8A-n78A | DC_3A_n1A DC_3A_n8A DC_3A_n78A DC_7A_n1A DC_7A_n8A DC_7A_n78A | Yes |  |
| DC_3A-21A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |  |  |
| DC_3A-21A-42A_n1A-n77A5,6 DC_3A-21A-42C_n1A-n77A5,6 | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A |  |  |
| DC_3A-21A-42A_n1A-n78A5,6 DC_3A-21A-42C_n1A-n78A5,6 | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A |  |  |
| DC_3A-21A-42A_n1A-n79A DC_3A-21A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A |  |  |
| DC_3A-28A-41A-42A_n78A5,6 DC_3A-28A-41A-42C_n78A5,6 DC_3A-28A-41C-42A_n78A5,6 DC_3A-28A-41C-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |  |  |
| DC_7A-8A-20A-32A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |  |  |
| DC_7A-8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A |  |  |
| DC_7A-8A-32A-38A_n1A | DC_8A_n1A |  |  |
| DC_7A-8A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |  |  |
| DC_7A-8A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |  |  |
| DC_7A-20A-28A-32A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |  |  |
| DC_7A-20A-28A-32A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |  |  |
| DC_7A-20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A |  |  |
| DC_7A-20A-32A-38A_n1A | DC_20A_n1A |  |  |
| DC_7A-20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A |  |  |
| DC_7A-28A-32A-38A_n1A | DC_28A_n1A |  |  |
| DC_8A_n3A-n28A-n77A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_8A_n79A |  |  |
| DC_8A-11A_n3A-n28A-n77A | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_8A-11A_n3A-n28A-n77(2A) | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_8A-20A-32A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |  |  |
| DC_8A-42A_n3A-n28A-n77A5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |  |  |
| DC_8A-42A_n3A-n28A-n77(2A)5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |  |  |
| DC_8A-42C_n3A-n28A-n77A5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |  |  |
| DC_8A-42C_n3A-n28A-n77(2A)5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |  |  |
| DC_19A-21A-42A_n1A-n77A5,6 DC_19A-21A-42C_n1A-n77A5,6 | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A |  |  |
| DC_19A-21A-42A_n1A-n78A5,6 DC_19A-21A-42C_n1A-n78A5,6 | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A |  |  |
| DC_19A-21A-42A_n1A-n79A DC_19A-21A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A |  |  |
| DC_19A-21A-42A_n77A-n79A5,6 DC_19A-21A-42C_n77A-n79A5,6 | DC_19A_n77A DC_19A_n79A |  |  |
| DC_19A-21A-42A_n78A-n79A5,6 DC_19A-21A-42C_n78A-n79A5,6 | DC_19A_n78A DC_19A_n79A |  |  |
| DC_19A-42A_n1A-n77A-n79A5,6 | DC_19A_n1A DC_19A_n77A DC_19A_n79A |  |  |
| DC_19A-42A_n1A-n78A-n79A5,6 | DC_19A_n1A DC_19A_n78A DC_19A_n79A |  |  |
| DC_20A-28A-32A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |  |  |
| N NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 4: Only single switched UL is supported NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 8: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 9: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 10: The combination is not used alone as fall back mode of other band combinations. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |  |

#### 5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)

Table 5.5B.4.5-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Mandatory simultaneous Rx/Tx |  |
| --- | --- | --- | --- |
| DC_1A-3A-7A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |  |  |
| DC_1A-3A-7A-8A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |  |  |
| DC_1A-3A-7A-8A-40A_n78A DC_1A-3A-7A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |  |  |
| DC_1A-3A-7A-8A-40A_n78(2A) DC_1A-3A-7A-8A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |  |  |
| DC_1A-3A-7A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |  |  |
| DC_1A-3A-7A-20A_n28A-n78A3,6,7 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | Yes |  |
| DC_1A-3A-7A-20A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |  |  |
| DC_1A-3A-7A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |  |  |
| DC_1A-3A-7A-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_1A-3A-7C-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |  |  |
| DC_1A-3A-7A-28A_n5A-n78A DC_1A-3A-7C-28A_n5A-n78A DC_1A-3C-7A-28A_n5A-n78A DC_1A-3C-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |  |  |
| DC_1A-3A-7A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_1A-3C-7A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |  |  |
| DC_1A-3A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |  |  |
| DC_1A-3A-8A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-3A-8A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-3A-8A-20A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |  |  |
| DC_1A-7A-20A-28A-32A_n3A DC_1A-7C-20A-28A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A DC_28A_n3A |  |  |
| DC_1A-7A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |  |  |
| DC_1A-8A-11A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-8A-11A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A | Yes |  |
| DC_1A-8A-42A_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |  |  |
| DC_1A-8A-42A_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |  |  |
| DC_1A-8A-42C_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |  |  |
| DC_1A-8A-42C_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |  |  |
| DC_3A-7A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |  |  |
| DC_3A-7A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |  |  |
| DC_7A-8A-20A-32A-38A_n1A | DC_8A_n1A DC_20A_n1A |  |  |
| DC_7A-20A-28A-32A-38A_n1A | DC_20A_n1A DC_28A_n1A |  |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |  |

### 5.5B.4a	Inter-band NE-DC within FR1

#### 5.5B.4a.1	Inter-band NE-DC configurations within FR1 (two bands)

Table 5.5B.4a.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Single UL allowed | Mandatory simultaneous Rx/Tx |
| --- | --- | --- | --- |
| DC_n1A_28A | DC_n1A_28A | No |  |
| DC_n3A_1A | DC_n3A_1A | DC_n3_1 |  |
| DC_n3A_8A | DC_n3A_8A | No |  |
| DC_n8A_1A | DC_n8A_1A | No |  |
| DC_n8A_3A | DC_n8A_3A | No |  |
| DC_n28A_3A DC_n28A_3C | DC_n28A_3A | No |  |
| DC_n28A_8A DC_n28A_8B | DC_n28A_8A | No |  |
| DC_n28A_20A3 | DC_n28A_20A | No |  |
| DC_n28A_34A | DC_n28A_34A | No |  |
| DC_n28A_39A DC_n28A_39C | DC_n28A_39A | No |  |
| DC_n28A_40A DC_n28A_40C | DC_n28A_40A | No |  |
| DC_n41A_3A DC_n41A_3C DC_n41C_3A | DC_n41A_3A | No |  |
| DC_n41A_8A DC_n41A_8B DC_n41C_8A | DC_n41A_8A | No |  |
| DC_n41A_34A2 | DC_n41A_34A | No |  |
| DC_n41A_39A2 DC_n41A_39C2 DC_n41C_39A2 | DC_n41A_39A | No |  |
| DC_n41A_40A2 DC_n41A_40C2 DC_n41C_40A2 | DC_n41A_40A | No |  |
| DC_n77A_1A DC_n77(2A)_1A | DC_n77A_1A | DC_n77_1 |  |
| DC_n77A_3A DC_n77(2A)_3A | DC_n77A_3A | DC_n77_3 |  |
| DC_n77A_8A DC_n77(2A)_8A | DC_n77A_8A | No |  |
| DC_n78A_1A | DC_n78A_1A | No |  |
| DC_n78A_3A | DC_n78A_3A | No |  |
| DC_n78A_3C | DC_n78A_3A | No |  |
| DC_n78A_5A | DC_n78A_5A | No |  |
| DC_n78A_7A | DC_n78A_7A | No |  |
| DC_n78A_7A-7A | DC_n78A_7A | No |  |
| DC_n78A_8A | DC_n78A_8A | No |  |
| DC_n78A_26A | DC_n78A_26A | No |  |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

#### 5.5B.4a.2	Inter-band NE-DC configurations within FR1 (three bands)

Table 5.5B.4a.2-1: Inter-band NE-DC configurations within FR1 (three bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Mandatory simultaneous Rx/Tx |
| --- | --- | --- |
| DC_n3A_1A-8A | DC_n3A_1A DC_n3A-8A |  |
| DC_n77A_1A-8A DC_n77(2A)_1A-8A | DC_n77A_1A DC_n77A_8A |  |
| DC_n77A_3A-1A | DC_n77A_1A DC_n77A_3A |  |
| DC_n77A_3A-8A DC_n77(2A)_3A-8A | DC_n77A_3A DC_n77A_8A |  |
| DC_n78A_1A-3A DC_n78A_1A-3C | DC_n78A_1A DC_n78A_3A |  |
| DC_n78A_1A-5A | DC_n78A_1A DC_n78A_5A |  |
| DC_n78A_1A-7A | DC_n78A_1A DC_n78A_7A |  |
| DC_n78A_1A-7A-7A | DC_n78A_1A DC_n78A_7A |  |
| DC_n78A_1A-8A | DC_n78A_1A DC_n78A_8A |  |
| DC_n78A_3A-5A | DC_n78A_3A DC_n78A_5A |  |
| DC_n78A_3A-7A | DC_n78A_3A DC_n78A_7A |  |
| DC_n78A_3A-7A-7A | DC_n78A_3A DC_n78A_7A |  |
| DC_n78A_3A-8A DC_n78A_3C-8A | DC_n78A_3A DC_n78A_8A |  |
| DC_n78A_5A-7A | DC_n78A_5A DC_n78A_7A |  |
| DC_n78A_5A-7A-7A | DC_n78A_5A DC_n78A_7A |  |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. |  |  |

#### 5.5B.4a.3	Inter-band NE-DC configurations within FR1 (four bands)

Table 5.5B.4a.3-1: Inter-band NE-DC configurations within FR1 (four bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Mandatory simultaneous Rx/Tx |  |
| --- | --- | --- | --- |
| DC_n77A_1A-3A-8A DC_n77(2A)_1A-3A-8A | DC_n77A_1A DC_n77A_3A DC_n77A_8A |  |  |
| DC_n78A_1A-3A-5A | DC_n78A_1A DC_n78A_3A DC_n78A_5A |  |  |
| DC_n78A_1A-3A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_7A |  |  |
| DC_n78A_1A-3A-7A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_7A |  |  |
| DC_n78A_1A-3A-8A DC_n78A_1A-3C-8A | DC_n78A_1A DC_n78A_3A DC_n78A_8A |  |  |
| DC_n78A_1A-5A-7A | DC_n78A_1A DC_n78A_5A DC_n78A_7A |  |  |
| DC_n78A_1A-5A-7A-7A | DC_n78A_1A DC_n78A_5A DC_n78A_7A |  |  |
| DC_n78A_3A-5A-7A | DC_n78A_3A DC_n78A_5A DC_n78A_7A |  |  |
| DC_n78A_3A-5A-7A-7A | DC_n78A_3A DC_n78A_5A DC_n78A_7A |  |  |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. |  |  |

#### 5.5B.4a.4	Inter-band NE-DC configurations within FR1 (five bands)

Table 5.5B.4a.4-1: Inter-band NE-DC configurations within FR1 (five bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Mandatory simultaneous Rx/Tx |
| --- | --- | --- |
| DC_n78A_1A-3A-5A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_5A DC_n78A_7A |  |
| DC_n78A_1A-3A-5A-7A-7A | DC_n78A_1A DC_n78A_3A DC_n78A_5A DC_n78A_7A |  |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. |  |  |