3GPP TSG-RAN WG4 Meeting #103e	R4-2210809

Electronic Meeting, 9th – 20th May, 2022

Source:	ZTE Corporation

Title:	TP revision on Section 6.2.3.1

Agenda Item:	11.2.3.3

Document for:	Discussion

### 6.2.3	Overlapping CA (two cells)

#### 6.2.3.1      General Aspects

Using the Overlapping CA approach to support irregular spectrum is one of the methods without the need of introduction of new dedicated channel bandwidths for both UE and BS. Figure 6.2.3-1 shows overlapping CA from network perspective, in which BS supports the intra-band overlapping CA, while UEs only supports the single CC with the existing channel bandwidth. And figure 6.2.3-2 shows overlapping CA from UE perspective, in which both BS and UE support the intra-band overlapping CA to use all RBs, and the UE supports intra-band non-contiguous CA for the combination as well.

Even two CCs are overlapped in frequency domain, the network can configure different CCs with non-overlapping BWP to avoid the conflict of physical signals or channels.

UE1: legacy channel bandwidth

UE2: legacy channel bandwidth

Overlapping CA for gNB

Figure 6.2.3-1: Overlapping CA from network perspective

Overlapping CA for gNB

UE3: overlapping CA of CC1 and CC2

CC1: BWP#1

CC2: BWP#2

Figure 6.2.3-2: Overlapping CA from UE perspective

In summary,

-	No new gNB channel bandwidth is required

-	Legacy UE using existing lower regular NR channel bandwidth can operate in either carrier

-	Overlapping CA approach needs to have PRB grid alignment between overlapping CCs

-	From UE perspective, overlapping CA is optional supported in DL only.

-   Irregular bandwidth utilizing overlapping CA approach cannot be combined with NR CA combinations within the same irregular spectrum, where the number of CCs per spectrum block with an irregular BW shall be exactly 2 and spending already two CCs for the irregular BW reduces the remaining capacity for CA with any further CCs.

- From UE perspective, the solution requires UE support of intra-band non-contiguous CA for the combination

# Reference

TR 38.844 v0.0.8, Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, Ericsson