3GPP TSG-RAN4 Meeting # 103-e	R4-2210829

# Electronic Meeting, 9th  - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | 0003 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.106: Correction of definitions, symbols and abbreviations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The list of definitions, symbols and abbreviations does not contain some terms used in the specification and contains some redundant terms. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The sections are updated to align with the rest of the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete definitions, symbols and abbreviations lists, potential confusion when readin the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Antenna connector: connector at the conducted interface of the repeater type 1-C

Beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

Beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

Beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

Beam peak direction: direction where the maximum EIRP is found

Beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

directional requirement: requirement which is applied in a specific direction within the OTA coverage range.

Equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

Fractional bandwidth: fractional bandwidth FBW is defined as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%

gap between passbands: frequency gap between two consecutive passbands that belong to the same operating band, where the RF requirements in the gap are based on co-existence for un-coordinated operation

Inter-passband gap: The frequency gap between two supported consecutive passbands that belong to different operating bands.

Maximum passband output power: mean power level measured per passband at the antenna connector, during the transmitter ON state in a specified reference condition

Maximum passband TRP output power: mean power level measured per passband during the transmitter ON state in a specified reference condition and corresponding to the declared rated passband TRP output power (Prated,p,,TRP)

Measurement bandwidth: RF bandwidth in which an emission level is specified

multi-band connector: Antenna Connector for a Multi-band repeater.

Multi-band repeater: Repeater Type 1-C whose antenna connector is associated with a transmitter and/or receiver that is characterized by the ability to process two or more passband(s) in common active RF components simultaneously, where at least one passband is configured at a different operating band than the other passband(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

Non-contiguous spectrum: spectrum consisting of two or more passbands separated by inter-passband gap(s).

Operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

OTA coverage range: a common range of directions within which OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met

OTA peak directions set: set(s) of beam peak directions within which certain OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range

Passband: The frequency range in which the repeater operates in with operational configuration, this frequency range can correspond to one or several consecutive nominal channels, if they are not consecutive each subset of channels shall be considered as an individual passband, a repeater can have one or several passbands, all channels within the passband(s) shall belong to a single operator or collaborating operators.

passband edge: Frequency at the edge of the passband

Radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

Rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the repeater is declared to radiate at the associated beam peak direction during the transmitter ON period

Rated passband output power: mean power level associated with a passband the manufacturer has declared to be available at the antenna connector, during the transmitter ON period in a specified reference condition

Rated passband TRP output power: mean power level declared by the manufacturer per passband, that the manufacturer has declared to be available at the RIB during the transmitter ON state

Rated total output power: mean power level associated with a particular operating band the manufacturer has declared to be available at the antenna connector, during the transmitter ON state in a specified reference condition

Rated total TRP output power: mean power level associated with a particular operating band, that the manufacturer has declared to be available at the RIB during the transmitter ON state in a specified reference condition

Reference beam direction pair: Beam direction pair in the reference direction declared by the manufacturer.

Repeater type 1-C:	Repeater operating at FR1 with a requirement set consisting only of conducted requirements defined at individual antenna connectors.

Repeater type 2-O: Repeater operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

Requirement set: one of the NR requirements set as defined for NR repeater

single-band connector: Repeater type 1-C antenna connector supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band connector.

Sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

sub-block: one contiguous allocated block of spectrum for transmission and reception by the repeater.

Superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

Total radiated power: is the total power radiated by the antenna

NOTE:	The total radiated power is the power radiating in all direction for two orthogonal polarizations. Total radiated power is defined in both the near-field region and the far-field region

Transmitter OFF state: Time period during which the repeater downlink or uplink is not allowed to transmit in the corresponding direction

Transmitter ON state: Time period during which the repeater downlink or uplink is transmitting in the corresponding direction

Transmitter transient period: Time period during which the transmitter is switching from the OFF state to the ON state or vice versa

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWPassband	Passband bandwidth

f	Separation between the passband edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the operating band edge

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

FFBWhigh	Highest supported frequency within supportedoperating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

Ffilter	Filter centre frequency

Foffset,high	Frequency offset from FC,high to the upper passband edge

Foffset,low	Frequency offset from FC,low to the lower passband edge

f_offset	Separation between the passband edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the operating band

Fstep,X	Frequency steps for the OTA transmitter spurious emissions (Category B)

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

PEM,n50/n75,ind	Declared emission level for Band n50/n75; ind = a, b

Prated,p,AC	Rated passband output power per antenna connector

Prated,t,AC	Rated total output power declared per antenna connector

Prated,p,EIRP 	Rated passband EIRP output power

Prated,p,TRP	Rated passband TRP output power declared per RIB

Prated,t,TRP	Rated total TRP output power declared per RIB

Pin,p,AC	Input power intended to produce the maximum rated output power (Prated,p,AC) at the antenna connector

Pin,p,EIRP	Input power intended to produce the maximum rated output power (Prated,p,TRP) at the RIB

Prated,out,FBWhigh	The rated output EIRP for the higher supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,out,FBWlow	The rated output EIRP for the lower supported frequency range within supported operating band, for which fractional bandwidth support was declared

Pmax,p,AC	Maximum passband output power measured per antenna connector

Pmax,p,EIRP	Maximum passband EIRP output power when repeater is configured at the rated passband TRP output power (Prated,p,TRP)

Pmax,p,TRP	Maximum passband TRP output power measured per RIB

Wgap	Inter passband Bandwidth gap size

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

AoA	Angle of Arrival

BW	Bandwidth

CACLR	Cumulative ACLR

CP-OFDM	Cyclic Prefix-OFDM

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DL	Downlink

EIRP	Effective Isotropic Radiated Power

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

MR	Medium Range

NR	New Radio

OBUE	Operating Band Unwanted Emissions

OOB	Out-of-band

OTA	Over-The-Air

QAM	Quadrature Amplitude Modulation

RF	Radio Frequency

RIB	Radiated Interface Boundary

RX	Receiver

SCS	Sub-Carrier Spacing

TX	Transmitter

TRP	Total Radiated Power

UL	Uplink

WA	Wide Area