3GPP TSG-RAN WG4 Meeting # 103-e	R4-2210930

# Electronic Meeting,  May.9th - 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: Introduction of CRS-IM receiver for 30kHz SCS TDD scenario 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN 4 agreed to introduce CRS-IM receiver for 30kHz SCS TDD scenario if obvious gain can be observed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requirements for CRS-IM receiver for 30kHz SCS TDD scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5.2.2.1.19 and 5.2.3.1.19 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4. |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

##### 5.2.2.2.X	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.2.2.X-4 and Table 5.2.2.2.X-5, with the addition of test parameters in Table 5.2.2.2.X-2 for the serving cell and Table 5.2.2.2.X-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.X-1.

Table 5.2.2.2.X-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1 and 2-1 |

Table 5.2.2.2.X-2: Tests parameters for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.X-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause TBA | As specified in clause TBA |
| Time offset to the serving cell |  | us | 1.5 | -0.5 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: Refer to [X.Y.Z] Note 2: The channel for the LTE interference cells and the serving cell are independent. Note 3: No MBSFN is configured on LTE carrier. Note 4: Network-based CRS interference mitigation is disabled on LTE carrier. Note 5: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame |  |  |  |  |

The requirements for UE capable of performing CRS-IM without the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.2.2.X-4 with following test procedure:

[TBA]

Table 5.2.2.2.X-4: Minimum performance for Rank 1 without the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBA | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBA |

The requirements for UE capable of performing CRS-IM with the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.2.2.X-5:

Table 5.2.2.2.X-5 Minimum performance for Rank 1 with the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | TBA | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBA |

<End of change 1>

<Start of change 2>

##### 5.2.3.2.X	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.3.2.X-4 and Table 5.2.3.2.X-5, with the addition of test parameters in Table 5.2.3.2.X-2 for the serving cell and Table 5.2.3.2.X-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.X-1.

Table 5.2.3.2.X-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1 and 2-1 |

Table 5.2.3.2.X-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD UL-DL pattern |  |  | FR1.15-1 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.X-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause TBA | As specified in clause TBA |
| Time offset to the serving cell |  | us | 1.5 | -0.5 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: Refer to [X.Y.Z] Note 2: The channel for the LTE interference cells and the serving cell are independent. Note 3: No MBSFN is configured on LTE carrier. Note 4: Network-based CRS interference mitigation is disabled on LTE carrier. Note 5: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame |  |  |  |  |

The requirements for UE capable of performing CRS-IM without the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.3.2.X-4 with following test procedure:

[TBA]

Table 5.2.3.2.X-4: Minimum performance for Rank 1 without the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBA | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | TBA |

The requirements for UE capable of performing CRS-IM with the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.3.2.X-5:

Table 5.2.3.2.X-5 Minimum performance for Rank 1 with the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | TBA | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | TBA |

<End of Change 1>