3GPP TSG-RAN WG4 Meeting # 103-e              									           R4-2211146

Electronic Meeting, 9st of May – 20th of May, 2022                         (revision of R4-2211145)

(revision of R4-2210163)

Agenda item:			9.12.3.2

Source:	THALES

Title:	Draft text proposal for Clause 7.3.3.3.2 OTA reference sensitivity - TR 38.863

Document for:	Approval

# Introduction

During RAN WG4 Meeting RAN4#102-e, a work split has been proposed to complete Section 6 and Section 7 of TR 38.863. In this document, a TP is proposed to update some of the subsections related to RF requirements.

# Discussion

In this contribution is proposed to add information with respect to Section 7.3.3.3.2 OTA reference Sensitivity of TR 38.863 [1] by including some information related to agreements for TS 38.108.

Proposal 1: To update the TR with the attached TP

# Conclusions

In this contribution, the following proposal is made:

Proposal 1: To update the TR with the attached TP

# Reference

[1]	TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

# Text proposals for TR 38.863

### ------------------------------------------------<Start of TP>---------------------------------------------------

## 7.3.3.3.2              OTA reference Sensitivity

OTA reference sensitivity level is a directional requirement specified as an EIS level.

As OTA reference sensitivity is used as a reference for many of the interference requirements where it is important that the interferer is polarization matched to the receiver antenna it is also important that the wanted signal is polarization matched. Hence OTA reference sensitivity is specifically declared per polarization and the EIS values are under the assumption of polarization match.

For SAN type 1-O OTA reference sensitivity is intended to provide equivalent protection and performance as the conducted reference sensitivity level. The OTA reference sensitivity level is calculated based on the conducted reference sensitivity level for SAN type 1-H and antenna gain of a passive antenna which would be used with a non-AAS SAN in the same deployment scenario.

The requirement is valid over the OTA antenna coverage.

### ------------------------------------------------<End of TP>---------------------------------------------------