3GPP TSG-RAN WG4 Meeting #103	R4-2211172

Electronic Meeting, 09 ‒ 20, 2022

Title: 	WF on A-MPR for NS_05

Agenda Item:	5.1.8.1

Source: 	Huawei, HiSilicon

Document for:	Approval

# Background

- Square brackets remain in the NS_05 PC2 A-MPR spec.

- It is observed [1][2] that certain PC2 A-MPR values for NS_05 are too stringent for practical implementations, especially for APT/ET PAs which are optimised for power efficiency.

- It is observed [2] that PC2 operations are not allowed in Japan, for which NS_05 is designated.

- It is observed [2] that for system performance the output power of a PC2 UE should not be lower than that of a PC3 UE.

## References

[1] R4-2209181 Consideration on practical PA implementation for NS_05 A-MPR, Huawei, HiSilicon, RAN4#103-e

[2] R4-2210240 Email discussion summary for [103-e][104] NR_PC2_UE_FDD_maintenance, Moderator (China Unicom), RAN4#103-e

# Agreements

RAN4 will study the NS_05 A-MPR requirements by exploring the following options:

- Option 1: Applicability of the PC2 A-MPR requirements for NS_05, considering the regulations in Japan;

- Option 2: Revisit the large A-MPR values (e.g. ≥10 dB) for NS_05 in both PC2 and PC3 tables; Revise the A-MPR values if found necessary;

  - System impact should be considered where lowering PC3 and/or PC2 A-MPR values is also a possibility

- Option 3: Other options are not precluded.

Interested companies are encourage to submit contributions to the next RAN4 meeting.