3GPP TSG-RAN4 Meeting #103-e		R4-2211215

# Electronic Meeting, May 09th - 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on reduced capability UEs for general measurements and intra-frequency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Provides skeleton of new sections for RedCap requirements There is no agreement to support CSI-RS based L3 measurements and CSI-RS based L3 is not mentioned in the exception sheet and hence it shall be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Skeleton structure contains high level sections for: - Measurement procedures - 9.1A General measurement requirement for RedCap - 9.2B NR intra-frequency measurements for RedCap More sections can be added based on agreements in other areas. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap requirements will be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: 9.1A, 9.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## 9.1A General measurement requirement for RedCap

### 9.1A.1	Introduction

This clause contains general requirements on the RedCap UE regarding measurement reporting in RRC_CONNECTED state. The requirements are split in intra-frequency, inter-frequency, inter-RAT E-UTRAN FDD, inter-RAT E-UTRAN TDD, and L1-RSRP measurements requirements. These measurements may be used by the NG-RAN. The measurement quantities are defined in TS38.215 [4], the measurement model is defined in TS38.300 [10], TS37.340 [17] and measurement accuracies are specified in clause 10. Control of measurement reporting is specified in TS 38.331 [2].

The SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified.

### 9.1A.2	Measurement gap

If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE does not support independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply the network must provide a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers.

If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply the network must provide either  per-FR measurement gap patterns for frequency range where UE requires per-FR measurement gap for concurrent monitoring of all frequency layers of each frequency range independently, or a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers of all frequency ranges.

During the per-UE or per-FR measurement gaps the UE:

-	is not required to conduct reception/transmission from/to the corresponding NR serving cell for SA (with single carrier) except the reception of signals used for RRM measurement(s), and the signals used for random access procedure according to [7].

UEs shall support the measurement gap patterns listed in Table 9.1A.2-1 based on the applicability specified in Table 9.1A.2-2. UE determines measurement gap timing based on gap offset configuration and measurement gap timing advance configuration provided by higher layer signalling as specified in TS 38.331 [2] and TS 36.331 [16].

Table 9.1A.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |

Table 9.1A.2-2: Applicability for Gap Pattern Configurations supported by the RedCap UE with NR standalone operation (with single carrier)

| Measurement gap pattern configuration | Serving cell | Measurement Purpose | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
|  | FR1 | non-NR RAT NOTE2 | 0,1,2,3 |
|  |  | FR1 and/or FR2 | 0-11 |
|  |  | non-NR RAT and FR1 and/or FR2 NOTE2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
| Per-UE measurement | FR2 | non-NR RAT only NOTE2 | 0,1,2,3 |
| gap |  | FR1 only | 0-11 |
|  |  | FR1 and FR2 | 0-11 |
|  |  | non-NR RAT and FR1 and/or FR2 NOTE2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  |  | FR2 only | 12-23 |
|  | FR1 if configured | non-NR RAT only | 0,1,2,3 |
|  | FR2 if configured | NOTE2 | No gap |
|  | FR1 if configured | FR1 only | 0-11 |
|  | FR2 if configured |  | No gap |
|  | FR1 if configured | FR2 only | No gap |
| Per-FR | FR2 if configured |  | 12-23 |
| measurement | FR1 if configured | non-NR RAT and | 0, 1, 2, 3, 4, 6, 7, 8,10 |
| gap | FR2 if configured | FR1 NOTE2 | No gap |
|  | FR1 if configured | FR1 and FR2 | 0-11 |
|  | FR2 if configured |  | 12-23 |
|  | FR1 if configured | non-NR RAT and | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured | FR2 NOTE2 | 12-23 |
|  | FR1 if configured | non-NR RAT and FR1 and FR2 NOTE2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | 12-23 |
| NOTE1: If per-UE measurement gap is configured with MG timing advance of TMG ms, the measurement gap starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among all serving cells subframes. If per-FR measurement gap for FR1 is configured with MG timing advance of TMG ms, the measurement gap for FR1 starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among serving cells subframes in FR1. If per-FR measurement gap for FR2 is configured with MG timing advance of TMG ms, the measurement gap for FR2 starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among serving cells subframes in FR2. TMG is the MG timing advance value provided in mgta according to [2]. In determining the measurement gap starting point, UE shall use the DL timing of the latest subframe occurring immediately before the configured measurement gap among serving cells. NOTE 2: In RedCap, non-NR RAT means E-UTRA only. |  |  |  |

Editor’s note: The case of whether to support Per-FR gap/per-UE gap may need updated depends on RAN4 discussion outcome.

If measurement gap is configured in one FR but measurement object is not configured in the FR, the scheduling opportunity in the FR depends on the configured measurement gap pattern.

For NR standalone operation (with single carrier), if UE is not capable of per-FR-gap, total interruption time on a serving cell during MGL is defined when MGL(N) = 6ms, 5.5ms, 4ms, 3.5ms, 3ms, and 1.5ms. And if UE is capable of per-FR-gap, total interruption time on FR1 serving cell during MGL is defined only when MGL(N) = 6ms, 4ms, and 3ms, and total interruption time on FR2 serving cell during MGL is defined only when MGL(N) = 5.5ms, 3.5ms, and 1.5ms.

(a)	Measurement gap with MGL = N(ms) with MG timing advance of 0ms for serving cell in synchronous NR standalone operation (with single carrier)

(b)	Measurement gap with MGL = N(ms) with MG timing advance of 0.5ms for serving cell in synchronous NR standalone operation (with single carrier)

Figure 9.1A.2-1: Measurement GAP and total interruption time on serving cell for NR standalone operation (with single carrier)

The corresponding total number of interrupted slots on serving cell is listed in Table [9.1A.2-3] for all serving cell in NR standalone (with single carrier).

Table 9.1A.2-3: Total number of interrupted slots on serving cell during MGL for NR standalone operation (with single carrier) with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cell |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |
| (kHz) | MGL=6ms | MGL=4ms | MGL=3ms | MGL=6ms | MGL=4ms |  | MGL=3ms |
| 15 | 6 | 4 | 3 | 7Note3 | 5Note3 |  | 4Note3 |
| 30 | 12 | 8 | 6 | 12 | 8 |  | 6 |
| 60 | 24 | 16 | 12 | 24 | 16 |  | 12 |
| 120 | 48 | 32 | 24 | 48 | 32 |  | 24 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on serving cell is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz is only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a RedCap UE can receive and/or transmit in those half-slots is up to UE implementation. |  |  |  |  |  |  |  |  |

In case that UE capable of per-FR measurement gap is configured with per-UE measurement gap or per-FR measurement gap for FR2 serving cell, total number of interrupted slots on FR2 serving cells during MGL is listed in Table 9.1A.2-3a.

Table 9.1A.2-3a: Total number of interrupted slots on FR2 serving cell during MGL for NR standalone operation (with single carrier) with per-UE measurement gap or per-FR measurement gap for FR2

| NR | Total number of interrupted slots on FR2 serving cell |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.25ms is applied |  |  |
| (kHz) | MGL= 5.5ms | MGL= 3.5ms | MGL= 1.5ms | MGL= 5.5ms | MGL= 3.5ms |  | MGL= 1.5ms |
| 60 | 22 | 14 | 6 | 22 | 14 |  | 6 |
| 120 | 44 | 28 | 12 | 44 | 28 |  | 12 |
| NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell. |  |  |  |  |  |  |  |  |

It is up to UE implementation whether or not the UE is able to conduct transmission in the following slot(s),

-	when MGTA is not applied, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap

-	when MGTA is applied and the SCS of the UL carrier is other than 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap

-	when MGTA is applied and the SCS of the UL carrier is 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after the slot partially overlapped with measurement gap

where UL slot denotes that all the symbols in the slot are uplink symbols, and L=1 if  for the UL transmission is less than the length of one slot; L=2 otherwise.

#### 9.1A.2.1	SA: Measurement Gap Sharing

For NR standalone UE configured with per-UE measurement gap, measurement gap sharing shall be applies when UE requires measurement gaps to identify and measure cells on an intra-frequency carrier or when SMTC configured for intra-frequency measurement are fully overlapping with per-UE measurement gaps, and when UE requires measurement gaps to identify and measure cells on inter-frequency carriers for SSB based L3 measurement, or when all of SMTC configured for inter-frequency SSB based measurement without measurement gaps are fully overlapping with per-UE measurement gaps or per-FR measurement gap, and/or inter-RAT E-UTRAN carriers.

For NR standalone UE configured with per-FR1 measurement gap, measurement gap sharing shall be applied when UE requires measurement gaps to identify and measure cells on FR1 intra-frequency carriers or when SMTC configured for FR1 intra-frequency measurement are fully overlapping with per-FR1 measurement gaps, and when UE requires measurement gaps to identify and measure cells on FR1 inter-frequency carriers for SSB based L3 measurement and/or inter-RAT E-UTRAN carriers, or when all of SMTC configured for inter-frequency SSB based measurement without measurement gaps are fully overlapping with per-FR1 measurement gaps.

For NR standalone UE configured with per-FR2 measurement gap, measurement gap sharing shall be applied when UE requires measurement gaps to identify and measure cells on FR2 intra-frequency carriers or when SMTC configured for FR2 intra-frequency measurement are fully overlapping with per-FR2 measurement gaps, and when UE requires measurement gaps to identify and measure cells on FR2 inter-frequency carriers for SSB based L3 measurement, or when all of SMTC configured for inter-frequency SSB based measurement without measurement gaps are fully overlapping with per-UE measurement gaps.

When network signals “01”, “10” or “11” with RRC parameter MeasGapSharingScheme [2] and the value of X is defined as in Table 9.1.2.1a-1, and

-	Kintra = 1 / X * 100,

-	Kinter = 1 / (100 – X) * 100,

When network signals “00” indicating equal splitting gap sharing, X is not applied.

The RRC parameter MeasGapSharingScheme shall be applied to the calculation of carrier specific scaling factor as specified in clause 9.1A.5.2.1.

### 9.1A.3	UE Measurement capability

#### 9.1A.3.1	SA: Monitoring of multiple layers using gaps

The requirements in this clause are applicable for UE configured with SA NR operation mode.

When monitoring of multiple inter-RAT E-UTRAN carriers and inter-frequency NR carriers ([or without using gaps provided the UE supports such capability] or the effective MGRP is applied for per-FR measurement gap capable UE) is configured by PCell, the UE shall be capable of performing one measurement of the configured measurement type (SS-RSRP, SS-RSRQ, SS-SINR, E-UTRAN RSRP, E-UTRAN RSRQ, E-UTRAN RS-SINR measurements, etc.) of detected cells on all the layers.

For UE configured with the NR SA operation, the effective total number of frequencies, excluding the frequencies of the PCell being monitored, is Nfreq, SA, RedCap, which is defined as:

Nfreq, SA, RedCap = Nfreq, SA, NR, RedCap + Nfreq, SA, E-UTRA, RedCap ,

where

Nfreq, SA, E-UTRA, RedCap is the number of E-UTRA inter-RAT carriers being monitored (FDD and TDD) as configured by PCell,

Nfreq, SA, NR, RedCap is the number of NR inter-frequency carriers being monitored as configured by PCell.

#### 9.1A.3.2	SA: Maximum allowed layers for multiple monitoring

If a UE is configured with SA NR operation mode, the UE shall be capable of monitoring at least:

-	Depending on UE capability, 6 NR SSB inter-frequency carriers configured by PCell, and

-	Depending on UE capability, 7 NR inter-frequency carriers including SSB and CSI-RS in total configured by PCell, and

-	Depending on UE capability, 6 E-UTRA TDD inter-RAT carriers configured by PCell, and

-	Depending on UE capability, 6 E-UTRA FDD inter-RAT carriers configured by PCell.

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least [10] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, and E-UTRA TDD layers.

If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one SSB frequency layer.

### 9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria

#### 9.1A.4.1	Introduction

This clause contains requirements on UE capabilities for support of event triggering and reporting criteria. As long as the measurement configuration does not exceed the requirements stated in clause [9.1A.4.2], the UE shall meet all other performance requirements defined in clause 9 and clause 10.

The UE can be requested to make measurements under different measurement identities defined in TS 38.331 [2]. Each measurement identity corresponds to either event-based reporting, periodic reporting, or no reporting. In case of event-based reporting, each measurement identity is associated with an event triggering criterion. In case of periodic reporting, a measurement identity is associated with one periodic reporting criterion. In case of no reporting, a measurement identity is associated with one no reporting criterion.

The purpose of this clause is to set some limits on the number of different event triggering, periodic, and no reporting criteria the UE may be requested to track in parallel.

#### 9.1A.4.2	Requirements

In this clause a reporting criterion corresponds to either one event (in the case of event-based reporting), or one periodic reporting criterion (in case of periodic reporting), or one no reporting criterion (in case of no reporting). For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table [9.1A.4.2-1].

The UE shall be able to support in parallel per category up to Ecat reporting criteria according to Table [9.1A.4.2-1]. For the measurement categories belonging to intra-frequency, inter-frequency, and inter-RAT measurements (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the total number of reporting criteria as follows:

-	For UE configured with SA operation mode: , where

is the total number of NR reporting criteria according to Table [9.1A.4.2-1],

is the total number of inter-RAT E-UTRA reporting criteria according to Table [9.1A.4.2-1].

Table 9.1A.4.2-1: Requirements for reporting criteria per measurement category

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, and SS-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 1 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SS-RSRQ, and SS-SINRfor NG-RAN inter-frequency cells |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 1 | 10 | Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies. |
| NOTE 1: Applicable for UE configured with SA NR operation mode. |  |  |

### 9.1A.5	Carrier-specific scaling factor

This clause specifies the derivation of carrier-specific scaling factor (CSSF) values, which scales the measurement delay requirements given in clause 9.2B, 9.3Band 9.4A when UE is configured to monitor multiple measurement objects. The CSSF values are categorized into CSSFoutside_gap_RedCap,i and CSSFwithin_gap_RedCap,i, for the measurements conducted outside measurement gaps and within measurement gaps, respectively.

#### 9.1A.5.1	Monitoring of multiple layers outside gaps

The carrier-specific scaling factor CSSFoutside_gap_RedCap,i for measurement object i derived in this chapter is applied to following measurement types:

-	SSB-based intra-frequency measurement with no measurement gap in clause [9.2B.5], when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	SSB-based intra-frequency measurement with no measurement gap in clause [9.2B.5], when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	SSB-based inter-frequency measurement with no measurement gap in clause [9.3B.7], when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

-	SSB-based inter-frequency measurement with no measurement gap in clause [9.3B.7], when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps.

If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one SSB frequency layer.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFoutside_gap_RedCap,i and requirements derived from CSSFoutside_gap_RedCap,i are not specified.

##### 9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap_RedCap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, will be specified as follows:

- CSSFoutside_gap,i =1, if only one measurement object is configured to be measured outside of MG for RedCap.

- Otherwise, CSSFoutside_gap,i =2 for intra-frequency measurement, and CSSFoutside_gap,i = 2*Y for inter-frequency measurement with no measurement gap, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG.

Note: Only inter-frequency MOs without MG when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap are measured outside of MG.

#### 9.1A.5.2	Monitoring of multiple layers within gaps

The carrier-specific scaling factor CSSFwithin_gap_RedCap,i for a measurement object i derived in this chapter is applied to following measurement types:

-	SSB-based intra-frequency measurement object with no measurement gap in clause 9.2B.5, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	SSB-based intra-frequency measurement object with measurement gap in clause 9.2B.6.

-	SSB-based inter-frequency measurement object with measurement gap in clause 9.3B.4.

-	SSB-based inter-frequency measurement object without measurement gap for UE capable of interFrequencyMeas-NoGap in clause 9.3B.7, when

-	all of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, or

-		part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap.

-	E-UTRA Inter-RAT measurement object in clauses 9.4A.2 and 9.4A.3.

UE is expected to conduct the measurement of this measurement object i only within the measurement gaps.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFwithin_gap_RedCap,i and requirements derived from CSSFoutside_gap_RedCap,i are not specified.

##### 9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap_RedCap,i and is derived as described in this clause.

For each measurement gap j count the total number of intra-frequency measurement object and inter-frequency/inter-RAT measurement objects which are candidates to be measured within the gap j.

-	An NR measurement object with SSB measurement configured is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement object, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

-	Mintra_RedCap,i,j: Number of intra-frequency measurement objects, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter_RedCap,i,j : Number of NR inter-frequency layers and EUTRA inter-RAT, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter_RedCap,i,j  equals 0.

-	Mtot_RedCap,i,j = Mintra_RedCap,i,j + Minter_RedCap,i,j : Total number of intra-frequency, inter-frequency and inter-RAT frequncy layers, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot_RedCap,i,j equals 0.

The carrier specific scaling factor CSSFwithin_gap_RedCap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap_RedCap,i= max(ceil(Ri×Mtot_RedCap,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap_RedCap,i is the maximum among

-	ceil(Ri×Kintra×Mintra_RedCap,i,j) in gaps where Minter_RedCap,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra_RedCap,i,j) in gaps where Minter_RedCap,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object, CSSFwithin_gap_RedCap,i is the maximum among

-	ceil(Ri×Kinter×Minter_RedCap,i,j) in gaps where Mintra_RedCap,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter_RedCap,i,j) in gaps where Mintra_RedCap,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a.

### 9.1A.6	Minimum requirement at transitions

When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply.

The carrier-specific scaling factor specified in clause 9.1A.5 that applies to the other impacted measurement objects will also apply based on the longer measurement or cell identification delay before or after the transition.

When the UE transitions between DRX and non-DRX or when DRX cycle periodicity changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

Subsequent to this measurement period, the cell identification and measurement period requirements on each measurement object are corresponding to the second mode after transition.

## 9.2B NR intra-frequency measurements for RedCap

### 9.2B.1	Introduction

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP, if the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2].

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified intra-frequency cells if carrier frequency information is provided by PCell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE can perform intra-frequency SSB based measurements without measurement gaps if

-	the SSB is completely contained in the active BWP of the UE, or

-	the active downlink BWP is initial BWP [3].

For intra-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.2B.5.3.

SSB based measurements are configured along with one or two measurement timing configuration(s) (SMTC(s)) which provides periodicity, duration and offset information on a window of up to 5ms where the measurements are to be performed. For intra-frequency connected mode measurements, up to two measurement window periodicities may be configured. A single measurement window offset and measurement duration are configured per intra-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. Switching time is 0.5ms for frequency range FR1 and 0.25ms for frequency range FR2.

### 9.2B.2	Requirements applicability

The requirements in clause 9.2B apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	For 2Rx RedCap:

-	SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7 and 10.1.8 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12 and 10.1.13 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

-	For 1Rx RedCap:

-	SS-RSRP related side conditions given in clauses [x.y.z] and [x.y.z] for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses [x.y.z] and [x.y.z] for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses [x.y.z] and [x.y.z] for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex [x.y.z] for a corresponding Band.

### 9.2B.3	Number of cells and number of SSB

#### 9.2B.3.1	Requirements for FR1

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	8 identified cells, and

-	14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

#### 9.2B.3.2	Requirements for FR2

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band.

### 9.2B.4	Measurement Reporting Requirements

#### 9.2B.4.1	Periodic Reporting

For 2Rx RedCap: The requirements in clause 9.2.4.1 shall apply.

For 1Rx RedCap: Reported RSRP, RSRQ, and RS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2).

#### 9.2B.4.2	Event-triggered Periodic Reporting

For 2Rx RedCap: The requirements in clause 9.2.4.2 shall apply.

For 1Rx RedCap: Reported RSRP, RSRQ, and RS-SINR measurements contained in event-triggered periodic measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2).

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.2B.4.3.

#### 9.2B.4.3	Event Triggered Reporting

For 2Rx RedCap: The requirements in clause 9.2.4.3 shall apply.

For 1Rx RedCap: Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2).

The UE shall not send any event triggered measurement reports as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index_RedCap or T identify intra without index_RedCap defined in clause 9.2B.5.1 or clause 9.2B.6.2. When L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index_RedCap or Tidentify_intra_with_index_RedCap as defined in clause 9.2B.5.1 or clause 9.2B.6.2. If a cell which has been detectable at least for the time period Tidentify intra without index_RedCap or Tidentify intra with index_RedCap defined in clause 9.2B.5.1 or clause 9.2B.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra_RedCap provided the timing to that cell has not changed more than  3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

### 9.2B.5	Intra-frequency measurements without measurement gaps for RedCap

#### 9.2B.5.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index_RedCap if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_RedCap. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_RedCap. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index_RedCap = (TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap) ms

Tidentify_intra_with_index_RedCap = (TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TSSB_time_index_intra_RedCap) ms

Where:

TPSS/SSS_sync_intra_RedCap: it is the time period used in PSS/SSS detection given in tables 9.2B.5.1-1, 9.2B.5.1-2, 9.2B.5.1-3.

TSSB_time_index_intra_RedCap: it is the time period used to acquire the index of the SSB being measured given in tables 9.2B.5.1-4, 9.2B.5.1-5

T SSB_measurement_period_intra_RedCap: equal to a measurement period of SSB based measurement given in table 9.2B.5.2-1, table 9.2B.5.2-2, table 9.2B.5.2-3.

CSSFintra_RedCap: it is a carrier specific scaling factor and is determined

according to CSSFoutside_gap_RedCap,i in clause 9.1A.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps,  or according to CSSFwithin_gap_RedCap,i in clause 9.1A.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps.

if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

For 2Rx RedCap:

Mpss/sss_sync_w/o_gaps_RedCap : For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync_w/o_gaps_RedCap =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps_RedCap =24.  For a UE supporting FR2 power class 3, Mpss/sss_sync_w/o_gaps_RedCap =24. For a UE supporting FR2 power class 4, Mpss/sss_sync_w/o_gaps_RedCap =24, For a UE supporting FR2 power class 7, Mpss/sss_sync_w/o_gaps_RedCap = 24.

Mmeas_period_w/o_gaps_RedCap : For a UE supporting power class 1 or 5, Mmeas_period_w/o_gaps_RedCap =40. For a UE supporting FR2 power class 2, Mmeas_period_w/o_gaps_RedCap =24. For a UE supporting power class 3, Mmeas_period_w/o_gaps_RedCap =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps_RedCap =24, For a UE supporting power class 7, Mmeas_period_w/o_gaps_RedCap = 24.

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index_RedCap or Tidentify_intra_with_index_RedCap

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

Table 9.2B.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max( 600ms, ceil(5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2B.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps_RedCap x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_RedCap x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps_RedCap x Kp x Klayer1_measurement) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2B.5.1-3: Time period for PSS/SSS detection, (Frequency range FR1) for 1Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max( 600ms, ceil( 7 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max( 600ms, ceil(7 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(7 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2B.5.1-4: Time period for time index detection (FR1) for 2Rx RedCap UE

| DRX cycle | TSSB_time_index_intra_RedCap |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(120ms, ceil (3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2B.5.1-5: Time period for time index detection (FR1) for 1Rx RedCap UE

| DRX cycle | TSSB_time_index_intra_RedCap |
| --- | --- |
| No DRX | max(160ms, ceil( [6] x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(160ms, ceil ([6] x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | Ceil([6] x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

#### 9.2B.5.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 9.2B.5.2-1, 9.2B.5.2-2, 9.2B.5.2-3.

If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra_RedCap.

For FR2, a longer measurement period is allowed, if aperiodic CSI-RS resource is measured for L1-RSRP measurement on any FR2 serving frequency in the same band, and the CSI-RS resource is outside measurement gap and overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols. If SSB-ToMeasure or SS-RSSI-Measurement is configured, the SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same band which can be merged and the RSSI symbols are indicated by SS-RSSI-Measurement.

Table 9.2B.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2B.5.2-2: Measurement period for intra-frequency measurements without gaps (FR2) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps_RedCap x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_RedCap x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps_RedCap xKp x Klayer1_measurement ) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2B.5.2-3: Measurement period for intra-frequency measurements without gaps (FR1) for 1Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max([200]ms, ceil( [5] x Kp) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max([200]ms, ceil(1.5x [5] x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil( [5] x Kp ) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

#### 9.2B.5.3	Scheduling availability of UE during intra-frequency measurements

UE shall be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that the SSB symbols indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged [2], if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [3] are included.

##### 9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

##### 9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

##### 9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2

The following scheduling restriction applies due to SS-RSRP or SS-SINR measurement on an FR2 intra-frequency cell

The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration (The signaling deriveSSB_IndexFromCell is always enabled for FR2). If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

The following scheduling restriction applies to SS-RSRQ measurement on an FR2 intra-frequency cell

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration (The signaling deriveSSB_IndexFromCellc is always enabled for FR2). If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

If following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots.

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, the UE is expected to receive the PDCCH that the UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, the UE is expected to receive PDSCH that corresponds to the PDCCH that the UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured.

##### 9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1

When the UE performs intra-frequency measurements in a HD-FDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When the UE performs intra-frequency measurements in a HD-FDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

### 9.2B.6	Intra-frequency measurements with measurement gaps

#### 9.2B.6.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index_RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_RedCap. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_RedCap. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index_RedCap = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap  ms

Tidentify_intra_with_index_RedCap = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TSSB_time_index_intra_RedCap ms

Where:

TPSS/SSS_sync_intra_RedCap: it is the time period used in PSS/SSS detection given in table 9.2B.6.1-1 or 9.2B.6.1-2 or 9.2B.6.1-3.

TSSB_time_index_intra_RedCap: it is the time period used to acquire the index of the SSB being measured given in table 9.2B.6.1-4 or 9.2B.6.1-5.

T SSB_measurement_period_intra_RedCap: equal to a measurement period of SSB based measurement given in table 9.2B.6.2-1 or 9.2B.6.2-2 or 9.2B.6.2-3.

CSSFintra_RedCap: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap_RedCap,i in clause 9.1A.5.2 for measurement conducted within measurement gaps.

For 2Rx RedCap:

Mpss/sss_sync_with_gaps_RedCap : For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync with_gaps_RedCap =40. For a UE supporting FR2 power class 2, Mpss/sss_sync with_gaps_RedCap =24.  For a UE supporting FR2 power class 3, Mpss/sss_sync with_gaps_RedCap =24. For a UE supporting power class 4, Mpss/sss_sync with_gaps_RedCap =24, For a UE supporting FR2 power class 7, Mpss/sss_sync with_gaps_RedCap =24.

Mmeas_period_ with_gaps_RedCap: For a UE supporting power class 1 or 5, Mmeas_period_ with_gaps_RedCap =40. For a UE supporting power class 2, Mmeas_period_ with_gaps_RedCap =24. For a UE supporting power class 3, Mmeas_period_ with_gaps_RedCap =24. For a UE supporting power class 4, Mmeas_period with_gaps_RedCap =24, For a UE supporting power class 7, Mmeas_period_ with_gaps_RedCap =24.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index_RedCap or Tidentify_intra_with_index_RedCap.

Table 9.2B.6.1-1: Time period for PSS/SSS detection (FR1) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max(600ms, 5 x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600ms, 5 x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 9.2B.6.1-2: Time period for PSS/SSS detection (FR2) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max(600ms, Mpss/sss_sync_with_gaps_RedCap x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5x Mpss/sss_sync_with_gaps_RedCap) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra_RedCap_RedCap |
| DRX cycle>320ms | Mpss/sss_sync_with_gaps_RedCap x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 9.2B.6.1-3: Time period for PSS/SSS detection (FR1) for 1Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra_RedCap |
| --- | --- |
| No DRX | max(600 ms, 7 x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600 ms, 7 x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 9.2B.6.1-4: Time period for time index detection (Frequency range FR1) for 2Rx RedCap UE

| DRX cycle | TSSB_time_index_intra_RedCap |
| --- | --- |
| No DRX | max(120ms, 3 x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(120ms, 3 x max(MGRP, SMTC period,DRX cycle) x CSSFintra_RedCap) |
| DRX cycle>320ms | 3 x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 9.2B.6.1-5: Time period for time index detection (Frequency range FR1) for 1Rx RedCap UE

| DRX cycle | TSSB_time_index_intra_RedCap |
| --- | --- |
| No DRX | max(160ms, [6] x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(160ms, [6] x max(MGRP, SMTC period,DRX cycle) x CSSFintra_RedCap) |
| DRX cycle>320ms | [6] x max(MGRP, DRX cycle) x CSSFintra_RedCap |

#### 9.2B.6.2	Intra-frequency Measurement Period

The measurement period for FR1 intra-frequency measurements with gaps is as shown in table 9.2B.6.2-1 and in table 9.2B.6.2-3 for 1 Rx RedCap.

The measurement period for FR2 intra-frequency measurements with gaps is as shown in table 9.2B.6.2-2.

For either an FR1 or FR2 serving cell, longer measurement period would be expected during the period Tidentify_CGI_RedCap when the UE is requested to decode an NR CGI.

Table 9.2B.6.2-1: Measurement period for intra-frequency measurements with gaps (FR1) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max(200ms, 5 x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 9.2B.6.2-2: Measurement period for intra-frequency measurements with gaps (FR2) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max(400ms, Mmeas_period with_gaps_RedCap x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5 x Mmeas_period with_gaps_RedCap) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra_RedCap |
| DRX cycle>320ms | Mmeas_period with_gaps_RedCap x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 9.2B.6.2-3: Measurement period for intra-frequency measurements with gaps (FR1) for 1Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra_RedCap |
| --- | --- |
| No DRX | max([200] ms, [5] x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max([200] ms, ceil(1.5x [5]) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | [5] x max(MGRP, DRX cycle) x CSSFintra_RedCap |

----------------------END OF CHANGES----------------------------