Annex 1

3GPP 5G

The present Annex includes unwanted emission requirements from NR carriers for NR base stations. The requirements shall apply to BS that support NR, or NR with NB-IoT operation in NR in-band.

A NR base station is characterized by the ability of its receiver and transmitter to process only NR carriers or NR carriers with NB-IoT operation in NR in-band.

This Annex is outlined as follows:

–	Chapter 1 specifies the frequency ranges and operating bands for which the requirements in the present Annex apply.

–	Chapter 2 specifies definitions, symbols and abbreviations.

–	Chapter 3 includes the NR BS unwanted emission requirements for BS type 1-C and BS type 1-H.

–	Chapter 4 includes the NR BS unwanted emission requirements for BS type 1-O and BS type 2-O.

–	Chapter 5 specifies references.

Values specified in the present Annex incorporate test tolerances defined in Recommendation ITU-R M.1545 [4].

# 1	Operating bands

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to the present version of the specification are identified as described in Table 1-1.

TABLE 1-1

NR frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

The unwanted emission limits defined in the present Annex are for NR BS operating at least one of the bands in Table 1-2 or Table 1-3:

TABLE 1-2

NR operating bands in FR1

| NR operating band number | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n13 | 777 MHz – 787 MHz | 746 MHz – 756 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n24(7) | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n26 | 814 MHz – 849 MHz | 859 MHz – 894 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n30 | 2305 MHz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD(3) |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n53 | 2483.5 MHz – 2495 MHz | 2483.5 MHz – 2495 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n67 | N/A | 738 MHz – 758 MHz | SDL |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n85 | 698 MHz – 716 MHz | 728 MHz – 746 MHz | FDD |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n89 | 824 MHz – 849 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n91 | 832 MHz – 862 MHz | 1427 MHz – 1432 MHz | FDD(2) |
| n92 | 832 MHz – 862 MHz | 1432 MHz – 1517 MHz | FDD(2) |
| n93 | 880 MHz – 915 MHz | 1427 MHz – 1432 MHz | FDD(2) |
| n94 | 880 MHz – 915 MHz | 1432 MHz – 1517 MHz | FDD(2) |
| n95(1) | 2010 MHz – 2025 MHz | N/A | SUL |
| n96(4) | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD(3) |
| n97(5) | 2300 MHz – 2400 MHz | N/A | SUL |
| n98(5) | 1880 MHz – 1920 MHz | N/A | SUL |
| n99(6) | 1626.5 MHz -1660.5 MHz | N/A | SUL |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD |
| n102(4) | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD(3) |
| n104(8) | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD |
| (1) This band is applicable in China only. (2) Variable duplex operation does not enable dynamic variable duplex configuration by the network and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. (3) This band is restricted to operation with shared spectrum channel access as defined in TS 37.213 [20]. (4) This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. (5) The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. (6) UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. (7) DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. (8) [This band is applicable only in countries/regions designating this band for IMT licensed operation in accordance with RCC Recommendation 1/21.] |  |  |  |

TABLE 1-3

NR operating bands in FR2

| NR operating band number | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n259 | 39500 MHz – 43500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |
| n262 | 47200 MHz – 48200 MHz | TDD |
| n263 | 57000 MHz – 71000 MHz | TDD |

# 2	Definitions, symbols and abbreviations

## 2.1	Definitions

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in ITU-R SM.329 [3] used for the formulation of unwanted emission requirements for FR1

antenna connector: connector at the conducted interface of the BS type 1-C

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

NB-IoT operation in NR in-band: NB-IoT is operating in-band when it is located within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel.

Operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

NOTE:	The operating band(s) for a BS is declared by the manufacturer according to the designations in tables 1-2 and 1-3.

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply

[NOTE:	For requirements based on EIRP, the radiated interface boundary is associated to the far-field region]

TAB connector: transceiver array boundary connector

TAB connector RX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted RX requirements are applied

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with reception on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

TAB connector TX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted TX requirements are applied.

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

## 2.2	Symbols

FDL_low		Lowest frequency of the downlink operating band

FDL_high		Highest frequency of the downlink operating band

FUL_low		Lowest frequency of the uplink operating band

FUL_high		Highest frequency of the uplink operating band

ΔfOBUE		Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

NTXU,counted	The number of active transmitter units as calculated in clause 3.1, that are taken into account for conducted TX output power limit in clause 3.1.2, and for unwanted TX emissions scaling

NTXU,countedpercell	The number of active transmitter units that are taken into account for conducted TX emissions scaling per cell, as calculated in clause 3.1

## 2.3	Abbreviations

ACLR	      Adjacent Channel Leakage Ratio

CA	      Carrier Aggregation

CACLR      Cumulative ACLR

CLTA		Co-Location Test Antenna

DTT	      Digital terrestrial television

EIRP		Effective Isotropic Radiated Power

FR		Frequency Range

GPS	      The Global Positioning System

OTA		Over-The-Air

PHS		Personal Handyphone System

RIB		Radiated Interface Boundary

TAB		Transceiver Array Boundary

TRP		Total Radiated Power

## 2.4	Conducted and radiated reference points

## 2.5	Base station classes

Some requirements apply specifically to a specific BS class. The BS classes for NR are Wide Area Base Stations, Medium Range Base Stations and Local Area Base Stations.

BS classes for BS type 1-H are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum coupling loss equal to 45 dB.

BS classes for BS type 1-O and BS type 2-O are defined as indicated below:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

# 3	NR generic unwanted emission characteristics for BS type 1-C and BS type 1-H

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. Unwanted emissions outside of this frequency range are limited by a spurious emissions requirement.

The values of ΔfOBUE are defined in table 3-1 for the NR operating bands.

TABLE 3-1

Maximum offset of OBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-C | FDL_high – FDL_low 200 MHz | 10 |
|  | 200 MHz < FDL_high – FDL_low 900 MHz | 40 |
| BS type 1-H | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz FDL_high – FDL_low 900 MHz | 40 |

For band n46, n96 and n102, the values of ΔfOBUE are defined in table 3-2.

TABLE 3-2

Maximum offset of OBUE outside the downlink operating band

| Operating band | ΔfOBUE (MHz) |
| --- | --- |
| n46, n102 | 40 |
| n96 | 50 |

## 3.1	Conducted operating band unwanted emissions

### 3.1.1	Conducted Operating band unwanted emissions for BS type 1-C

The operating band unwanted emissions for BS type 1-C for each antenna connector shall be below the applicable basic limits defined in clauses 3.1.3 – 3.1.8.

For Band n41 and n90 operation in Japan, the operating band unwanted emissions limits shall be applied to the sum of the emission power over all antenna connectors for BS type 1-C.

### 3.1.2	Conducted Operating band unwanted emissions BS type 1-H

The operating band unwanted emissions for BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clauses 3.1.3 – 3.1.8, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the BS type 1-H emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector shall be less than or equal to the BS type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

### 3.1.3	Basic limits for Wide Area BS (Category A)

For BS operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n71, n85, basic limits are specified in table 6.6.4.5.2-1 of TS 38.141-1 [1].

For BS operating in Bands n1, n2, n3, n7, n24, n25, n30, n34, n38, n39, n40, n41, n50, n65, n66, n70, n74, n75, n92, n94, basic limits are specified in table 6.6.4.5.2-2 of TS 38.141-1 [1]:

For BS operating in Bands n48, n77, n78, n79, basic limits are specified in table 6.6.4.5.2-3 of TS 38.141-1 [1].

### 3.1.4	Basic limits for Wide Area BS (Category B)

For Category B Operating band unwanted emissions, there are two options for the basic limits that may be applied regionally. Either the basic limits in clause 6.6.4.2.2.1 or clause 6.6.4.2.2.2 shall be applied.

#### 3.1.4.1	Category B requirements (Option 1)

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, basic limits are specified in table 6.6.4.5.3.1-1 of TS 38.141-1 [1].

For BS operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n50, n65, n66, n70, n75, n92, n94, basic limits are specified in tables 6.6.4.5.3.1-2 of TS 38.141-1 [1].

For BS operating in Bands n48, n77, n78, n79, basic limits are specified in table 6.6.4.5.3.1-3 of TS 38.141-1 [1].

#### 3.1.4.2	Category B requirements (Option 2)

The limits in this clause are intended for Europe and may be applied regionally for BS operating in Bands n1, n3, n7, n8, n38, n65 or n101.

For a BS operating in Bands n1, n3, n8, n65 or BS type 1-C operating in bands n7, n38 or n101 basic limits are specified in table 6.6.4.5.3.2-1 of TS 38.141-1 [1].

### 3.1.5	Basic limits for Medium Range BS (Category A and B)

For Medium Range BS in NR bands ≤ 3 GHz, basic limits are specified in table 6.6.4.5.4-1 and table 6.6.4.5.4-2 of TS 38.141-1 [1].

For Medium Range BS in NR bands > 3 GHz, basic limits are specified in table 6.6.4.5.4-3 and table 6.6.4.5.4-4 of TS 38.141-1 [1].

### 3.1.6	Basic limits for Local Area BS (Category A and B)

For Local Area BS in NR bands ≤ 3 GHz, basic limits are specified in table 6.6.4.5.5-1 of TS 38.141-1 [1].

For Local Area BS in NR bands > 3 GHz, basic limits are specified in table 6.6.4.5.5-2 of TS 38.141-1 [1].

### 3.1.7	Basic limits for Local Area and Medium Range BS for band n46, n96 and n102 (Category A and B)

For Local Area and Medium Range BS operating in Band n46, basic limits for 10 MHz channel bandwidth are specified in table 6.6.4.5.5A-1 of TS 38.141-1 [1]. For Local Area and Medium Range BS operating in Band n46, n96 and Band n102, basic limits for 20 MHz, 40 MHz, 60 MHz and 80 MHz channel bandwidth are specified in table 6.6.4.5.5A-2 of TS 38.141-1 [1]. For one non-transmitted channel basic limits are specified in table 6.6.4.5.5A-3 of TS 38.141-1 [1], and for two non-transmitted channels basic limits are specified in table 6.6.4.5.5A-4 of TS 38.141-1 [1].

In the case of one or two non-transmitted 20 MHz channels between transmitted channels, when a NR-U channel bandwidth of 60 MHz or 80 MHz have been assigned, the spectrum emission mask for non-transmitted channels specified in table 6.6.4.5.5A-3 and table 6.6.4.5.5A-4 of TS 38.141-1 [1] applies for one and two non-transmitted channels respectively. The relative power of any BS emission shall not exceed the most stringent levels given by table 6.6.4.5.5A-2 and table 6.6.4.5.5A-3 of TS 38.141-1 [1]in the case of non-transmitted channels between transmitted channels.

### 3.1.8	Basic limits for additional requirements

In certain regions the following additional requirements may apply:

– 	For protection of DTT, the requirement in clause 6.6.4.5.6.2 of TS 38.141-1 [1].

–	For operation in Band n48, the requirement in clause 6.6.4.5.6.3 of TS 38.141-1 [1].

–	For operation in Band n53, the requirement in clause 6.6.4.5.6.4 of TS 38.141-1 [1].

–	For protection of GPS, the requirement in clause 6.6.4.5.6.5 of TS 38.141-1 [1].

–	For operation with shared spectrum channel access, the requirement in clause 6.6.4.5.6.6 of TS 38.141-1 [1].

## 3.2	Conducted Adjacent channel leakage ratio (ACLR)

## 3.3	Conducted Cumulative adjacent channel leakage ratio (CACLR)

## 3.4	Conducted Transmitter spurious emissions

## 3.5	Conducted receiver spurious emissions

# 4	NR generic unwanted emission characteristics for BS type 1-O and BS type 2-O

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The value of ΔfOBUE is defined in table 4-1 for BS type 1-O and BS type 2-O for the NR operating bands.

TABLE 4-1

Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL_high – FDL_low ≤ 900 MHz | 40 |
| BS type 2-O | FDL_high – FDL_low ≤ 4000 MHz | 1500 |

## 4.1	OTA Operating band unwanted emissions

### 4.1.1	OTA Operating band unwanted emissions BS type 1-O

For BS type 1-O, for a RIB operating in multi-carrier or contiguous CA, the requirements apply to BS channel bandwidths of the outermost carrier. In addition, for a RIB operating in non-contiguous spectrum, the requirements shall apply inside any sub-block gap. In addition, for a multi-band RIB, the requirements shall apply inside any Inter RF Bandwidth gap.

#### 4.1.1.1	Wide Area BS (Category A)

For a RIB operating in Bands n5, n8, n12, n13, n14, n26, n28, n29, n71, n85, emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.1-1 of TS 38.141-2 [2].

For a RIB operating in Bands n1, n2, n3, n7, n25, n30, n34, n38, n39, n40, n41, n50, n65, n66, n70, n74, n75, n77, n78, n79, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.1-2 to 6.7.4.5.1.1-4 of TS 38.141-2 [2].

#### 4.1.1.2	Wide Area BS Category B (Option 1)

For a RIB operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.2-1 of TS 38.141-2 [2].

For a RIB operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n50, n65, n66, n70, n75, n77, n78, n79, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.2-2 to 6.7.4.5.1.2-4 of TS 38.141-2 [2].

#### 4.1.1.3	Wide Area BS Category B (Option 2)

The limits in this clause are intended for Europe and may be applied regionally for a RIB operating in bands n1, n3, n8 and n65.

For a RIB operating in bands n1, n3, n8, n65 emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.3-1 of TS 38.141-2 [2].

#### 4.1.1.4	Medium Range BS (Category A and Category B)

For Medium Range BS class in NR bands ≤ 3 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.4-1 and 6.7.4.5.1.4-4 of TS 38.141-2 [2].

For Medium Range BS class in 3GHz <NR bands ≤ 4.2 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.4-2 and 6.7.4.5.1.4-5 of TS 38.141-2 [2].

For Medium Range BS class in 4.2GHz <NR bands ≤ 6 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.4-3 and 6.7.4.5.1.4-6 of TS 38.141-2 [2].

#### 4.1.1.5	Local Area BS (Category A and Category B)

For Local Area BS class in NR bands ≤ 3 GHz, emissions shall not exceed the maximum levels specified in table 6.7.4.5.1.5-1 of TS 38.141-2 [2].

For Local Area BS class in 3 GHz < NR bands ≤ 4.2 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.5-2 of TS 38.141-2 [2].

For Local Area BS class in 4.2 GHz < NR bands ≤ 6 GHz, emissions shall not exceed the maximum levels specified in tables 6.7.4.5.1.5-3 of TS 38.141-2 [2].

#### 4.1.1.6	Additional requirements

In certain regions the following additional requirements may apply:

– 	For protection of DTT, the requirement in clause 6.7.4.5.1.6.2 of TS 38.141-1 [1].

– 	For Protection of GPS, the requirement in clause 6.7.4.5.1.6.3 of TS 38.141-1 [1].

– 	For operation in Bands n50, n51, n74, n75, n76, the requirement in clause 6.7.4.5.1.6.4 of TS 38.141-1 [1].

### 4.1.2	OTA Operating band unwanted emissions BS type 2-O

For BS type 2-O, for a RIB operating in multi-carrier or contiguous CA, the requirements apply to the frequencies starting from the edge of the contiguous transmission bandwidth. In addition, for a RIB operating in non-contiguous spectrum, the requirements apply inside any sub-block gap.

#### 4.1.2.1	OTA operating band unwanted emission limits (Category A)

The power of unwanted emission shall not exceed the limits in table 6.7.4.5.2.2-1, 6.7.4.5.2.2-2 or 6.7.4.5.2.2-3 of TS 38.141-2 [2].

#### 4.1.2.2	OTA operating band unwanted emission limits (Category B)

The power of unwanted emission shall not exceed the limits in table 6.7.4.5.2.3-1, 6.7.4.5.2.3-2 or 6.7.4.5.2.3-3 of TS 38.141-2 [2].

#### 4.1.2.3	Additional OTA operating band unwanted emission limits

The following additional requirement shall apply:

– 	For protection of Earth Exploration Satellite Service, the requirement in clause 6.6.4.5.6.2 of TS 38.141-1 [1].

## 4.2	OTA Adjacent channel leakage ratio (ACLR)

## 4.3	OTA Cumulative adjacent channel leakage ratio (CACLR)

## 4.4	OTA Transmitter spurious emissions

## 4.5	OTA receiver spurious emissions

# 5	References

[1]	3GPP TS 38.141-1 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing (Release 17)”.

[2]	3GPP TS 38.141-2 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing (Release 17)”.

[3]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[4]	ITU-R Recommendation M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[5]	3GPP TS 37.213 v17.2.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Physical layer procedures for shared spectrum channel access (Release 17)”.

Attachment 1 
to Annex 1

Definition of test tolerance

Test tolerance

With reference to Recommendation ITU-R M.1545 [4], “test tolerance” is the relaxation value referred to in recommends 2 of Recommendation ITU-R M.1545, i.e. the difference between the core specification value and the test limit, evaluated applying the shared risk principle as per Figure 2 and Figure 3 of Annex 1 of Recommendation ITU-R M.1545. In case the core specification value is equal to the test limit (Figure 3 of Annex 1 of Recommendation ITU-R M.1545) the “test tolerances” are equal to 0 dB.