3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211600

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.8.1.1

Source: 	Qualcomm Incorporated

Title: 	On SRS antenna switching Test

Document for:	Discussion

# Introduction

In this contribution, we present our view on SRS antenna port switching test scope and configurations.

# Discussion

## Test Configuration

SRS carrier switching tests have the following TDD configurations:

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 |  |  |
| referenceSubcarrierSpacing | kHz | 15 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=’10DL:2GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 10 |  |  |
| nrofUplinkSlot |  | 2 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| Note 1: As specified in TS 38.213 [3] and TS 38.331 [2]. Note 2: For information |  |  |  |  |

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 |  |  |
| referenceSubcarrierSpacing | kHz | 30 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=’6DL:4GP:4UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 6 |  |  |
| nrofUplinkSlot |  | 4 |  |  |
| nrofUplinkSymbols |  | 4 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 2 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| Note 1: As specified in TS 38.213 [3] and TS 38.331 [2]. Note 2: For information |  |  |  |  |

We can apply the UL/DL slot pattern to SRS antenna switching test. However, since the start position is 5, we need 6 UL symbols in the special slot to accommodate SRS symbols. Therefore, we propose the following special slot configurations:

Proposal 1: Special slot configuration:

| SCS | Special slot configuration |
| --- | --- |
| 15kHz | S = ‘6DL: 2GP: 6UL’ |
| 30kHz | S = ‘4DL: 4GP: 6UL’ |

Based on the previous meeting WF [1]: Offset is determined by TDD pattern: consider special slot for SRS transmission, we proposed the following offset configuration for SRS in different configurations:

Proposal 2: In order to have SRS antenna switching on special slots, we propose the following tables for test configurations of each SRS, scenario and xTyR:

| 15kHz, scenario 1 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl40, 1 | sl40, 5 | sl40, 1 | sl40, 5 | sl40, 1 | sl40, 5 | sl40, 9 | sl40, 13 |

| 15kHz, scenario 2 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | [2] | 5 | [2] | 5 | [2] | 5 | [2] |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 5 | sl40, 5 |

| 30kHz, scenario 1 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl80, 3 | sl80, 11 | sl80, 3 | sl80, 11 | sl80, 3 | sl80, 11 | sl80, 19 | sl80, 27 |

| 30kHz, scenario 2 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | [2] | 5 | [2] | 5 | [2] | 5 | [2] |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 11 | sl80, 11 |

Observation 1: In practice, network schedules SRS carrier switching mostly on special slots, same as SRS antenna switching. In addition, since the start position is 5, we need at least 6 UL symbols in a special slot to allocate SRS symbols. It is also beneficial to align SRS carrier switching and antenna switching test configuration from test implementation perspective.

Proposal 3: Apply srs-ResourceId 0 offset configuration from proposal 2 and special slot configuration from proposal 1 to SRS carrier switching tests.

# Conclusion

Proposal 1: Special slot configuration to accommodate 6 SRS symbols:

| SCS | Special slot configuration |
| --- | --- |
| 15kHz | S = ‘6DL: 2GP: 6UL’ |
| 30kHz | S = ‘4DL: 4GP: 6UL’ |

Proposal 2: In order to have SRS antenna switching on special slots, we propose the following tables for test configurations of each SRS, scenario and xTyR:

| 15kHz, scenario 1 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl40, 1 | sl40, 5 | sl40, 1 | sl40, 5 | sl40, 1 | sl40, 5 | sl40, 9 | sl40, 13 |

| 15kHz, scenario 2 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | [2] | 5 | [2] | 5 | [2] | 5 | [2] |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 1 | sl40, 5 | sl40, 5 |

| 30kHz, scenario 1 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl80, 3 | sl80, 11 | sl80, 3 | sl80, 11 | sl80, 3 | sl80, 11 | sl80, 19 | sl80, 27 |

| 30kHz, scenario 2 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | [2] | 5 | [2] | 5 | [2] | 5 | [2] |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 3 | sl80, 11 | sl80, 11 |

Observation 1: In practice, network schedules SRS carrier switching mostly on special slots, same as SRS antenna switching. In addition, since the start position is 5, we need at least 6 UL symbols in a special slot to allocate SRS symbols. It is also beneficial to align SRS carrier switching and antenna switching test configuration from test implementation perspective.

Proposal 3: Apply srs-ResourceId 0 offset configuration from proposal 2 and special slot configuration from proposal 1 to SRS carrier switching tests.

# Reference

[1] R4-2210996