3GPP TSG-RAN4 Meeting #104-e	R4-2212089

E-meeting, 15 – 26 August, 2022

Source: 	Ligado Networks, Nokia, Nokia Shanghai Bell

Title: 	System Parameters for the new LTE TDD Band in 1670 – 1675 MHz

Agenda item:	12.3.1

Document for: 	Approval

# Introduction

In RAN#95e, a new work item (WI) to introduce a new LTE TDD band in 1670 – 1675 MHz was approved [1].  The following objectives related to UE aspects were specified:

- Specify UE/BS RF requirements for a new LTE TDD operating band:

  - BS transmit frequency range: 1670 MHz –1675 MHz, UE transmit frequency range: 1670 MHz–1675 MHz

  - Channel bandwidths of 1.4, 3, 5 MHz

- This new LTE band is expected to be release independent.

This document proposes system parameters for the new band.

Note: After the agreement on assignment of band number 105 to the new LTE TDD band in 1670 – 1675 MHz at RAN4#103-e, in an offline discussion it was suggested that since the new band is a TDD band it might be better to assign number #54 since it is available instead of #105. Therefore, the new band is being highlighted as 54 or 105 pending agreement within 3GPP on the number to use for this band.

# Proposed System Parameters

## Operating Band and Channel Bandwidths

In RAN4#103-e [2], it was also agreed to assign band number 105 for the new band since n104 was assigned to a TDD band operating in 6425 – 7125 MHz [3].

Table 1: E-UTRA Operating Band definition for the new LTE TDD Band in 1670 – 1675 MHz

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 54 or 105 | 1670 MHz | – | 1675 MHz | 1670 MHz | – | 1675 MHz | TDD |

Table 2: E-UTRA channel bandwidth for the new LTE TDD Band in 1670 – 1675 MHz

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 54 or 105 | Yes | Yes | Yes |  |  |  |

Proposal 1: Approve the proposed system parameters for operating band and channel bandwidths for the new LTE TDD band in 1670 – 1675 MHz so that draft CR(s) can be submitted at the RAN4#104-bis-e meeting for endorsement.

## EARFCN

In an offline discussion post RAN4#103e meeting, since there are unused TDD band numbers after Band 53 it was suggested to revise the band number assignment for the new TDD band in 1670 – 1675 MHz to Band 54.

Depending on the Band number assignment, there are two options to allocate EARFCN for the new LTE TDD band in 1670 – 1675 MHz.

Option 1

Table 3: Option 1 for E-UTRA channel number assignment for the new LTE TDD Band

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 103 | 757 | 70646 | 70646 – 70655 | 787 | 134282 | 134282 – 134291 |
| 105 | 1670 | 70656 | 70656 – 70705 | 1670 | 70656 | 70656 – 70705 |

Option 2

Table 4: Option 2 for E-UTRA channel number assignment for the new LTE TDD Band

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 53 | 2483.5 | 60140 | 60140 - 60254 | 2483.5 | 60140 | 60140 - 60254 |
| 54 | 1670 | 60255 | 60255 – 60304 | 1670 | 60255 | 60255 – 60304 |

Proposal 2: At RAN4#104-e, select and agree to either Option 1 (Table 3) or Option 2 (Table 4) for the EARFCN range for the new LTE TDD Band in 1670 – 1675 MHz so that draft CR(s) can be submitted at the RAN4#104-bis-e meeting for endorsement.

# Proposals

Proposal 1: Approve the proposed system parameters for operating band and channel bandwidths for the new LTE TDD band in 1670 – 1675 MHz so that draft CR(s) can be submitted at the RAN4#104-bis-e meeting for endorsement.

Proposal 2: Select and agree to either Option 1 (Table 3) or Option 2 (Table 4) for the EARFCN range for the new LTE TDD Band in 1670 – 1675 MHz so that draft CR(s) can be submitted at the RAN4#104-bis-e meeting for endorsement.

# References

[1] RP-220971, New WID proposal for Introduction of LTE TDD band in 1670 – 1675  MHz, RAN#95-e

[2] R4-2210572, WF on Introduction of LTE TDD band in 1670-1675 MHz, RAN4#103-e

[3] TS 38.101-1, v17.6.0, NR: UE Radio transmission and reception, June 2022