3GPP TSG-RAN4 Meeting #104-e	R4-2212263

# Online meeting,  15 August – 26 August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | 0015 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.106: NR repeater receiver spurious emissions requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For repeater type 1-C, concept of “basic limit” is not applicable. For receiver spurious emission requirements, requirements for frequency range over 12.75 GHz may be duplicated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “basic limits” to “minimum requirements” Remove the receiver spurious emissions requirement for frequency range of 12.75 – 26 GHz from the requirement table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Receiver spurious emissions requirements are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------ Start of change -----------

6.5.5	Receiver spurious emissions

6.5.5.1	General

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at the antenna connector. The requirements only apply to repeater type 1-C for TDD operation.

For each antenna connectors on BS-side and UE-side supporting both RX and TX in TDD, the requirements apply during the transmitter OFF state. For antenna connectors both BS-side and UE-side in FDD, the RX spurious emissions requirements are superseded by the TX spurious emissions requirements, as specified in clause 6.5.4.

For multi-band connectors that both transmit and receive in operating band supporting TDD, RX spurious emissions requirements are applicable during the TX OFF state, and are subject to exclusion zones in each supported operating band.

For Band n41 and n90 operation in Japan, the sum of receiver spurious emissions requirements over all antenna connectors for repeater type 1-C shall not exceed minimum requirements defined in clause 6.5.5.2.

6.5.5.2	Minimum requirements

The receiver spurious emissions requirements for repeater type 1-C are that for each antenna connector, the power of emissions shall not exceed the value specified in table 6.5.5.2-1.

Table 6.5.5.2-1: Repeater type 1-C receiver spurious emissions minimum requirements

| Spurious frequency range | Minimum requirements | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz | -47 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| - |  |  |  |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the UL operating band is reaching beyond 12.75 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the repeater transmitter operating band to ΔfOBUE above the highest frequency of the repeater transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.5.1. For multi-band connectors, the exclusion applies for all supported operating bands. |  |  |  |

------ End of change -----------