3GPP TSG-RAN WG4 Meeting #104-e                                               R4-2212556

Electronic Meeting, 15 Aug. – 26 Aug. 2022

Agenda Item: 	9.10.1.3

Source:	Ericsson

Title:	Simulation results for CRS-IM

Document for:	Information

# 1	Introduction

In this contribution, we bring our alignment and impairment results for 15kHz and 30kHz SCS scenarios.

Results for 15kHz SCS scenario are based on the simulation assumption [1].

Results for 30kHz SCS scenario are based on the simulation assumption [2].

# 2	Discussion

## 2.1 15kHz SCS scenario

## 2.1.1 FDD

Summary:

Table 1 Test cases for 15kHz SCS scenario FDD

| Case Number | Scenario | MIMO | Alignment result | Impairment result |
| --- | --- | --- | --- | --- |
| Case 1 | Scenario 1 | 2Tx 2Rx Low | 9.2dB | 11.2dB |
| Case 2 |  | 2Tx 4Rx Low | 4.9dB | 6.9dB |
| Case 5 | Scenario 2 | 4Tx 2Rx Low | 9.0dB | 10.5dB |
| Case 6 |  | 4Tx 4Rx Low | 4.7dB | 6.2dB |

## 2.1.2 TDD

Summary:

Table 2 Test cases for 15kHz SCS scenario TDD

| Case Number | Scenario | MIMO | Alignment result | Impairment result |
| --- | --- | --- | --- | --- |
| Case 3 | Scenario 1 | 4Tx 2Rx Low | 10.4dB | 12.4dB |
| Case 4 |  | 4Tx 4Rx Low | 6.9dB | 8.9dB |
| Case 5 | Scenario 2 | 4Tx 2Rx Low | 9.6dB | 11.1dB |
| Case 6 |  | 4Tx 4Rx Low | 5.4dB | 6.9dB |

## 2.2 30kHz SCS scenario for TDD

Summary:

Table 3 Test cases for 30kHz SCS scenario TDD

| Case Number | Scenario | Interference cell loading level | CBW & SCS | MIMO | Baseline | CRS-IM |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | Scenario 2 | 10% | 20MHz, 30kHz | 4Tx 2Rx Low | 9.59dB | 8.4dB |
| Case 2 | Scenario 2 | 10% | 20MHz, 30kHz | 4Tx 4Rx Low | 6.4dB | 5.1dB |

# 3	Summary

In this contribution, we provided our alignment and impairment results for the TDD/FDD for 15kHz SCS scenario and TDD for 30kHz SCS scenario.

# References

[1]	R4-2210923 Simulation assumptions for CRS-IM (15 kHz FDD and TDD), China Telecom

[2]	R4-2210658 Simulation assumptions for CRS-IM (30 kHz SCS TDD), CMCC