3GPP TSG-RAN4 Meeting #104-e	R4-2212656

# Electronic meeting, August 15-26, 2022

Source:			Ericsson

Title:			LS Reply to RAN1 on UE antenna gain

Agenda item:		13.1.3

Document for:		Approval

# Introduction

In the scope of the NR_NTN_enh WI, RAN1 has sent a LS to RAN4 ([1]) requesting for information on the UE antenna gain value.

This contribution further elaborates on a realistic UE antenna gain value, LS Reply to RAN1 is proposed in annex 4.

# Discussion

## Context

RAN1 has sent the following request to RAN4 in its LS ([1]), asking RAN4 if -5dBi UE antenna gain would be a more realistic value to consider for their link level evaluation in the scope of NR NTN enhancements WI.

## UE antenna gain discussion

In the following, as assumed for the Rel-17 NTN satellite UE specification, we are assuming a “typical” handheld UE also supporting TN operations, without any external antenna.

RAN4 usually assumes a 0 dBi UE antenna gain for all system simulations and when specifying UE RF Rx requirements in TS 36.101/38.101-1:

Nevertheless, some studies made more or less recently investigated UE antenna performance, one of them being the one from Aalborg University showing a wide spread of measurement results (TRP/TRS) over the available phone on the market at that time, highlighting consequences on the perceived coverage. The outcomes of those studies have even triggered the attention of some European Administrations who were getting concerned on some UE antenna bad performance. This ended up with the European Commission mandating the specification of UE OTA parameters for the EU Certification, this to ensure UE antenna performance was also considered for the mobile certification.

Since then, other studies have been carried out and their outcomes are available on the web.

Also, UE antenna gain highly depends on:

- UE design, i.e., where the antenna is placed in the mobile.

- The operating frequency.

The gain might have several dBis difference depending on those criteria.

Some information is also available in FCC ID database, where antennas gain value for the certified FCC UEs is mentioned in the measurement reports. It should be noted that, for some recent mobiles, the antenna gain might be relatively high (from -4 dBi up to +2 dBi, depending on the considered band).

Based on the above consideration, considering all types and all generations of UEs we think a realistic UE antenna gain should be in the -3/-5 dBi values range.

It should also be noted that RAN1 is as

A LS Reply is proposed in the following Annex 4.

# References

- R1-2205623, LS on UE antenna gain for NR NTN coverage enhancement, NTT Docomo

# Annex: LS Reply to RAN1

3GPP TSG-RAN WG4 Meeting #104-e	R4-2xxxx

# Electronic meeting, August 15-26, 2022

Title:	LS Reply on UE antenna gain for NR NTN coverage enhancement

Response to:	LS on UE antenna gain for NR NTN coverage enhancement

Release:	Rel-18

Work Item:	-

Source:	RAN WG4

To:	RAN WG1

Cc:	-

Contact person:	Dominique Everaere

dominique.everaere@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	-

# 1	Overall description

RAN4 would like to thank RAN1 for the LS R1-2205623 on UE antenna gain for NR NTN coverage enhancement. Regarding the following action:

RAN1 respectfully asks RAN4 to provide feedback on the above question and any additional information that may help RAN1 determine the realistic assumptions on smartphone antenna gain.

RAN4 has discussed this topic in RAN4#104-e meeting.

The UE antenna gain varies depending on the operating frequency and UE design, where the antenna is placed. If previous generations of UEs had antenna gain much lower than 0 dBi, more recent generations have improved gain (at least for some of them).

Based on the information available on various websites (e.g., FCC certification database) and after further consideration, RAN4 thinks that a realistic UE antenna gain value would be in the range of [-3 ;-5] dBi. RAN4 would then recommend RAN1 then to take [-4] dBi as an assumption for their link budget evaluation.

# 2	Actions

To RAN WG1

ACTION: 	RAN WG4 respectfully asks RAN WG1 to take the above information into account when assuming a realistic UE antenna gain for their link study in the scope of the NT NTN enhancements WI.

# 3	Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #104bis-e	October 10-19, 2022	Online

TSG-RAN4 Meeting #105	November 14 – 18, 2022	Canada