3GPP TSG-RAN WG4 Meeting #104-e	R4-2213090

Electronic Meeting, 15 August – 26 August 2022

Source:	Ericsson, Verizon

Title:	TP for TR 36.718-02-01: CA_13-48

Agenda item:			12.1.3.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 36.718-02-01 to add dual UL for existing CA_13-48 according to the request in [1].

# 2. Reference

- RP-221831, New WID: Rel-18 LTE Advanced Carrier Aggregation for x bands (x<= 6) DL with y bands (y=1, 2) UL, Huawei, HiSilicon

# 3. Text Proposal

---Start of changes---

## 5.3.x	CA_13-48

### 5.3.x.1	Channel bandwidths per operating band for CA

Table 5.3.x.1-1: E-UTRA CA configurations and bandwidth combination sets

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_13A-48A | CA_13A-48A | 13 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_13A-48C | CA_13A-48A | 13 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_13A-48D | CA_13A-48A | 13 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 48 | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 5.3.x.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.x.2-1.

Table 5.3.x.2-1: Harmonic and IMD analysis

| UE UL carriers |  |  | fx_low |  |  | fx_high |  |  | fy_low |  |  | fy_high |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) |  | 777 |  |  | 787 |  |  | 3550 |  |  | 3700 |  |
| 2nd harmonics frequency limits |  |  | 2*fx_low |  |  | 2*fx_high |  |  | 2* fy_low |  |  | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 |  |  | 1574 |  |  | 7100 |  |  | 7400 |  |  |
| 3rd harmonics frequency limits |  |  | 3*fx_low |  |  | 3*fx_high |  |  | 3* fy_low |  |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) |  |  | 2331 |  |  | 2361 |  |  | 10650 |  |  | 11100 |
| Two tone 2nd order IMD products |  |  | ¦fy_high – fx_low¦ |  |  | ¦fy_low – fx_high¦ |  |  | ¦fy_low + fx_low¦ |  |  | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) |  |  | 2923 |  |  | 2763 |  |  | 4327 |  |  | 4487 |
| Two-tone 3rd order IMD products |  |  | ¦fy_high – 2*fx_low¦ |  |  | ¦fy_low – 2*fx_high¦ |  |  | ¦2*fy_low – fx_high¦ |  |  | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) |  |  | 2146 |  |  | 1976 |  |  | 6313 |  |  | 6623 |
| Two-tone 3rd order IMD products |  |  | ¦2*fx_low + fy_low¦ |  |  | ¦2*fx_high + fy_high¦ |  |  | ¦2*fy_low + fx_low¦ |  |  | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) |  |  | 5104 |  |  | 5274 |  |  | 7877 |  |  | 8187 |
| Two-tone 4th order IMD products |  |  | ¦2*fx_low –2* fy_high¦ |  |  | ¦2*fx_high – 2*fy_low¦ |  |  | ¦2*fx_low +2* fy_low¦ |  |  | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) |  |  | 5846 |  |  | 5526 |  |  | 8654 |  |  | 8974 |
| Two-tone 4th order IMD products |  |  | ¦3*fx_low –1* fy_high¦ |  |  | ¦3*fx_high – 1*fy_low¦ |  |  | ¦3*fy_low – 1*fx_high¦ |  |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) |  |  | 1369 |  |  | 1189 |  |  | 9863 |  |  | 10323 |
| Two-tone 4th order IMD products |  |  | ¦3*fx_low +1* fy_low¦ |  |  | ¦3*fx_high +1* fy_high¦ |  |  | ¦3*fy_low + 1*fx_low¦ |  |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) |  |  | 5881 |  |  | 6061 |  |  | 11427 |  |  | 11887 |
| Two-tone 5th order IMD products |  |  | ¦fx_low – 4*fy_high¦ |  |  | ¦fx_high – 4*fy_low¦ |  |  | ¦fy_low – 4*fx_high¦ |  |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) |  |  | 14023 |  |  | 13413 |  |  | 402 |  |  | 592 |
| Two-tone 5th order IMD products |  |  | ¦fx_low + 4*fy_low¦ |  |  | ¦fx_high + 4*fy_high¦ |  |  | ¦fy_low + 4*fx_low¦ |  |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) |  |  | 14977 |  |  | 15587 |  |  | 6658 |  |  | 6848 |
| Two-tone 5th order IMD products |  |  | ¦2*fx_low – 3*fy_high¦ |  |  | ¦2*fx_high – 3*fy_low¦ |  |  | ¦2*fy_low – 3*fx_high¦ |  |  | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) |  |  | 9546 |  |  | 9076 |  |  | 4739 |  |  | 5069 |
| Two-tone 5th order IMD products |  |  | ¦2*fx_low + 3*fy_low¦ |  |  | ¦2*fx_high + 3*fy_high¦ |  |  | ¦2*fy_low + 3*fx_low¦ |  |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) |  |  | 12204 |  |  | 12674 |  |  | 9431 |  |  | 9761 |

Based on analysis on above table, there is no IMD issue for this combination.

Table 5.3.x.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_13-48 |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 29, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 24, 30 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | E-UTRA band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |

### 5.3.x.3	∆TIB and ∆RIB values

Already included in TS 36.101.

### 5.3.x.4	REFSENS requirements

Based on analysis of 5.3.x.2, there are no additional MSD requirements for this combination.

---End of changes---