3GPP TSG-RAN WG4 Meeting #104-e	R4-2213551

Electronic Meeting, 15 – 26 August, 2022

Title: 	Simulation results for PRS-RSRPP

Source: 	Huawei, HiSilicon

Agenda item:	9.19.2.2.3

Document for:	Discussion

Introduction

Measurement accuracy for PRS-RSRPP are discussed in RAN4#103-e, and the outcomes are captured in WF [1], and the following are agreed.

Issue 4-4-1: Need for PRS-RSRPP measurement accuracy Agreements: - PRS-RSRPP measurement accuracy requirements need to be defined. Issue 4-4-2: Conditions/scenarios for PRS-RSRPP measurement accuracy Agreements: - RAN4 to conduct link level simulations to evaluate the baseband performance of PRS-RSRPP measurement.

The simulation assumption are approved in [2]. In this paper we will provide our simulation results for PRS-RSRPP measurement performance.

Discussion

Ideal PRS-RSRPP

To evaluate the measurement performance of PRS-RSRPP, PRS-RSRPP error is defined as

estimated PRS-RSRPP – ideal PRS-RSRPP

The ideal PRS-RSRPP is based on perfect channel knowledge and perfect knowledge of the first path TOA.

Based on the definition of PRS-RSRPP in 38.215, PRS-RSRPP is the power of the channel response at the i-th path of the RE. As the channel response is in time domain, how it translates into a power on RE level in frequency domain needs to clarified, otherwise the simulated PRS-RSRPP error from different companies are not comparable (as different companies may have different ideal values).

| Definition | DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

In RAN4#103-e, we proposed to define the ideal PRS-RSRPP for the p-th path as

RSRPPp∝kHkexpj2πDpkNIFFT2

where Hk is the channel response in frequency domain for the k-th RE, Dp is the delay of the p-th path.

The physical meaning of the calculation is that channel response at the p-th path delay on a resource element corresponds to the response on the resource element of a shifted version of the channel where the i-th path delay is shifted to 0.

In last meeting, some companies commented that IFFT size is up to UE implementation, so it should not be used in the definition of the ideal PRS-RSRPP. We would like to note that Dp and NIFFT in the formula are corresponding to each other, i.e. Dp is the sample number/index for the ideal TOA corresponding to the IFFT size NIFFT, so e.g. if Dp = 5 when UE is assuming IFFT size 512, then Dp = 10 when UE is assuming IFFT size 1024. We suggest to use this formula to define the ideal PRS-RSRPP given the ideal channel Hk and ideal TOA of the path Dp.

Proposal 1: RAN4 to use the following formula to define ideal PRS-RSRPP.

RSRPPp∝kHkexpj2πDpkNIFFT2

where Hk is the channel response in frequency domain for the k-th resource element, Dp is the delay of the p-th path.

In particular, for the two tap static channel as agreed in [2], as the distance between the two paths is quite large (450ns), there is almost no inter-path component when estimating the path power. Using the above formula, the ideal PRS-RSRPP would be fixed as 0.5.

Proposal 2: For the two tap static channel as agreed in [2] the ideal PRS-RSRPP is 0.5.

Simulation results

In this section we provide simulation results based on [2]. Table 1 lists the PRS-RSRPP error for 4-sample measurement, Table 2 for 1-sample.

Table 1: PRS-RSRPP error for 4-sample, Es/Iot of (-6, -13, -13)dB

| Parameter | Value |  |  | PRS-RSRPP error |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 5% | 50% | 95% |
|  | 15 | 24 | 4 | -6.5 | -3.5 | 2.9 |
|  |  | 52 | 1 | -3.1 | -2.3 | 2.5 |
|  |  | 104 | 1 | -2.2 | -1.5 | 1.8 |
|  | 30 | 24 | 4 | -6.3 | -2.7 | 2.9 |
|  |  | 48 | 1 | -4.0 | -1.5 | 2.6 |
|  |  | 132 | 1 | -3.2 | -1.5 | 2.5 |
|  | 60, FR1 | 24 | 4 | -5.8 | -3.0 | 2.8 |
|  |  | 64 | 1 | -3.7 | -1.0 | 2.4 |
|  |  | 132 | 1 | -3.0 | -1.5 | 2.3 |
|  | 60, FR2 | 24 | 4 | -5.8 | -3.0 | 2.8 |
|  |  | 64 | 1 | -3.7 | -1.0 | 2.4 |
|  |  | 132 | 1 | -3.0 | -1.5 | 2.3 |
|  | 120 | 32 | 4 | -5.3 | -3.0 | 2.3 |
|  |  | 64 | 1 | -2.6 | -1.0 | 1.6 |
|  |  | 128 | 1 | -2.3 | -1.5 | 1.5 |

Table 2: PRS-RSRPP error for 1-sample, Es/Iot of (-3, -6, -6)dB

| Parameter | Value |  |  | PRS-RSRPP error |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 5% | 50% | 95% |
|  | 15 | 24 | 4 | -6.0 | -2.5 | 2.1 |
|  |  | 52 | 1 | -3.2 | -1.5 | 2.6 |
|  |  | 104 | 1 | -2.3 | -1.0 | 1.6 |
|  | 30 | 24 | 4 | -6.1 | -3.5 | 2.1 |
|  |  | 48 | 1 | -2.4 | -1.4 | 1.6 |
|  |  | 132 | 1 | -2.1 | -1.0 | 1.4 |
|  | 60, FR1 | 24 | 4 | -5.5 | -3.0 | 2.0 |
|  |  | 64 | 1 | -2.3 | -1.0 | 1.4 |
|  |  | 132 | 1 | -2.0 | -1.5 | 1.3 |
|  | 60, FR2 | 24 | 4 | -5.5 | -3.0 | 2.0 |
|  |  | 64 | 1 | -2.3 | -1.0 | 1.4 |
|  |  | 132 | 1 | -2.0 | -1.5 | 1.3 |
|  | 120 | 32 | 4 | -5.0 | -3.0 | 1.9 |
|  |  | 64 | 1 | -2.4 | -1.0 | 1.5 |
|  |  | 128 | 1 | -2.1 | -1.5 | 1.2 |

Proposal 3: Take the simulation results in Table 1 and Table 2 for deriving the PRS-RSRPP accuracy.

Conclusions

In this paper we provided our simulation results for PRS-RSRPP measurement performance.

Proposal 1: RAN4 to use the following formula to define ideal PRS-RSRPP.

RSRPPp∝kHkexpj2πDpkNIFFT2

where Hk is the channel response in frequency domain for the k-th resource element, Dp is the delay of the p-th path.

Proposal 2: For the two tap static channel as agreed in [2] the ideal PRS-RSRPP is 0.5.

Proposal 3: Take the simulation results in Table 1 and Table 2 for deriving the PRS-RSRPP accuracy.

Reference

- R4-2210600, WF on NR Positioning Enhancements (Part 1), Ericsson

- R4-2211061, Simulation assumption for PRS-RSRPP, Huawei, HiSilicon