3GPP TSG-RAN WG4 Meeting # 104-e 	R4-2213646

Electronic Meeting, August 15th - 26th, 2022

Agenda Item: 9.18.3.1.5

Source: MediaTek Inc.

Title: Measurement procedure for RedCap

Document for: Discussion

# Introduction

In this contribution paper, we discuss the remaining open issue on measurement procedure captured in the way forward (WF) [1].

# Discussion on remaining open issues

The open issue from the WF is given below:

Whether to support for per-FR/per-UE gap Candidate options: - Option 1 (OPPO, Ericsson, CMCC, Nokia): If a RedCap UE support both FR1 and FR2, whether RedCap UE can support per-FR gap(e.g., independentGapConfigdf) depends on UE capability. - Option 2 (MTK, vivo, Apple): If MG is needed, both per-UE gap and per-FR gap can be supported, but UE behavior is same as per-UE gap.

Whether to support for per-FR/per-UE gap

Whether the UE can support the per-FR gap or not depends on the UE and in general there is no issue that the UE supports per-FR gap. However, given that the UE will have one serving cell (i.e. One CC), hence, having per-FR gap is meaningless. This is because whether the UE is configured with per-FR gap or per-UE gap, the UE behaviour is always the same as per-UE gap. In addition, this issue shall impact the spec writing only and no technical impact to the UE performance.

- If MG is needed, both per-UE and per-FR MG can be supported by UE, but they both share the same per-UE MG based cell identification/measurement requirements.

# Summary

In this contribution, we discussed whether to support for per-FR/per-UE measurement gap and we have the following proposal:

Proposal 1:  If MG is needed, both per-UE and per-FR MG can be supported by UE, but they both share the same per-UE MG based cell identification/measurement requirements.

# References

- R4-2210592, “WF on RedCap RRM requirements”, Ericsson, May 2022.