3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213679

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	10.19.2

Source: 	ZTE Corporation

Title: 	Discussion on system parameter for APT600MHz

Document for:	Approval

Introduction

The topic of APT 600MHz have already been extensively discussed during R17 time phase. At that time, it was a SID, which mainly focus on feasibility analyses for options B1, B2 and B2a, where the feasibility studies and conclusions can be found in the TR38.860[1]. In this contribution, we want to share some initial views from system parameter perspective.

- Specify a new NR FDD operating band to include band numbering and core requirements to operate in APT region by considering that:

  - UE transmit: 663 - 703 MHz, BS transmit: 612 MHz – 652 MHz,

  - To support the following channel bandwidths and SCS as in the Table 2 below

  - UL channel bandwidth is limited to 20 MHz to limit the MSD as was the case with n71

  - The use of UE power class 3 (23dBm).

Table 2: Channel bandwidth and SCS parameters

| SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |
| 30 |  | 10 | 15 | 20 | 25 | 30 | 35 |  |

- Develop a technical specification for the APT 600 MHz band for option B1 (as defined in table 1) for the APT region 600 MHz band with a view to developing the normative text on:

- Operating band, channel bandwidth and system parameters- see Table 1 and Table 2

- BS and UE RF core requirement taking into account potential coexistence issues

- RRM requirement

Discussion

## 2.1. Operating bands and band numbering

As indicated in the WID objective, the frequency range and the supported channel bandwidths for the new FDD bands are shown in the following tables and band number is proposed as n105.

Table 2.1-1. frequency range for APT600MHz

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Proposal 1: to define the band in Table 2.1-1 for APT600MHz.

## 2.2. Channel bandwidth

For the supported channel bandwidth for APT600MHz, it has been captured in the WID and it could be captured as following Table 2.2-1 similar as that of band n71. However one issue might need more discussions on whether the NOTE 4 in Table 2.2-1 should be also applicable for n105.

Table 2.2-1. supported chennel bandwidth and SCS for band n105 in TS 38.101-1

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353,4 |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353,4 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n105 | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <...> NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.2-2. supported chennel bandwidth and SCS for band n105 in TS 38.104

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n105 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <...> . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: to define the supported channel bandwidth for APT600MHz as in Table 2.2-1 and Table 2.2-2 and further discuss the applicability of NOTE 4 in Table 2.2-1 for n105.

In addition, from UE RF perspective, the largest supported channel bandwidth in downlink is 35MHzr and the largetest supported channel bandwidth in uplink is limited to 20 MHz, which means UE need to support asymmetric channel bandwidths similar as band n71.

In current spec TS38.101-1, there are three asymmetric channel bandwidth combination set defined for band n71, each set corresponding to sets of UL/DL channel bandwidth, where UL channel bandwidth is smaller than DL channel bandwidth, as shown in table 3.

For the new added band for APT600MHz, more discussion would be needed to support the asymmetric channel bandwidths due to it may largely depends on the operator’s demands, inputs from operator would be needed.

Table 2.2-3. Asymmetric channel bandwidth for band n71

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
|  | 20 | 25, 30, 35 | 2 |
| nX | ffs | ffs | 0 |

Proposal 3: to further discuss the asymmetric channel bandwidth set for APT600MHz.

## 2.3. Channel raster and sync raster

For APT600MHz, its frequency ranges can be seen as the extension of band n71. Thus the channel raster should be the same as band n71, i.e. 100kHz to the simplify the UE implementation complexity for initial access. Similar reason could be also applied for sync raster of APT600MHz.

Proposal 4: to use the same channel raster 100kHz as n71 and the NR-ARFCN in the Table 2.3-1 and GSCN in Table 2.3-2.

Table 2.3-1. NR-ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n105 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |

Table 2.3-2. Applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n71 | 15 kHz | Case A | 1547 – <1> – 1624 |
| n105 | 15 kHz | Case A | 1535 – <1> – 1624 |

.

Conclusions

In this contribution, we want to share some initial views from system parameter perspective for APT600MHz.and proposals are made as following:

Proposal 1: to define the band in Table 2.1-1 for APT600MHz.

Proposal 2: to define the supported channel bandwidth for APT600MHz as in Table 2.2-1 and Table 2.2-2 and further discuss the applicability of NOTE 4 in Table 2.2-1 for n105.

Proposal 3: to further discuss the asymmetric channel bandwidth set for APT600MHz

Proposal 4: to use the same channel raster 100kHz as n71 and the NR-ARFCN in the Table 2.3-1 and GSCN in Table 2.3-2.

References

- TR38.860, Study on Extended 600 MHz NR band, v17.0.0

- RP-221778, New WID on APT 600 MHz NR band, Spark NZ, Qualcomm Incorporated

- TR36.775, US 600 MHz Band for LTE, v15.0.0