3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213811

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Discussions on remain issues on CSI reporting requirements for FR2-2

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.2.4

Document for:	Discussion

Background

In the last meeting, we provide our simulation results for CQI reporting requirements for FR2-2

Simulation results

The simulation results for CQI distribution and BLER requirements are captured in Table 2-1 (Without Phase noise modelled) and Table 2-2(With Phase noise). We use bandwidth/SCS 100MHz/120kHz and reuse the test parameters of Rel-15 FR2 CQI AWGN test and CQI table 1 is assumed.

Table 2-1: Simulation results without phase noise modelled

| SNR | Median CQI | Prob(Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 1 | 100% | 0 | 0 | 100% |
| -4 | 2 | 100% | 0 | 0 | 100% |
| -2 | 3 | 100% | 0 | 0 | 100% |
| 0 | 4 | 100% | 0 | 0 | 100% |
| 2 | 5 | 100% | 0 | 0 | 100% |
| 4 | 6 | 100% | 0 | 0 | 100% |
| 6 | 7 | 100% | 0 | 0 | 100% |
| 8 | 8 | 100% | 0 | 0 | 100% |
| 10 | 9 | 100% | 0 | 0 | 87.5% |
| 12 | 10 | 100% | 0 | 0 | 88.9% |
| 14 | 11 | 100% | 0 | 0 | 100% |
| 16 | 12 | 100% | 0 | 0 | 24.0% |
| 18 | 13 | 100% | 0 | 0 | 53.8% |

Table 2-2: Simulation results without phase noise modelled

| SNR | Median CQI | Prob(Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 0 | 100% | / | / | 0.07 |
| -4 | 2 | 100% | 0 | 0 | 100% |
| -2 | 3 | 100% | 0 | 0 | 100% |
| 0 | 4 | 100% | 0 | 0 | 100% |
| 2 | 5 | 100% | 0 | 0 | 100% |
| 4 | 6 | 100% | 0 | 0 | 100% |
| 6 | 7 | 100% | 0 | 0 | 100% |
| 8 | 8 | 100% | 0 | 0 | 100% |
| 10 | 9 | 100% | 0 | 0 | 100% |
| 12 | 10 | 100% | 0 | 0 | 100% |
| 14 | 11 | 100% | 0 | 0 | 100% |
| 16 | 12 | 100% | 0 | 11% | 100% |
| 18 | 13 | 100% | 0% | 100% | 100% |

Discussions

SCS/Bandwidth

We have following options for this issue:

- Option 1: Only 100MHz/120kHz - Option 2: 100MHz/120kHz and 400MHz/480kHz

Based on our understanding,120kHz SCS is more susceptible to ICI caused by phase noise, hence we think it is more feasible to only consider 120kHz/100MHz since RAN4 always consider minimum requirements. Furthermore, as the same problems with other requirements, 480kHz/400MHz means quite low testable SNR which means only CQI index related to QPSK can be tested. Hence we propose to only consider 100MHz/120kHz.

Observation 1: 120kHz SCS is more susceptible to ICI caused by phase noise.

Observation 2: 480kHz/400MHz means quite low testable SNR which means only CQI index related to QPSK can be tested.

Proposal 1: Only consider 120kHz/100MHz

SNR value

We have following options for this issue:

- Option 1: Not consider SNR related to 64QAM with following reasons: - The lowest SNR with CQI reporting related to 64QAM is 12dB which is rather higher than maximum achievable SNRBB specified by RF part. - Phase noise has big impact on the BLER performance when SNR is equal or larger than 14dB which CQI reporting index is larger than 12. - Option 2: Decide based on simulation results.

The test SNR should be lower than maximum testable SNR defined by RF part which is 7.7dB for 120kHz/100MHz bandwidth. Based on our simulation results, 7dB SNR is related to 16QAM and phase noise impact can be ignored. Hence we suggest to select 6/7 dB to verify CQI reporting related to 16QAM and -2/-1 dB to verify CQI reporting related to QPSK.

Proposal 2: Select SNR=6/7 dB to verify CQI reporting related to 16QAM and -2/-1 dB to verify CQI reporting related to QPSK.

Conclusion

In the last meeting, we provide our views on remain issues on CQI reporting requirements for FR2-2. The observations and proposals are:

Observation 1: 120kHz SCS is more susceptible to ICI caused by phase noise.

Observation 2: 480kHz/400MHz means quite low testable SNR which means only CQI index related to QPSK can be tested.

Proposal 1: Only consider 120kHz/100MHz

Proposal 2: Select SNR=6/7 dB to verify CQI reporting related to 16QAM and -2/-1 dB to verify CQI reporting related to QPSK.

Reference

[1]   R4-2210665 WF on UE demodulation performance requirements definition for 52.6 - 71 GHz. Qualcomm Incorporated