3GPP TSG-RAN WG4 # 104-e	R4-2213904

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.18.5.1.1

Source:	MediaTek Inc.

Title:	Simulation results and discussion on PDSCH requirements for RedCap

Document for:	Discussion

- Introduction

In RAN4#103-e, a way forward on RedCap UE requirements was agreed [1]. In this contribution, we provide our simulation results for PDSCH requirements and our views on PDSCH requirements for 256QAM.

- Discussion

RAN4 discussed 256QAM PDSCH requirements for RedCap UE in RAN4#103-e and had the following agreement and way forward.

Agreement: Define 256QAM demodulation requirements for FR1 - Introduce 256QAM requirement for 2Rx Way forward: Define 256QAM demodulation requirements for FR1 - For 256QAM requirements for 1Rx, - Option 1: Define 256QAM demodulation requirements for 1Rx in FR1 - Option 1a: MCS20 - Option 1b: MCS is based on the operating SNR - Option 2: Not to define 256QAM demodulation requirements for 1Rx in FR1

The reaming issue for 256QAM demodulation requirements is whether to define requirements for 1Rx. It is highly possible that RedCap UE supports only 1 Rx. Therefore, we suggest defining 256QAM demodulation requirements for 1Rx. For existing 256QAM test cases, the SNR requirement is around 24/25 dB for 2 Rx. If we consider 1 Rx UE with the same MCS, it will increase the operating SNR to a higher level. Hence, we suggest using MCS20 to define requirements for 256QAM modulation where the simulation results for FDD and TDD are shown as below.

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Reference from TS38.101-4 | SNR for 70% of max T-put, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | 256QAM 0.67 Rank 1 | TDLA30-10 | 2x1 low | 5.2.2.1.1-3 Test 1-3 | 22.7 |

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Reference from TS38.101-4 | SNR for 70% of max T-put, [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | 256QAM 0.67 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 5.2.2.2.1-3 Test 1-3 | 22.9 |

Proposal 1: Use MCS20 to define 1Rx 256QAM demodulation requirements for RedCap UE.

- Simulation Results

We provide the simulation results for FDD and TDD in Table 1 and Table 2, respectively.

Table 1. Summary of simulation results for FDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Reference from TS38.101-4 Table | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 5.2.2.1.1-3 Test 1-1 | 1.3 | 3.3 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 5.2.2.1.1-3 Test 1-4 | 10.0 | 12.0 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 5.2.2.1.1-4 Test 2-1 | 14.2 | 16.2 |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 5.2.2.1.1-3 Test 1-4 | 5.9 | 7.9 |

Table 2. Summary of simulation results for TDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Reference from TS38.101-4 Table | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 5.2.2.2.1-3 Test 1-1 | 1.4 | 3.4 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 5.2.2.2.1-3 Test 1-4 | 10.1 | 12.1 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 5.2.2.2.1-4 Test 2-1 | 14.4 | 16.4 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 5.2.2.2.1-3 Test 1-1 | -2.2 | -0.2 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 5.2.2.2.1-3 Test 1-4 | 5.9 | 7.9 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 5.2.2.2.1-4 Test 2-1 | 17.3 | 19.3 |

- Conclusion

In this contribution, we provide simulation results for RedCap PDCSH based on the agreed simulation assumption in [1]. Also, we have a proposal about 1Rx 256QAM demodulation requirements.

Proposal 1: Use MCS20 to define 1Rx 256QAM demodulation requirements for RedCap UE.

- Reference

- R4-2210672, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson