3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213909

# Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draft | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-4, TDD UL-DL pattern for HST-DPS (Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-Perf |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No TDD UL-DL pattern defined in HST-DPS requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add TDD UL-DL pattern defined in HST-DPS requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TDD UL-DL pattern in HST-DPS requirement is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sections 5.2.2.2.10, 5.2.3.2.10 and 7.2.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

<SKIP UNCHANGED PART>

Table 5.2.2.2.10-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 |  | R.PDSCH.2-10.5 TDD |  | 40 / 30 |  | 64QAM, 0.43 |  | FR1.30-1 |  | HST-DPS |  | 1 |  | 2x2 |  | 70 |  | 13.0 |  |
| 1-2 |  | R.PDSCH.2-10.5 TDD |  | 40 / 30 |  | 64QAM, 0.43 |  | FR1.30-1 |  | HST-DPS |  | 2 |  | 2x2 |  | 70 |  | 13.0 |  |

END OF CHANGE 1

START OF CHANGE 2

<SKIP UNCHANGED PART>

Table 5.2.3.2.10-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 |  | R.PDSCH.2-10.5 TDD |  | 40 / 30 |  | 64QAM, 0.43 |  | FR1.30-1 |  | HST-DPS |  | 1 |  | 2x4 |  | 70 |  | 10.2 |  |
| 1-2 |  | R.PDSCH.2-10.5 TDD |  | 40 / 30 |  | 64QAM, 0.43 |  | FR1.30-1 |  | HST-DPS |  | 2 |  | 2x4 |  | 70 |  | 10.2 |  |

END OF CHANGE 2

START OF CHANGE 3

<SKIP UNCHANGED PART>

Table 7.2.2.2.4-3: Minimum performance for HST-DPS

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) |  | Modulation format and code rate |  | TDD UL-DL pattern |  | Propagation condition |  | Number of active PDSCH TCI states | Correlation matrix and antenna configuration |  |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1-1 |  | [R.PDSCH.5-12.2 TDD] |  | 200 / 120 |  | 64QAM, 0.43 |  | FR2.120-1 |  | HST-DPS-FR2-BI-B |  |  | 1 | 2x2 |  |  | 70 |  | [13.9] |
| 1-2 |  | [R.PDSCH.5-12.1 TDD] |  | 200 / 120 |  | 64QAM, 0.43 |  | FR2.120-1 |  | HST-DPS-FR2-UNI-A |  |  | 2 | 2x2 |  |  | 70 |  | [13.7] |

END OF CHANGE 3