3GPP TSG-RAN WG4 Meeting # 104-e 													R4-2214169

Electronic Meeting, 15  Aug– 26 Aug 2022

Agenda item:			9.11.4

Source:	Moderator (ZTE)

Title:	Email discussion summary for [104-e][309] NTN_Solutions_UERF_Maintenance

Document for:	Information

Introduction

The e-mail discussion covers NTN UE RF requirement, TP to TS 38.101-5 and TP to TR 38.863 on NTN UE RF part.

All contributions submitted are divided into the following Topics:

- NTN UE Tx requirement

- NTN UE Rx requirement

- TP to TS 38.101-5

- TP to TR 38.863

Topic #1: 	UE Tx requirement

Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212158 | MediaTek (Chengdu) Inc. | NR NTN UE frequency error requirement testing Proposal 1: Clarify the intended modelling of doppler shift in the requirements description of inform RAN5. Proposal 2: For NGSO, recommend to RAN5 that the frequency error be measured under varying doppler shift for the non-zero doppler shift case. For zero doppler shift case, a GSO satellite is proposed be modelled. Proposal 3: GSO-only NTN-supporting UEs should not be required to pass the non-zero doppler test, and maybe NGSO-only NTN supporting UEs could not be required to pass the zero doppler test. Proposal 4: Re-use the existing AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates for usage with precompensation in the Frequency Error test case. |
| R4-2214046 | Qualcomm Incorporated | Doppler pre-compensation in RF requirements Proposal 1: Doppler is set to zero for other than frequency error UE RF requirements Observation: Doppler is real NTN system is not necessary constant Proposal 2: Frequency error with non-zero doppler is required to be within +/- 0.1 ppm in constant doppler conditions. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 frequency error for NTN UE conformance testing

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1:  Frequency error assumption for UE RF requirements other than frequency error testing

- Proposals

  - Option 1: Doppler is set to zero for other than frequency error UE RF requirements [Qualcomm]

  - Option 2: others

- Recommended WF

Issue 1-2:   Frequency error assumption for non-zero doppler

- Proposals

  - Option 1:  Frequency error with non-zero doppler is required to be within +/- 0.1 ppm in constant doppler conditions. [Qualcomm]

  - Option 2: under varying doppler shift for the non-zero doppler shift case [MTK]

  - Option 3: other

- Recommended WF

Issue 1-3:    Recommendation to RAN5 on frequency error measurement

- Proposals

  - Proposal 1: Clarify the intended modelling of doppler shift in the requirements description of inform RAN5.

  - Proposal 2: For NGSO, recommend to RAN5 that the frequency error be measured under varying doppler shift for the non-zero doppler shift case. For zero doppler shift case, a GSO satellite is proposed be modelled.

  - Proposal 3: GSO-only NTN-supporting UEs should not be required to pass the non-zero doppler test, and maybe NGSO-only NTN supporting UEs could not be required to pass the zero doppler test.

  - Proposal 4: Re-use the existing AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates for usage with precompensation in the Frequency Error test case.

- Recommended WF

Companies views’ collection for 1st round

Open issues

Issue 1-1:

| Company | Comments |
| --- | --- |
| Company A | Issue 1-1: Comment |
| Company B |  |
| Huawei | OK with option 1. |
| ZTE | Okay with option 1 |
| Xiaomi | OK with option 1 |
| Ericsson | Option 1 is acceptable |
| THALES | Option 1 seems acceptable. |
| MediaTek | Option 1 would be ideal, but how would it be enabled in practice? It would be good to understand that before we commit to an agreement. |
| Nokia | We are okay with Option 1. |

Issue 1-2:

| Company | Comments |
| --- | --- |
| Company A | Issue 1-2-1: Comment Issue 1-2-2: Comment Issue 1-2-3: Comment |
| Company B |  |
| Huawei | Option 3, Since EphemerisInfo in SIB19 doesn’t include dUT1 (The difference between UTC and UT1 (The definition can be found at: https://www.britannica.com/science/Universal-Time)) and leap second. The time error between UTC and UT1 can reach 0.9 second. Since the location information of satellite is a function of UT1, the satellite location error can be 30km with 0.9 second time error! That means the frequency error after pre-compensation can be hundreds of Hertz. Thus, we should consider how UE can acquire the UT1 and leap second when testing frequency error and compensating the Doppler shift. Otherwise, the 0.1PPM can’t be reached. |
| ZTE | Open for further discussion and it seems that how to do the UL compensation still needs more discussions; only under certain condition, then 0.1ppm requirement could be reached, however from our understanding, it might a bit later after the completion of core requirements. |
| Xiaomi | Before we have agreement on this issue, it seems the impact due to introduction Doppler shift pre-compensation compared to TN UE for frequency error needs more time study. |
| Ericsson | Option 1 would be acceptable |
| Qualcomm | To Huawei, there is no assumption of a clock, the location is a function of DL SFN at UL synchronization reference point. It is not defined based on a UTC time nor UT1 time |
| THALES | The requirement should be in +/- 0.1 ppm, but the Doppler Shift should be varied in a realistic manner, in order to demonstrate that UE is capable of correctly pre-compensating. Since several UE pre-compensation algorithms can exist on UE side, we need to be sure that all UEs will react similarly to the test environment and in real conditions, with respect to given requirements. |
| MediaTek | Just to clarify, we proposed Option 2 on the basis that the UE will be predicting the movement of the satellite in real life (i.e. considering gravitational pull etc when predicting this), so we did not find any way to configure the UE with real ephemeris data such that it would lead to a constant doppler. Please could Qualcomm explain specifically how this is achieved? The issue we see with just stating an agreement and throwing it to RAN5 is in case some (wrong) assumptions are made about UE behaviour when defining the test. We would be happy to ask RAN5 some specific questions. We agree with Qualcomm that, by design, the absolute time is not required to be known at the UE. this is why SFN an Epoch Time info are provided, this was consciously decided by other WGs. |
| Nokia | We are okay with Option 1 as a baseline but would also like to better understand how the pre-compensating is verified. The method described by MTK/Thales could serve as a good starting point. |
| Hughes/EchoStar | frequency error assumption for non-zero Doppler - Need clarification: are any of the options considering Doppler due to vehicular speeds (in addition to Doppler due to satellite velocity)? . |

Issue 1-3:

| Company | Comments |
| --- | --- |
| Company A | Issue 1-3-1: Comment Issue 1-3-2: Comment Issue 1-3-3: Comment |
| Company B |  |
| Huawei | Not only the precise UE Location but also the satellite location have an impact on the accuracy of Doppler shift pre-compensation. The location information of satellite is a function of UT1. However, EphemerisInfo in SIB19 doesn’t include dUT1 (The difference between UTC and UT1) and leap second. The UT1 and leap second should be provided to UE when testing frequency error and compensating the Doppler shift. |
| ZTE | This could be further discussed in RAN5. |
| Xiaomi | Depends the outcome of issue 1-2. |
| Ericsson | If the doppler estimation algorithm should be tested, it should be done separately, and not most likely in the scope of this RF requirement. It has been agreed that the pre-compensate Doppler estimation error was neglectable comparing to the 0.1ppm budget for UE frequency error (whatever UE location is considered), that’s what should be checked here. If needed, several frequency error tests could be done with several Doppler pre-compensation value, but during the test, the Doppler should remain constant. All proposals 1-4 might not be relevant in this context then (frequency error requirement), but they should be discussed when checking the Doppler estimation algorithm. As ZTE suggested, to avoid duplicate discussion, such discussion might better take place in RAN5? |
| Qualcomm | Could Mediatek provide more information how does UE indicate being “GSO-only NTN-supporting UEs ”. It is our understanding the RAN2 requirement in 38.300 applies for all UEs. Also the test method should be discussed in ran5 and if something is difficult to test, that should not be a reason to revisit the core requirement. We have some very bad examples of this in the thread [140] originating from Rel-15. Not sure why doppler has to be changing. What about if UE is in airplane? And in GEO max doppler is 0.93 ppm as it is documented. Seems high compared to 0.1 ppm accuracu requirement. |
| THALES | Even for GSO there is some (residual) Doppler shift, even if it is much lower than for NGSO. To be further discussed. In any case, the UE should be able to pre-compensate the UL transmission based on ephemeris data and GNSS. Is the precision/accuracy fine within the given requirements? This is the question. The test should be as much realistic as possible, taking into account realistic Doppler shift value. We could also assume (separately or together) 1) GNSS information correctly provided to the UE/decoded by the UE AND/OR 2) ephemeris data correctly decoded by the UE when implementing the test. |
| MediaTek | @ Qualcomm: The UE may indicate “GSO-only” this via the ntn-ScenarioSupport-r17 UE capability field. I do not believe that our contribution suggested that there is ONLY a zero doppler effect for GSO, or that pre-compensation is not applicable in such a scenario. But equally it would be strange to require a supporting GSO to achieve a requirement that is only applicable in a scenario (NGSO) that it has already indicated that it doesn’t support. We hope it is agreeable at least that in the zero doppler case we model a GSO satellite ephemeris, or at least not an NGSO satellite? Finally, we support Proposal 4. |
| Nokia | We understand the intention of this proposed test but, as others, believe this discussion is more relevant in RAN5. |

Issue 1-4:

| Company | Comments |
| --- | --- |
| Company A | Issue 1-4-1: Comment Issue 1-4-2: Comment Issue 1-4-3: Comment |
| Company B |  |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1: Frequency error assumption for UE RF requirements other than frequency error testing All companies except for MTK are fine with option 1, MTK think that Option 1 would be ideal and need more clarification on it. We propose to use option 1 as baseline. Tentative agreements: - Option 1: Doppler is set to zero for other than frequency error UE RF requirement Recommendations for 2nd round: Further clarify how to match with the reality as requested by MTK |
| Sub-topic #1-2 | Issue 1-2: Frequency error assumption for non-zero doppler Ericsson , Nokia and Qualcomm support the option 1, however other companies want to further investigate it. Regarding the UT1 and leap second proposed by Huawei, this might need more discussions. There is no agreement reached in 1st round, further discuss in 2nd round. Candidate options: - Option 1: Frequency error with non-zero doppler is required to be within +/- 0.1 ppm in constant doppler conditions. [Qualcomm] - Option 2: under varying doppler shift for the non-zero doppler shift case [MTK] - Option 3: other Recommendations for 2nd round: Further discuss in the 2nd round for above options. |
| Sub-topic #1-3 | Issue 1-3: Recommendation to RAN5 on frequency error measurement there is lots of discussion on MTK’r proposals for conformance, ZTE, Ercisson, Qualcomm, Nokia prefer it to be discussed in RAN5. Regarding for UE supporting the GEO scenario, whether it’s necessary to test with doppler shift, it seems that companies have different views on that. MTK propose to have zero-value, however thales and Qualcomm, GEO still have some doppler shift in practice. However we also have the following agreement in the TS 38.101-5 spec as following in which zero doppler shift condition is mainly for GEO scenario. Requirement will be verified for at least two cases of which one has zero Doppler conditions Recommendations for 2nd round: For GEO scenario, whether to revisit the previous agreement as following: Requirement will be verified for at least two cases of which one has zero Doppler conditions For LEO scenario, if NGSO-only NTN supporting UEs could not be required to pass the zero doppler test. For the other testing setup as proposed in proposal 5, to discuss it in RAN4 or RAN5. From the moderator point of view, it should be discussed in RAN5 for the proposal 4. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #3: UE Rx requirement

Companies’ contributions summary

(Cat A CRs are not listed)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212165 | Skyworks Solutions Inc. | Band n256 requirement gap for out of-band blocking |
| R4-2212652 | Ericsson | NTN satellite UE out of band blocking requirement |
| R4-2213711 | ZTE Corporation | Further discussion on NTN UE Rx RF requirements |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 OOBB requirements for n256

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1:

- Proposals

  - Option 1: Skyworks, ZTE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -303 | -154 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | [-100] < f – FDL_low < -15 or 15 < f – FDL_high < 60 | [-145] < f – FDL_low ≤ [-100] or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – [145] or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Void NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be modified as [-30] NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be modified as [-15] |  |  |  |  |  |

  - Option 2:  Ericsson [ keep the same width for range 2 of n256 as the 25 MHz width of usual TN range 2 ]

Table 1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | [-30] | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: For band n256 in Range 1 requirement, the applicable lower frequency range should be modified as -100 < f – FDL_low ≤ -15. For band n256 in Range 2 requirement, the applicable lower frequency range should be modified as -125 < f – FDL_low ≤ -100. NOTE 2: For band n256 in Range 3 requirement, the applicable lower frequency range should be modified as 1 ≤ f ≤ FDL_low –125 NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be modified as [-30] NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be modified as [-15] |  |  |  |  |  |

Companies views’ collection for 1st round

Open issues

Issue 2-1:

| Company | Comments |
| --- | --- |
| Company A | Issue 2-1: Comment |
| Company B |  |
| Huawei | Option 1. And all the notes under this table can be removed since the requirements are clear enough for each band. |
| ZTE | Option 1 is more preferred. |
| Xiaomi | Prefer option 1, and suggest removing the bracket and all the notes. |
| Ericsson | We actually prefer the option 1 approach (separate row for n256). Regarding range 2, in last meeting some companies wanted to have more time to check if -125 was acceptable. It would be good to get their conclusion before choosing -145 or -125 then. Note 1, 2, 3 and 4 should be removed. |
| Qualcomm | We prefer option 1 and we are OK to remove all the square brackets in the table. |
| MediaTek | Option 1 is preferred. |
| THALES | Option 1. Please correct/remove square brackets. |
| Nokia | We prefer option 1 |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 | Issue 2-1: OOBB requirements for n256 Since all companies support the option 1 and we propose to agree with option 1 and remove the note 2,3, 4 in the table since this is redundant. In addition, regarding range 2, in last meeting some companies wanted to have more time to check if -125 was acceptable. It is encouraged to provide the views in the 2nd round or GTW session. Candidate options: - Option 1: Operating Band; Parameter; Unit; Range 1; Range 2; Range 3. Pinterferer; dBm; -44; -303; -154. n255; Finterferer (CW); MHz; -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750. n2561; Finterferer (CW); MHz; -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750. NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations N. - Option 2: [ keep the same width for range 2 of n256 as the 25 MHz width of usual TN range 2 ] Table 1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz Operating Band; Parameter; Unit; Range 1; Range 2; Range 3. Pinterferer; dBm; -44; [-30]; -15. n255, n256; Finterferer (CW); MHz; -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60; -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85; 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750. NOTE 1: For band n256 in Range 1 requirement, the applicable lower frequency range should be modified as -100 < f – FDL_low ≤ -15. For band n256 in Range 2 requirement, the applicable lower frequency range should be modified as -125 < f – FDL_low ≤ -100. NOTE 2: For band n256 in Range 3 requirement, the applicable lower frequency range should be modified as 1 ≤ f ≤ FDL_low –125 NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be modified as [-30] NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be modified as [-15]. Recommendations for 2nd round: Further discussion in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: TP to TS 38.101-5

Companies views’ collection for 1st round

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212645 CR to TS 38.101-5 - Tx requirements issues fixes Ericsson | Company A |
|  | Company B |
| R4-2213156 CR for 38.101-5 to further improve the wording for frequency error requirements Huawei, HiSilicon | Company A |
|  | Company B |
|  | Ericsson: we should keep the note, there is no need to put that text out of this note. |
| R4-2212601 CR to 38.101-5: Corrections on Rx requirements for NTN UE Xiaomi | Huawei: It’s recommend to remove superscripts in Table 7.3.2-2 together. |
|  | Xiaomi: Thanks Huawei for the comment, either removing the superscripts or changing the “NOTE” to “NOTE 1” in table 7.3.2-2 is ok for us. |
| R4-2212646 CR to TS 38.101-5 - Rx requirements issues fixes Ericsson | Huawei: Out-of-band blocking can be updated based on the agreement in Issue 2-1 |
|  | Qualcomm: Need to wait for the conclusion in issue 2-1 |

Topic #4: TP to TR 38.863

Companies views’ collection for 1st round

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213155 CR for 38.863 to maintain UE RF parts Huawei, HiSilicon | Company A |
|  | Company B |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on NR NTN UE RF requirement maintenance | ZTE |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2212158 |  | NR NTN UE frequency error requirement testing | MTK | Noted |  |
| R4-2214046 |  | Doppler pre-compensation in RF requirements | Qualcomm | Noted |  |
| R4-2212165 |  | Band n256 requirement gap for out of-band blocking | Skyworks Solutions Inc. | Noted |  |
| R4-2212652 |  | NTN satellite UE out of band blocking requirement | Ericsson | Noted |  |
| R4-2213711 |  | Further discussion on NTN UE Rx RF requirements | ZTE | Noted |  |
| R4-2212645 |  | CR to TS 38.101-5 - Tx requirements issues fixes | Ericsson | endorsed |  |
| R4-2213156 |  | CR for 38.101-5 to further improve the wording for frequency error requirements | Huawei, HiSilicon | Return to |  |
| R4-2212601 |  | CR to 38.101-5: Corrections on Rx requirements for NTN UE | Xiaomi | Revised to |  |
| R4-2212646 |  | CR to TS 38.101-5 - Rx requirements issues fixes | Ericssson | Revised to | To update with the latest agreement reached |
| R4-2213155 |  | CR for 38.863 to maintain UE RF parts | Huawei, HiSilicon | endorsed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Dominique Everaere | dominique.everaere@ericsson.com |
| Qualcomm | Bin Han (Topic#2/3/4) | binhan@qti.qualcomm.com |
| THALES | Dorin Panaitopol |  |
| Hughes/EchoStar | Munira Jaffar | Munira.Jaffar@EchoStar.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)