3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214182

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.11.7.1, 9.11.7.3

Source:	Moderator (Qualcomm Incorporated)

Title:	Email discussion summary for [104-e][322] NR_NTN_Demod_Part1

Document for:	Information

# Introduction

The summary is to discuss the NR NTN demodulation requirements. It covers the contributions submitted under the following agendas:

- 9.11.7.1 General

- 9.11.7.3 UE demodulation requirements

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Collect the views from companies on the simulation assumptions for general and UE demodulation requirements

- 2nd round: Finalize all the remaining simulation assumptions for UE demodulation. Collect the simulation results submitted by companies.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Rohde & Schwarz | Niels Petrovic | Niels.petrovic@rohde-schwarz.com |
| Nokia, Nokia Shanghai Bell | Anthony Lo | Anthony.Lo@nokia.com |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |
| Qualcomm | Jahidur Rahman | rahman@qti.qualcomm.com |
| Huawei | Zehan Zhao | zhaozehan@hisilicon.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: General assumptions

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212234 | Ericsson | Observation 1: Elevation angle 30o could be considered as the worst case from link budget perspective. Observation 2: Delay spread 100ns could basically fulfill the UL timing limitation and represent the worst case in NLOS scenarios regarding to >=30o elevation angel. Observation 3: Smaller K-factor leads to worse performance for LOS NTN channel. Delay spread impact is very small regarding to NTN LOS scenarios. Proposal 1: RAN4 take following channel model for NTN demodulation. - NLOS: NTN-TDLA, DS =100ns, Doppler = 200Hz, K_offset use the value for 30o elevation angle - LOS: NTN-TDLC - Option 1: by a certain elevation angle. DS = 10ns, Doppler =200Hz, K=3.59dB, K_offset use the value for 90o elevation angle - Option 2: by “artificial” parameters. DS=12ns, Doppler=200Hz, K=3dB, K_offset use the value for 30o elevation angle. |
| R4-2213279 | Nokia, Nokia Shanghai Bell | Observation 1: There is no need to add further complexity to demodulation requirements other than decide the desired k-parameter value(s). Observation 2: K-factor value could be chosen based on worst case elevation angle discussed in RAN1 (30 degrees for LEO) and the most appropriate scenario (e.g., rural). Observation 3: Agreement related to Issue 1-2-1 could be clarified to be about “maximum Doppler spread”. |
| R4-2213859 | Huawei, HiSilicon | Proposal 1: Keep previous agreements about the delay and Doppler configuration for NTN demodulation requirements. Proposal 2: Do not explicitly specify the detailed elevation angle for NTN demodulation requirements. Proposal 3: For Doppler, UE compensation functionality is already covered by RF requirements. Do not verify the UE compensation prior to the baseband processing. Proposal 4: For timing, UE compensation functionality is already covered by RRM requirements. Do not verify the UE compensation prior to the baseband processing. Proposal 5: Use NTN-TDLX <DS>-<Fd> for channel parameter combination for NTN demodulation. Proposal 6: Keep previous k-factor for NTN demodulation requirements. |
| R4-2211867 | Apple | Proposal #1: Do not consider delay spread / Doppler shift due to worst case elevation angle since we have agreed to define requirements with assumption of compensation of Doppler shift, frequency drift and time drift prior to baseband processing. Proposal #2: Define requirements with max delay spread of 100ns and nominal residual frequency offset. Proposal #3: Define channel model parameter combination as NTN-TDLX <DS>-<Doppler>. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: Channel model for NTN-TDLA (NLOS)

- Proposals

  - Option 1 (Ericsson, Huawei, Apple, Nokia): DS =100ns, Doppler = 200Hz

  - Option 2: specify other option if any.

- Recommended WF

  - TBA

Moderator notes: K_offset is discussed in Issue 2-3

### Sub-topic 1-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2: Channel model for NTN-TDLC (LOS)

- Proposals

  - Option 1 (Ericsson): DS = 10ns, Doppler =200Hz, K-factor=3.59dB

  - Option 2 (Ericsson): DS=12ns, Doppler=200Hz, K-factor=3dB

  - Option 3 (Huawei, Nokia): DS=100ns, Doppler=200Hz, K-factor=21.6dB

  - Option 4: specify other option if any.

  - Option 5: DS=3.5ns, Doppler=200Hz, K-factor=8.05dB

- Recommended WF

  - TBA

Moderator notes: K_offset is discussed in Issue 2-3

### Sub-topic 1-3

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3: Channel model parameter combination

- Proposals

  - Option 1 (Huawei, Apple): Use NTN-TDLX <DS>-<Fd> for channel parameter combination for NTN demodulation

  - Option 2: specify other option if any.

- Recommended WF

  - TBA

### Sub-topic 1-4

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-4: Doppler shift due to satellite motion for DL in service link

- Proposals

  - Option 1 (Huawei, Nokia): Confirm the agreement that do not verify the UE compensation prior to the baseband processing.

  - Option 2: specify other option if any.

- Recommended WF

  - Option 1

### Sub-topic 1-5

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-5: Timing drift and sampling frequency offset

- Proposals

  - Option 1 (Huawei, Nokia): Confirm the agreement that do not define sampling offset model.

  - Option 2: specify other option if any.

- Recommended WF

  - Option 1

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Nokia | Sub topic 1-1: Support Option 1. Sub topic 1-2: Support Option 3, but k-factor could rather be 8.05. The worst-case elevation angle considered in RAN1 discussion and TR38.821 [3] Table 6.1.3.2-1 is 30 degrees for LEO, and thus k-factor value could be set to the mean k-factor value according to a specific scenario at this elevation from Table 6.7.2-x in TR38.811. For example, as rural case is the most common scenario, k-factor=8.05 dB at worst case elevation angle for LEO, which is why we would support using that number. However, in our simulations 8.05 dB compared to our earlier proposed 21.6 dB doesn’t seem to make a big difference in PDSCH performance. Hence, we are fine with either. Sub topic 1-3: We are fine with Option 1, however we don’t really see the necessity of this agreement since delay spread and residual Doppler are rather constant in the agreements. Sub topic 1-4: Support Option 1. Sub topic 1-5: Support Option 1. |
| Apple | Issue 1-1: Channel model for NTN-TDLA (NLOS) We support option 1. For clarification – is the 200Hz including the residual freq error/ Doppler after UE compensation and the Doppler due to UE motion? Issue 1-3: Channel model parameter combination We support option 1. We need to specify it since we might have different DS for LOS and NLOS channel models. Issue 1-4: Doppler shift due to satellite motion for DL in service link We support option 1. Issue 1-5: Timing drift and sampling frequency offset We support option 1. …. Others: |
| Qualcomm | Sub topic 1-1: We support Option 1. Our understanding is that this Doppler is due to UE motion. During the last meeting, we already agreed that Doppler due to satellite mobility will be fully compensated by other requirements, such as RRM/RF. Sub topic 1-2: We would like to understand more on the choice of K-factor in Option 3. We note that 38.811 specifies a K-factor of 10.22 dB for the LOS path at 50-degree elevation. Could proponent of Option 3 clarify? Sub topic 1-4: We are okay with the recommended WF. Sub topic 1-5: We are okay with the recommended WF. |
| Ericsson | Issue 1-1: Channel model for NTN-TDLA (NLOS) We support Option 1. Issue 1-2: Channel model for NTN-TDLC (LOS) We don’t quite get the motivation of Option 3. - For LOS channel, the delay spread would be expected smaller than NLOS channel and the maximum DS is ~12ns in TR38.811 for high LOS probability scenarios. - Why K-factor =21.6dB? Based on our simulation, the performance of NTN-TDL-C is very close to AWGN when K-factor >18dB, and K-factor =8dB would have 1dB degradation than 18dB. In that case, we don’t see a necessary to test LOS channel with very high K-factor, especially for UL. K-factor = 3.59dB happens when elevation angle is 90 deg in Rural. The performance degradation is ~2dB for 16QAM. From demodulation perspective, it is the worst case. In summary, if we want to take the parameters from the worst link budget angle, we can go for Rural 30o. then DS = 3.5ns, K=8.05dB (Option 5). If we want to use “artificial” channel, we could choose parameters form candidate scenarios and angles (see table below), then Option 2 could be the choice. We also suggest companies checking the necessary of LOS channel for low MCS cases when high K-factor is configured. Issue 1-3: Channel model parameter combination We support Option 1. Issue 1-4: Doppler shift due to satellite motion for DL in service link We support Option 1. Issue 1-5: Timing drift and sampling frequency offset We support Option 1. |
| Huawei | Issue 1-1: Channel model for NTN-TDLA (NLOS) We are OK with Option 1. Issue 1-2: Channel model for NTN-TDLC (LOS) We are OK to consider 30 degree as the worst case. The new proposed Option 5 is OK for us. Issue 1-3: Channel model parameter combination We are OK with Option 1. Issue 1-4: Doppler shift due to satellite motion for DL in service link We are OK with Option 1. Issue 1-5: Timing drift and sampling frequency offset We are OK with Option 1. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1: Channel model for NTN-TDLA (NLOS) Tentative agreements: Option 1: DS =100ns, Doppler = 200Hz (residual Doppler after UE compensation) Candidate options: N/A Recommendations for 2nd round: Confirm the tentative agreements. |
| Sub-topic #1-2 | Issue 1-2: Channel model for NTN-TDLC (LOS) Tentative agreements: N/A Candidate options: - Option 1 (Ericsson): DS = 10ns, Doppler =200Hz, K-factor=3.59dB - Option 2 (Ericsson): DS=12ns, Doppler=200Hz, K-factor=3dB - Option 3 (Huawei, Nokia): DS=100ns, Doppler=200Hz, K-factor=21.6dB - Option 4: DS=3.5ns, Doppler=200Hz, K-factor=8.05dB Recommendations for 2nd round: continue to discuss DS and K-factor for NTN-TDLC (LOS). |
| Sub-topic #1-3 | Issue 1-3: Channel model parameter combination Tentative agreements: - Option 1: Use NTN-TDLX <DS>-<Fd> for channel parameter combination for NTN demodulation Candidate options: N/A Recommendations for 2nd round: Confirm the tentative agreements. |
| Sub-topic #1-4 | Issue 1-4: Doppler shift due to satellite motion for DL in service link Tentative agreements: - Option 1: Confirm the agreement that do not verify the UE compensation prior to the baseband processing Candidate options: N/A Recommendations for 2nd round: Confirm the tentative agreements |
| Sub-topic #1-5 | Issue 1-5: Timing drift and sampling frequency offset Tentative agreements: - Option 1: Confirm the agreement that do not define sampling offset model Candidate options: N/A Recommendations for 2nd round: Confirm the tentative agreements |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: PDSCH demodulation

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211867 | Apple | Proposal #4: Define PDSCH demod requirements for LEO and discuss how requirements can be extended to GEO by defining large enough k-offset value. Do not define separate set of requirements for GEO. Proposal #5: Do not define requirements with 30KHz SCS for NTN. Proposal #6: Target low MCS for NTN UE demod requirements for low SNR conditions. |
| R4-2212553 | Ericsson | Proposal 1: Define the requirements for LEO only Proposal 2: The applicability for GSO only is proposed to be the same as ‘nonTerrestrialNetwork-r17’: UE needs to pass TS38.101-4 requirements only. Proposal 3: k_offset refers to 30o elevation angle for NTN-TDLA test cases and refers to 90° elevation angle for NTN-TDLC test cases Observation 1: No obvious performance difference between configuring delay spread 100ns, 150ns and 200ns for NTN-TDLA Proposal 4: Consider the following test cases for NTN-TDLA channel model: Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB) DMRS. 1-1; R.PDSCH.1-1.1 FDD (QPSK); 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; TBD. 1-2; R.PDSCH.1-2.1 FDD (16QAM); 10 / 15; 16QAM, 0.48; NTN-TDLA100-200; 1x2, ULA Low; 70; TBD. Observation 2: Negligible performance impact can be observed for configuring different delay spread under a fixed k-factor Observation 3: k-factor make more influence and configuring k-factor = 3dB can have the worst performance Proposal 5: Consider the following test cases for NTN-TDLC channel model: Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB) DMRS. 2-1; R.PDSCH.1-1.1 FDD (QPSK); 10 / 15; QPSK, 0.30; NTN-TDLC10-200; 1x2, ULA Low; 70; TBD. 2-2; R.PDSCH.1-2.1 FDD (16QAM); 10 / 15; 16QAM, 0.48; NTN-TDLC10-200; 1x2, ULA Low; 70; TBD. |
| R4-2212608 | Qualcomm Incorporated | Observation 1: AoA of the LoS path has minimal impact on the performance requirement. Proposal 1: Assume an AoA of Ө = 60 degree for the LOS path in NTN TDL-C channel. Proposal 2: Feasibility of the large k_offset value should be confirmed by the TE vendors. |
| R4-2213280 | Nokia, Nokia Shanghai Bell | Proposal 1: Define NTN PDSCH demodulation requirements for LEO only. Observation 1: Legacy requirements can be applied for GEO, no need for LEO requirements. Observation 2: The link budget results in TR 38.821 [3] clearly show that SNRs required by 64QAM do not exist in any of the cases with 2GHz carrier frequency. Observation 3: Existing specification has only one RMC with 64QAM for 30kHz PDSCH |
| R4-2213860 | Huawei, HiSilicon | Proposal 1: Define the requirements for LEO-600 only. Proposal 2: If only LEO cases are defined, all UE supports nonTerrestrialNetwork-r17 needs to pass the additional LEO test in TS38.101-5 and TS38.101-4 requirements. Proposal 3: Select the K_offset value as per Table 2.3.2 based on detailed channel model. We slightly prefer to consider α = 10° as the starting point. Proposal 4: Define PDSCH performance requirements for 64QAM. Proposal 5: For NTN UE performance requirements, select both 10MHz bandwidth for 15kHz SCS and 20MHz bandwidth for 30kHz SCS. Proposal 6: For disabled HARQ and HARQ process 32 requirements for NTN PDSCH, use following simulation assumptions respectively. |
| R4-2212569 | Qualcomm Incorporated | For information: Simulation Results on NTN UE PDSCH Requirements |
| R4-2212554 | Ericsson | For information: Simulation results for PDSCH NTN |
| R4-2213336 | Nokia, Nokia Shanghai Bell | Proposal 1: Use existing RMC:s R.PDSCH.1-1.1 FDD (QPSK) and R.PDSCH.1-2.1 FDD (16QAM) for NTN PDSCH demodulation requirements. Proposal 2: Use parameters in Table 1 for NTN PDSCH demodulation requirements. Observation 1: Performance difference between the two K-factor approaches (21.6 vs. 8.05dB) is rather small |
| R4-2213861 | Huawei, HiSilicon | For information: Simulation results on satellite NTN demod PDSCH |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: PDSCH requirements for GEO and LEO

- Proposals

  - Option 1 (Apple, Ericsson, Nokia, Huawei): Define PDSCH demod requirements for LEO-600. Do not define separate set of requirements for GEO

  - Option 2: Specify other option if any

- Recommended WF

  - TBA

### Sub-topic 2-2

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-2: Applicability rules for LEO requirements

- Proposals

  - Option 1 (Ericsson, Nokia): The applicability for GSO only is proposed to be the same as ‘nonTerrestrialNetwork-r17’: UE needs to pass TS38.101-4 requirements only

  - Option 2 (Huawei): If only LEO cases are defined, all UE supports nonTerrestrialNetwork-r17 needs to pass the additional LEO test in TS38.101-5 and TS38.101-4 requirements.

- Recommended WF

  - TBA

### Sub-topic 2-3

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-3: K-offset value

- Proposals

  - Option 1 (Ericsson): k_offset refers to 30o elevation angle for NTN-TDLA test cases and refers to 90° elevation angle for NTN-TDLC test cases

  - Option 2 (Apple): Discuss how requirements can be extended to GEO by defining large enough k-offset value

  - Option 3 (Qualcomm): k_offset value is equal to or greater than twice the satellite-UE one-way delay. The feasibility of the large k_offset value should be confirmed by the TE vendors

  - Option 4 (Huawei): Select the K_offset value as per Table 2.3.2 based on detailed channel model. We slightly prefer to consider α = 10° as the starting point.

Table 2.3.2 K_offset value in slot

| Tilt angle/° | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEO-600 | 13 | 10 | 8 | 6 | 6 | 5 | 5 | 5 | 4 |
| LEO-1200 | 21 | 17 | 14 | 12 | 10 | 10 | 9 | 9 | 9 |
| GEO | 271 | 264 | 258 | 253 | 248 | 244 | 241 | 240 | 239 |

- Recommended WF

  - Discuss the specific values of k_offset based on the table in option 4

Moderator notes: Suggest to focusing on the specific values discussion for k_offset based on option 4. Meanwhile, encourage TE vendors’ input on the feasibility.

### Sub-topic 2-4

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-4: AoA of the LOS Path for the NTN TDL-C Channel

- Proposals

  - Option 1 (Qualcomm): Assume an AoA of Ө = 60 degree for the LOS path in NTN TDL-C channel

  - Option 2: specify other option if any

- Recommended WF

### Sub-topic 2-5

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-5: Modulation order

- Proposals

  - Option 1 (Huawei): Define PDSCH performance requirements for 64QAM

  - Option 2 (Apple, Ericsson, Nokia): Do not define PDSCH performance requirements for 64QAM

- Recommended WF

  - Do not define PDSCH for performance requirements for 64QAM

### Sub-topic 2-6

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-6: SCS/CBW set

- Proposals

  - Option 1 (Huawei): Define requirements with 30KHz/20MHz SCS/CBW for NTN

  - Option 2 (Apple, Ericsson): Do not define requirements with 30KHz SCS for NTN

- Recommended WF

  - TBA

Moderator’s note: It has been agreed that 15kHz/10MHz SCS/CBW should be defined.

### Sub-topic 2-7

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-7: MCS for NTN UE demod requirements

- Proposals

  - Option 1 (Apple): Considering the following test cases as starting point for UE demod requirements

| CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric |
| --- | --- | --- | --- | --- |
| 10MHz / 15kHz | MCS 4 Rank 1 | NTN-TDLA30-30 | 1x2 low | 70% of peak rate |
| 10MHz / 15kHz | 16QAM MCS 11 Rank 1 | NTN-TDLC100-10 | 1x2 low | 70% of peak rate |

Moderator note: To clarify what’s the DL and Doppler for above propagation condition? We have agreed to use 200Hz as the doppler.

  - Option 2 (Ericsson):

Consider the following test cases for NTN-TDLA channel model:

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) DMRS |
| 1-1 | R.PDSCH.1-1.1 FDD (QPSK) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.1-2.1 FDD (16QAM) | 10 / 15 | 16QAM, 0.48 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |

Consider the following test cases for NTN-TDLC channel model

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) DMRS |
| 2-1 | R.PDSCH.1-1.1 FDD (QPSK) | 10 / 15 | QPSK, 0.30 | NTN-TDLC10-200 | 1x2, ULA Low | 70 | TBD |
| 2-2 | R.PDSCH.1-2.1 FDD (16QAM) | 10 / 15 | 16QAM, 0.48 | NTN-TDLC10-200 | 1x2, ULA Low | 70 | TBD |

- Recommended WF

  - TBA

### Sub-topic 2-8

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-8: Simulation assumption for PDSCH performance with disabled HARQ

- Proposals

  - Option 1 (Huawei): To use the following assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 16 (8 with HARQ enabled, 8 with HARQ disabled) |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2+K_offset for enabled HARQ process, otherwise N/A |
| Maximum number of HARQ transmission |  |  | 4 for enabled HARQ process, 1 for disabled HARQ process |

  - Option 2 (Nokia): To use the following assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Number of SAN antennas/UE antennas |  | 1x2 |
| Carrier frequency |  | 2 GHz |
| Residual frequency offset |  | 0.1ppm (200Hz) |
| Channel model |  | NTN-TDLA 100 ns (NLOS) NTN-TDLC 100ns (LOS) |
| Channel model k-factor |  | 21.6 (corresponds to suburban scenario with 50 degrees elevation) 8.05 (corresponds to rural scenario with 30 degrees elevation) |
| UE velocity |  | 3 km/h |
| MCS |  | 4 (QPSK, 64QAM MCS table) 13 (16QAM, 64QAM MCS table) |
| Reference measurement channel |  | R.PDSCH.1-1.1 FDD (QPSK) R.PDSCH.1-2.1 FDD (16QAM) |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | N/A |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Depends on RMC |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | Ratio of PDSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=49, nSCID =0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 2 |
|  | Allocation length | 12 |
| Frequency domain resource assignment | RB assignment | Depends on RMC |

- Recommended WF

  - TBA

### Sub-topic 2-9

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-9: Simulation assumption for PDSCH performance with HARQ Processes 32

- Proposals

  - Option 1 (Huawei): To use the following assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 32 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2+K_offset |
| Maximum number of HARQ transmission |  |  | 4 |

  - Option 2: specify other option if any

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Rohde & Schwarz | Sub topic 2-3: We would like to clarify a few points before confirming the values. From TE point of view not only the delay is important, but also e.g. the number of HARQ processes used for testing in this scenario. Which scenario would apply here, no HARQ? Fixed number of HARQ (32?)? Also we assume the general assumption of 10 MHz BW and single carrier testing also holds in this case. |
| Nokia | Sub topic 2-1: Support Option 1. Sub topic 2-2: Support Option 1. Sub topic 2-3: Considering sub topic 2-1 we could use RTT for LEO-600 and 30 degree elevation angle which is the worst case discussed earlier in RAN1. Final number of slots depends also on the agreement related to SCS. Sub topic 2-4: We don’t see necessity to discuss this issue separately here but it should follow the other discussion. Sub topic 2-5: Support Option 2. Sub topic 2-6: Support Option 2. Sub topic 2-7: Support TDL-A parameters in Option 2. We have earlier agreement of 200Hz Doppler and discussion Sub-topic 1-2 about LOS parameters. Sub topic 2-8: Support Option 2. Sub topic 2-9: This topic should depend on discussion of sub-topic 2-8 and similar parameters should be used whenever possible. |
| Apple | Issue 2-1: PDSCH requirements for GEO and LEO We support option 1. Issue 2-2: Applicability rules for LEO requirements Option 1 is fine for GSO. If UE supports both NGSO and GSO or only NGSO, we already defined applicability in last meeting. Option 2 is suggesting that if UE supports GSO it still needs to meet the requirements in 38.101-5. We don’t see the need. Issue 2-3: K-offset value We don’t think large K-offset can be supported by TE based on R&S feedback above, hence we don’t think requirements can be extended to GEO. We propose to de-prioritize defining requirements for GEO and only define for LEO-600. We are fine to use option 1as starting point and also request feedback from TE vendors on feasibility with other agreed parameters. We have agreed on one test with 32 HARQ processes. What is the feasible K-offset for that? Issue 2-4: AoA of the LOS Path for the NTN TDL-C Channel Could proponents please clarify how this impacts the final channel model parameters for NTN TDL-C channel? Issue 2-5: Modulation order We support the recommended WF> Issue 2-6: SCS/CBW set Support option 2. We had agreement in last meeting that 15KHz is prioritized and requirements with 30KHz will be revisited if needed. Issue 2-7: MCS for NTN UE demod requirements Option 1 with some modifications, based on the assumption that 200Hz including the residual freq error/ Doppler after UE compensation and the Doppler due to UE motion. No additional freq offset is introduced. NTN-TDLA100-200 NTN-TDLC10-200 Don’t see the necessity of testing both modulations with both Channels. Issue 2-8: Simulation assumption for PDSCH performance with disabled HARQ On option 1 why do we have 16 HARQ processes 8 with enabled and 8 with disabled HARQ? Are we testing both in same test case? In option 2 how is the residual freq offset added/modeled in simulation. Issue 2-9: Simulation assumption for PDSCH performance with HARQ Processes 32 Okay with option 1 as baseline, |
| Qualcomm | Issue 2-1: PDSCH requirements for GEO and LEO We are okay with option 1. Issue 2-2: Applicability rules for LEO requirements We support option 1. Issue 2-3: K-offset value We think that a large k_offset value may not be feasible during TE testing, hence we request feedback from the TE vendors. Issue 2-4: AoA of the LOS Path for the NTN TDL-C Channel We support option 1. To Apple: Our understanding is that the AoA of the LOS path (denoted by theta in the above figure) will impact the maximum Doppler shift experienced by the LOS path. Issue 2-5: Modulation order We support the recommended WF. Issue 2-6: SCS/CBW set We support option 2. We agreed in the last meeting that 15KHz SCS will be prioritized. Issue 2-7: MCS for NTN UE demod requirements We are okay with the modulation and code rate, i.e., with MCS4 and MCS13, antenna configuration as proposed in option 2. We don’t think we have discussed the DS for the NTN TDLA channel (please correct us if we are missing anything here), we are open to discuss this. For NTN TDLC channel, we would like to stick to a DS of 100ns. Issue 2-8: Simulation assumption for PDSCH performance with disabled HARQ Same question as Apple. It is not clear as to why we have half the HARQ processes disabled in Option 1. For disabled HARQ testing, we agreed to set the number of reTx to 1. Issue 2-9: Simulation assumption for PDSCH performance with HARQ Processes 32 Could the proponent of Option 1 clarify what is different here other than the number of HARQ processes compared to the regular TN FDD 15KHz/10MHz test as specified in 38.101-4? |
| Ericsson | Issue 2-1: PDSCH requirements for GEO and LEO Support option 1. Issue 2-2: Applicability rules for LEO requirements Support option 1. Issue 2-3: K-offset value For NLOS channel NTN-TDLA, we can select k_offset corresponding to the elevation angle 30o(k_offset = 8), since it can be considered as the worst case. For LOS channel NTN-TDLC, based on our discussion paper [R4-2212234], smaller K-factor leads to worse performance for LOS NTN channel. Delay spread impact is very small regarding to NTN LOS scenarios. In this case, we propose to consider k_offset corresponding to the elevation angle 90°(k_offset = 4), to reflect the worst case. We also encourage TE vendors to share their views. Issue 2-4: AoA of the LOS Path for the NTN TDL-C Channel Have we already agreed to only consider 200Hz Doppler shift for both NTN-TDLA and NTN-TDLC channel model? If so, maybe we can skip this discussion. Issue 2-5: Modulation order Support the recommended WF. Issue 2-6: SCS/CBW set Support option 2. We don’t see the need to include such an additional scenario. Issue 2-7: MCS for NTN UE demod requirements We support option 2. As for the delay spread for NTN-TDLA, from our simulation results we observed similar performance between configuring 100ns, 150ns and 200ns. So then we propose 100ns as it can fulfil the UL timing limitation and represent the worst case in NLOS scenarios regarding to >=30o elevation angel. For NTN-TDLC, from our simulation results, k-factor make more influence and configuring k-factor = 3dB can have the worst performance. Therefore, we choose 10ns accordingly. Issue 2-8: Simulation assumption for PDSCH performance with disabled HARQ Generally fine with option 2. The k_factor and the time offset should be modified later according to the following agreements on the scenario and elevation angle in this meeting. HARQ configuration should be included as well (or maybe in the table of test case). Issue 2-9: Simulation assumption for PDSCH performance with HARQ Processes 32 Fine with option 1. |
| Huawei | Issue 2-1: PDSCH requirements for GEO and LEO We are OK with Option 1. Issue 2-2: Applicability rules for LEO requirements Considering the limited use scenario for GEO, we are also OK to only consider LEO requirements. Issue 2-3: K-offset value As per TE vendor feedback, the k_offset value selection is impacted by many factors. Further feedback from TE vendor is needed for the test feasibility. Issue 2-4: AoA of the LOS Path for the NTN TDL-C Channel We don't think the AOA should be considered for the TDL channel. Also, the maximum Doppler shift should be 200Hz as per previous agreement. Issue 2-5: Modulation order Option 1. There is agreement on RF part to introduce 64QAM for RF requirements. Also, based on our link budget evaluation, 64QAM is feasible at least for rank1 and MCS17. Issue 2-6: SCS/CBW set Option 1. Considering that 30kHz SCS has been introduced in satellite performance requirements, it is reasonable to define 30kHz SCS performance requirements for NTN UE. Issue 2-7: MCS for NTN UE demod requirements We are OK to use MCS4, MCS13 and MCS17 for NTN UE demod requirements. Issue 2-8: Simulation assumption for PDSCH performance with disabled HARQ Option 1. For disabled HARQ feedback, it is invalid to disable all HARQ process since the network should ensure the signalling transmitted via PDSCH is correctly received by UE, so we propose to consider half HARQ process enabled while another half HARQ process disabled. Issue 2-9: Simulation assumption for PDSCH performance with HARQ Processes 32 Option 1. For the HARQ 32 NTN requirements we should consider the K_offset value for HARQ-ACK and different number of HARQ process numbers. For other parameters, we can reuse from Rel-15 requirements. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1: PDSCH requirements for GEO and LEO Tentative agreements: Define PDSCH demod requirements for LEO-600. Do not define separate set of requirements for GEO Candidate options: N/A Recommendations for 2nd round: Confirm the tentative agreements |
| Sub-topic#2-2 | Issue 2-2: Applicability rules for LEO requirements Tentative agreements: The applicability for GSO only is proposed to be the same as ‘nonTerrestrialNetwork-r17’: UE needs to pass TS38.101-4 requirements only Candidate options: N/A Recommendations for 2nd round: Confirm the tentative agreements |
| Sub-topic#2-3 | Issue 2-3: K-offset value Tentative agreements: N/A Candidate options: Only specify the k_offset values for LEO-600. Encourage TE vendors to provide the input on the feasibility of supporting a large k_offset value referring to 30o elevation angle for NTN-TDLA test cases and refers to 90° elevation angle for NTN-TDLC test cases. The following two cases need to be checked: - HARQ is disabled - HARQ process is 32 Recommendations for 2nd round: continue to discuss candidate options |
| Sub-topic#2-4 | Issue 2-4: AoA of the LOS Path for the NTN TDL-C Channel Tentative agreements: N/A Candidate options: To further discuss whether to consider the additional parameters for AoA in NTN TDL-C considering 200Hz was agreed as the maximum doppler shift - Option 1: Yes, specify the reason - Option 2: No Recommendations for 2nd round: continue to discuss candidate options |
| Sub-topic#2-5 | Issue 2-5: Modulation order The majority view is not to define PDSCH for performance requirements for 64QAM Tentative agreements: Do not to define PDSCH for performance requirements for 64QAM Candidate options: N/A Recommendations for 2nd round: confirm the tentative agreements. |
| Sub-topic#2-6 | Issue 2-6: SCS/CBW set The majority view is not to define requirements with additional 30KHz SCS for NTN Tentative agreements: Do not define requirements with additional 30KHz SCS for NTN. Moderator’s note: It has been agreed that 15kHz/10MHz SCS/CBW should be defined. Candidate options: N/A Recommendations for 2nd round: confirm the tentative agreements |
| Sub-topic#2-7 | Issue 2-7: MCS for NTN UE demod requirements Tentative agreements: N/A Candidate options: N/A - Option 1: - Consider the following test cases for NTN-TDLA channel model: Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB) DMRS. 1-1; R.PDSCH.1-1.1 FDD (QPSK); 10 / 15; QPSK, 0.30; NTN-TDLA100-200; 1x2, ULA Low; 70; TBD. 1-2; R.PDSCH.1-2.1 FDD (16QAM); 10 / 15; 16QAM, 0.48; NTN-TDLA100-200; 1x2, ULA Low; 70; TBD. - Consider the following test cases for NTN-TDLC channel model: Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB) DMRS. 2-1; R.PDSCH.1-1.1 FDD (QPSK); 10 / 15; QPSK, 0.30; NTN-TDLC[10]-200; 1x2, ULA Low; 70; TBD. 2-2; R.PDSCH.1-2.1 FDD (16QAM); 10 / 15; 16QAM, 0.48; NTN-TDLC[10]-200; 1x2, ULA Low; 70; TBD. - Option 2: Specify other option if any Recommendations for 2nd round: continue to discuss in 2nd round. |
| Sub-topic#2-8 | Issue 2-8: Simulation assumption for PDSCH performance with disabled HARQ Tentative agreements: N/A. Candidate options: Focus on the following options and proposed the specific change in the 2nd round discussion: - Option 1: Parameter; Unit; Value. Duplex mode; FDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 0. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. CSI-RS for tracking; CSI-RS periodicity; Slots; 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4. CSI-RS offset; Slots; 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4. Number of HARQ Processes; 16 (8 with HARQ enabled, 8 with HARQ disabled). The number of slots between PDSCH and corresponding HARQ-ACK information; 2+K_offset for enabled HARQ process, otherwise N/A. Maximum number of HARQ transmission; 4 for enabled HARQ process, 1 for disabled HARQ process. - Option 2: Parameter; Unit; Value. Duplex mode; FDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 0. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. CSI-RS for tracking; CSI-RS periodicity; Slots; 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4. CSI-RS offset; Slots; 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4. Number of HARQ Processes; Disabled. The number of slots between PDSCH and corresponding HARQ-ACK information; N/A. Maximum number of HARQ transmission; 1. Recommendations for 2nd round: continue to discuss above options in the 2nd round. |
| Sub-topic#2-9 | Issue 2-9: Simulation assumption for PDSCH performance with HARQ Processes 32 Candidate options: Focus on the following option and proposed the specific change in the 2nd round discussion: - Option 1: Parameter; Unit; Value. Duplex mode; FDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 0. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. CSI-RS for tracking; CSI-RS periodicity; Slots; 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4. CSI-RS offset; Slots; 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4. Number of HARQ Processes; 32. The number of slots between PDSCH and corresponding HARQ-ACK information; 2+K_offset. Maximum number of HARQ transmission; 4. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF for NTN demodulation requirements - general and PDSCH | Qualcomm Incorporated | Capture the WF |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2211867 |  | Discussion on PDSCH demod requirements for NTN | Apple | Noted |  |
| R4-2212234 |  | Discussion on general issue on NTN demodulation | Ericsson | Noted |  |
| R4-2212553 |  | Discussion on the SA UE PDSCH demodulation requirement | Ericsson | Noted |  |
| R4-2212554 |  | Simulation results for SA UE PDSCH | Ericsson | Noted |  |
| R4-2212569 |  | Simulation Results on NTN UE PDSCH Requirements | Qualcomm Incorporated | Noted |  |
| R4-2212608 |  | Views on NTN UE PDSCH Requirements | Qualcomm Incorporated | Noted |  |
| R4-2213279 |  | Discussion on general issues for NTN demodulation requirements | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213280 |  | Discussion on PDSCH demodulation requirements for NTN | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213336 |  | Simulation results on PDSCH demodulation requirements for NTN | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213859 |  | Discussion on UE NTN demod general | Huawei,HiSilicon | Noted |  |
| R4-2213860 |  | Discussion on UE NTN demod PDSCH | Huawei,HiSilicon | Noted |  |
| R4-2213861 |  | Simulation results on satellite NTN demod PDSCH | Huawei,HiSilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents