3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214279

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.21.4

Source:	Moderator (Nokia)

Title:	Email Discussion Summary for [104-e][228] NR_IIOT_URLLC_enh

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Agenda 9.21:

  - Enhanced IIoT and URLLC support	[NR_IIOT_URLLC_enh]

    - RRM core requirement maintenance	[NR_IIOT_URLLC_enh-Core]

      - Propagation delay compensation enhancements	[NR_IIOT_URLLC_enh-Core]

      - Reference point for Te requirements	[NR_IIOT_URLLC_enh-Core]

      - Others	[NR_IIOT_URLLC_enh-Core]

    - RRM performance requirements	[NR_IIOT_URLLC_enh-Perf]

      - General (test configurations, conditions and etc)	[NR_IIOT_URLLC_enh-Perf]

      - Measurement period and accuracy requirements	[NR_IIOT_URLLC_enh-Perf]

      - Test cases for FR1	[NR_IIOT_URLLC_enh-Perf]

        - Test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC	[NR_IIOT_URLLC_enh-Perf]

        - Test cases for UE Rx-Tx time difference measurement with TRS for RTT-based PDC	[NR_IIOT_URLLC_enh-Perf]

      - Test cases for FR2	[NR_IIOT_URLLC_enh-Perf]

        - Test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC	[NR_IIOT_URLLC_enh-Perf]

        - Test cases for UE Rx-Tx time difference measurement with TRS for RTT-based PDC	[NR_IIOT_URLLC_enh-Perf]

    - Demodulation performance and CSI requirements	[NR_IIOT_URLLC_enh-Perf]

      - PUCCH requirements 	[NR_IIOT_URLLC_enh-Perf]

    - Moderator summary and conclusions	[NR_IIOT_URLLC_enh]

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Li Zhang | zhangli164@huawei.com |
| Qualcomm, Inc. | Carlos Cabrera-Mercader | ccmercad@qti.qualcomm.com |
| Ericsson | Magnus Larsson | magnus.k.larsson@ericsson.com |
| vivo | Qian Yang | qian9.yang@vivo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: RRM core requirement maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

This topic discusses contributions submitted under 9.21.1.1 and 9.21.1.2:

1) Propagation delay compensation enhancements: 7 contributions (including 3 CRs)

2) Reference point for Te requirements: one contribution (including 1 CR)

3) Others: no contributions

Moderator would like to remind that the Core part of the WI is closed.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212206 | Qualcomm | Proposal 1: Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC, using either PRS or TRS as the DL reference signal, are based on 4 samples. Proposal 2: Introduce a scaling factor Kgap, like the scaling factor P for L1-RSRP, RLM and BFD measurements, to account for collisions between PDC RS resources and measurement gaps, including concurrent measurement gaps. Proposal 3a: For UEs configured to perform PRS measurements with Type 1A/1B PPW, the measurement period for PDC can be longer if some of the PDC RS resources collide with PPW instances where PRS has higher priority than CSI-RS (PRS for PDC is considered to have the same priority as CSI-RS). No measurement period for PDC is defined in that case. Proposal 3b: For UEs configured to perform PRS measurements with Type 1A/1B PPW, no measurement requirements for PDC apply if the PDC RS resources always collide with PPW instances where PRS has higher priority than CSI-RS (PRS for PDC is considered to have the same priority as CSI-RS). Proposal 4a: For UEs configured to perform PRS measurements with Type 2 PPW, the measurement period for PDC can be longer if some of the PDC RS resources collide with PRS symbols where PRS has higher priority than CSI-RS (PRS for PDC is considered to have the same priority as CSI-RS). No measurement period for PDC is defined in that case. Proposal 4b: For UEs configured to perform PRS measurements with Type 2 PPW, no measurement requirements for PDC apply if the PDC RS resources always collide with PRS symbols where PRS has higher priority than CSI-RS (PRS for PDC is considered to have the same priority as CSI-RS). Proposal 5: Define PDC requirements so that one sample per DRX cycle is assumed when DRX is configured. - For the measurement period requirement with PRS, - Define Teffect= TLCMTPRS, TDRX*LCMTPRS, TDRX, where TDRX is the DRX cycle length - FFS: How to define Lavailable_PRS when DRX is configured - For the measurement period requirement with TRS, - Define TUERxTx,TRS=Nsample *LCMTTRS, TDRX, where TDRX is the DRX cycle length |
| R4-2213048 | vivo | Proposal 1: Measurement period requirements for PRS based UE Rx-Tx time difference measurement is TUERxTx,PRS=Nsample *TPRS . Proposal 2: Accuracy requirements for RTT-based PDC with TRS are based on measurements with 4 samples under AWGN channel. Proposal 3: No requirements are defined for the case when PDC resources are collided with gaps. Proposal 4: For PRS/TRS based UE Rx-Tx time difference measurement for TT-based PDC, DRX related requirements are specified with the assumption that UE measures once per DRX cycle. |
| R4-2213552 | Huawei | Proposal 1: Update the PRS based PDC requirements as follow. TUERxTx, PRS=NPRSslotN'Lavailable_PRSN*Nsample-1*TeffectPRS+Tlast Proposal 2: Define measurement period for TRS based PDC based on 4 samples. Proposal 3: Introduce a scaling factor Kgap to account for the PDC resources occasions dropped due to collision with MG. Proposal 4: Define DRX requirements for PDC such that UE is assumed to take one sample per DRX cycle. |
| R4-2213777 | Nokia | - TRS measurement sample number has a minor effect on the TUE-RX error on FR1 and FR2, AWGN and fading channel. - Define UE Rx-Tx time difference measurement accuracy with 1 TRS measurement sample for FR1 and FR2, AWGN and fading channel. - Account when the PDC resources occasions collide with legacy MG. - A collision between PDC measurements and measurement gaps is captured by allowing an additional delay in the RRM measurements. - Do not define PDC UE Rx-TX measurement period requirements under DRX conditions. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Following agenda items are covered:

9.21.1.1 Propagation delay compensation enhancements: 7 contributions (including 3 CRs)

9.21.1.2 Reference point for Te requirements: one contribution (CR)

9.21.1.3 Others: no contributions

Based on the contributed papers and the proposals following open issues have been identified:

- Number of measurement samples assumed for defining the Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC (PRS and TRS based).

- Introduction of a scaling factor if the PDC-RS collides with a measurement gap

- Measurement period for PDC if some of the PDC RS resources collide with PPW:

  - PRS measurements with Type 1A/1B PPW

  - PRS measurements with Type 2 PPW

- PDC requirements when DRX is in use

- Measurement period requirements for PRS based UE Rx-Tx time difference measurements

All will be addressed individually in the following sub-topics.

### Sub-topic 1-1

Sub-topic description:

Number of measurement samples assumed for defining the Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC (PRS and TRS based).

4 companies have provided input on the matter and there are 2 different proposals.

Open issues and candidate options before e-meeting:

Issue 1-1: Number of measurement samples assumed for defining the Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC (PRS and TRS based)

- Proposals

  - Option 1: 4 samples

  - Option 2: 1 sample

- Recommended WF

  - 3 companies support using 4 samples while 1 company support using 1 sample. Based on this moderator would like to hear if it would be acceptable for companies agreeing that 4 samples are assumed for defining the Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC (PRS and TRS based)?

  - Moderator propose following agreement:

    - RAN4 assume 4 measurement samples are used for defining the Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC (PRS and TRS based).

### Sub-topic 1-2

Sub-topic description

Introduction of a scaling factor if the PDC-RS collides with a measurement gap.

4 companies have provided input on the matter and there are different proposals.

Open issues and candidate options before e-meeting:

Issue 1-2: Introduction of a scaling factor if the PDC-RS collides with a measurement gap

- Proposals

  - Option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and MG.

  - Option 2: Allow for additional delay if there is any overlap between PDC RSs and MG’s.

  - Option 3: Do not define requirements if there is any overlap between PDC RSs and MG’s.

- Recommended WF

  - 3 different proposals have been proposed regarding how to address if there is overlap between PDC RS and a measurement gap.

  - All proposals support that as minimum RAN4 need to account such scenario in the requirements (i.e.  an overlap occur).

  - Moderator propose to initially agree that RAN4 need to define requirements that account a possible overlap between PDC RS and a measurement gap.

  - How to account such overlap in the requirements should be discussed together with Sub-topic 1-3.

  - Moderator propose following agreement:

    - Agree that RAN4 need to define requirements that account a possible overlap between PDC RS and a measurement gap.

    - How to account such overlap in the requirements (options 1-3) will be discussed together with Sub-topic 1-3.

### Sub-topic 1-3

Sub-topic description

How to define the measurement period for PDC if some of the PDC RS resources collide with PPW accounting the following scenarios:

- PRS measurements with Type 1A/1B PPW

  - occasional overlap

  - continuous overlap

- PRS measurements with Type 2 PPW

  - occasional overlap

  - continuous overlap

1 company have provided input on the matter and there are different proposals how to address the different scenarios.

Open issues and candidate options before e-meeting:

Issue 1-3: PDC measurement period if PRS measurements occasionally collide with Type 1A/1B PPW

- Proposals

  - Option 1: PDC measurement period is extended.

  - Option 2: Other.

- Recommended WF

  - More discussion needed. Moderator suggest discuss all Issues 1-3 – 1-6 together and provide a possible compromise proposal below.

Issue 1-4: PDC measurement period if PRS measurements continuously collide with Type 1A/1B PPW

- Proposals

  - Option 1: No measurements requirements for PDC are defined.

  - Option 2: Other.

- Recommended WF

  - More discussion needed. Moderator suggest discuss all Issues 1-3 – 1-6 together and provide a possible compromise proposal below.

Issue 1-5: PDC measurement period if PRS measurements occasionally collide with Type 2 PPW

- Proposals

  - Option 1: PDC measurement period is extended.

  - Option 2: Other.

- Recommended WF

  - More discussion needed. Moderator suggest discuss all Issues 1-3 – 1-6 together and provide a possible compromise proposal below.

Issue 1-6: PDC measurement period if PRS measurements continuously collide with Type 2 PPW

- Proposals

  - Option 1: No measurements requirements for PDC are defined.

  - Option 2: Other.

- Recommended WF

  - More discussion needed. Moderator suggest discuss all Issues 1-3 – 1-6 together and provide a possible compromise proposal below.

- Combined recommended WF for Issues 1-3 – 1-6 (and Issue 1-2):

  - Moderator would like to open for a more general discussion for Issues 1-3 – 1-6 (and maybe also including Issue 1-2).

  - For Issue 1-3 – 1-6:

    - Moderator would like to hear if companies can compromise to a solution where RAN4 handles all raised Issues 1-3 – 1-6, by defining that if an overlap as described in Issue 1-3 – 1-6 occur, the UE is allowed longer measurement period for PDC measurements if the PDC RS has higher priority (assuming such priority is already defined).

    - RAN4 will not be specific about how long additional measurement delay the UE is allowed and will not define such detailed requirements.

    - A continuous overlap of PDC RS and PPW (type 1A/1B/2) can be regarded as a special case of occasional overlap.

      - Hence, the general requirement addressing overlap will address both occasional and continuous overlap.

  - Moderator propose following agreement (Issue 1-3 – 1-6):

    - If PDC measurements overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PDC RS has higher priority. (Assuming such priority is already defined).

  - For Issue 1-2:

    - Similar solution as proposed for Issues 1-3 – 1-6 can apply for PCD RS overlap with any measurement gap:

      - UE is allowed longer measurement period for PDC measurements if one or more PDC RS overlap with a measurement gap.

  - Moderator propose following agreement (Issue 1-2):

    - If one or more PDC RS overlap with a measurement gap UE is allowed longer measurement period for PDC measurements.

### Sub-topic 1-4

Sub-topic description

PDC requirements when DRX is in use.

4 companies have provided input on the matter and there are two different proposals.

Open issues and candidate options before e-meeting:

Issue 1-7: Shall RAN4 define PDC requirements when DRX is in use

- Proposals

  - Option 1: Define DRX requirements for PDC such that UE is assumed to take one sample per DRX cycle.

  - Option 2: Do not define PDC UE Rx-TX measurement period requirements under DRX conditions.

- Recommended WF

  - 3 companies support option 1 and moderator would like to hear if option 1 can be accepted as a compromise from other companies?

  - Moderator propose following agreement:

    - RAN4 also defines PDC requirements when DRX is in use

### Sub-topic 1-5

Sub-topic description

How to define the measurement period requirements for PRS based UE Rx-Tx time difference measurements.

2 companies have provided input on the matter.

Open issues and candidate options before e-meeting:

Issue 1-8: Measurement period requirements for PRS based UE Rx-Tx time difference measurements

- Proposals

  - Option 1: Measurement period requirements for PRS based UE Rx-Tx time difference measurement is TUERxTx,PRS=Nsample *TPRS .

  - Option 2: Update the PRS based PDC requirements as follows:

    - TUERxTx, PRS=NPRSslotN'Lavailable_PRSN*Nsample-1*TeffectPRS+Tlast

- Recommended WF

  - More discussion is needed. The proposals are very similar. Further discuss the final definition.

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 1-1

Issue 1-1: Number of measurement samples assumed for defining the Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC (PRS and TRS based)

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support option 1 and moderator’s suggestion. |
| Nokia | We can compromise to agree on using 4 samples of TRS to align with the legacy. |
| Qualcomm | Support the recommended WF |
| Ericsson | Recommended WF is fine. “RAN4 assume 4 measurement samples are used for defining the Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC (PRS and TRS based).” |
| vivo | Support the recommended WF |

Sub topic 1-2

Issue 1-2: Introduction of a scaling factor if the PDC-RS collides with a measurement gap

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We are fine with all 3 options since all of them are addressing the overlap between PDC RS and MG. We slightly prefer option 1 which gives more flexibility to NW. We are also fine with moderator’s suggestion. |
| Nokia | Support Moderator’s suggestion |
| Qualcomm | We agree that RAN4 should account for possible overlap between PDC RS and measurement gaps in the requirements for PDC. Our preference is option 1. This has been done for other measurements. Are there any impediments to adding a scaling factor? |
| Ericsson | Recommended WF is fine. “Agree that RAN4 need to define requirements that account a possible overlap between PDC RS and a measurement gap. How to account such overlap in the requirements (options 1-3) will be discussed together with Sub-topic 1-3.” |
| vivo | Option 2 or 3. Since it is only serving cell measurement and 4 samples would be allowed for measurement, it can be up to UE implementation to handle some overlapping cases. In addition, PDC RS measurement would belong to L1 measurements. If scaling factor is defined, not only colliding with measurement gaps, but also colliding with SMTC occasions for L3 measurement without gap need to be considered. The requirements would be like L1 measurement requirements (RLM/BFD) and it is quite complicated. Furthermore, measurement restrictions for PDC RS measurement were also agreed. It means even if scaling factor is introduced, it doesn’t provide entire picture of how long the measurement period would be under all the possible cases. Therefore, we think option 2 or 3 should be good enough for PDC RS measurement. |

Sub topic 1-3:

Issue 1-3: PDC measurement period if PRS measurements occasionally collide with Type 1A/1B PPW

Issue 1-4: PDC measurement period if PRS measurements continuously collide with Type 1A/1B PPW

Issue 1-5: PDC measurement period if PRS measurements occasionally collide with Type 2 PPW

Issue 1-6: PDC measurement period if PRS measurements continuously collide with Type 2 PPW

Please at least comment on the proposed compromise proposals:

  - Moderator propose following agreement (Issue 1-3 – 1-6):

    - If PDC measurements overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PDC RS has higher priority.

  - Moderator propose following agreement (Issue 1-2):

    - If one or more PDC RS overlap with a measurement gap UE is allowed longer measurement period for PDC measurements.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support to consider the collision between PPW and PDC RS. We are also fine with moderator’s suggestion, except that for issue 1-3 – 1-6, longer PDC measurement period should be allowed in the cases where PRS (for positioning) has higher priority. |
| Qualcomm | On the moderator’s proposal for issues 1-3 to 1-6: Is there a typo in the proposal? Did it mean to say “if PRS has higher priority”? Edited below: - If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS. The proposal treats all PPW the same regardless of type 1A/1B/2? Also, we assume the proposal is for “occasional” overlap. If all the PDC RS resources overlap as described above then there would be no PDC measurement requirements. On the moderator’s proposal for issue 1-2, the proposal is to extend the PDC measurement period by an unspecified amount? |
| Ericsson | Recommended WF is fine. |
| vivo | The following revision would be fine for us. o Moderator propose following agreement (Issue 1-3 – 1-6):  If PDC measurements overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PDC RS has higher lower priority. o Moderator propose following agreement (Issue 1-2):  If one or more PDC RS overlap with a measurement gap UE is allowed longer measurement period for PDC measurements. |

Sub topic 1-4

Issue 1-7: Shall RAN4 define PDC requirements when DRX is in use

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support option 1 and moderator’s suggestion. |
| Nokia | We can accept the agreement proposed by the Moderator |
| Qualcomm | Support option 1 |
| Ericsson | OK to define requirements for DRX. One concern is how to develop requirements, given the performance simulations we already have developed (for TRS) and the requirements we propose to reuse from positioning (PRS). Samples will now be spread out differently. Do we need yet another TRS run and a new run for PRS? |
| vivo | Support option 1 |

Sub topic 1-5

Issue 1-8: Measurement period requirements for PRS based UE Rx-Tx time difference measurements

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | The two options are similar in addressing the applicable capability for PRS based PDC. We prefer option 2 because the “max number of DL PRS resources that UE can process in a slot” is applicable for PRS based PDC, as in Component 3 for FG 25-19a. |
| Qualcomm | Support option 2. |
| Ericsson | Option 2. |
| vivo | Fine with option 2. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213049, CR to TS 38.133 Correction to measurements requirements for PDC, vivo | Huawei: for PRS based PDC, it depends on outcome of issue 1-8. For both PRS and TRS based PDC, outcomes from issue 1-3 – 1-6 and 1-7 need to be captured. |
|  | Ericsson: Proposal 1: Measurement period requirements for PRS based UE Rx-Tx time difference measurement is T_{UERxTx,PRS}=N_{sample}\ \ast T_{PRS}\ . We prefer option 2 in issue 1-8. TUERxTx, PRS=NPRSslotN'*Nsample*TPRS Proposal 2: Accuracy requirements for RTT-based PDC with TRS are based on measurements with 4 samples under AWGN channel. Ericsson: OK. Good to have fading robustness in practice, even if no formal fading reqs. However in our simulations we see no difference for AWGN between 1 and 4 samples. We do see a difference between 1 and 4 samples for fading channels. Proposal 3: No requirements are defined for the case when PDC resources are collided with gaps. Ericsson: This is discussed in issue 1-2 and needs to be aligned with conclusion. Proposal 4: For PRS/TRS based UE Rx-Tx time difference measurement for TT-based PDC, DRX related requirements are specified with the assumption that UE measures once per DRX cycle. OK, but what happens with performance? Do we need new requirements? |
|  | vivo: Will be updated based on conclusion of open issues. |
| R4-2213553, CR on requirements for UE Rx-Tx measurement for PDC, Huawei | Qualcomm: No agreement yet on the measurement period requirement with DRX. It has not been discussed in the first round. so this change needs discussion: Teffect=maxTPRS,TDRX, when DRX is in use. Same for this change: maxTTRS,TDRX |
|  | Company B |
| R4-2213657, CR on reporting mapping for URLLC in TS 38.133, MediaTek | Qualcomm: Is there a prior agreement for this change? The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the measurement report mapping requirements specified in clause 10.1.25.3, with k = 5. MediaTek: Yes, this is based on agreement in RAN1 which is sent to RAN4 in LS [R1-2112834]. The agreement is given as: ‘Agreement: If RTT-based PDC is supported, a single granularity 32Tc (i.e. k=5) is supported for Rx-Tx measurement report.’ However, RAN4 didn’t discuss this agreement but we believe RAN4 should follow RAN1 for such configurations. |
|  | Company B |
| R4-2212156, draftCR to clarify timing reference point for UE UL timing test cases, Intel | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1: Number of measurement samples assumed for defining the Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC (PRS and TRS based) Tentative agreements: Yes. All companies can agree to the proposed agreement. Moderator thanks companies for the willingness to compromise. Agreement: - RAN4 assume 4 measurement samples are used for defining the Rel-17 UE Rx-Tx measurement requirements for RTT-based PDC (PRS and TRS based). Candidate options: Not needed Recommendations for 2nd round: Sub-topic 1-1 is closed |

|  | Status summary |
| --- | --- |
| Sub-topic #1-2 | Issue 1-2: Introduction of a scaling factor if the PDC-RS collides with a measurement gap Tentative agreements: Yes, all companies agree RAN4 need to define requirements that account a possible overlap between PDC RS and a measurement gap. Agreement: - RAN4 need to define requirements that account a possible overlap between PDC RS and a measurement gap. Candidate options: Not needed. How to account such overlap in the requirements (options 1-3) will be discussed together with Sub-topic 1-3. Recommendations for 2nd round: Sub-topic 1-2 is closed |

|  | Status summary |
| --- | --- |
| Sub-topic #1-3 | How to define the measurement period for PDC if some of the PDC RS resources collide with PPW Moderator comment: Yes, it is correct that an error sneaked its way into the proposal and ‘higher’ should be ‘lower’ or alternatively ‘PDC’ should be ‘PRS’. Thanks for pointing out the mistake. Originally the intention was a like listed by Qualcomm: - If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS. To address Qualcomm’s question ‘On the moderator’s proposal for issue 1-2, the proposal is to extend the PDC measurement period by an unspecified amount?’: The delay would not be explicitly specified, but one could expect the delay to be depending on the amount of overlap – similar to defining Kgap scaling factor. Tentative agreements: Yes and No. There is still need for more discussion to agree on the details. Candidate options: Based on the discussion it is moderator’s impression that all companies can agree on defining the measurement period for PDC accounting if some of the PDC RS resources collide with PPW (issue 1-3 – 1-6). For Issue 1-3 – 1-6: - If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS. For issue 1-2 (PDC-RS collides with one or more measurement gap) topics are now down to following two options: For Issue 1-2: - Option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and MG. - Option 2: Allow for additional delay if there is any overlap between PDC RSs and MG’s. Recommendations for 2nd round: Confirm following agreement for Issues 1-3 – 1-6: - If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS. Further discuss how to account for overlap between PDC RS and MG, and agree on one of the two options for Issue 1-2: - Option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and MG. - Option 2: Allow for additional delay if there is any overlap between PDC RSs and MG’s. |

|  | Status summary |
| --- | --- |
| Sub-topic #1-4 | Issue 1-7: Shall RAN4 define PDC requirements when DRX is in use Tentative agreements: Yes. All companies can agree to defining PDC requirements when DRX is in use Agreement: - RAN4 will define PDC requirements when DRX is in use Candidate options: Not needed Recommendations for 2nd round: Ericsson raised following question in the first round: How to develop requirements, given the performance simulations we already have developed (for TRS) and the requirements we propose to reuse from positioning (PRS). Samples will now be spread out differently. Do we need yet another TRS run and a new run for PRS? |

|  | Status summary |
| --- | --- |
| Sub-topic #1-5 | Issue 1-8: Measurement period requirements for PRS based UE Rx-Tx time difference measurements Tentative agreements: Yes. All companies can support option 2. Agreement: - Update the PRS based PDC requirements as follows: - TUERxTx, PRS=NPRSslotN'*Nsample*TPRS Candidate options: Not needed Recommendations for 2nd round: For 2nd round RAN4 need to agree on how to account the agreement for Issue 1-7, Issue 1-2 and Issues 1-3 – 1-6 in the measurement period: - DRX - Overlap with measurement gaps (pending agree in Issue 1-2) - PDC RS resources overlap with Type 1A/1B/2 PPW This can be done by directly discuss the wording in the CR. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2213049, CR to TS 38.133 Correction to measurements requirements for PDC, vivo | Noted |
| R4-2213553, CR on requirements for UE Rx-Tx measurement for PDC, Huawei | Revised |
| R4-2213657, CR on reporting mapping for URLLC in TS 38.133, MediaTek | Qualcomm commented on the CR in first round and MTK provided an answer. Return to |
| R4-2212156, draftCR to clarify timing reference point for UE UL timing test cases, Intel | Agreeable |

## Open issues 2nd round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1-3 | Issue 1-3 – 1-6: PDC measurement period if PRS measurements occasionally collide with Type 1A/1B PPW Confirm following agreement for Issues 1-3 – 1-6: - If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS. Issue 1-2: Further discuss how to account for overlap between PDC RS and MG, and agree on one of the two options for Issue 1-2: - Option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and MG. - Option 2: Allow for additional delay if there is any overlap between PDC RSs and MG’s. |

|  | Status summary |
| --- | --- |
| Sub-topic #1-4 | Issue 1-9: How to develop requirements when DRX is in use In first round it was agreed to define requirements also under DRX in use conditions. One company raised following question: How to develop requirements, given the performance simulations we already have developed (for TRS) and the requirements we propose to reuse from positioning (PRS). Samples will now be spread out differently. Do we need yet another TRS run and a new run for PRS? |

|  | Status summary |
| --- | --- |
| Sub-topic #1-5 | For 2nd round RAN4 need to agree on how to account the agreement for Issue 1-7, Issue 1-2 and Issues 1-3 – 1-6 in the measurement period: - DRX - Overlap with measurement gaps (pending agree in Issue 1-2) - PDC RS resources overlap with Type 1A/1B/2 PPW This can be done by directly discuss the wording in the CR. |

## Companies views’ collection for 2nd round

### Open issues

Sub-topic 1-3:

Issue 1-3 1-6: Confirm agreement?

and Issue 1-2: How to account for overlap between PDC RS and MG?

| Company | Comments |
| --- | --- |
| Ericsson | Priorities of this kind is a matter for RAN1. |
| Qualcomm | Issue 1-3 1-6: We’re OK with the agreement proposed by the moderator. To avoid the issue of defining the priority for PDC RS, we could also compromise to the modified version proposed by vivo in GTW: - If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS. Issue 1-2: We prefer option 1 but could compromise to option 2. |
| Huawei | Issue 1-3 – 1-6: We also support the compromised proposal from vivo during GTW: - If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS. Issue 1-2: We prefer option 1 but could compromise to option 2. |
| Ericsson | For issue 1-3- 1-6: - If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS or PRS used for PDC measurements. If I understand it correctly, the question is about how to resolve collision between PRS for PDC -vs- other DL signal (e.g., CSI-RS, PRS for positioning, SIB …). For this question, RAN1 has discussed and agreed to the following. Basically, gNB should configure properly to avoid collision. The reasons were: - PRS for PDC is only with active BWP of PCell. It should be easily to configure such that there is no collision. RAN1#108e Agreement For a UE configured with DL PRS for RTT-based PDC, the UE doesn't expect to be configured/scheduled to receive any other DL channel/signal in the PRBs that overlap those of the DL PRS for PDC in the OFDM symbols occupied by the DL PRS for PDC. - Spec change(s) if any is up to the editor. If UE doesn't expect to be configured/scheduled to receive any other DL channel/signal in the PRBs that overlap those of the DL PRS for PDC in the OFDM symbols occupied by the DL PRS for PDC, then we think that we do not need a rule to allow longer measurement period. Issue 1-2: How to account for overlap between PDC RS and MG: Same comment as for issue 1-3- 1-6: If UE doesn't expect to be configured/scheduled to receive any other DL channel/signal in the PRBs that overlap those of the DL PRS for PDC in the OFDM symbols occupied by the DL PRS for PDC, then we think that we do not need a rule. |
| vivo | Issue 1-3 – 1-6: We support the compromised proposal: - If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS. Issue 1-2: We prefer option 2. On one hand, it is possible for NW for avoid such collision since both MG and PDC RS are both UE specific and related to current serving cell. On the other hand, if collision between MG is considered, it is rational also to consider collision between PDC-RS and SMTC which is used for intra-frequency measurement. Defining scaling factor for MG collision only is not a complete solution. |

Sub-topic 1-4:

Issue 1-9: How to develop requirements when DRX is in use

| Company | Comments |
| --- | --- |
| Ericsson | IN RAN4#103 we commented: Is long DRX cycles compatible with accurate time distribution and services integrated with TSN (Time Sensitive Networks)? For integration with TSN networks then periodic delivery of timing packets are expected and accurate 5GS clock is needed for packet residence time compensation. Accurate 5GS timing would require regular connection to gNB for time and frequency stability. Waking up after DRX is an inaccurate process the Te value alone is much bigger than the whole control-to-control budget of +/- 145-275 ns. For PDC, specifically, it needs to reflect the delay and channel present when the 5GS time was distributed and hence there is a dependency between the 5GS delivery and PDC occasion (when PDC measurement is executed). A couple of points for RAN4#104: - As pointed out above, accurate TSN clocks are delivered periodically. Will there be time for DRX sleep? - Waking up after DRX is an inaccurate process the Te value alone is much bigger than the whole control-to-control budget of +/- 145-275 ns. We need to add the Te after waking up to the requirements for DRX. We then get accurate requirements (without DRX) and inaccurate requirements (with DRX). ### 7.1.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies: - when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS |
| Huawei | To Ericsson: PDC measurement is configured by NW via RRC signalling, and same as DRX, so we assume if the NW sees the PDC measurement as time critical or urgently needed, it can configure non-DRX or small DRX cycles, i.e. everything is up to NW control. On the PDC accuracy, we do not think UL timing error is an issue for the PDC because UE or gNB will combine the Rx-Tx measurement from both UE and gNB. As to the UE Rx-Tx measurement accuracy, the impact of UL and DL sync error would be accounted in the group delay calibration margin, so there is no need to account for Te (UL timing error) again. Also, even without DRX, the UL timing requirement is still Te, so we do not see any difference in the measurement accuracy. |
| vivo | We are open to further study how to define DRX related requirements. |

Sub-topic 1-5:

Issue 1-9: How to develop requirements when DRX is in use

| Company | Comments |
| --- | --- |
| Moderator | No need for company’s comments. Please account the agreement from 1st round: - DRX - Overlap with measurement gaps (pending agree in Issue 1-2) - PDC RS resources overlap with Type 1A/1B/2 PPW into the measurement period requirements for PRS based UE Rx-Tx time difference measurements, agreed in the 1st round. |

### CRs/TPs comments collection

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2213553_revised, CR on requirements for UE Rx-Tx measurement for PDC, Huawei |  |
| R4-2213657_revised, CR on reporting mapping for URLLC in TS 38.133, MediaTek |  |

## Summary for 2nd round

### Open issues

Sub-topic 1-3, Issue 1-3 – 1-6: PDC measurement period if PRS measurements occasionally collide with Type 1A/1B PPW

Confirm following agreement for Issues 1-3 – 1-6:

- If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS.

Outcome:

3 companies can support following updated proposal:

- If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements.

1 company has concerns.

Agreement: No

Way Forward:

Companies to bring discussion and proposal for next meeting addressing the open issue:

- Discuss possible UE requirements if PDC RS resources overlap with Type 1A/1B/2 PPW.

Sub-topic 1-3, Issue 1-2: how to account for overlap between PDC RS and MG, and agree on one of the two options for Issue 1-2

Further discuss how to account for overlap between PDC RS and MG, and agree on one of the two options for Issue 1-2:

- Option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and MG.

- Option 2: Allow for additional delay if there is any overlap between PDC RSs and MG’s.

Outcome:

2 companies prefer option 1 but can compromise to option 2.

1 company prefer option 2

1 company have concern on agreeing either of the options

Agreement: No

Way Forward:

Companies to bring discussion and proposal for next meeting addressing the open issue:

Discuss how to account for overlap between PDC RS and MG, and agree on one of the two options for Issue 1-2:

- Option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and MG.

- Option 2: Allow for additional delay if there is any overlap between PDC RSs and MG’s.

Sub-topic 1-4, Issue 1-9: How to develop requirements when DRX is in use

Outcome:

No consensus among companies

Agreement: No

Way forward:

Companies to bring discussion and proposal for next meeting addressing the open issue: How to develop requirements when DRX is in use.

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2213553_revised, CR on requirements for UE Rx-Tx measurement for PDC, Huawei | Return To. Moderator: Based on the 2nd round discussion outcome of Sub-topics 1-3 and 1-4. New draft CR available removing requirements related to Sub-topics 1-3 and 1-4. |
| R4-2213657_revised, CR on reporting mapping for URLLC in TS 38.133, MediaTek | Agreeable |

# Topic #2: RRM performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

This topic discusses contributions submitted under 9.21.2.1 - 9.21.2.4:

- General (test configurations, conditions and etc) (3 contributions including 1 draftCR)

- Measurement period and accuracy requirements (5 contributions including 1 draftCR)

- Test cases for FR1

  - Test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC (2 contributions including 1 draftCR)

  - Test cases for UE Rx-Tx time difference measurement with TRS for RTT-based PDC (1 contributions including 1 draftCR)

- Test cases for FR2]

  - Test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC (2 contributions including 2 draftCR)

  - Test cases for UE Rx-Tx time difference measurement with TRS for RTT-based PDC (0 contributions)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| 9.21.2.1 |  |  |
| R4-2213050 | vivo | Proposal 1: Accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on PRS and gNB Rx-Tx time difference measurement based on SRS, with measurement of 4 samples. Proposal 2: Accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on TRS and gNB Rx-Tx time difference measurement based on SRS, with measurement of 4 samples. |
| R4-2213776 | Nokia | Proposal 1: Consider table 1 and table 2 for usagePDC-r17 SRS Configurations. |
| 9.21.2.2 |  |  |
| R4-2212207 | Qualcomm | Includes simulation result. Observation 1: Simulation results for UE Rx-Tx time difference using TRS are shown in Table 1 and Table 2. The results do not include any margin to account for UE Rx/Tx group delay calibration error. |
| R4-2212901 | Ericsson | Includes simulation result. Consider Tables 1 to 8 in the development of PDC requirements based on TRS |
| R4-2213555 | Huawei | Proposal 1: Define accuracy requirements for PRS and TRS based Rx-Tx in AWGN only. Proposal 2: Verify measurement period requirements and accuracy requirements in the same TC. Proposal 3: In each TC, include sub-tests for two different PRS/TRS BWs. Define new TRS RMC with 24 RB for 15k and 30kHz SCS, and 32 RB for 120kHz SCS. |
| R4-2213775 | Nokia | Includes simulation results. For UE Rx-Tx time difference measurement accuracy with TRS, we have the following observations and proposals: Observation 1: The TRS measurement sample number has minor effect on the TUE-RX error. Observation 2: The TRS TUE-RX error changes significantly between AWGN and fading channels in both FR1 and FR2. |
| 9.21.2.3.1 |  |  |
| R4-2213890 | Nokia | Proposal 1: The test case for PDC PRS measurement period can reuse the current test case defined in TS 38.133-A6.6.14.1 by adjusting the scenario to single cell and updating with the PDC PRS configuration and SRS with usagePDC-r17 defined in R2-2203766. Proposal 2: The test case for PDC PRS measurement accuracy can reuse the current test case defined in TS 38.133-A6.7.15.1 by adjusting the scenario to single cell and updating with the PDC PRS configuration and SRS with usagePDC-r17 defined in R2-2203766 Observation 1: The current positioning PRS test case for measurement period and accuracy are separated. Observation 2: The test case procedures and part of the test configurations for PRS measurement period and accuracy are different. Proposal 3: Define the test cases for PDC measurement period and accuracy separately. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Shall RAN4 define accuracy requirements for PRS and TRS based Rx-Tx in fading channel conditions

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed.

### Sub-topic 2-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2: Consider table 1 and table 2 in R4-2213776 for usagePDC-r17 SRS Configurations

- Proposals

  - Option 1: Yes

  - Option 2: No (Please also provide necessary changes)

- Recommended WF

  - More discussion needed.

### Sub-topic 2-3

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-3: Verify PDC measurement period requirements and accuracy requirements in the same TC

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed.

### Sub-topic 2-4

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-4: Each TC, include sub-tests for two different PRS/TRS BWs

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed.

### Sub-topic 2-5

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-5: Define new TRS RMC with 24 RB for 15k and 30kHz SCS, and 32 RB for 120kHz SCS

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed.

### Sub-topic 2-6

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-6: The test case for PDC PRS measurement period can reuse the current test case defined in TS 38.133-A6.6.14.1 with modifications

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed.

### Sub-topic 2-7

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-7: The test case for PDC PRS measurement accuracy can reuse the current test case defined in TS 38.133-A6.7.15.1 with modifications

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed.

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1

Issue 2-1: Shall RAN4 define accuracy requirements for PRS and TRS based Rx-Tx in fading channel conditions

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 2. In our view, the typical scenario for PDC is IIOT, where fading channel is not quite relevant. Also, the accuracy under fading channel is clearly worse than AWGN, making it more challenging to meet the error budget for PDC. Last, it was agreed that there is no corresponding gNB measurement requirements for fading, and it is reasonable to have same scope for UE and gNB requirements. |
| Qualcomm | Option 2 (No) |
| Ericsson | There could be large area use cases with mobility subjected to fading. To save time we could rely on fading results already developed for UE TRS in this WI and existing results related to UE positioning. |
| vivo | Option 2 |

Sub topic 2-2

Issue 2-2: Consider table 1 and table 2 in R4-2213776 for usagePDC-r17 SRS Configurations

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1 is fine, but do we also need a new SRS RMC for 120kHz SCS? |
| Nokia | Support Option 1 |
| Qualcomm | Option 1 looks OK. |
| Ericsson | OK |

Sub topic 2-3

Issue 2-3: Verify PDC measurement period requirements and accuracy requirements in the same TC

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. If we look into the test setup in Rel-16 Rx-Tx test cases, it can be seen that the test conditions are almost the same in delay and accuracy TCs. Therefore, we suggest to merge delay and accuracy TCs, i.e. to verify both delay and accuracy in a single TC. This will save RAN4 spec efforts and UE testing efforts. |
| Nokia | We agree that most of the test parameter configuration will be same for both accuracy test and delay test. But the test case procedures are different, and the current spec separates the test cases for accuracy and delay. We don’t see any significant benefit for combining the test cases. |
| Qualcomm | Option 1 (Yes) |
| Ericsson | We prefer the existing specification structure where measurement period and accuracy are separate sections. |
| vivo | Option 1. We have such agreement in the last meeting. Agreement: - RAN4 agree to use one set of test cases to verify both the delay and accuracy of PDC measurement |

Sub topic 2-4

Issue 2-4: Each TC, include sub-tests for two different PRS/TRS BWs

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. This is following same principle as Rel-16 Rx-Tx accuracy test cases. |
| Qualcomm | RTT-based PDC is a bit different from UE Rx-Tx for positioning. The RS is received only from serving cell and within active DL BWP. Also, in the UE capability for RTT-based Propagation delay compensation based on DL PRS and SRS (25-19a) there is no constraint on the maximum PRS bandwidth supported by the UE. It can be up to the bandwidth of the BWP. We assume the test case will support multiple test configurations for FDD vs. TDD and SCS. For each configuration it should suffice to test one PRS/TRS configuration occupying the channel BW. |
| Ericsson | We have used rel-16 base for RMC for our CRs for PDC case in R4-2212902-03 and do two different PRS/BW. |
| vivo | Slightly prefer Option 1. |

Sub topic 2-5

Issue 2-5: Define new TRS RMC with 24 RB for 15k and 30kHz SCS, and 32 RB for 120kHz SCS

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. For PDC measurement, the PRS BW needs to be within the active BW. Without new RMC, only the smallest PRS BW can be used for test. In order to test different RS BWs, we suggest to define new RMC based on serving cell RF BW. |
| Qualcomm | See response to issue 2-4. Why not set TRS BW to the channel BW for each test configuration? |
| Ericsson | We have used rel-16 base for RMC for our CRs for PDC case in R4-2212902-03 and do not rely on new RMC. |
| vivo | Depending on issue 2-4 |

Sub topic 2-6

Issue 2-6: The test case for PDC PRS measurement period can reuse the current test case defined in TS 38.133-A6.6.14.1 with modifications

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1 is fine. |
| Nokia | Support Option 1 |
| Qualcomm | Yes, it can be used as the baseline. |
| Ericsson | We agree. This is basis for our CR R4-2212902, with adjustments to single cell and configurations. |
| vivo | Option 1 is fine. |

Sub topic 2-7

Issue 2-7: The test case for PDC PRS measurement accuracy can reuse the current test case defined in TS 38.133-A6.7.15.1 with modifications

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1 is fine. |
| Nokia | Support Option 1 |
| Qualcomm | Yes, it can be used as the baseline. |
| Ericsson | We agree. This is basis for our CR R4-2212903, with adjustments to single cell and configurations. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213554. CR to introduce TRS RMC for PDC tests, Huawei | Qualcomm: Pending issue 2-5. |
|  | Company B |
| R4-2213556, CR on PDC measurement accuracy requirements, Huawei | Qualcomm: Was there a prior agreement to define the report mapping with 32*Tc resolution? |
|  | Company B |
| R4-2213863, draftCR on test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA, Nokia | Huawei: pending on issue 2-3 |
|  | Company B |
| R4-2213557, CR to introduce TC#2 for PDC measurement, Huawei | Qualcomm: - Cell 1 does not have PRSTRS transmission during T1 and transmits PRSTRS during T2. - Could you clarify where T = 10 ms comes from? - TRS configs depend on issue 2-5. |
|  | Ericsson, OK but if we agree on subtests (like this CR) then all CR has to change to this as per work split. |
| R4-2212902, UE Rx-Tx time difference delay PDC test based on PRS/SRS, Ericsson | Huawei: pending on issue 2-3 |
|  | Company B |
| R4-2212903, UE Rx-Tx time difference accuracy PDC test based on PRS/SRS, Ericsson | Huawei: pending on issue 2-3 |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1: Shall RAN4 define accuracy requirements for PRS and TRS based Rx-Tx in fading channel conditions Tentative agreements: No. 3 companies support option 2 (RAN4 will not define accuracy requirements for PRS and TRS based Rx-Tx in fading channel conditions) while one company questions this approach Candidate options: Issue 2-1: Shall RAN4 define accuracy requirements for PRS and TRS based Rx-Tx in fading channel conditions - Proposals - Option 1: Yes - Option 2: No Recommendations for 2nd round: More discussion needed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-2 | Issue 2-2: Consider table 1 and table 2 in R4-2213776 for usagePDC-r17 SRS Configurations Tentative agreements: Yes. All companies support option 1. Agreement: - RAN4 will consider table 1 and table 2 in R4-2213776 for usagePDC-r17 SRS Configurations Candidate options: Not needed Recommendations for 2nd round: Sub-topic 2-2 is closed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-3 | Issue 2-3: Verify PDC measurement period requirements and accuracy requirements in the same TC Tentative agreements: Yes. As pointed out by vivo: In the agreed WF from RAN4#103: ## Sub-topic 3-7 Issue 3-8: Test case list for measurement core requirements Issue 3-9: Test case list for measurement accuracy requirements Agreement: - RAN4 agree to use one set of test cases to verify both the delay and accuracy of PDC measurement Moderator suggest not to open the discussion further and keep the agreement from 103 meeting. Candidate options: Not needed Recommendations for 2nd round: sub-topic 2-3 is closed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-4 | Issue 2-4: Each TC, include sub-tests for two different PRS/TRS BWs Tentative agreements: No Candidate options: Issue 2-4: Each TC, include sub-tests for two different PRS/TRS BWs - Proposals - Option 1: Yes - Option 2: No Recommendations for 2nd round: More discussion needed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-5 | Issue 2-5: Define new TRS RMC with 24 RB for 15k and 30kHz SCS, and 32 RB for 120kHz SCS Tentative agreements: No Candidate options: Issue 2-5: Define new TRS RMC with 24 RB for 15k and 30kHz SCS, and 32 RB for 120kHz SCS - Proposals - Option 1: Yes - Option 2: No Recommendations for 2nd round: More discussion needed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-6 | Issue 2-6: The test case for PDC PRS measurement period can reuse the current test case defined in TS 38.133-A6.6.14.1 with modifications Tentative agreements: Yes. All companies support that the test case for PDC PRS measurement period can reuse the current test case defined in TS 38.133-A6.6.14.1 with modifications. Agreement: - The test case for PDC PRS measurement period can reuse the current test case defined in TS 38.133-A6.6.14.1 with modifications Candidate options: Not needed Recommendations for 2nd round: Sub-topic 2-6 is closed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-7 | Issue 2-7: The test case for PDC PRS measurement accuracy can reuse the current test case defined in TS 38.133-A6.7.15.1 with modifications Tentative agreements: Yes. All companies support that the test case for PDC PRS measurement accuracy can reuse the current test case defined in TS 38.133-A6.7.15.1 with modifications. Agreement: - The test case for PDC PRS measurement accuracy can reuse the current test case defined in TS 38.133-A6.7.15.1 with modifications. Candidate options: Not needed Recommendations for 2nd round: Sub-topic 2-7 is closed |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2213554. CR to introduce TRS RMC for PDC tests, Huawei | To be revised |
| R4-2213556, CR on PDC measurement accuracy requirements, Huawei | Return to |
| R4-2213863, draftCR on test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA, Nokia | To be revised |
| R4-2213557, CR to introduce TC#2 for PDC measurement, Huawei | To be revised |
| R4-2212902, UE Rx-Tx time difference delay PDC test based on PRS/SRS, Ericsson | To be revised |
| R4-2212903, UE Rx-Tx time difference accuracy PDC test based on PRS/SRS, Ericsson | To be revised |

## Open issues 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1: Shall RAN4 define accuracy requirements for PRS and TRS based Rx-Tx in fading channel conditions Tentative agreements: No. 3 companies support option 2 (RAN4 will not define accuracy requirements for PRS and TRS based Rx-Tx in fading channel conditions) while one company questions this approach Candidate options: Issue 2-1: Shall RAN4 define accuracy requirements for PRS and TRS based Rx-Tx in fading channel conditions - Proposals - Option 1: Yes - Option 2: No Recommendations for 2nd round: More discussion needed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-4 | Issue 2-4: Each TC, include sub-tests for two different PRS/TRS BWs Tentative agreements: No Candidate options: Issue 2-4: Each TC, include sub-tests for two different PRS/TRS BWs - Proposals - Option 1: Yes - Option 2: No Recommendations for 2nd round: More discussion needed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-5 | Issue 2-5: Define new TRS RMC with 24 RB for 15k and 30kHz SCS, and 32 RB for 120kHz SCS Tentative agreements: No Candidate options: Issue 2-5: Define new TRS RMC with 24 RB for 15k and 30kHz SCS, and 32 RB for 120kHz SCS - Proposals - Option 1: Yes - Option 2: No Recommendations for 2nd round: More discussion needed |

## Companies views’ collection for 2nd round

### Open issues

Issue 2-1: Shall RAN4 define accuracy requirements for PRS and TRS based Rx-Tx in fading channel conditions

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Agreed in GTW |

Issue 2-4: Each TC, include sub-tests for two different PRS/TRS BWs

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Agreed in GTW |

Issue 2-5: Define new TRS RMC with 24 RB for 15k and 30kHz SCS, and 32 RB for 120kHz SCS

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Agreed in GTW |

### CRs/TPs comments collection

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2213554_revised. CR to introduce TRS RMC for PDC tests, Huawei |  |
| R4-2213556_revised, CR on PDC measurement accuracy requirements, Huawei |  |
| R4-2213863_revised, draftCR on test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA, Nokia |  |
| R4-2213557_revised, CR to introduce TC#2 for PDC measurement, Huawei |  |
| R4-2212902_revised, UE Rx-Tx time difference delay PDC test based on PRS/SRS, Ericsson |  |
| R4-2212903_revised, UE Rx-Tx time difference accuracy PDC test based on PRS/SRS, Ericsson |  |

## Summary for 2nd round

### Open issues

All Issues closed in GTW.

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2213554_revised. CR to introduce TRS RMC for PDC tests, Huawei | Noted |
| R4-2213556_revised, CR on PDC measurement accuracy requirements, Huawei |  |
| R4-2213863_revised, draftCR on test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA, Nokia | Agreeable |
| R4-2213557_revised, CR to introduce TC#2 for PDC measurement, Huawei | Agreeable |
| R4-2212902_revised, UE Rx-Tx time difference delay PDC test based on PRS/SRS, Ericsson | Agreeable |
| R4-2212903_revised, UE Rx-Tx time difference accuracy PDC test based on PRS/SRS, Ericsson | Not pursued |

# Topic #3: RRM performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

This topic discusses contributions submitted under 9.21.3.1:

- PUCCH requirements  (4 contributions including 0 draftCR)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| 9.21.3.1 |  |  |
| R4-2212248 | Ericsson | Proposal 1: Define new requirement for sub-slot PUCCH repetition HARQ-ACK. Proposal 2: Take following parameter configurations (see Tdoc for details) for sub-slot PUCCH format 0 repetition demodulation requirements if it is agreed to be introduced |
| R4-2213670 | Samsung | Proposal 1: RAN4 defines PUCCH sub-slot repetition requirement with subslotlenght as 7 Proposal 2: RAN4 defines PUCCH sub-slot repetition requirement with sub slot repetition as 2 Proposal 3: RAN4 defines PUCCH sub-slot repetition requirement with inter-slot hopping enable - 5 as the startingsymbol index for first sub-slot repetition - 12 as the startingsymbol index for second sub-slot repetition Proposal 4: Reuse existing Rel-15 PUCCH format 0 requirement with 2 symbols for PUCCH requirement with sub-slot repetition. |
| R4-2213773 | Nokia | Proposal 1: RAN4 to define PUCCH demodulation performance requirements for PF0, with sub slot repetition and inter sub-slot hopping, but condition introduction in the specification on the availability of at least 2 simulation inputs. Proposal 2: Enable inter sub-slot frequency hopping. Proposal 3: Take the above table of test parameters for multi-subslot PUCCH format 0, as the baseline of the discussion in RAN4. Proposal 4: Use DTX->Ack, miss-ACK and NACK-ACK as performance indicators for multiple sub-slot PUCCH with format0. |
| R4-2213820 | Huawei | Proposal 1: Introduce PF0 performance requirements with sub slot repetition. Proposal 2: Use test assumptions in Table 2-1: |

### Sub-topic 3-1

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-1: Test assumptions for PUCCH requirements

- Recommended WF

  - More discussion needed. Companies are encouraged to fill in the ‘combined table’ below.

Further below each of the tables provided by the different companies are listed. Here below is one additional table which is used to collect a potential combined set of parameters. Some parameters have been filled based on initial reading of proposals in the company tables. Companies can edit and add their preferences in the table. Please also fill in new rows with any parameters are missing.

Combined table from moderator (please edit as needed):

| Parameter | Test | status |  |
| --- | --- | --- | --- |
| SCS(kHz)/Bandwidth(MHz) | 30/20 | more discussion | Nokia: The PUCCH discussion has been treated in [104-e][329] NR_IIOT_URLLC_enh_Demod And most of the parameters have achieved agreements. |
| Propagation conditions | TDLC300-100 Low | more discussion | Company XX |
| Antenna condition | 1T2R | more discussion | Company XX |
| Number of sub-slots | 2 | more discussion | Company XX |
| Number of UCI information bits | 1 | 1 agreeable? | Company XX |
| Number of PRBs | 1 | 1 agreeable? | Company XX |
| Number of symbols in a sub-slot | 2 | 2 agreeable? | Company XX |
| First PRB prior to frequency hopping | 0 | 0 agreeable? | Company XX |
| Intra-sub-slot frequency hopping | N/A | agree: disabled | Company XX |
| Inter-sub-slot frequency hopping | Enabled | agree: enabled | Company XX |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) | agree: The largest PRB index – (Number of PRBs – 1) | Company XX |
| Group and sequence hopping | neither | agree: neither | Company XX |
| Hopping ID | 0 | agree: 0 | Company XX |
| Initial cyclic shift | 0 | agree: 0 | Company XX |
| First symbol | 5, 12 | agree: 5, 12 | Company XX |

R4-2212248:

| Parameter | Test |
| --- | --- |
| Number of sub-slots | 2 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| Number of symbols in a sub-slot | 2 |
| First PRB prior to frequency hopping | 0 |
| Intra-sub-slot frequency hopping | N/A |
| Inter-sub-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 5, 12 |

R4-2213670:

| Parameter | Test |
| --- | --- |
| SCS/BW | 15KHz SCS/10MHz |
| Number of UCI information bits | 1 |
| SubslotLengthfor PUCCH | 7 |
| Number of repetition | 2 |
| Inter sub-slot hopping | Enabled |
| First PRB prior to frequency hopping | 0 |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of symbols | 2 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 5 for first sub-slot repetition 12 for second sub-slot repetition |

R4-2213773:

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of PUCCH symbols | 2 |
| Number of PUCCH repetitions (nrofSlots) | 4 |
| Number of Sub-slot symbols (subslotLengthForPUCCH-r16) | 7 |
| Number of slots | 2 |
| First symbol of sub-slot (startingSymbolIndex) | 5 |
| First PRB prior to frequency hopping | 0 |
| Intra-subslot frequency hopping (intraSlotFrequencyHopping) | disabled |
| Inter-subslot frequency hopping (interslotFrequencyHopping) | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |

R4-2213820:

| Parameter | Test |
| --- | --- |
| SCS(kHz)/Bandwidth(MHz) | 30/20 |
| Propagation conditions | TDLC300-100 Low |
| Antenna condition | 1T2R |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| Number of symbols | 2 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Disabled |
| Inter-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Sub slot length | 2 symbols |
| Repetition number | 2 |
| Test metric | Prob(ACK miss)<1% Prob(DTX to ACK)<1% |

## Open issues 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Open issues

Please fill in comany vies where indicated.

| Parameter | Test | status | Company comments |
| --- | --- | --- | --- |
| SCS(kHz)/Bandwidth(MHz) | 30/20 | Agreed: 30kHz/10MHz | N/A. |
| Propagation conditions | TDLC300-100 Low | more discussion | Company XX |
| Antenna condition | 1T2R | more discussion | Company XX |
| Number of sub-slots | 2 | Agreement: 2 | N/A |
| Number of UCI information bits | 1 | agreement:1 bit | N/A |
| Number of PRBs | 1 | 1 agreeable? | Company XX |
| Number of symbols in a sub-slot | 2 | agreement: 2 | N/A |
| First PRB prior to frequency hopping | 0 | 0 agreeable? | Company XX |
| Intra-sub-slot frequency hopping | N/A | agree: disabled | Company XX |
| Inter-sub-slot frequency hopping | Enabled | agree: enabled | Company XX |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) | agree: The largest PRB index – (Number of PRBs – 1) | Company XX |
| Group and sequence hopping | neither | agree: neither | Company XX |
| Hopping ID | 0 | agree: 0 | Company XX |
| Initial cyclic shift | 0 | agree: 0 | Company XX |
| First symbol | 5, 12 | agree: 5, 12 | N/A |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NR_IIOT_URLLC_enh | Nokia, Nokia Shanghai Bell |  |
|  | Collection of simulation result for NR_IIOT_URLLC_enh | Nokia, Nokia Shanghai Bell |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2213049 |  | CR to TS 38.133 Correction to measurements requirements for PDC | vivo | Noted |  |
| R4-2213553 |  | CR on requirements for UE Rx-Tx measurement for PDC | Huawei, HiSilicon | Revised |  |
| R4-2213657 |  | CR on reporting mapping for URLLC in TS 38.133 | MediaTek | Return To |  |
| R4-2212156 |  | draftCR to clarify timing reference point for UE UL timing test cases | Intel | Endorsed |  |
| R4-2213554 |  | CR to introduce TRS RMC for PDC tests | Huawei, HiSilicon | Revised |  |
| R4-2213556 |  | CR on PDC measurement accuracy requirements | Huawei, HiSilicon | Return To |  |
| R4-2213863 |  | draftCR on test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2213557 |  | CR to introduce TC#2 for PDC measurement | Huawei, HiSilicon | Revised |  |
| R4-2212902 |  | UE Rx-Tx time difference delay PDC test based on PRS/SRS | Ericsson | Revised |  |
| R4-2212903 |  | UE Rx-Tx time difference accuracy PDC test based on PRS/SRS | Ericsson | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2213553 | R4-2214524 | CR on requirements for UE Rx-Tx measurement for PDC | Huawei, HiSilicon | Return To | Moderator: Based on the 2nd round discussion outcome of Sub-topics 1-3 and 1-4. New draft CR available removing requirements related to Sub-topics 1-3 and 1-4. |
| R4-2213657 |  | CR on reporting mapping for URLLC in TS 38.133 | MediaTek | Agreeable |  |
| R4-2213554 |  | CR to introduce TRS RMC for PDC tests | Huawei, HiSilicon | Noted |  |
| R4-2213556 |  | CR on PDC measurement accuracy requirements | Huawei, HiSilicon | Agreeable |  |
| R4-2213863 | R4-2214728 | draftCR on test cases for UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2213557 | R4-2214727 | CR to introduce TC#2 for PDC measurement | Huawei, HiSilicon | Agreeable |  |
| R4-2212902 | R4-2214700 | UE Rx-Tx time difference delay PDC test based on PRS/SRS | Ericsson | Agreeable |  |
| R4-2212903 |  | UE Rx-Tx time difference accuracy PDC test based on PRS/SRS | Ericsson | Not Pursued |  |
| R4-2214338 |  | WF on NR_IIOT_URLLC_enh | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2214339 |  | Collection of simulation result for NR_IIOT_URLLC_enh | Nokia, Nokia Shanghai Bell | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents