3GPP TSG-RAN WG4 Meeting #104-e	R4-2214324

Electronic Meeting, 15 – 26 August, 2022

Title: 	WF on R15 and R16 RRM maintenance

Agenda Item:	4.5

Source: 	Huawei, Hisilicon

Document for:	Approval

Topic #1: Rel-15 NR RRM maintenance

Sub-topic 1-1: Applicability of FR1+FR2 test

- Issue 1-1-1: Applicability of the test considering FR1+FR2 testability

- Add A.5.7.1.3 and A.7.7.1.3 in A.3.13A to allow UE not to pass the tests

| A.5.7.1.3 | EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |
| --- | --- |
| A.7.7.1.3 | SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |

Sub-topic 1-2: Margin in relative accuracy for FR2 inter-frequency RSRP tests

- Issue 1-1-2: Whether to add E to the upper bound

- Add additional margins E=[3]dB to the upper bound for FR2 inter-frequency relative RSRP accuracy test requirements

Topic #2: Rel-16 NR RRM maintenance

Sub-topic 2-1: eMIMO

- Issue 2-1-1: FR2 PL-RS switching delay when the target PL-RS is SSB and used for L1-RSRP measurements

- Postpone until the issue is concluded in Rel-17 feMIMO.

Sub-topic 2-2: Positioning

- Issue 2-2-1: Start of measurement period for deferred MT-LR

Candidate options:

  - Option 1 (HW, CATT, E///, Nokia)

    - For the event of periodic location in deferred MT-LR, update the start point of measurement period as:

      - the time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

  - Option 2 (QC)

    - For the event of periodic location in deferred MT-LR, update the start point of measurement period as:

      - For deferred MT-LR with “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall finish the measurements by time T where T is the time when “Periodic Location” event occurs. The requirements apply when the time T is known by the UE no later than Tref, where Tref=T-TRSTD,Total-maxTavailable_PRS,i.

  - Option 3 (vivo)

    - No need to specify starting point or ending point for periodic location report.

      - As long as UE finishes the measurements during the periodic interval, UE can report the measurements periodically