3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214328

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.8.2.1

Source:	Apple

Title:	WF on R17 FeRRM - SRS antenna port switching

Document for:	Approval

# Topic #3: SRS antenna port switching performance part

Sub-topic 3-1: Test configuration principles

Issue 3-1-1: General configuration – SCS and BW

Agreements in GTW (Aug 22)

- for configuration of test cases for SRS antenna port switching, to consider following cases:

  - 15 kHz SSB SCS with 10 MHz bandwidth for FDD

  - 15 kHz SSB SCS with 10 MHz bandwidth for TDD

  - 30 kHz SSB SCS with 40 MHz bandwidth for TDD

- Above configurations apply for both aggressor cell and victim cell, and such configuration may be different between aggressor cell and victim cell. The configuration of victim and aggressor cells can be chosen independently in the test.

- Add a note in the test requirements that the UE is required to test only one of the supported combinations.

Issue 3-1-2: SRS configuration for scenario 1 sync case (symbol-level interruption)

Agreements in GTW (Aug 22)

- Define test cases for scenario 1 sync case by allocating the SRS resource before the last two symbols or more of slot i.e., at the 10th symbol of a slot and the interruption on victim CCs should be no longer than 1 slot (subframe for E-UTRA) when the SCS of aggressor CC and victim CC is 15/30 KHz.

Sub-topic 3-2: testing configuration details

Issue 3-2-1: Special slot configuration to accommodate 6 SRS symbols

Agreements in GTW (Aug 22)

For both Scenario 1 and Scenario 2 in SRS antenna port switching test cases, new TDD configurations are introduced to have 6UL symbols in the special slot.

| SCS | Special slot configuration |
| --- | --- |
| 15kHz | S = ‘6DL: 2GP: 6UL’ |
| 30kHz | S = ‘4DL: 4GP: 6UL’ |