3GPP TSG-RAN WG4 Meeting #104-e	R4-2214437

Electronic Meeting, 15 Aug. – 26 Aug. 2022

Agenda Item:	10.20.3 UE RF requirements for SP and LPI [NR_unlic_enh]

Source:	Charter Communications, Inc.

Title:	WF on NR-U PC3 requirements

Document for:	Approval

# Introduction

This email discussion is for R18 NR_unlic_enh WI and the scope covers the following agenda items:

This is the WF to capture all agreements and open issues in [104-e][125] NR_unlic_enh within the topic of introduction of Power Class 3 email related to Rel-18 NR_unlic_enh discussions at RAN4#104-bis-e meeting.

# Introduction of Power Class 3

### Sub-topic 1-1 (PC3 ACLR)

Issue 1-1: PC3 ACLR requirement

- Proposals

  - Option 1: Remove ACLR requirement for both PC5 and PC3

  - Option 2: Same 27dB ACLR requirement for PC5 and PC3 (Skyworks, Apple)

  - Option 3: Same 30dB ACLR requirement for PC3 (Charter)

  - Option 4: Remove ACLR requirement for PC3 (Charter)

  - Option 5: Other (please specify)

Agreement from GTW

- Evaluate ACLR for 27 and 30dB to be able to make an informed decision

  - Co-existence study is needed for evaluation of 27dB ACLR

### Way Forward on PC3 ACLR requirement:

- Evaluate ACLR for 27 and 30dB to be able to make an informed decision

  - Co-existence study is needed for evaluation of 27dB ACLR

### Sub-topic 1-2 (MPR/A-MPR for 1Tx)

This section discusses the important parameters for calibration point and post PA loss

Issue 1-2-1: Calibration

- Proposals

  - Option 1: 0.5dB MPR for QPSK DFT-s-OFDM 20MHz 100RB0 waveform at 30dB ACLR

  - Option 2: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB0 waveform at 30dB ACLR (Skyworks, Charter, Apple)

  - Option 3: MPR is evaluated for the same SEM, EVM and IBE requirements than for PC5 (Skyworks)

  - Option 4: MPR is evaluated at 30 and 27dB ACLR

  - Option 5: PA calibration point at 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR, where the emission levels exactly meet the allowed emission limit of the mask.

  - Option 6: Other (please specify)

- Way Forward

  - Option 3 may be taken out an own issue about evaluating MPR.

    - Option 1: MPR is evaluated for the same SEM, EVM and IBE requirements than for PC5

    - Option 2: Other (please specify)

- Agreement

  - PC3 calibration point: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB0 waveform at 30dB ACLR

  - Option 4 is handled in the agreement in Issue 1-1.

Issue 1-2-2: Post PA loss

- Proposals

  - Option 1: 4dB post PA losses (Skyworks, Charter, Apple)

  - Option 2: Other (please specify)

- Agreement

  - Post PA loss: 4dB

Issue 1-2-3: MPR evaluation

- Proposals

  - Option 1: Focus on single CC MPR for 20 to 100MHz channels (Skyworks, Charter, Apple)

  - Option 2: Other (please specify)

- Agreement

  - Focus MPR evaluation on single CC

### Way Forward on PC3 MPR/A-MPR for 1Tx:

- PC3 calibration point: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB0 waveform at 30dB ACLR

- MPR is evaluated for the same SEM, EVM and IBE requirements than for PC5

- Focus MPR evaluation on single CC

- Post PA losses 4dB

### Sub-topic 1-3 (MPR/A-MPR for 2Tx)

This section discusses the important parameters for calibration point and post PA loss

Issue 1-3-1: Calibration

- Proposals

  - Option 1: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB3 waveform at 27dB ACLR and 20MHz NR-U SEM (Skyworks, Charter, Apple)

  - Option 2: MPR is evaluated for the same SEM, EVM and IBE requirements than for PC5

  - Option 3: MPR is evaluated at 30 and 27dB ACLR

  - Option 4: Reuse the PC5 PA calibration point for individual PAs. Verify with measurement to detect any elevated intermodulation distortion.

  - Option 5: Other (please specify)

- Agreement

  - 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB3 waveform at 27dB ACLR and 20MHZ NR-U SEM

Issue 1-3-2: Post PA loss

- Proposals

  - Option 1: 4dB post PA losses (Skyworks, Charter, Apple)

  - Option 2: Other (please specify)

- Agreement

  - Post PA loss: 4dB

Issue 1-3-3: Antenna isolation

- Proposals

  - Option 1: 16dB antenna isolation (Skyworks, Charter)

  - Option 2: 10dB antenna isolation (Apple, Intel)

  - Option 3: Other (please specify)

- Agreement

  - Evaluate both 16dB and 10dB antenna isolation

Issue 1-3-4: MPR evaluation

- Proposals

  - Option 1: Focus on single CC MPR for 20 to 100MHz channels (Skyworks, Charter, Apple, Intel)

  - Option 2: Other (please specify)

- Agreement

  - Focus MPR evaluation on single CC

### Way Forward on PC3 MPR/A-MPR for 2Tx:

- Calibration point for each PC5 PA: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB3 waveform at 27dB ACLR and 20MHz NR-U SEM

- Focus MPR evaluation on single CC

- Post PA losses 4dB

- MPR Evaluation

  - MPR is evaluated for the same SEM, EVM and IBE requirements than for PC5

  - MPR is evaluated for 16dB and 10dB antenna isolation

### Sub-topic 1-4 (Miscellaneous Topics)

Issue 1-4-1: Priority order

- Proposals

  - Option 1: (Skyworks, Apple)

- n96 NS_54 and m102 NS_58 then possibly NS_59

- n46 NS 28/29/30/31 in-band PSD limited channels

- n96 NS_53 and NS_60

- NS_61

  - Option 2: (Charter)

- n96 NS_54, n96 NS_53, and m102 NS_58

- n46 NS 28/29/30/31 in-band PSD limited channels

- n96 NS_59 and n96 NS_60

- NS_61

  - Option 3: Other (please specify)

- Recommended WF

  - MPR evaluation for

    - 1Tx and 2TX PC3

  - A-MPR

    - n96 NS_54 and m102 NS_58 then possibly NS_59

    - n46 NS 28/29/30/31 in-band PSD limited channels

    - n96 NS_53 and NS_60

    - NS_61

Issue 1-4-2: PC3 and existing NS values

- Proposals

  - Option 1: Add PC3 A-MPR to existing NS values for SP and LPI modes in all regions (Skyworks, Charter, Apple, Intel)

  - Option 2: Other (please specify)

- Agreement

  - Agreement on Option 1

### Way Forward on modes for PC3:

### PC3 A-MPR is only applicable to NS values for SP and LPI modes

Issue 1-4-3: Channel allocations and placement

- Proposals

  - Option 1: Re-use channel allocations (full and partial allocations) and channel placement from PC5 for PC3 simulations

  - Option 2: Other (please specify)

- Recommended WF

  - Agreement on option 1 (Skyworks, Charter, Apple, Intel)

  - Evaluate in-band PSD/EIRP limited channels should have dedicated A-MPR for PC3 (Skyworks, Apple)

# Reference

[1] R4-2214236, Email Discussion Summary for [104-e][125] NR_unlic_enh, Apple