3GPP TSG-RAN WG4 Meeting #104-bis-e                                                                                  R4-2215414

Electronic Meeting, October 10 – October 19, 2022

Source: 	CATT

Title: 	General consideration on BS RF core requirements for ATG network for NR

Agenda item:	6.13.4

Document for:	Approval

- Introduction

In RAN4 #104-e meeting, A WF [1] on ATG BS RF requirements was agreed. Some remaining issues in WF need further discussion in next meeting.

In this contribution, we provide our views on remaining issues for BS RF core requirements for ATG network for NR.

- Discussion

ATG BS class

As per the WF [1], the remaining issues concerning ATG BS class are shown as below:

Agreements: - Option1: to follow the HAPS approach - ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude range [TBD km]. - Further discuss the typical vertical altitude range Agreement: follow the HAPS approach

We think that take-off and landing phases are not typical ATG scenarios, so the altitude during take-off and landing phases are not considered for ATG BS class. For IFC in commercial aircraft typically flying at an altitude between 9.5 km and 12 km. So, altitude between 9.5 km and 12 km can be as typical vertical altitude range.

Proposal 1: ATG BS class can be,

ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude range 9.5-12km.

ATG BS type

As per the WF [1], the remaining issues concerning ATG BS type are shown as below:

Agreement: Use BS type 1-H, 1-O as baseline. FFS for BS type I-C.

Figure 2-1: Operating bands in FR1 for n1, n78 and n79

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

According the WID, Example bands include n1, n78 and n79. The UL/DL operating band for n1, n78 and n79 are shown in Figure 2-1. Referring to BS product in 2GHz, BS type 1-C is also used in TN scenario. When it comes to ATG, the BS type 1-C with fixed high antenna gain is possible. So to define BS type 1-C is acceptable.

Observation 1: To define BS type 1-C for ATG is acceptable.

ATG BS RF requirements

As per the WF [1], the TAE, transmitter intermodulation, and receiver intermodulation are not applicable for ATG. The ACLR, OBUE, and ACS requirement depend on the coexistence study outcome. We can follow HAPS approach to define ATG BS requirements. For example, unless otherwise stated, ATG BS class would refer to Wide Area BS class with some exceptions.

Proposal 2: Follow HAPS approach to define ATG BS requirements.

- Conclusion

This contribution provides our general consideration on BS RF core requirement for ATG network for NR. The following observation and proposals are concluded as follows:

Proposal 1: ATG BS class can be,

ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude range 9.5-12km.

Observation 1: To define BS type 1-C for ATG is acceptable.

Proposal 2: Follow HAPS approach to define ATG BS requirements.

- References

- R4-2214461, WF on ATG BS RF requirements, ZTE, RAN4#104-e