3GPP TSG-RAN WG4 Meeting # 104bis-e   										  			R4-2215489

Electronic Meeting, October 10 – October 19, 2022

Source:	CMCC

Title:	TP for 38.718-02-01 CA_n3A-n41C with UL_n41C

Agenda item:	5.10.2

Document for:	Approval

# 1 Background

In last meeting, a TP CA_n3A-n41C with UL_n41C was proposed but finally postponed. This contribution provides updated text proposal on the NR CA band combination CA_n3A-n41C with UL_n41C with different MSD value compared with in [1].

# 2 Text Proposal

##### ---Start of changes---

## 6.X	CA_n3-n41

### 6.X.1	Common for 1 band UL and 2 bands UL CA

#### 6.X.1.1 Operating bands for CA

Table 6.X.1.1-1: CA band combination CA_n3-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n41 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

#### 6.X.1.2	Channel bandwidths per operating band for CA

Table 6.X.1.2-1: Supported bandwidths per CA band combination CA_n3-n41

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41C | CA_n41C | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |

#### 6.X.1.3	UE Co-existence studies

The harmonic / harmonic mixing issue have been analyzed in previous release.

IMD order for contiguous UL CA as below.

WGap = lowest band edge of n41 - highest DL band edge of n3 = 2496 – 1880 = 616 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(190, 194) = 190 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 9.

IMD9 occurs which usually is negligible. Therefore, no MSD requirements need to be considered.

#### 6.X.1.4	∆TIB and ∆RIB values

The TIB,c and RIB have been specified in current spec.

#### 6.X.1.5	REFSENs requirements

There is no MSD issue for the CA combination.

#### 6.X.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements

##### ---End of changes---

# Reference

[1]	R4-2213126, TP for 38.718-02-01 CA_n3A-n41C with UL_n41C, Huawei, HiSilicon