3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215600

# Electronic Meeting, Oct 10 – Oct 19, 2022

Agenda item:	6.9.2.1

Source:	Apple

Title:	On L3 part enhancement for FR2 SCell activation

Document for:	Discussion

# Introduction

As discussed in last RAN4#104e meeting, some of the FR2 RRM requirements allow excessively long delay for UE in certain RRM operations, and it’s observed that there are feasible ways to enhance specific requirements to guarantee fair performance in the field for FR2 networks, e.g., FR2 SCell activation delay requirement. The WF of eFeRRM has been agreed in [1], and the open issues were also captured.

## Sub-topic 2-1 General Issue 2-1-1: Baseline scope for FR2 SCell activation enhancement Agreement: - 1st phase: RAN4 starts the discussion of the R18 unknown FR2 SCell activation enhancement based on baseline FR2 SCell activation requirement in TS38.133 section 8.3.2 - Discuss the feasibility and method of SCell activation enhancement based on AP-CSI-RS and/or A-TRS - 2nd phase: the other SCell activation cases, e.g., multiple SCell activation, direct SCell activation, and PUCCH SCell activation, to be discussed after baseline case (i.e. single FR2 SCell activation) is completed. - 2nd phase will start from RAN4 #107 meeting. Issue 2-1-2: Prioritization for initial phase Agreement: - RAN4 to prioritize at least FR2 unknown Scell delay reduction in the 1st phase of the WI. - FR2 unknown Scell without intra-band serving cell is considered for 1st phase. - The extension of the enhancement solutions to FR1 can also be discussed.

In this paper, we further discuss the possible L3 part enhancement to shorten the FR2 SCell activation delay.

# Discussion on L3 part enhancement for FR2 SCell activation

During the L3 cell synchronization(including AGC), cell measurement and T/F tracking, there are multiple parts may be considered for enhancement, as shown in figure 1.

Figure 1. L3 enhancement part during activation

#### 2.1 Beam related enhancement for L3 part

Issue 2-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

The options we discussed in last meeting were:

- FFS on beam sweeping factor reduction during unknown FR2 SCell activation. - Option 1a (CMCC): The agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or positioning enhancement can be considered as baseline. - Option 1b (vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 for FR2 - Option 1c (Ericsson): - RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X=1280ms]. - If UE performed a full RX beam sweeping for a procedure, next procedures or steps fall within RX beam constant time do not need to perform RX beam sweeping. - UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before - Other option is not precluded

Beam sweeping factor is assumed to be 8 in the L3 cell synchronization and measurement during FR2 SCell activation. However, since SSB is used for L3 measurement including PSS/SSS/DMRS for PBCH, UE may be able to perform symbol level beam sweeping inside SSB; and moreover, the SNR side condition is -2dB for activation requirement. Thus, it may be possible to reduce the Rx beam sweeping factor for L3 measurement for FR2 SCell activation enhancement. Since the beam sweeping is related with the UE implementation, we also agree with other companies that the same methodology from FR2 HST and positioning enhancement can be reused here, i.e., introduce the UE capability to indicate Rx beam sweeping factor (≤8).

Proposal 1: introduce the UE capability to indicate Rx beam sweeping factor (≤8) for L3 cell synchronization and measurement of FR2 unknown SCell activation enhancement.

Issue 2-2-2: Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception

The options we discussed in last meeting were:

- FFS whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction.

In multi-Rx chain DL reception WID, only single carrier case is considered for enhancement, as following,

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - The starting point is the enhancements related to L1-RSRP measurement enhancements - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs)

Moreover, it would be very difficult to maintain a CR for FR2 SCell activation enhancement if it was also discussed and specified in multi-Rx chain DL reception WI, i.e., there will be 2 CRs from two WIs for the same enhancement. Thus, our view is to use the conclusion of single FR2 carrier enhancement from multi-Rx chain DL reception WI to the R18 eFeRRM WI, but the FR2 SCell activation enhancement shall be discussed in R18 eFeRRM WI.

Proposal 2: RAN4 will discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case.

#### 2.2 Sample number related enhancement for L3 part

Issue 2-2-3: Cell measurement/synchronization sample number related enhancement of FR2 unknown SCell activation

The options we discussed in last meeting were:

- FFS to align the understanding of cell measurement/synchronization/AGC in the existing FR2 unknown SCell activation delay requirement. - FFS on all options for issue 2-2-3 - Option 1 (Apple, OPPO, CMCC, LGE, MTK, NTT DCM): RAN4 to study sample number reduction for L3 measurement/synchronization during unknown FR2 SCell activation (with -2dB SINR side condition) - Option 1a(Apple, CMCC): RAN4 to study if we can remove or reduce L3 T/F tracking time (8 Trs) - Option 2 (ZTE): Based on the assumption of scaling factor 8, whether the processing latency of AGC can be reduced, should be further discussed.

In FR2 unknown SCell activation Requirement in section 8.3.2 TS38.133, the following two cases are considered:

| Case 2-2-1: unknown FR2 SCell and SP CSI-RS is used | 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |
| --- | --- |
| Case 2-2-2: unknown FR2 SCell and Periodic CSI-RS is used | 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}. |

In our understanding, for legacy requirement, TFirstSSB_MAX + 15*TSMTC_MAX is used for AGC and PSS/SSS detection, and 8*Trs is used for cell measurement, T/F tracking and SSB index reading. However, in the final delay equation, how to schedule the UE behavior is up to UE implementation.

Observation: for legacy requirement, TFirstSSB_MAX + 15*TSMTC_MAX is used for AGC and PSS/SSS detection, and 8*Trs is used for cell measurement, T/F tracking and SSB index reading.

In the legacy unknown FR2 SCell activation, we assumed that the synchronization with AGC takes two sample for each Rx beam and cell measurement with T/F tracking takes one more sample. However, since SCell activation has much higher SNR side condition compared with other mobility requirement, i.e., -2dB. It may be also possible to consider if the sample number for L3 measurement/synchronization could be reduced during the SCell activation. We did some simulation on the PSS/SSS detection based on the simulation assumption of R4-2115359, but changed the Rx number to 2 and effective SINR to -2dB; and the simulation results are summarized as following,

| Channel | SCS | SINR | Success Rate of cell detection | sample num |
| --- | --- | --- | --- | --- |
| AWGN | 120kHz | -2dB | 100.00% | 1 |
|  |  |  | 100.00% | 2 |
|  |  |  | 100.00% | 3 |
| TDLA-120KHz | 120kHz | -2dB | 98.60% | 1 |
|  |  |  | 100.00% | 2 |
|  |  |  | 100.00% | 3 |
| TDLC-120KHz | 120kHz | -2dB | 99.10% | 1 |
|  |  |  | 100.00% | 2 |
|  |  |  | 100.00% | 3 |

Thus, the sample number of PSS/SSS detection can be reduced to 1, and we also think with such high SNR condition AGC can work well with 1 sample.

Proposal 3: reduce the sample number to 1 for L3 measurement/synchronization during unknown FR2 SCell activation (with -2dB SINR side condition); i.e., change TFirstSSB_MAX + 15*TSMTC_MAX to TFirstSSB_MAX + 7*TSMTC_MAX.

During the cell synchronization, UE is able to acquire the cell phase timing (coarse timing in legacy activation requirement) and then UE would perform T/F tracking on corresponding SSBs to derive the timing for following L1-RSRP measurement. However, we think during the cell synchronization, UE may be able to acquire the sufficient timing information for the L1-RSRP measurement with enhancement capability and the time delay for L3 T/F tracking may be saved, as shown in the figure 2.

Figure 2. example for the enhanced unknown FR2 SCell activation without L3 T/F tracking (example case is case 2-2-1)

Proposal 4: for unknown R18 FR2 SCell activation enhancement, RAN4 to further discuss remove or reduce L3 T/F tracking time (8 Trs) after concluding beam sweeping factor and PSS/SSS detection enhancement.

#### 2.3 RS related enhancement for L3 part

Issue 2-2-4: RS related enhancement of FR2 unknown SCell activation

The options we discussed in last meeting were:

- FFS: AP-CSI-RS and/or A-TRS based fast SCell activation may also apply to unknown FR2 SCell activation enhancement

Since stage 1 is for enhancement on FR2 unknown SCell activation, the PSS/SSS detection and DL synchronization is needed to identify the target SCell. The AP-CSI-RS and A-TRS is not supposed to be used for cell synchronization or coarse timing acquisition (time correlation based on CSI-RS is not even considered in CSI-RS L3). If the coarse timing of AP-CSI-RS and A-TRS is known to UE, then it could be directly used for L1 measurement rather than L3 UE behavior on the SCell. In order to narrow down the possible solutions for unknown FR2 SCell activation enhancement, we think RAN4 can preclude to use AP-CSI-RS and/or A-TRS for L3 part in unknown FR2 SCell activation enhancement.

Proposal 5: AP-CSI-RS and/or A-TRS based fast SCell activation is not apply to L3 part for unknown FR2 SCell activation enhancement.

# Conclusion

In this discussion, we further discuss the possible L3 part enhancement to shorten the FR2 SCell activation delay.

Proposal 1: introduce the UE capability to indicate Rx beam sweeping factor (≤8) for L3 cell synchronization and measurement of FR2 unknown SCell activation enhancement.

Proposal 2: RAN4 will discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case.

Observation: for legacy requirement, TFirstSSB_MAX + 15*TSMTC_MAX is used for AGC and PSS/SSS detection, and 8*Trs is used for cell measurement, T/F tracking and SSB index reading.

Proposal 3: reduce the sample number to 1 for L3 measurement/synchronization during unknown FR2 SCell activation (with -2dB SINR side condition); i.e., change TFirstSSB_MAX + 15*TSMTC_MAX to TFirstSSB_MAX + 7*TSMTC_MAX.

Proposal 4: for unknown R18 FR2 SCell activation enhancement, RAN4 to further discuss remove or reduce L3 T/F tracking time (8 Trs) after concluding beam sweeping factor and PSS/SSS detection enhancement.

Proposal 5: AP-CSI-RS and/or A-TRS based fast SCell activation is not apply to L3 part for unknown FR2 SCell activation enhancement.

# References

[1] R4-2214345, WF on R18 eFeRRM, Apple, RAN4 #104e