3GPP RAN WG4 Meeting #104bis-e		R4-2215663

Online, October 10th – 19th, 2022

Agenda item:	6.2.3

Source:	Apple

Title:	On CA_n5-n28 architectures

WI/SI:	FS_NR_700800900_combo_enh

Release:	Rel-18

Document for:	Approval

# 1	Introduction

Carrier aggregation has been successful in progressively achieving higher data rate since its inception in LTE Rel-10. Despite the number of integrated bands in FR1 range alone has been up to 6, the sub-1GHz band combinations have not yet gained much traction in smartphones, due to the known implementation challenges in RF front-end multiplexing filter design for closely spaced frequency bands and limited form factor to provide sufficient antenna frequency range coverage. Nonetheless, the desire for combining the narrowly and fragmentally allocated sub-1GHz spectrum to achieve higher data rate still remains which has prompted a new study item in RAN #96 meeting to identify the issues and investigate the solution to enable the support of band combinations made up of 700/800/900MHz for a smartphone, specifically for the three example band combinations, CA_n5-n8, CA_n5-n28, and CA_n8-n20-n28 [1]. In last RAN4 meeting, the general aspects of the frequency range restriction, architecture assumption, channel band width support, and UL configuration for the three intended band combinations have been first discussed which were concluded with an approved WF [2]. To follow up on the WF, in this contribution we share our views on the potential architecture variants and their implications on UE RF requirements for CA_n5-n28 based on the WF architecture assumption for the number of antenna.

# Discussion

Figure 2-1 shows the aggregated spectrum allocation for CA_n5-n28 where no frequency range restriction has been indicated for either of the constituent bands.

Figure 2-1 CA_n5-n28 aggregated spectrum allocation

For CA_n5-n28 with 2-antenna implementation, there can be two potential architecture variants, one with a quadplexer (or dual quadplexer) in the main signal path and a duplexer in the diversity Rx path, as shown in Figure 2-2 (a), the other with a triplexer in both main and diversity paths, as shown in Figure 2-2 (b).

Considering the maximum simultaneously covered spectrum range and closely spaced UL and DL bands, the following challenges and issues impacting UE RF performance for 2-antenna architecture variant (a) can be anticipated:

- The main antenna design needs to cover the entire spectrum range of 191 MHz simultaneously as shown in Figure 2-1, which is equivalent to a 24% bandwidth ratio and that would exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination likely would be compromised.

- The feasibility of a single quadplexer is questionable as n28 alone already requires a dual duplexer to reduce the band range to duplex gap ratio in order to achieve the acceptable filter isolation and insertion loss performance. Dual quadplexer has the implication on UE front-end design complexity, area, and cost impacts.

- REFSENS degradation (MSD) on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation.

Figure 2-2 Potential UE architecture variants to support CA_n5-n28 with 2 antenna

For 2-antenna architecture variant (b), the following challenges and issues impacting UE RF performance can be anticipated:

- Compared to variant (a), though the main path frequency range is reduced from 191 MHz to 136 MHz, the diversity path frequency range however is increased from 136 MHz to 191 MHz. From antenna design perspective, the same challenge with 24% antenna bandwidth ratio as in variant (a) is expected.

- The feasibility of a single triplexer for diversity path is questionable as n28 alone already requires a dual duplexer to reduce the band range to duplex gap ratio in order to achieve the acceptable filter isolation and insertion loss performance. Dual triplexer in diversity path in conjunction with a single or dual triplexer in main path has the implication on UE front-end design complexity, area, and cost impacts.

- REFSENS degradation (MSD) on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation.

Observation 1: For CA_n5-n28 with 2-antenna implementation, the antenna design needs to cover the entire spectrum range of 191 MHz simultaneously which is equivalent to a 24% bandwidth ratio that would exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination likely would be compromised.

Proposal 1: For CA_n5-n28 with 2-antenna implementation, the feasibility of a single quadplexer in main path and a single triplexer in both main path and diversity path needs to be studied.

For the 3-antenna implementation, the two of the three antenna are used in the main path to aggregate the n5 and n28 signals over the air, as shown in Figure 2-3. As n5 and n28 signals do not need to be combined through a multiplexer, there is no additional insertion loss in both n5 and n28 main signal paths as compared to single-band implementation.

Observation 2: For CA_n5-n8 with 3-antenna implementation, there is no additional insertion loss in both n5 and n8 main signal paths as compared to single-band implementation since n5 and n8 signals do not need to be combined through a multiplexer.

Figure 2-3 CA_n5-n28 UE architecture based on the 3-antenna implementation

Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antenna as compared to single-antenna implementation. However, the additional antenna cannot be added without occupying more phone space. Therefore, the feasibility on placing more than two low-band antenna in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor.

Lastly, despite the 3-antenna implementation can help reduce n5 UL interfering n28 DL via antenna isolation, the overall isolation still may not be sufficient due to frequency proximity between n5 UL and n28 DL. Therefore, the REFSENS impact on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation needs to be addressed.

Observation 3: Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antenna as compared to single-antenna implementation.

Proposal 2: Implementation of more than 2 low-band antennas in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor.

Proposal 3: For CA_n5-n28, the REFSENS impact on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation needs to be addressed.

# 3	Conclusion

In this contribution we share our views on the potential architecture variants and their implications on UE RF requirements for CA_n5-n28 based on the WF architecture assumption for the number of antenna.

Observation 1: For CA_n5-n28 with 2-antenna implementation, the antenna design needs to cover the entire spectrum range of 191 MHz simultaneously which is equivalent to a 24% bandwidth ratio that would exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination likely would be compromised.

Observation 2: For CA_n5-n8 with 3-antenna implementation, there is no additional insertion loss in both n5 and n8 main signal paths as compared to single-band implementation since n5 and n8 signals do not need to be combined through a multiplexer.

Observation 3: Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antenna as compared to single-antenna implementation.

Proposal 1: For CA_n5-n28 with 2-antenna implementation, the feasibility of a single quadplexer in main path and a single triplexer in both main path and diversity path needs to be studied.

Proposal 2: Implementation of more than 2 low-band antennas in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor.

Proposal 3: For CA_n5-n28, the REFSENS impact on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation needs to be addressed.

# 4	References

RP-221464 “Revised SI: Study on enhancement for 700/800/900MHz band combinations for NR”, CATT, 3GPP TSG-RAN Meeting #96, Budapest, Hungary, June 6th – 9th, 2022

R4-2214445 “WF on study on FS_NR_700800900”, CATT, 3GPP TSG-RAN WG4 Meeting #104-e, online, August 15th – 26th, 2022