# 3GPP TSG-RAN WG4 Meeting # 104bis-e                            R4-2215897

# Electronic Meeting, Oct. 10-Oct. 19, 2022

# Title: 	RF requirements on CA band combination of n5-n8

# Source: 	ZTE Corporation

# Agenda item:	6.2.2.2

# Document for:	Approval

# 1	Introduction

The CA band combination of the 700MHz, 800MHz and 900MHz bands (i.e. LB bands) were initial discussed in last meeting. For n5-n8, due to the n5 DL frequency range is partial overlapped with n8 UL frequency range, so the frequency restriction to avoid the overlapping are studied and be approved based on the operator’s input in last meeting.

For the supported channel bandwidth and RF requirements, it was agreed:

## BCS information：

The following is the assumption of BCS0:

n5: {5,10}MHz, n8{5, 10}MHz

## RF requirement

  - The MSD due to IMD and cross band isolation.

  - The impact on n5 out-of-band blocking requirement.

In this contributions, we give some discussion on the RF requirements on n5-n28 for smartphone to support the NR CA configurations of LB bands.

# 2	Discussion

In general, the studies on the RF requirements should base on the RF architecture. For the RF implementation, there are two options for the RF filter, one is traditional full band filter and the other one is new n-plexer (i.e.dedicated filter). For the antenna implementation, either 2 antenna or 3 antenna is approved in the WF, where the antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

In terms of WF, the BCS information for 2UL/2DL CA_n5-n8 is:

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A | CA_n5A-n8A | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |

1. UL CA (2UL/2DL)

In terms of the WF, to support UL CA_n5-n8, the follow frequency ranges in table 1 are used for further discussion.

Table 1. Frequency ranges restriction for UL CA_n5-n8

|  | UL | DL |
| --- | --- | --- |
| Frequency 1 (800MHz) -->n5 | 824MHz - 835MHz | 869MHz - 880MHz |
| Frequency 2 (900MHz) -->n8 | 904MHz - 915MHz | 949MHz - 960MHz |

For UL CA, only UL CA specific requirements if any should be defined, that is the IMD MSD, which can be verified from co-existence studies for the two simultaneous UL to check whether the intermodulation products will falls in its own FDD band DL CA. Usually, when we calculation the intermodulation products, full frequency range for the two bands are selected.

For n5-n8, the IMD calculations with full frequency ranges and frequency range restriction are shown in Table 2 and Table 3, respectively.

Table 2. IMD products for UL CA_n5-n8 with full frequency ranges.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 880 | 915 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 733 | 766 | 911 | 1006 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1097 | 942 | 697 | 642 |

Table 3. IMD products for UL CA_n5-n8 with frequency ranges restriction.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 835 | 904 | 915 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 733 | 766 | 973 | 1006 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1097 | 1042 | 697 | 642 |

When the full frequency ranges of n5 UL and n8 UL are used in the calculation, IMD3 and IMD5 may falls into band n8 DL which cause sensitivity degradation, as shown in table 2. However, if the frequency range restriction of n5 UL and n8 UL are used, it can be seen in table 3 that the IMD product frequency ranges are changed which is not overlapped with band n8 DL any more. In other words, no IMD issues. Therefore, there is no IMD MSD requirements needed to be defined considering the frequency ranges restriction of n5 and n8.

Observation 1. No IMD products falls into n8 DL considering the frequency ranges restriction of n5 and n8.

Although the WF said that MSD due to IMD should be considered below, it seems it is based on the full range.

As mentioned above, there is no need to define the IMD MSD for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8. Of course, if UL full frequency range of n5 and n8 are considered, indeed there exists IMD issues. But the UL CA_n5-n8 become impossible which was already discussed in last meeting.

Proposal 1. No need to define the IMD MSD requirement for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8.

2. Single UL(1UL/2DL)

For the fallback configuration of inter-band UL CA, some additional requirements need to be defined, including delta Tib,c/Rib,c, harmonic/cross-band isolation MSD (if any) and OOB blocking exception (if any).

After some calculations, there are no harmonic issues for DL CA_n5-n8 with single band UL configuration. However, cross-band isolation MSD should be considered due to poor filter rejection of the two close bands which depends on the RF architecture.

Observation 2. No harmonic issues for DL CA_n5-n8 with single band UL configuration no matter with full frequency range and frequency restriction.

In our understanding, like other normal CA band combination, RF architecture is shared by 1UL single band operation and 2UL CA to support the xUL/2DL CA. For delta Tib,c/Rib,c and cross band MSD, it depend on the filter performance used in different RF architectures. For 2 antenna and 3 antenna implementation, we believe the IL and performance for new n-plexer (i.e.dedicated filter) and traditional filter are different, where:

- For 3 antennas implementation, delta Tib,c equals to 0dB due to no common n-plexer for Tx path

- For 2 antennas implementation, the IL for new n-plexer should be studied.

Currently, there is no performance of dedicated filter available. Therefore, the value for delta Tib,c/Rib,c and cross band MSD should be postponed or discussed together with filter performance study.

Observation 3. The values of delta Tib,c/Rib,c and cross band MSD relay on the filter performance.

For the n5 OOB blocking requirements, if the UL is restricted in band n5 in 1UL/2DL CA_n5-n8, which means band n8 is only for Rx, then the single band n5 OOB blocking requirements could be applied.

Observation 4. If the UL is restricted in band n5 in 1UL/2DL CA_n5-n8, the single band n5 OOB blocking requirements could be applied.

# 3 Conclusion

In this contribution, we give some discussion on the RF requirements for smartphone to support 2DL/2UL  CA_n5-n8.

The conclusions are summarized below:

Observation 1. No IMD products falls into n8 DL considering the frequency ranges restriction of n5 and n8.

Proposal 1. No need to define the IMD MSD requirement for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8.

Observation 2. No harmonic issues for DL CA_n5-n8 with single band UL configuration no matter with full frequency range and frequency restriction

Observation 3. The values of delta Tib,c/Rib,c and cross band MSD relay on the filter performance.

Observation 4. If the UL is restricted in band n5 in 1UL/2DL CA_n5-n8, the single band n5 OOB blocking requirements could be applied.

# 4	Reference

[1] R4-2214445, WF on study on FS_NR_700800900, CATT