GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216271

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	8.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on UL Segmented Transmission for UL synchronization for IoT NTN

Document for:	Discussion

# Introduction

The Rel-18 RAN4 WID was approved in [1] to define requirements for Rel-17 IoT NTN [2]. In Rel-17 IoT NTN, RAN1/2/3 specifications were defined to enable NB-IoT and eMTC operating over NTN. RAN1 sent an LS to RAN4 [3] about segmented UL transmission. The impact on RRM timing requirements were discussed in RAN4#104e meeting [4][5]. In this paper, we further provide our views on the remaining issues.

# Discussion

For gradual timing adjustment, the discussion in RAN4#104-e are summarized as follows:

#### __Issue 4-1-2: Gradual timing adjustment - FFS For gradual timing adjustment, the reference timing shall be (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Ts before the downlink timing of the reference cell. Clarify the adjustment with “apart from a change of NTA,UE-specific and NTA,common” - FFS the gradual timing adjustment figures (max step size, max, min rate etc) - FFS the impact of “segmented UL transmission

Sub-topic 3-1: timing adjustment during a UL repetition period - The following proposals are discussed in this meeting and can be further discussed. - Proposal 1: RAN4 shall clarify the IoT NTN behaviour in TS 36.133 for Rel-17 to allow UEs to adjust TA during the ongoing repetition. - Proposal 1a: The restriction on UL transmission adjustment shall be updated according to RAN1 LS on per-segment TA pre-compensation. - Proposal 2: RAN4 shall specify the IoT NTN behaviour in TS 36.133 to ensure the UE keeps a constant TA value within each segment - Proposal 3: Segmented UL transmission can be covered by NTN UE transmit timing requirements, i.e. Te_NTN. FFS whether and how to capture in RAN4

Based on the discussion in last meeting, companies had different interpretations on UE behavior based on RAN1’s LS. The very controversial discussion in last RAN4 meeting is whether UE is allowed to adjust TA during a segment. According to RAN1 spec TS 36.213 clause 16.1.2 which is shown below, the time and frequency pre-compensation is down per uplink segment, which means UE shall keep a constant TA within one segment. In this way, there could be big step accumulated timing drift after the transmission of the segment, and the quantity of adjustment for the next segment could be large which may overlap with the previous one. This is why RAN1 had introduced the transmission gap.

For a NB-IoT UE communicating over NTN, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of Nsegmentprecompensation time units, where the quantity Nsegmentprecompensation is provided by system information, as specified in 3GPP TS 36.331.

Observation 1: RAN1’s agreements imply that UE shall not do time pre-compensation within one segment.

The above RAN1 agreements may have impacts on RAN4 spec on the restrictions on uplink timing adjustment during one repetition period, which is provided below:

TS 36.133 clause 7.20 When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.

In TN scenarios, the motivation to have above restrictions is that the autonomous adjustment during repetition may have impact on UL demodulation performance as eNB may need to proceed multiple repetitions before it can assess the timing of UE. The restriction is valid in TN IoT where the mobility is negligible. However, in IoT NTN, if such restriction applies to IoT NTN UE no matter on the whole repetition period or one per segment basis, it means large timing drift is experienced at eNB side. For instance, at the beginning of a segment, the timing error of UL transmission could be ranged from [-Te, Te] considering the UE implementation margin. During the segment, eNB will experience constant timing drift caused by the dramatic changing of the propagation delay, and at the end of the segment, the accumulated timing error could be Te + D, where D is the total timing during the segment.

Observation 2: If UE is not allowed to perform time pre-compensation during a segment, gNB will experience large timing drift during one segment.

An essential problem is whether UE is allowed do pre-compensation during one segment which seems contradicts with RAN1’s agreement. With UE specific TA adjustment, the timing error shall meet Te requirements for each transmission. For instance, for transmission on a segment, UE shall adjust the transmit timing constantly during the segment based on UE specific TA. From eNB side, the timing drift caused by moving of satellite is eliminated for each subframe of the segment. Besides, the large step timing adjustment can also be avoided.

Observation 3: If UE is allowed to perform time pre-compensation during a segment, timing drift caused by moving of satellite is eliminated.

Proposal 1: UE is allowed to perform time pre-compensation during a segment.

Based on analysis above, it is more reasonable that UE can perform time pre-compensation during a segment. Then the following question is how to define transmit timing requirements for IoT NTN and whether the restriction on UE autonomous timing adjustment is valid. From our understanding, the similar approach as NR NTN transmit timing requirement can be adopted. For IoT NTN, the reference point for the UE initial transmit timing shall be the downlink timing of the serving cell minus NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs.

Proposal 2: For IoT NTN, the reference point for the UE initial transmit timing shall be the downlink timing of the serving cell minus NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs.

For the gradual timing adjustment, similar statement as NR NTN can be adopted. Current requirements can apply apart from the change of NTA,adjUE due to satellite position update and NTA,adjcommon.

Proposal 3: Current requirements on gradual timing adjustment can apply excluding the change of NTA,adjUE due to satellite position update and NTA,adjcommon .

Since UE is allowed to perform timing pre-compensation, the restriction on autonomous uplink timing adjustment during an ongoing repetition period does not apply to NTN IoT.

Proposal 4: The restriction on autonomous uplink timing adjustment during an ongoing repetition period does not apply to NTN IoT.

# Conclusions

Observation 1: RAN1’s agreements imply that UE shall not do time pre-compensation within one segment.

Observation 2: If UE is not allowed to perform time pre-compensation during a segment, gNB will experience large timing drift during one segment.

Observation 3: If UE is allowed to perform time pre-compensation during a segment, timing drift caused by moving of satellite is eliminated.

Proposal 1: UE is allowed to perform time pre-compensation during a segment.

Proposal 2: For IoT NTN, the reference point for the UE initial transmit timing shall be the downlink timing of the serving cell minus NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs.

Proposal 3: Current requirements on gradual timing adjustment can apply excluding the change of NTA,adjUE due to satellite position update and NTA,adjcommon .

Proposal 4: The restriction on autonomous uplink timing adjustment during an ongoing repetition period does not apply to NTN IoT.

# References

[1] RP-221556 New WID: WID on NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks

[2] RP-211601 NB-IoT/eMTC support for Non-Terrestrial Networks

[3] R1-2205642 LS on UL Segmented Transmission for UL synchronization for IoT NTN

[4] R4-2214350 WF on LTE IoT NTN RRM requirements, MediaTek inc

[5] R4-2214353 WF on UL Segmented Transmission for UL synchronization for IoT NTN, MediaTek inc.