3GPP TSG-RAN WG4 Meeting # 104bis-e 	R4-2216835

# Electronic Meeting, 10th – 19th October, 2022

Title:	NR NTN Frequency Error

Agenda Item:	4.2.4

Source:	MediaTek Inc.

Document for:		Discussion

# Introduction

This document discusses and aims to help finalise the remaining open issues related to the conditions for NR NTN frequency error requirement and related testing.

- Status from RAN4#104-e

At RAN4#104-e, there was lots of discussion related to the conditions for verifying the Frequency Error requirement, covering the following aspects:

- Whether a constant or varying doppler should be assumed

- Whether for non-frequency error test cases only a zero doppler should be assumed

- Whether a non-zero doppler should be limited to NGSO-supporting devices

- How the UE should acquire the location, i.e. via an AT command or some other means such as GNSS simulator

As a result, the following was agreed in [1]:

Agreements:

  - Option 1: Doppler is set to zero for other than frequency error UE RF requirement

# Discussion on remaining points

## 3.1	Varying or constant doppler for Frequency Error test

Firstly, we would like to highlight that the applying frequency pre-compensation is not just about shifting the carrier frequency to +/– Fd, as the UE needs to predict the full and adapt to pre-compensate the full characteristics of the channel in frequency and time simultaneously. In order to be realistic, the downlink would also need to apply such characteristics.

RAN4 RF requirements/tests normally rely on use of a “static” channel for verifying RF requirements, on the basis that uncertainties can be mitigated. Therefore, introducing a doppler channel into an RF test is a new concept.

Observation 1: To “realistically” test pre-compensation, the channel used would need to apply all of the characteristics of the channel, in uplink and downlink. Purely applying a shift in UL frequency with everything else remaining the same is not realistic in our view.

Observation 2: Existing RF test cases seems to always apply static channels until now.

## 3.2	Verifying UE Tx RF requirements with zero doppler

Based on the agreement at the last meeting, it would make sense to liaise RAN5 to trigger this development work.

Proposal: Liaise RAN5 to trigger the development of the zero doppler test configuration.

# Conclusion

The following is concluded:

Observation 1: To “realistically” test pre-compensation, the channel used would need to apply all of the characteristics of the channel, in uplink and downlink. Purely applying a shift in UL frequency with everything else remaining the same is not realistic in our view.

Observation 2: Existing RF test cases seems to always apply static channels until now.

Proposal: Liaise RAN5 to trigger the development of the zero doppler test configuration.

# References

[1]	R4-2214372, WF on NTN UE RF requirement maintenance