3GPP TSG-RAN WG4 Meeting #104-bis-e                           	R4-2216845

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	TP to TS 38.115-2: Annex B: Test tolerances and derivation of test requirements

Agenda Item:	4.1.5.3.3

Document for:	Approval

# Introduction

Based on the pre-arranged work-split, we provide TP to the radiated conformance test specification TS 38.115-2 for the NR repeater.

# Discussion

Content of this TP was drafted based on TS 38.141-2 content.

For the FR2-1 frequency bands, the frequency sub-ranges from TS 38.141-2 were reused.

# Conclusions

Based on the above discussion, the following proposal was formulated:

Proposal 1: Approve the attached TP to TS 38.115-2.

# References

[1]	3GPP TS 38.115-2, v 0.0.1

# Annex A: TP to TS 38.115-2

------------------------------ Modified section ------------------------------

# Annex B (informative):
Test tolerances and derivation of test requirements

# B.1	Measurement of radiated characteristics

Table B.1-1: Derivation of test requirements (radiated tests)

| Test | Minimum requirement in TS 38.106 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.2 Radiated transmit power (EIRP) | See TS 38.106 [2], clause 7.2 | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz 2.2 dB, 43.5GHz < f ≦ 48.2GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.5 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 7.x OTA repeater output power (TRP) |  | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43,5GHz 2.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 7.3 OTA frequency stability | See TS 38.106 [2], clause 7.3 | 12 Hz | Formula: Frequency Error limit + TT |
| 7.4 OTA out of band gain | See TS 38.106 [2], clause 7.4 | TBD |  |
| 7.5.2 OTA ACLR | See TS 38.106 [2], clause 7.5.2 | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦43.5GHz 2.8 dB, 43.5GHz < f ≦ 48.2GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz | Formula: Relative limit - TT Absolute limit +TT |
| 7.5.3 OTA operating band unwanted emission | See TS 38.106 [2], clause 7.5.3 | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz 0.1*BWcontiguous f < fmax 0 dB | Formula: Minimum Requirement + TT |
| 7.5.4 OTA spurious emissions | See TS 38.106 [2], clause 7.5.4 | 0 dB | Formula: Minimum Requirement + TT |
| 7.6 OTA EVM | See TS 38.106 [2], clause 7.6 | [1 %] | Formula: EVM limit + TT |
| 7.7 OTA input intermodulation | See TS 38.106 [2], clause 7.7 | TBD |  |
| 7.8 OTA ACRR | See TS 38.106 [2], clause 7.8 | TBD |  |
| 7.9.1 OTA transmitter OFF power | See TS 38.106 [2], clause 7.9.2 | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Minimum Requirement + TT |
| 7.9.2 OTA transient period | See TS 38.106 [2], clause 7.9.3 | N/A |  |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

------------------------------ End of modified section ------------------------------