3GPP TSG-RAN WG4 Meeting RAN4#104-bis-e     				R4-2216875 Online

October 10 – 19, 2022

Agenda Item:	6.8.2

Source:	Lenovo

Title:	On Defining Coverage Requirements for Multi-Rx Chain Downlink Reception

Document for:  Approval

- Introduction

A new work item on NR frequency range 2 (FR2) multi-Rx chain DL reception was agreed in [1] and revised in [2] with the following objective:

- Introduce necessary requirement(s) for enhanced FR2 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO

  - Enhanced RF requirements:

    - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs

In this contribution, we consider how to define spherical coverage requirements with simultaneous reception from different directions.

- Spherical Coverage with Simultaneous Reception

The single direction spherical coverage is defined in terms of the complementary distribution function of the effective isotropic sensitivity (EIS). Let EISϕ,θ denote the EIS (in dB) in the direction ϕ,θ in the absence of any interfering signal. For a given EIS value α, let Aα denote the set of directions ϕ,θ such that

Aα=ϕ,θ: EISϕ,θ≤α .

With this definition, the complementary cumulative distribution function CCDF of the EIS for a single direction can be expressed as

CCDFα=1-14πϕ=02πθ=0πIAαϕ,θsinθdθ dϕ

where IAαϕ,θ is the indicator function given by

IAαϕ,θ=1ϕ,θ∈Aα0else .

When limited to a finite number of sampling points N, the complementary cumulative distribution function for a single direction can be expressed as

CCDFαi=1NIAαϕi,θi Wϕi,θi .

Simultaneous reception

When receiving from two directions simultaneously, let

EISS1ϕ1,θ1,ϕ2,θ2

EISS2ϕ1,θ1,ϕ2,θ2

denote the EIS in directions ϕ1,θ1 and ϕ2,θ2, respectively. Furthermore, define the set

ASα=ϕ1,θ1,ϕ2,θ2:EISS1ϕ1,θ1,ϕ2,θ2≤α                          and EISS2ϕ1,θ1,ϕ2,θ2≤α .

If it is assumed that the directions ϕ1,θ1 and ϕ2,θ2 are chosen randomly, then the complementary cumulative distribution function for simultaneous reception of signals from two directions can be expressed as

CCDFSα=1-116π2ϕ1=02πθ1=0πϕ2=02πθ2=0πIASαϕ1,θ1,ϕ2,θ2sinθ2sinθ1dθ2 dϕ2 dθ1 dϕ1

where IASαϕ1,θ1,ϕ2,θ2 is the indicator function given by

IASαϕ1,θ1,ϕ2,θ2=1ϕ1,θ1,ϕ2,θ2∈ASα0else .

From the definition of ASα, it follows that

IASαϕ1,θ1,ϕ2,θ2=IASαϕ2,θ2,ϕ1,θ1

from which it follows that

CCDFSα=1-18 π2ϕ1=02πθ1=0πϕ2=0ϕ1θ2=0θ1IASαϕ1,θ1,ϕ2,θ2sinθ2sinθ1dθ2 dϕ2 dθ1 dϕ1

When limited to a finite number of sampling points N, the complementary cumulative distribution function can be expressed as

CCDFSα=1-116π2i=1Nj=1NIASαϕi,θi,ϕj,θjWϕi,θiWϕj,θj

where Wϕi,θi reflects the area of the unit sphere corresponding to ϕi,θi and

i=1NWϕi,θi=4 π .

Using the same symmetry argument as in the continuous case, the complementary cumulative distribution function can be simplified as

CCDFSα=1-18 π2i=1Nj=1i-1IASαϕi,θi,ϕj,θjWϕi,θiWϕj,θj

where it has further been assumed that the UE cannot receive both signals from the same direction so that

IASαϕi,θi,ϕi,θi=0.

For this last case, the required number of measurements is NN-12

- Complexity Reduction

We consider the case that the single Rx chain EIS spherical coverage measurements are taken prior to the multiple Rx chain measurements.  Furthermore, we consider the case that the purpose of the measurement is not to evaluate complementary cumulative distribution of the multi-Rx EIS.  Instead, we assume that the purpose of the measurement is to determine if the spherical coverage requirement is met.  Let the coverage requirement be given by

CCDFSα0≤p0 .

When verifying the multi-Rx spherical coverage requirement, it is not necessary to take measurements for a pair of angles ϕi,θi and ϕj,θj unless both

EISϕi,θi≤α0

and					      EISϕj,θj≤α0 ,

since the requirement cannot be passed for simultaneous reception unless it is passed for single Rx reception.  Let Bα0 denote the set of indices given by

Bα0=i,1≤i≤N: EISϕi,θi≤α0 .

With this notation, CCDFSα0 can be expressed as

CCDFSα0=1-116 π2iϵBα0jϵBα0IASα0ϕi,θi,ϕj,θjWϕi,θiWϕj,θj

Let M=Bα0 and let the indices within Bα0 be indexed from 1 to M as k1, k2, ⋯, kM.  With this indexing, CCDFSα0 can be further expressed as

CCDFSα0=1-18 π2i=k1kMj=k1i-1IASα0ϕi,θi,ϕj,θjWϕi,θiWϕj,θj .

Thus, by restricting the set of indices measured to the set Bα0=i,1≤i≤N: EISϕi,θi≤α0, the number of measurements needed to verify the requirement that

CCDFSα0≤p0

is reduced from NN-12 to MM-12.  In case that

M=Bα0=N/2  ,

the required number of measurements required to verify the coverage requirement that

CCDFSα0≤p0

is given by

M M-12=N/2 N/2-12=N28-N4 .

Limitation on the Minimum Angular Separation

In general, the UE will see interference between two TRP’s which are too close together, and this interference will not only significantly desense the receiver, but it will also limit the maximum achievable signal-to-noise ratio.  For this reason, it may be reasonable to assume that simultaneous reception will require a minimum angular separation ψ between the two TRP’s.  With such an assumption, it would not be necessary to take measurements for pairs of angles ϕi,θi and ϕj,θj for which the angular separation is less than or equal to ψ.

If it is assumed that the limitation on the angular separation is sufficient to eliminate at least the nearest neighbor for each grid point ϕi,θi, the number of measurement pairs can be reduced by M, where M is the size of the set

Bα0=i,1≤i≤N: EISϕi,θi≤α0 .

If it is assumed as above that M=N/2, then the number of measurements needed to verify the coverage requirement that

CCDFSα0≤p0

is equal to

N28-N4 .

- Summary

# In this contribution, we have considered how the spherical complementary cumulative distribution function can be defined for simultaneous reception for two directions. We have also considered methods for reducing complexity in the event that the purpose of the test is only to verify that the coverage requirement is met, and not to evaluate the complementary cumulative distribution function. Based on this investigation, we have the following propoals.

# Proposal 1:  Define the complementary cumulative distribution function for simultaneous reception from two randomly selected directions as

CCDFSα=1-116π2i=1Nj=1NIASαϕi,θi,ϕj,θjWϕi,θiWϕj,θj .

where Wϕi,θi reflects the area of the unit sphere corresponding to ϕi,θi and i=1NWϕi,θi=4 π .

# This expression can be simplified as

# CCDFSα=1-18 π2i=1Nj=1i-1IASαϕi,θi,ϕj,θjWϕi,θiWϕj,θj .

# Proposal 2:  In the case that the purpose of the measurement is only to verify that the requirement

CCDFSα0≤p0

is met, the measurement can be simplified as

# CCDFSα0=1-18 π2i=k1kMj=k1i-1IASα0ϕi,θi,ϕj,θjWϕi,θiWϕj,θj

# where k1, k2, ⋯, kM are the M elements of the set

# Bα0=i,1≤i≤N: EISϕi,θi≤α0 .

# References

- RP-220974, “New| WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception,” Qualcomm Incorporated, 3GPP TSG-RAN Meeting #95e.

- RP-221753, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception,” Qualcomm Incorporated, 3GPP TSG-RAN Meeting #96e.