3GPP TSG-RAN4 Meeting #104Bis-e	R4-2216884

Oct 2022

Title:	Discussion and TP on Suffix information for 36.102

Source:	Qualcomm Incorporated

Agenda item:	7.5.5

Release:	Rel-18

WI: 	LTE_NBIOT_eMTC_NTN_req-Core

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

For IoT NTN, the specification skeleton was approved [1]. This paper discusses use of suffixes, proposes changes and text for section 4.2.

# 2. 	Discussion

This specification is to host requirements for NB IoT based NTN and Cat-M1 based NTN system. Currently in 36.101 dedicated requirements for these systems are under suffixes denoted by “E” and “F” as descried in clause 4.3A of TS 36.101. The 4.3A says also that:

“A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional subclause (suffix A, B, C, D, E, F and G) in clauses 5, 6 and 7.”

Similar approach with suffixes was agreed in [2] with suffixes but without general and additional requirements clause. It is not specified but the 36.101 uses suffixes consistently on third level.

The current skeleton uses suffix’s as follows:

| Section | Level 2 | Level 3 | Level 4 |
| --- | --- | --- | --- |
| 5 | 5.2A, 5.2B, 5.3B | 5.4.1B, 5.4.2B, 5.4.3B |  |
| 6 |  | 6.2.1A, 6.2.1B, 6.2.2A, 6.2.2B, 6.2.3A, 6.2.3B, 6.2.4B, 6.3.1B, 6.3.2B, 6.3.3B, 6.3.4A, 6.3.4B, 6.4.1A, 6.4.1B, 6.4.2A, 6.4.2B, 6.5.1B, 6.5.2B, 6.5.3B, 6.6.1B | 6.5.2B.1, 6.5.2B.2, 6.5.2B.3 |
| 7 |  | 7.3.1A, 7.3.1B, 7.4.1B, 7.5.1B, 7.7.1B, 7.8.1B | 7.6.1.1B, 7.6.2.1B |

In some cases there are only 4th level suffix clauses and is some cases there are only 2nd level and in some cases there are 3rd level clauses. This maybe problematic in future and our proposal is to align the use of suffix clauses to the same level through out the specification.

In NR specification 38.101 series, the suffix use is specified in clause 4.3 with the sentence:

“Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.”

We propose that the suffix use is specified to improve readability of the specification.

Proposal: Use suffixes on 2nd level in the specification 36.102 and approve the TP in appendix.

We also open to adopt the 36.101 approach and use suffixes on 3rd level as long as the highest level is the same for all sections.

# Conclusion

Proposal: Use suffixes on 2nd level in the specification 36.102 and approve the TP in appendix.

# References

[1]	R4-2215118, “Draft spec for TS 36.102”, Mediatek India Technology Pvt., RAN4#104e, Aug 2022

[2]	R4-2214467, “WF on NB-IoT/eMTC NTN agenda items 12.5.1 and 12.5.4”, MediaTek, RAN4#104e, Aug 2022

# Text proposal

## 4.3	Specification Suffix Information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes for Iot NTN

| Clause suffix | Variant |
| --- | --- |
| A | Cat-M1 |
| B | NB1, NB2 |