3GPP TSG-RAN WG4 Meeting # 104bis-e												R4-2217154

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			6.19.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [104-bis-e][221] NR_MC_enh_RRM

Document for:	Information

Introduction

This email thread discusses the RRM core requirements of WI on Multi-carrier enhancements.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to comment in each sub-topic.

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Fahad Syed Muhammad | Fahad.syed_muhammad@nokia.com |
| China Telecom | Shan YANG | yangshan@chinatelecom.cn |
| Huawei | Jing Han | Hw.hanjing@huawei.com |
| Apple | Qiming Li | Li_qiming@apple.com |
| CMCC | Xiaoran ZHANG | zhangxiaoran@chinamobile.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1:	DL interruption for Tx switching across ¾ bands

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215496 | CMCC | Proposal 1: For Rel-18 Tx switching across 3 or 4 bands, DL interruption requirements are to be specified. Proposal 2: For Rel-18 Tx switching across 3 or 4 bands, reuse Rel-16/Rel-17 values for length of DL interruption. Proposal 3: Define DL interruption requirements for Rel-18 Tx switching across 3 or 4 bands applicable to all scenarios including: - Inter-band UL-CA without SUL band - Inter-band UL CA for {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s) - Intra-band two contiguous aggregated carriers within one non-SUL band out of 3 or 4 bands - {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band} |
| R4-2215788 | Nokia, Nokia Shanghai Bell | Observation 1: The UL Tx switching behaviour between 2 UL bands (source-band before the switch and target band after the switch) selected from 3 or 4 bands is the same as the UL Tx switching in Rel16/17. Proposal 1: For each band pair selected from the 3 or 4 bands, the interruption length for UL Tx switching across 3 or 4 bands is the same as those for UL Tx switching across 2 bands in Rel16/17. Proposal 2: Wait for RAN1 conclusion on the interrupted carriers at UL Tx switching across 3 or 4 bands. |
| R4-2215798 | China Telecom | Proposal 1: For Rel-18 Tx switching across ¾ bands with single TAG, reuse the Rel-16/17 values for the length of DL interruption as specified in TS 38.133. Proposal 2: For Rel-18 Tx switching across ¾ bands with single TAG, reuse the Rel-16/17 agreement on the starting symbol of DL interruption, i.e., the DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL switching period. |
| R4-2215872 | vivo | Proposal 1: For UL Tx switching across up to 3 or 4 bands with single TAG, DL interruption length is to reuse Rel-16/17 DL interruption length. |
| R4-2216310 | Huawei, HiSilicon | Observation 5: DL interruption requirements in R17 UE Tx switching between two bands can be reused regardless of single TAG and 2TAGs. |
| R4-2216424 | Ericsson | Observation 1: In Rel-16 the introduction of TX switching between two UL carriers were confined to changes related to DL interruptions in TS 38.133. Observation 2: DL interruptions in TS 38.133 are not affected, for CA, since the switching period during which the UE is not expected to transmit on any carrier is the same for non-collocation. The capability uplinkTxSwitching-DL-Interruption-r16 is unaffected in the multiple TAG case. Observation 3: Same principle applies for ENDC, when it comes to DL interruptions, i.e., DL interruptions in TS 38.133 are not affected, even if the time masks in UE RF requirements might have to be different. |
| R4-2216715 | Samsung | Observation 1: In Rel-16 and Rel-17 UL TX switching, RAN4 requirements apply for the case of co-located and synchronized network deployment with the max receiving timing difference of 3us between the two carriers, which is reflected in time mask requirement in TS38.101-1/3. Observation 2: In RF session, it is agreed to ”limit number of TAGs to up to 2 for all the cases in the Rel-18 WI”. Proposal 1: No impact on the existing MTTD requirement due to the extension of UL TX switching for 2 bands from 1 TAG to 2 TAGs. Observation 3: Neither the starting point nor the length of DL interruption specified for UL TX switching for 2 bands is relevant to the timing relationship between two carriers. Proposal 2: No impact on the existing DL interruption requirement due to the extension of UL TX switching for 2 bands from 1 TAG to 2 TAGs. Observation 5: In the signaling design for uplinkTxSwitching-DL-Interruption, there is full flexibility for UE to claim whether the interruption exists or not on any DL carrier(s). Proposal 5: The DL interruption requirement for UL TX switching across ¾ bands can be defined comparable to the requirements in 8.2.2.2.10/10A/10B/10C for two bands. |
| R4-2215613 | Apple | Proposal 1: RAN4 needs to study interruption related requirement to support UL Tx switching across multiple bands. Proposal 2: interruption on other serving cells when UL Tx switching occurs across multiple bands shall be defined as: Proposal 3: switching period is being discussed in RF session and the outcome can be reused in interruption design. Proposal 4: TA adjustment uncertainty remains same as legacy. Proposal 5: RAN4 shall discuss the supported MRTD in this work item. |

Open issues summary

Issue 1-1: RRM impact due to Rel-18 Tx switching across 3 or 4 bands

- Proposals

  - Option 1(CMCC, Nokia, China Telecom, vivo, Huawei, Ericsson, Samsung, Apple): DL interruption requirements are to be specified

  - Option 2 (Samsung): No impact on the existing MTTD requirement due to the extension of UL TX switching for 2 bands from 1 TAG to 2 TAGs.

Note: Option 1 and option 2 don’t conflict. Please comment on both options.

- Recommended WF

Further discussion

| Company | Comments |
| --- | --- |
| China Telecom | We agree with both option 1 and option 2. |
| Huawei | We agree with both option 1 and option 2. |
| Ericsson | We agree with both option 1 and option 2. |
| Nokia | Option 1 and Option 2. For Option2, MTTD requirement has been specified in different scenarios considering pTAG and sTAG. This should not be impacted by UL Tx switching. |
| Apple | Support option 1 and 2. |
| vivo | We agree with option 1. For option 2, the understanding is that existing MTTD requirements for collocated (1 TAG) and non-collocated (2 TAG) are not changed, respectively. If so, we agree. |
| CMCC | We agree with option 1 and option 2 |
| Samsung | Support option 1 and option 2. |

Issue 1-2: DL interruption length

- Proposals

  - Option 1(CMCC, Nokia): reuse Rel-16/Rel-17 values for length of DL interruption.

  - Option 1a (Huawei, Ericsson, Samsung): For R18 Tx switching across 3/4 bands with both single TAG and 2 TAGs, reuse Rel-16/Rel-17 values for length of DL interruption.

  - Option 1b (China Telecom, vivo): For Rel-18 Tx switching across 3/4 bands with single TAG, reuse Rel-16/Rel-17 values for length of DL interruption.

  - Option 2 (Apple): interruption on other serving cells when UL Tx switching occurs across multiple bands shall be defined as:

Tinterrupt = ceil((switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1

Where

- Switching period is being discussed in RF session and the outcome can be reused in interruption design.

- TA adjustment uncertainty remains same as legacy.

- RAN4 shall discuss the supported MRTD in this work item.

- Recommended WF

Further discussion

| Company | Comments |
| --- | --- |
| China Telecom | Currently we agree with option 1b for single TAG case. In 2-TAG case, although the DL interruption time in unit of us can be the same as the singe-TAG, the number of interrupted OFDM symbols can be further discussed. |
| Huawei | Considering option 3, as non-collocated scenarios are to be considered for multiple TAGs, some company the possible change on MRTD, then the DL interruption may need further discussion. Therefore we think at least for single TAG, Rel-16/Rel-17 values for length of DL interruption can be reused. So we can agree with option 1b. Regarding option 3, we think it is for 2-TAGs scenario. We are open to further discuss MRTD. |
| Ericsson | Option 1a |
| Nokia | We support Option 1 and 1a. On Option 2, the difference from option 1 is only the value of MRTD. In legacy Tx switching, 3us MRTD is adopted assuming co-located scenario. Is there any particular reason to change the assumption when Tx switching is selected from 3 or 4 bands? We are fine to discuss the MRTD assumption when UL switching is extended across 3 or 4 bands but it is irrespective of 1 or 2 TAGs. |
| Apple | We are fine to reuse Rel-16/Rel-17 values for single TAG (option 1b). We propose option 2 with focus on 2 TAGs. The assumption of MRTD=3us for single TAG may not be valid here. If so, interruption length need to be increased. |
| vivo | We support option 1b, which should be agreeable in our view. For 2-TAG case, we need to align the understanding firstly of DL interruption and UL outage time which is discussed in Topic #2. Whether requirements for both DL interruption and UL outage time are needed or not. What would be the difference between the two. |
| CMCC | We can first agree on option 1b and further discuss 2TAG case based on option 2. |
| Samsung | At least, option 1b is common understanding needs no more discussion. For 2-TAG case, we have a general question that why MRTD should be included? The interruption on DL from RF switching is not lasting for the whole period if MRTD is a non-zero value. Considering the interruption on the whole period including MRTD is too pessimistic assumption. |

Issue 1-3: Starting point of the DL interruption

- Proposals

  - Option 1(China Telecom, vivo): For Rel-18 Tx switching across 3/4 bands with single TAG, reuse the Rel-16/17 agreement on the starting symbol of DL interruption, i.e., the DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL switching period.

  - Option 1a (Samsung, Huawei): For R18 Tx switching across 3/4 bands with both single TAG and 2 TAGs, reuse the Rel-16/17 agreement on the starting symbol of DL interruption

- Recommended WF

Further discussion

| Company | Comments |
| --- | --- |
| China Telecom | Option 1a is also ok. |
| Huawei | Support Option 1a, as the starting point is generic to cover both single TAG and 2TAGs scenarios. |
| Ericsson | Option 1a |
| Nokia | Option 1a. We think that the text in the existing specifications can be reused to indicate the starting point of DL interruption. |
| Apple | Option 1a is fine. |
| vivo | Option 1a may be fine. But we think it would be better to agree on single TAG firstly. 2-TAG case can be further studied together with other issues, e.g., DL interruption length, UL outage time etc. |
| CMCC | Option 1a |
| Samsung | Option 1a |

Issue 1-4: Interrupted cells (i.e., victim cells) where the DL interruption is allowed at UL Tx switching

- Proposals

  - Option 1(Nokia): Wait for RAN1 conclusion on the interrupted carriers at UL Tx switching across 3 or 4 bands.

  - Option 2 (Ericsson): The capability uplinkTxSwitching-DL-Interruption-r16 is unaffected in the multiple TAG case.

  - Option 2a (Samsung): For Rel-18 TX switching across 3/4 bands, no reason to change the existing requirement structure and the signaling design for uplinkTxSwitching-DL-Interruption.

Note: From Moderator’s view, option 2 and option 2a are the same. Please proponents confirm.

- Recommended WF

Further discussion

| Company | Comments |
| --- | --- |
| China Telecom | For 2-TAG case, we agree with option 2 that the DL interruption capability for single TAG can be applied. For Tx switching across 3 or 4 bands, the interrupted cells are being discussed in RF session (Sub-topic 1-5 in thread #139). It is ok to follow RAN4 RF and RAN1 agreements. |
| Huawei | So far we think either option 2 or option 2a is fine. If there are different conclusion from the legacy capability, we can revisit it. Anyway we think this issue is not important in RAN4 RRM, as RRM can directly use the conclusion of other group. |
| Ericsson | Option 2. |
| Nokia | We understood the impact on other carriers at Tx switching across 3 or 4 bands is being discussed in RAN1. So we can wait for RAN1 conclusion as proposed in Option 1. Unless RAN1 reaches different views, we also agree with Option 2 and 2a that existing capability can be reused to indicate the interrupted cells. The question is if the UE would indicate the same capability when UL switching is extended to 3 or 4 bands? |
| Apple | We do not disagree with option 2/2a. We are also fine to wait for more RF and RAN1 input. |
| vivo | In our understanding, the issue is under discussion in RF session. One particular case is as below. - For Tx switching across 4 bands, when the status of Tx chain on the 4 bands before and after switching is {1, 0, 1, 0} and {0, 1, 0, 1} respectively, there is ambiguity on the switched band pair of each Tx chain. In RAN4 understanding, RAN1 will discuss and resolve this ambiguity. Thus, we should wait for RF session conclusion. Our understanding is that this issue should be RF expertise rather than RAN1. |
| Samsung | By demonstrated in our paper, for the existing signaling design for uplinkTxSwitching-DL-Interruption, there is full flexibility for UE to claim whether the interruption exists or not on any DL carrier(s), so Option 2a is proposed. We believe Option 2 and 2a are similar proposal. |

Issue 1-5: DL interruption is applicable for ENDC

- Proposals

  - Option 1(Ericsson): Same principle applies for ENDC, i.e., DL interruptions in TS 38.133 are not affected, even if the time masks in UE RF requirements might have to be different.

- Recommended WF

Further discussion

| Company | Comments |
| --- | --- |
| China Telecom | For EN-DC with 2CC 1Tx-2Tx switching, the DL interruption capability for single TAG can be applied for 2-TAG case, and the length of DL interruption for 2-TAG can be further discussed. |
| Ericsson | Option 1 |
| Nokia | Fine with Option 1 that same DL interruption applies for ENDC. We’d just like to clarify what differences we might have in time masks. |
| vivo | In RAN1#109-e meeting, following conclusion was made. Conclusion EN-DC cases are out of scope for Rel-18 UL Tx switching Therefore, we think EN-DC should not be considered in RRM session also. |
| Samsung | Similar comment as vivo. |

Issue 1-6: Applicable scenario of DL interruption requirements

- Proposals

  - Option 1(CMCC): reuse Rel-16/Rel-17 values for length of DL interruption. Define DL interruption requirements for Rel-18 Tx switching across 3 or 4 bands applicable to all scenarios including:

    - Inter-band UL-CA without SUL band

    - Inter-band UL CA for {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s)

    - Intra-band two contiguous aggregated carriers within one non-SUL band out of 3 or 4 bands

    - {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band}

- Recommended WF

Further discussion

| Company | Comments |
| --- | --- |
| China Telecom | We support option 1. |
| Huawei | We support option 1. |
| Ericsson | We are ok with option 1. |
| Nokia | We understood that this has been discussed in RAN meetings. It is better to not discuss this in RAN4. In addition, the DL interruption requirement is defined without differentiating between legacy inter-band UL CA and SUL. Instead, the “uplink band pair” is used which can comprise of both cases. Hence we don’t think the discussions would have any impact to RAN4 spec. |
| CMCC | We support option1. Option 1 is to clarify that the sets of DL interruption length are applicable to all Tx switching scenarios. And to make it clear, we list the scenarios agreed in the WID. This has nothing to do with RAN discussion. |
| Samsung | Option 1 is good summary for the possible scenarios for R18 case. |

CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

None

Summary for 1st round

    - Open issues

Issue 1-1: RRM impact due to Rel-18 Tx switching across 3 or 4 bands

Based on 1st round discussion, all companies agree on option 1. Regarding option 2 (which doesn’t conflict with option1), as MTTD is for non-collocated TAG pairs, so from Moderator’s observation, all companies can agree on option 2 as well.

Tentative agreement:

  - Option 1: DL interruption requirements are to be specified

  - Option 2: No impact on the existing MTTD requirement due to the extension of UL TX switching for 2 bands from 1 TAG to 2 TAGs.

Note: Option 1 and option 2 don’t conflict

Recommendations for 2nd round: No discussion

Issue 1-2: DL interruption length

Based on 1st round discussion, all companies can agree that Rel-16/Rel-17 values for length of DL interruption can be reused for Rel-18 Tx switching across 3/4 bands with single TAG. For two TAGs, company proposed that assumption of MRTD=3us for single TAG may not be valid, so more discussion is needed.

Tentative agreement:

For Rel-18 Tx switching across 3/4 bands with single TAG, reuse Rel-16/Rel-17 values for length of DL interruption.

Candidate options:

For Rel-18 Tx switching across 3/4 bands with 2 TAGs

- Option 1: reuse Rel-16/Rel-17 values for length of DL interruption.

- Option 2: interruption on other serving cells when UL Tx switching occurs across multiple bands shall be defined as:

Tinterrupt = ceil((switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1

Where

- Switching period is being discussed in RF session and the outcome can be reused in interruption design.

- TA adjustment uncertainty remains same as legacy.

- MRTD=3us which is used to derive R16/R17 DL interruption length is not valid for non-collocated case, further discuss the concrete value of MTRD

Recommendations for 2nd round: further discuss the options in 2TAGs scenario.

Issue 1-3: Starting point of the DL interruption

All companies agree on option 1a.

Tentative agreement:

For R18 Tx switching across 3/4 bands with both single TAG and 2 TAGs, reuse the Rel-16/17 agreement on the starting symbol of DL interruption

Recommendations for 2nd round: No discussion

Issue 1-4: Interrupted cells (i.e., victim cells) where the DL interruption is allowed at UL Tx switching

No consensus was reached during 1st round discussion. From Moderator’s view, this issue is not important in RAN4 RRM. RRM can directly use the conclusion of RF session or other RAN group, when decisions are made.

Candidate options:

  - Option 1(Nokia, Apple, vivo): Wait for RAN1 /RF conclusion on the interrupted carriers at UL Tx switching across 3 or 4 bands.

  - Option 2 (China Telecom, Huawei, Ericsson, Samsung): The capability uplinkTxSwitching-DL-Interruption-r16 is unaffected in the multiple TAG case.

  - Option 2a (Samsung, Huawei): For Rel-18 TX switching across 3/4 bands, no reason to change the existing requirement structure and the signaling design for uplinkTxSwitching-DL-Interruption.

Recommendations for 2nd round: as this issue depends on the output from RF session or other RAN group, it will NOT be discussed in 2nd round.

Issue 1-5: DL interruption is applicable for ENDC

During 1st round discussion, one company pointed out there is the following conclusion in RAN1#109e,

Conclusion EN-DC cases are out of scope for Rel-18 UL Tx switching

Please companies check the above agreement and reconsider option 1.

Candidate options:

  - Option 1(Ericsson): Same principle applies for ENDC, i.e., DL interruptions in TS 38.133 are not affected, even if the time masks in UE RF requirements might have to be different.

Recommendations for 2nd round: in 2nd round, encourage companies check the above agreement and reconsider option1.

Issue 1-6: Applicable scenario of DL interruption requirements

No consensus was achieved on the applicable scenario of DL interruption requirements in first round.

Candidate options:

  - Option 1(CMCC): reuse Rel-16/Rel-17 values for length of DL interruption. Define DL interruption requirements for Rel-18 Tx switching across 3 or 4 bands applicable to all scenarios including:

    - Inter-band UL-CA without SUL band

    - Inter-band UL CA for {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s)

    - Intra-band two contiguous aggregated carriers within one non-SUL band out of 3 or 4 bands

    - {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band}

Recommendations for 2nd round: further discussion

    - CRs/TPs

None

Discussion on 2nd round (if applicable)

<Agreement>: Issue 1-1: RRM impact due to Rel-18 Tx switching across 3 or 4 bands

- DL interruption requirements are to be specified

- No impact on the existing MTTD requirement due to the extension of UL TX switching for 2 bands from 1 TAG to 2 TAGs.

Issue 1-2: DL interruption length

<Agreement>:

For Rel-18 Tx switching across 3/4 bands with single TAG, reuse Rel-16/Rel-17 values for length of DL interruption.

< Way forward >:

For Rel-18 Tx switching across 3/4 bands with 2 TAGs,

- Option 1: reuse Rel-16/Rel-17 values for length of DL interruption.

- Option 2: interruption on other serving cells when UL Tx switching occurs across multiple bands shall be defined as:

Tinterrupt = ceil((switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1

Where

- Switching period is being discussed in RF session and the outcome can be reused in interruption design.

- TA adjustment uncertainty remains same as legacy.

- MRTD=3us which is used to derive R16/R17 DL interruption length is not valid for non-collocated case, further discuss the concrete value of MTRD

| Company | Comments |
| --- | --- |
| Apple | We think it is better to understand the network scheduling behaviour first. There could be two different scheduling behaviours: - Alt 1: Network keeps scheduling the UE during the UL Tx switching: UE is allowed to cause interruption when UL Tx switching happens. Since the actual RTD at UE side is unknown by network, interruption could happen in a larger time window, which consists of switching period, TA adjustment uncertainty and MRTD. The benefit is the UE which is doing UL Tx switching would cause shorter interruption. Nevertheless, a downside is interruption indeed exists to system throughput. - Alt 2: Network avoid scheduling the UE during the UL Tx switching: NW would avoid scheduling the UE during the UL Tx switching time window. Although the UE which is doing UL Tx switching cannot enjoy scheduling timely, there is no interruption at the cell throughput level since NW can choose schedule other UEs during the time window. There is pro/con of the two different NW scheduling behaviours. According to the methodology approved in the early release on calculation of interruption (R4-2008623), it is more in line with alt 2. If we follow the same methodology, we may need to revisit MRTD, as it was agreed as one of the components. |
| China Telecom | We can further check whether the formula can be applied to 2-TAG case without any change as well as the applicable MRTD. For MRTD, we don’t agree to consider the worst case with the maximum value defined for non-collocated scenario in the spec. |
| Huawei | For 2TAGs, we are open to further check the concrete interruption length if the legacy equation (alt.2 in Apple’s above comments) is used and revised MRTD (larger than 3us) is applied. |
| Ericsson | Option 1 is fine. Option 2: Interruption only occur during switching period simultaneously on both bands no matter the timing period on carriers. |
| vivo | Option 2 is fine in general. MRTD for non-collocated deployment should be considered for 2-TAG case. |
| Nokia | We slightly prefer Option 1, but we are open to discuss Option 2. In Legacy rel16/17, for MRTD calculation, collocated assumption is used only for intra-band case (See 7.6.4 in 38.133). For inter-band, there is no such assumption. And since in rel16/17, inter-band case is considered for the calculation of DL-interruption with assumption of 3us MRTD for Tx switching, we don’t see any reason to have an impact on DL interruption time for ¾ bands case. Are we now considering the non-collocated deployment because of 2 TAGs? And in Option 2, probably we should change “interruptions on other serving cells” to “interruptions on victim cells” as not all the serving cells are interrupted? |
| Samsung | We are curious about if non-collocated deployment case is not considered (or, just a worst case rather than a typical case to be considered), why there are two TAGs needed in the concerned scenario? To follow Apple’s explanation, here we are discussing “interruption” rather than “scheduling restriction”, right? Alt.1 is our understanding, which is more aligned with Option 1. We are open to further examine the detailed timing if Alt.1 can be followed as common understanding firstly. |

<Agreement>: Issue 1-3: Starting point of the DL interruption

For R18 Tx switching across 3/4 bands with both single TAG and 2 TAGs, reuse the Rel-16/17 agreement on the starting symbol of DL interruption

Issue 1-4: Interrupted cells (i.e., victim cells) where the DL interruption is allowed at UL Tx switching

- Option 1(Nokia, Apple, vivo): Wait for RAN1 /RF conclusion on the interrupted carriers at UL Tx switching across 3 or 4 bands.

- Option 2 (China Telecom, Huawei, Ericsson, Samsung): The capability uplinkTxSwitching-DL-Interruption-r16 is unaffected in the multiple TAG case.

- Option 2a (Samsung, Huawei): For Rel-18 TX switching across 3/4 bands, no reason to change the existing requirement structure and the signaling design for uplinkTxSwitching-DL-Interruption.

RRM can directly use the conclusion of RF session or other RAN group, when decisions are made. So no discussion in 2nd round discussion.

Issue 1-5: DL interruption is applicable for ENDC

During 1st round discussion, one company pointed out there is the following conclusion in RAN1#109e,

Conclusion EN-DC cases are out of scope for Rel-18 UL Tx switching

Please companies check the above agreement and provide comments on option 1.

- Option 1: Same principle applies for ENDC, i.e., DL interruptions in TS 38.133 are not affected, even if the time masks in UE RF requirements might have to be different.

| Company | Comments |
| --- | --- |
| Apple | If RAN1 has already excluded EN-DC, then RAN4 doesn’t need to discuss it. |
| China Telecom | It is true that EN-DC is out of scope for Rel-18 Tx switching across 3 or 4 bands as discussed in RAN1. Meanwhile, if the option 1 is for 2CC 1Tx-2Tx switching across 2 bands and extend 1 TAG to 2 TAG case, we think ENDC scenario can be included. |
| Huawei | We confirmed that ENDC is out of scope [RP-221436] RAN1#109-e Conclusion EN-DC cases are out of scope for Rel-18 UL Tx switching. |
| Ericsson | Option 1: Same principle applies for ENDC, i.e., DL interruptions in TS 38.133 are not affected, even if the time masks in UE RF requirements might have to be different. |
| vivo | As we commented in the 1st round, it is out of scope based on RAN1 conclusion. |
| Nokia | We are fine to follow RAN1 conclusion. |
| Samsung | Same view as other companies to follow RAN1 agreement. |

< Way forward > Issue 1-6: Applicable scenario of DL interruption requirements

Candidate options:

- Option 1: reuse Rel-16/Rel-17 values for length of DL interruption. Define DL interruption requirements for Rel-18 Tx switching across 3 or 4 bands applicable to all scenarios including:

  - Inter-band UL-CA without SUL band

  - Inter-band UL CA for {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s)

  - Intra-band two contiguous aggregated carriers within one non-SUL band out of 3 or 4 bands

  - {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band}:

| Company | Comments |
| --- | --- |
| China Telecom | Support option 1. |
| Huawei | In general, we support this applicable scenario which is for R18 TX switching. We are just not sure whether there is DL interruption for SUL, as in R16/R17 TX switching, there is no DL interruption for SUL. |
| Ericsson | We are ok with option 1. |
| Nokia | We are fine with the first 3 bullets in Option 1. For the last bullet (SUL combo), as per our understanding, the last bullet scenario (SUL combo) was not concluded in RAN#96 meeting (as cited below) and no progress in RAN#97. Probably we should avoid the discussion in RAN4. |

Topic #2: UL outage time for TX switching with 2 TAGs

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215798 | China Telecom | Proposal 3: For UL outage time for Tx switching with 2 TAGs, if needed, the following proposals submitted to RF session in [4] can also be discussed in RRM session: - For deriving the UL outage time, use half of the difference between the actual TAs on the two TAGs. - For the timing and measurement error, 3 aspects need to be considered, including: a) BS synchronization accuracy, b) UE transmit timing error, c) TA quantization error. - For BS synchronization accuracy for synchronized network, the BS synchronization accuracy requirement of 3us defined in clause 7.4.2 of TS 38.133 can be used. - For UE transmit timing error, the requirement defined in clause 7.1 of TS 38.133 can be seen an upper bound, and the sum of maximum UE transmit timing error is 1.56 us for the carriers with 2 TAGs. - For TA quantization error, as defined in TS 38.213, it can be up to 5.2 us for 15 kHz SCS. |
| R4-2216310 | Huawei, HiSilicon | Observation 1: “timing and measurement error” is included in MTTD. Observation 2: The impacted UL OFDM symbols of Option 1 and option 2 @ 15kHz and 30kHz SCS of victim cell are the same, however they are different @60kHz SCS. - Option 1: Impacted UL OFDM symbols= Ceiling (switching period/symbol length) +1 OS - Option 2: Impacted UL OFDM symbols= Ceiling ((switching period+ MTTD)/symbol length) Observation 3: The UL timing difference on two bands in different TAG shall be considered, however Option 1 is more accurate for calculating the impacted UL OFDM symbols in 2 TAGs scenario. Observation 4: The concept of “UL outage” shall be clarified. It refers to the actual interruptions or the possible range of interrupted OSs (like a potential interruption window)? |
| R4-2216715 | Samsung | Proposal 3: Confirm the three factors should be considered in RF session for UL outage time: (1) UL switching time (UE capability), (2) the difference between the TA on the two TAGs, up to MTTD, and (3) timing and measurement error. Observation 4: For DL timing tracking, the estimation of DL timing error can be based on the tracking accuracy depending on the total bandwidth covered by SSB/TRS and the UE clock drift between two consecutive transmissions of SSB/TRS. Proposal 4: RRM session confirm the typical value of 27ns for the FR1 UE’s DL timing error based on TRS. |

Open issues summary

Background:

In RAN4 #104e, the following agreements were reached in RF session: Factors for UL outage time discussed in RF session, to be further checked in RRM session - UL switching time (UE capability) - The difference between the TA on the two TAGs, up to MTTD - Timing and measurement error

Issue 2-1: Concept of UL outage time

Please provide views on what’s UL outage time and how to define it.

- Proposals

  - Option 1(Huawei): The UL outage refers to the actual impacted UL OFDM symbols in 2 TAGs scenario

  - Option2 (Huawei): The UL outage refers to the possible range of interrupted UL OSs (like a potential interruption window)

- Recommended WF

  - Further discussion

| Company | Comments |
| --- | --- |
| China Telecom | Our understanding is option 1, the UL outage refers to the actual impacted UL transmission time or number of OFDM symbols for the leading or lagging bands respectively. |
| Huawei | We are open to both options. If the intention of RF session is to evaluate actual impacted UL OFDM symbols, then it is option 1. Option 2 comes from the idea which is used to define DL interruption in R16/17 this can be regarded as scheduling restriction window. |
| Ericsson | Option 1. The UL outage refers to the actual impacted UL OFDM symbols in 2 TAGs scenario |
| Nokia | We would also like to understand what is “UL outage time” in RF discussion. Is UL outage time defining the time where “UL” is restricted due to Tx switching? We understood there will not be any UE RF requirement for UL outage time. The background of the UL outage time needs to be clarified. In RRM, we have defined only DL interruption. It is understood UL is not allowed when doing Tx switching. Is there anything different from legacy switching? |
| Apple | Maybe some clarification can be helpful to align what need to be captured in RRM spec regarding UL outage. Is this about the unavailable UL symbols on serving carriers involved in the switching? Or is it about unavailable UL symbols on other victim serving carriers NOT involved in the switching? If it is the former one, can this be addressed in RF (there is some ongoing discussion in RF on ON/OFF time mask update)? |
| vivo | As commented for issue 1-2, for 2-TAG case, we need to align the understanding firstly of DL interruption and UL outage time. Whether requirements for both DL interruption and UL outage time are needed or not. What would be the difference between the two. |
| CMCC | Option 1 in our understanding. But we are wondering whether we need to define UL outage time in RRM spec? |
| Samsung | Option 1 is aligned with our understanding. |

Issue 2-2: Factors for UL outage time

- Proposals

  - Option 1(China Telecom): UL outage time for Tx switching with 2 TAGs includes:

    - UL switching time (UE capability)

    - Half of the difference between the actual TAs on the two TAGs

    - Timing and measurement error, including:

a) BS synchronization accuracy: 3us defined in clause 7.4.2 of TS 38.133 can be used;

b) UE transmit timing error: the requirement defined in clause 7.1 of TS 38.133 can be seen an upper bound, and the sum of maximum UE transmit timing error is 1.56 us for the carriers with 2 TAGs

c) TA quantization error: as defined in TS 38.213, it can be up to 5.2 us for 15 kHz SCS.

  - Option 2 (Huawei):

    - UL interrupted OFDM symbols shall consider the impact of MTTD, however directly using MTTD value to derive impacted UL OFDM symbols are not always correct

    - The third factor “Timing and measurement error” is already included in MTTD

Proposed: Impacted UL OFDM symbols= Ceiling (UL switching period/symbol length) +1 OS

  - Option 3 (Samsung): Confirm the three factors should be considered in RF session for UL outage time:

    - (1) UL switching time (UE capability),

    - (2) the difference between the TA on the two TAGs, up to MTTD,

    - (3) timing and measurement error: typical value is 27ns

- Recommended WF

  - Further discussion

| Company | Comments |
| --- | --- |
| China Telecom | For option 1, it is proposed by us and it is aiming to derive the UL outage time in terms of us. For option 2 on the number of impacted UL OFDM symbols, we think it also depends on: 1) The timing in which band is used as the reference timing when conducting switching. For example, if the timing in band A is used as the reference timing, no need to add one more symbol as the interrupted UL symbol. 2) Whether the outage falls in the UL OFDM symbol or DL OFDM symbol in TDD case. |
| Huawei | We would like to interpret option 2. Firstly MTTD is derived from MRTD, TA resolution error, UE Timing advance adjustment and Te. Therefore MTTD has already taken “timing and measurement error” (listed as third factor) into account. Secondly, the concrete interrupted UL outage depends on the conclusion from Issue 2-1. Reply to China Telecom, the outage is for victim cells. |
| Ericsson | Option 2. We agree with Huawei that MTTD contains UL TX uncertainties already, so we should avoid counting some of these factor twice. Regarding Option 1: “”TA quantization error: as defined in TS 38.213, it can be up to 5.2 us for 15 kHz SCS. The quantization error is +/- 8*64 / 2^µ, for µ = 0 and SCS = 15 kHz we get +/- 8Ts = +/- 0.26 µs. The example 5.2 µs seems to be too high. |
| Nokia | This should be discussed in RF session. |
| Apple | Before we go to detailed discussion, we would like to understand what need to be captured in RRM spec regarding UL outage. Similar comment as under issue 2-1. |
| vivo | RRM can discuss with RF conclusion, i.e., option 3, as starting point. The options are not exclusive. |
| Samsung | Confirm the understanding from last meeting’s RF discussion from RRM understanding. It is not necessary to discuss the number of impacted UL symbols, which is not asked by RF session. |

CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

None

Summary for 1st round

    - Open issues

Issue 2-1: Concept of UL outage time

During 1st round discussion, it seems the concept of “UL outage time” is not clear to RRM session. More information/progress in RF session are needed. Based on collected comments, the following options are suggested to be further discussed in 2nd round.

Candidate options:

  - Revised Option 1: The UL outage refers to the actual impacted UL OFDM symbols on serving carriers involved in the switching.

  - New Option 2: The UL outage refers to the actual impacted UL OFDM symbols on other victim serving carriers NOT involved in the switching.

  - Option 3: The UL outage refers to the possible range of interrupted UL OSs (like a potential interruption window)

Recommendations for 2nd round: further discussion.

Issue 2-2: Factors for UL outage time

In 1st round discussion, some company think this issue shall be discussed in RF session, and some companies are not sure what need to be captured RRM spec. To clarify, this issue is trigged by RF WF R4-2215163 (see below). RRM is just responsible to provide possible response to RF. Whether the UL outage time is captured in spec is still under discussion in RF session

- In RF WF [R4-2215163] - Issue 3-1-2: UL outage time WF Factors for UL outage time discussed in RF session, to be further checked in RRM session - UL switching time (UE capability) - The difference between the TA on the two TAGs, up to MTTD - Timing and measurement error

Candidate options:

  - Option 1(China Telecom): UL outage time for Tx switching with 2 TAGs includes:

    - UL switching time (UE capability)

    - Half of the difference between the actual TAs on the two TAGs

    - Timing and measurement error, including:

a) BS synchronization accuracy: 3us defined in clause 7.4.2 of TS 38.133 can be used;

b) UE transmit timing error: the requirement defined in clause 7.1 of TS 38.133 can be seen an upper bound, and the sum of maximum UE transmit timing error is 1.56 us for the carriers with 2 TAGs

c) TA quantization error: as defined in TS 38.213, it can be up to 5.2 us for 15 kHz SCS.

  - Option 2 (Huawei, Ericsson):

    - UL interrupted OFDM symbols shall consider the impact of MTTD, however directly using MTTD value to derive impacted UL OFDM symbols are not always correct

    - The third factor “Timing and measurement error” is already included in MTTD

Proposed: Impacted UL OFDM symbols= Ceiling (UL switching period/symbol length) +1 OS

  - Option 3 (Samsung, vivo): Confirm the three factors should be considered in RF session for UL outage time:

    - (1) UL switching time (UE capability),

    - (2) the difference between the TA on the two TAGs, up to MTTD,

    - (3) timing and measurement error: typical value is 27ns

Recommendations for 2nd round: to provide possible response to RF, further discussion is needed.

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Issue 2-1: Concept of UL outage time

During 1st round discussion, it seems the concept of “UL outage time” is not clear to RRM session. More information/progress in RF session are needed. Based on collected comments, the following options are suggested to be further discussed in 2nd round.

Candidate options:

- Revised Option 1: The UL outage refers to the actual impacted UL OFDM symbols on serving carriers involved in the switching.

- New Option 2: The UL outage refers to the actual impacted UL OFDM symbols on other victim serving carriers NOT involved in the switching.

- Option 3: The UL outage refers to the possible range of interrupted UL OSs (like a potential interruption window)

| Company | Comments |
| --- | --- |
| Apple | We were wondering if the revised option 1 can be covered by ongoing RF discussion. if so, we prefer to only capture it in RF spec. We are open to new option 2. |
| China Telecom | For clarification, it this issue just to align the understanding of the concept, or it talks about the requirements to be defined in RRM session? |
| Huawei | We are open to all options. The concept of outage shall be clarified in RF session. Reply to China Telecom, this is not for discussing RRM requirements as it is not yet decided whether outage is to be captured in spec or not, and if to be captured where to capture (RF or RRM). This discussion is to collect the views on the concept. |
| Ericsson | Same comment as China Telecom, it this issue just to align the understanding of the concept, or it talks about the requirements to be defined in RRM session? |
| vivo | We would understand further the purpose of UL outage time and then RRM impact if any. Whether is it the counterpart of DL interruption? The scenarios also need to be clarified, e.g., whether it is for the cases when simultaneous 2 Tx switching happen. |
| Nokia | As this UL outage is originated from RF, more clarification is required from RF session. We’d like to keep this issue open for more time to check. |
| Samsung | Our original understanding on UL outage time is revised Option 1. But it is the new concept we want to introduce here, and we are okay if some clear definition is given. |

Issue 2-2: Factors for UL outage time

In 1st round discussion, some company think this issue shall be discussed in RF session, and some companies are not sure what need to be captured RRM spec. To clarify, this issue is trigged by RF WF R4-2215163 (see below). RRM is just responsible to provide possible response to RF. Whether the UL outage time is captured in spec is still under discussion in RF session

- In RF WF [R4-2215163] - Issue 3-1-2: UL outage time Factors for UL outage time discussed in RF session, to be further checked in RRM session - UL switching time (UE capability) - The difference between the TA on the two TAGs, up to MTTD - Timing and measurement error

Candidate options:

  - Option 1(China Telecom): UL outage time for Tx switching with 2 TAGs includes:

    - UL switching time (UE capability)

    - Half of the difference between the actual TAs on the two TAGs(i.e., not directly use the MTTD defined in TS 38.133)

    - Timing and measurement error, including:

a) BS synchronization accuracy: 3us defined in clause 7.4.2 of TS 38.133 can be used;

b) UE transmit timing error: the requirement defined in clause 7.1 of TS 38.133 can be seen an upper bound, and the sum of maximum UE transmit timing error is 1.56 us for the carriers with 2 TAGs

c) TA quantization error: as defined in TS 38.213, it can be up to 5.2 us for 15 kHz SCS.

  - Option 2 (Huawei, Ericsson):

    - UL interrupted OFDM symbols shall consider the impact of MTTD, however directly using MTTD value to derive impacted UL OFDM symbols are not always correct

    - The third factor “Timing and measurement error” is already included in MTTD

Proposed: Impacted UL OFDM symbols= Ceiling (UL switching period/symbol length) +1 OS

  - Option 3 (Samsung): Confirm the three factors should be considered in RF session for UL outage time:

    - (1) UL switching time (UE capability),

    - (2) the difference between the TA on the two TAGs, up to MTTD,

    - (3) timing and measurement error: typical value is 27ns

| Company | Comments |
| --- | --- |
| Apple | It is better to focus on issue 2-1 first so that people can know what the target scenario is. |
| China Telecom | Responses to comments on Option l: 1) Regarding the comment that “The third factor “Timing and measurement error” is already included in MTTD: CTC response: We understand the other aspects are already included in the MTTD. But as seen in the 2nd bullet of our proposal, we suggest not to use the MTTD, but to use half of the difference between the actual TAs on the two TAGs. 2) On TA quantization error, E/// commented in GTW discussion that 5.26us is too large and 0.26 us is the correct number. CTC response: After further checking, should be 0.52us but not 5.2 us, sorry for my mistake! As specified in TS 38.213, for a SCS of kHz, the timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG in multiples of . With this, we are open to consider 0.52us or half of 0.52us. |
| Huawei | Although no consensus is achieved on issue 2-1, the below observation in option 2 is still true: - The third factor “Timing and measurement error” is already included in MTTD Regarding how to deduce outage, we are afraid that using actual TA value is not aligned the way we ever used to calculate interruptions. For network, it is complicated if specific TA value for each UE is used. Moreover max TA value is very large. MTTD/MRTD seems a good trade-off. |
| Ericsson | We prefer Option 2. Regarding Option 1: Please be advised: I need help understanding how TA quantization error can become 5.2 µs. Regarding Option 3: timing and measurement error: typical value is 27ns. We should develop minimum requirements |
| vivo | Depending on the concept of UL outage time as in Issue 2-1. It is not clear whether there is RRM impact. |
| Nokia | Target scenario needs to be identified first. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on R18 multi-carrier enhancements (RRM) | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2217258 | WF on multi-carrier enhancements | Huawei, HiSilicon | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents